3GPP TSG-RAN WG1 #85 	R1-164698

23th – 27th May 2016

Nanjing, China

Agenda item:	7.1.5.1

Source: 	Qualcomm Incorporated

Title: 	LDPC – Performance evaluation

Document for:	Discussion/Decision

# Introduction

In RAN1 #84b meeting it was agreed upon to evaluate channel codes under the following assumptions. This document compares the corresponding results for LDPC and LTE Turbo Codes, where the LDPC codes are new design proposals for NR following [3].

## eMBB Simulation Assumptions

For eMBB use cases, evaluation should provide the block error rate (BLER) performance versus SNR for the following cases which are more reflective of usage across mobile broadband data transmissions.

Table 1. eMBB simulation assumptions

| Channel* | AWGN |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | QPSK, 64 QAM |  |  |  |
| Coding Scheme | Turbo | LDPC | Polar |  |
| Code rate | 1/5, 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9 |  |  |  |
| Decoding algorithm** | Max-log-MAP (15) | BP (50) |  | List-32 |
| Info. block length*** (bits w/o CRC) | 100, 400, 1000, 2000, 4000, 6000, 8000 Optional(12K, 16K, 32K, 64K) |  |  |  |

Several notes for clarification were made as part of the evaluation agreement.

- * Fading channels will be simulated in the next stage

- ** These algorithms are starting points for further study. Other variants of agreed algorithms can be used for encoding and decoding (Complexity details should be illustrated)

- *** At least these info. block length and code rate shall be evaluated. Other info. block lengths and code rates are not precluded. Similar info. and encoded block lengths should be used for the evaluation. Total coded bits = info. Block length/code rate. Note: these info. block length and code rate are only for initial performance evaluations. They are not interpreted as design targets or assumptions for complexity analysis.

- General guidelines

  - Existing code constructions can be used for evaluation

  - Whenever feasible, performance comparison should adopt coding constructions with matching computational complexities

## URLLC/mMTC Simulation Assumptions

For URLLC and mMTC use cases, evaluation should provide the block error rate (BLER) performance versus SNR for the following cases which are more reflective of such services (beyond some configurations already included in eMBB evaluation).

Table 2. URLLC and mMTC simulation assumptions

| Channel* | AWGN |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | QPSK, 16 QAM |  |  |  |
| Coding Scheme | Convolutional codes | LDPC | Polar | Turbo |
| Code rate | 1/12, 1/6, 1/3 |  |  |  |
| Decoding algorithm** | List-X Viterbi | BP (50) | List-32 | Max-log-MAP (15) |
| Info. block length*** (bits w/o CRC) | 20, 40, 200, 600, 1000 |  |  |  |

Several notes for clarification were made as part of the evaluation agreement.

- * Fading channels will be simulated in the next stage

- ** These algorithms are starting points for further study. Other variants of agreed algorithms can be used for encoding and decoding (Complexity details should be illustrated)

- *** At least these info. block length and code rate shall be evaluated. Other info. block lengths and code rates are not precluded. Similar info and encoded block lengths should be used for the evaluation. Total coded bits = info. Block length/code rate. Note: these info. block length and code rate are only for initial performance evaluations. They are not interpreted as design targets or assumptions for complexity analysis.

- General guidelines

  - Existing code constructions can be used for evaluation

  - Whenever feasible, performance comparison should adopt coding constructions with matching computational complexities

  - BLER simulations down to 10-4 is recommended (to observe the error floor) for URLLC

# Results and Discussion

ME-LDPC code is designed as mentioned in the overview document [3]. ME-LDPC code has exact rates as prescribed by the above evaluation parameters. The information blocklengths are slightlly different and are mostly within +/- 5%. The exact blocklengths used are shown in the Appendix for the eMBB scenario. We show later that such differences in blocklength does not cause much differences in performance. As mentioned in the design overview document, the final PCMs are obtained by lifting the basegraph by the lifting size.

## eMBB Code Performance

In this section we compare the performance of the ME-LDPC design (see [3]) against the standard LTE Turbo codec. The simulations use the standard floating point BP decoder with 50 flooding iterations. In the following sections, we compare the performance of the ME-LDPC code with the Turbo code on various metrics.

Observation 1:  ME-LDPC outperforms LTE Turbo in all of the eMBB simulation scenarios at BLER of 1%. The detailed differences in performance is summarized below. This demonstrates the flexibility of design across operating points.

Summary of results for eMBB, 4QAM (BLER of 1%):

- ME-LDPC design outperforms the LTE Turbo code at all blocklengths. For all blocklengths the difference is uniformly between 0.4 dB – 0.5 dB.

- For the lowest rate of 0.2, the difference is uniformly (across all blocklengths) around 1.0 dB.

Summary of results for eMBB, 64QAM (BLER of 1%):

- ME-LDPC design outperforms the LTE Turbo code at all blocklengths. For all blocklengths the difference is uniformly between 0.5 dB – 0.8 dB.

- For the lowest rate of 0.2, the difference is uniformly (across all blocklengths) around 1.0 dB (larger in some cases to around 2 dB).

Proposal 1: NR should consider using ME-LDPC code family to replace LTE Turbo codes for EMBB based on performance.

### Spectral Efficiency versus SNR

In all the figures shown below, the dashed black curve corresponds to the unconstrained Shannon capacity curve.

#### 4QAM

Figure 2: Spectral Efficiency at 4QAM, K ~ 100

Figure 5: Spectral Efficiency at 4QAM, K ~ 2000

Figure 7: Spectral Efficiency at 4QAM, K ~ 6000

Summary of results for eMBB, 4QAM (BLER of 1%):

- ME-LDPC design outperforms the LTE Turbo code at all blocklengths. For all blocklengths the difference is uniformly between 0.4 dB – 0.5 dB.

- For the lowest rate of 0.2, the difference is uniformly (across all blocklengths) around 1.0 dB.

#### 64 QAM

Figure 9: Spectral Efficiency at 64QAM, K ~ 100

Figure 12: Spectral Efficiency at 64QAM, K ~ 2000

Figure 14: Spectral Efficiency at 64QAM, K ~ 6000

Summary of results for eMBB, 64QAM (BLER of 1%):

- ME-LDPC design outperforms the LTE Turbo code at all blocklengths. For all blocklengths the difference is uniformly between 0.5 dB – 0.8 dB.

- For the lowest rate of 0.2, the difference is uniformly (across all blocklengths) around 1.0 dB (larger in some cases to around 2 dB).

### Target SNR versus Blocklength

The figures below show the EsN0 required to achieve the target BLER of 1% for each rate as the blocklength increases. We observe that ME-LDPC consistently outperforms LTE Turbo.

Figure 30: EsN0 required to achieve 1% BLER at 4QAM

Figure 31: EsN0 required to achieve 1% BLER at 64QAM

## URLLC/mMTC Code Performance

The blocklenghts for K = 20 are obtained exactly from the ME-LDPC code. For K=40, the blocklengths used are K=36 and for other cases the blocklenghts used are within +/- 5% of the prescribed blocklengths.

Observation 2:  ME-LDPC outperforms LTE Turbo in all of the URLLC simulation scenarios down to BLER of 0.01% with minor exceptions which can be addressed with redesigns. ME-LDPC outperforms LTE Turbo across all scenarios for BLER 1% in performance with the gains summarized below.

Summary of results for URLLC/mMTC, 4QAM (BLER of 1%):

- ME-LDPC design outperforms the LTE Turbo code at all blocklengths. For blocklengths K >= 200, the difference is uniformly around 0.35 dB for rate = 0.33, around 0.9 dB for rate = 1/6 and around 1.5 dB for rate = 1/12.

- For K <= 40, the ME-LDPC outperforms Turbo by around 1.5 dB uniformly for all rates.

Summary of results for URLLC/mMTC, 16QAM (BLER of 1%):

- ME-LDPC design outperforms the LTE Turbo code at all blocklengths. For blocklengths K >= 200, the difference is uniformly around 0.3 dB for rate = 0.33, around 1.5 dB for rate = 1/6 and around 2.5 dB for rate = 1/12.

- For K <= 40, the ME-LDPC outperforms Turbo by around 1.7 dB uniformly for all rates.

Observation 3:  ME-LDPC outperforms LTE Turbo for very low rates (1/6 and 1/12) by almost 1 dB uniformly. This indicates the power of having lower rate ME-LDPC codes for efficient ultra-reliable communication.

Proposal 2: NR should consider using ME-LDPC code family to replace LTE Turbo codes for URLLC based on performance.

### Target SNR versus Blocklength

The figures below show the EsN0 required to achieve the target BLER of 1% for each rate as the blocklength increases. We observe that ME-LDPC consistently outperforms LTE Turbo.

Figure 42: EsN0 required to achieve 1% BLER at 4QAM

Figure 43: EsN0 required to achieve 1% BLER at 16QAM

# Shortening and Puncturing Robustness

The figures below show that there is not much performance loss when we considered blocklengths within +/- 5% of the prescribed blocklengths. The exact blocklengths and exact rates (prescribed by the evaluation parameters) were obtained by shortening and puncturing of the ME-LDPC code. We obeserve that the performance is within 0.05 dB and hence demonstrates the robustness of the design to shortening and puncturing.

Observation 4:  ME-LDPC design is robust to shortening and puncturing over the range of rates and blocklengths.

Figure 44: Performance comparison between K = 100 and for K within +/- 5% of K = 100 for different rates.

Figure 45: Performance comparison between K = 400 and for K within +/- 5% of K = 400 for different rates.

Figure 46: Performance comparison between K = 400 and for K within +/- 5% of K = 1000 for different rates.

Figure 47: Performance comparison between K = 400 and for K within +/- 5% of K = 2000 for different rates.

Figure 48: Performance comparison between K = 400 and for K within +/- 5% of K = 4000 for different rates.

Figure 49: Performance comparison between K = 400 and for K within +/- 5% of K = 6000 for different rates.

# Conclusions

Observation 1:  ME-LDPC outperforms LTE Turbo in all of the eMBB simulation scenarios at BLER of 1%. The detailed differences in performance is summarized below. This demonstrates the flexibility of design across operating points.

Summary of results for eMBB, 4QAM (BLER of 1%):

- ME-LDPC design outperforms the LTE Turbo code at all blocklengths. For all blocklengths the difference is uniformly between 0.4 dB – 0.5 dB.

- For the lowest rate of 0.2, the difference is uniformly (across all blocklengths) around 1.0 dB.

Summary of results for eMBB, 64QAM (BLER of 1%):

- ME-LDPC design outperforms the LTE Turbo code at all blocklengths. For all blocklengths the difference is uniformly between 0.5 dB – 0.8 dB.

- For the lowest rate of 0.2, the difference is uniformly (across all blocklengths) around 1.0 dB (larger in some cases to around 2 dB).

Proposal 1: NR should consider using ME-LDPC code family to replace LTE Turbo codes for EMBB based on performance.

Observation 2:  ME-LDPC outperforms LTE Turbo in all of the URLLC simulation scenarios down to BLER of 0.01% with minor exceptions which can be addressed with redesigns. ME-LDPC outperforms LTE Turbo across all scenarios for BLER 1% in performance with the gains summarized below.

Summary of results for URLLC/mMTC, 4QAM (BLER of 1%):

- ME-LDPC design outperforms the LTE Turbo code at all blocklengths. For blocklengths K >= 200, the difference is uniformly around 0.35 dB for rate = 0.33, around 0.9 dB for rate = 1/6 and around 1.5 dB for rate = 1/12.

- For K <= 40, the ME-LDPC outperforms Turbo by around 1.5 dB uniformly for all rates.

Summary of results for URLLC/mMTC, 16QAM (BLER of 1%):

- ME-LDPC design outperforms the LTE Turbo code at all blocklengths. For blocklengths K >= 200, the difference is uniformly around 0.3 dB for rate = 0.33, around 1.5 dB for rate = 1/6 and around 2.5 dB for rate = 1/12.

- For K <= 40, the ME-LDPC outperforms Turbo by around 1.7 dB uniformly for all rates.

Observation 3:  ME-LDPC outperforms LTE Turbo for very low rates (1/6 and 1/12) by almost 1 dB uniformly. This indicates the power of having lower rate ME-LDPC codes for efficient ultra-reliable communication.

Proposal 2: NR should consider using ME-LDPC code family to replace LTE Turbo codes for URLLC/mMTC based on performance.

Observation 4:  ME-LDPC design is robust to shortening and puncturing over the range of rates and blocklengths.

# References

- RP-160671, New SID Proposal: Study on New Radio Access Technology

- “Requirements and Evaluation”, R1-162209, Qualcomm Incorporated, RAN1 84b, Busan Korea.

- “LDPC Design Overview”, R1-164697, Qualcomm Incorporated, RAN1 85, Nanjing China.

# Appendix

## EMBB Blocklengths used for simulations

Marked in grey are not simulated. The code blocklengths were greater than 72K.

Table 3. EMBB Blocklengths

| Info Size/Rate | 1/5 (actual,%) | 1/3 (actual,%) | 2/5 (actual,%) | ½ (actual,%) | 2/3 (actual,%) | ¾ (actual,%) | 5/6 (actual,%) | 8/9 (actual,%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | (99,1.0) | (96,4.0) | (96,4.0) | (108,8.0) | (96,4.0) | (108,8.0) | (90,10.0) | (96,4.0) |
| 400 | (396,1.0) | (384,4.0) | (416,4.0) | (396,1.0) | (384,4.0) | (378,5.5) | (390,2.5) | (416,4.0) |
| 1000 | (1008,0.8) | (960,4.0) | (1024,2.4) | (1008,0.8) | (960,4.0) | (972,2.8) | (960,4.0) | (1024,2.4) |
| 2000 | (2016,0.8) | (1920,4.0) | (2048,2.4) | (2016,0.8) | (1920,4.0) | (1944,2.8) | (1920,4.0) | (2048,2.4) |
| 4000 | (4032,0.8) | (3840,4.0) | (4096,2.4) | (4032,0.8) | (3840,4.0) | (3888,2.8) | (3840,4.0) | (4096,2.4) |
| 6000 | (5760,4.0) | (6144,2.4) | (6144,2.4) | (5760,4.0) | (6144,2.4) | (6048,0.8) | (6240,4.0) | (6144,2.4) |
| 8000 | (8064,0.8) | (7680,4.0) | (8192,2.4) | (8064,0.8) | (7680,4.0) | (7776,2.8) | (7680,4.0) | (8192,2.4) |
| 12000 | (11520,4.0) | (12288,2.4) | (12288,2.4) | (11520,4.0) | (12288,2.4) | (12096,0.8) | (12480,4.0) | (12288,2.4) |
| 16000 | (16128,0.8) | (15360,4.0) | (16384,2.4) | (16128,0.8) | (15360,4.0) | (15552,2.8) | (15360,4.0) | (16384,2.4) |
| 32000 | (32256,0.8) | (30720,4.0) | (32768,2.4) | (32256,0.8) | (30720,4.0) | (31104,2.8) | (30720,4.0) | (32768,2.4) |
| 64000 | (59904,6.4) | (67584,5.6) | (57344,10.4) | (64512,0.8) | (61440,4.0) | (62208,2.8) | (61440,4.0) | (65536,2.4) |

## EMBB BLER vs SNR

### 4-QAM

Figure 16: ME-LDPC (solid; circles) versus Turbo (dashed squares); K ~ 100

Figure 17: ME-LDPC (solid; circles) versus Turbo (dashed squares); K ~ 400

Figure 18: ME-LDPC (solid; circles) versus Turbo (dashed squares); K ~ 1000

Figure 19: ME-LDPC (solid; circles) versus Turbo (dashed squares); K ~ 2000

Figure 20: ME-LDPC (solid; circles) versus Turbo (dashed squares); K ~ 4000

Figure 21: ME-LDPC (solid; circles) versus Turbo (dashed squares); K ~ 6000

### 64-QAM

Figure 23: ME-LDPC (solid; circles) versus Turbo (dashed squares); K ~ 100

Figure 24: ME-LDPC (solid; circles) versus Turbo (dashed squares); K ~ 400

Figure 25: ME-LDPC (solid; circles) versus Turbo (dashed squares); K ~ 1000

Figure 26: ME-LDPC (solid; circles) versus Turbo (dashed squares); K ~ 2000

Figure 27: ME-LDPC (solid; circles) versus Turbo (dashed squares); K ~ 4000

Figure 28: ME-LDPC (solid; circles) versus Turbo (dashed squares); K ~ 6000

## URLLC/mMTC BLER vs SNR

### 4-QAM

Figure 32: ME-LDPC (solid; circles) versus Turbo (dashed squares); K = 20

Figure 33: ME-LDPC (solid; circles) versus Turbo (dashed squares); K = 40

Figure 34: ME-LDPC (solid; circles) versus Turbo (dashed squares); K = 200

Figure 35: ME-LDPC (solid; circles) versus Turbo (dashed squares); K = 600

Figure 36: ME-LDPC (solid; circles) versus Turbo (dashed squares); K = 1000

A slight error-floor is observed for rate = 0.33 in some of the above figures. But this is not of much concern, since the design can be manipulated very slightly to get rid of the error-floor without affecting the waterfall performance.

### 16-QAM

Figure 37: ME-LDPC (solid; circles) versus Turbo (dashed squares); K = 20

Figure 38: ME-LDPC (solid; circles) versus Turbo (dashed squares); K = 40

Figure 39: ME-LDPC (solid; circles) versus Turbo (dashed squares); K = 200

Figure 40: ME-LDPC (solid; circles) versus Turbo (dashed squares); K = 600

Figure 41: ME-LDPC (solid; circles) versus Turbo (dashed squares); K = 1000

A slight error-floor is observed for rate = 0.33 in some of the above figures. But this is not of much concern, since the design can be manipulated very slightly to get rid of the error-floor without affecting the waterfall performance.