3GPP TSG RAN WG1 Meeting #85 	R1-164735

# Nanjing, China, 23rd – 27th May 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | CRNum | rev | - | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on CSI-Reporting-Type in TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_EBF_FDMIMO-Core |  |  |  |  |  |  | Date: |  | 2016-04-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In RAN1#84, mismatch between the names and values of RRC parameters in TS 36.331 and TS 36.213 was clarified. However, remaining mismatch between eMIMO-Type in TS 36.331 (RRC specification) and CSI-reporting-Type in TS36.213 (RAN1 specification) still exists in TS 36.213. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the field name ‘CSI-reporting-Type’ to ‘eMIMO-Type’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operational failure due to the mismatched field name for Rel-13 EBF/FD-MIMO. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1, 7.2.2, 7.2.3, 9.1.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

7.2.1	Aperiodic CSI Reporting using PUSCH

…

For non-BL/CE UE and for each of the transmission modes defined in subclause 7.1, the following reporting modes are supported on PUSCH:

Transmission mode 1	: Modes 2-0, 3-0, 1-0

Transmission mode 2	: Modes 2-0, 3-0, 1-0

Transmission mode 3	: Modes 2-0, 3-0, 1-0

Transmission mode 4	: Modes 1-2, 2-2, 3-1, 3-2, 1-1

Transmission mode 5	: Mode 3-1, 1-1

Transmission mode 6	: Modes 1-2, 2-2, 3-1, 3-2, 1-1

Transmission mode 7	: Modes 2-0, 3-0, 1-0

Transmission mode 8	: Modes 1-2, 2-2, 3-1, 3-2, 1-1 if the UE is configured with PMI/RI reporting; modes 2-0, 3-0, 1-0 if the UE is configured without PMI/RI reporting

Transmission mode 9 	: Modes 1-2, 2-2, 3-1, 3-2, 1-1 if the UE is configured with PMI/RI reporting and number of CSI-RS ports > 1; modes 2-0, 3-0, 1-0 if the UE is configured without PMI/RI reporting or without PMI reporting or number of CSI-RS ports=1 or the number of CSI-RS ports in each of one or more CSI-RS resources in a CSI process is one when eMIMO-Type is set to ‘CLASS B’.

Transmission mode 10 	: Modes 1-2, 2-2, 3-1, 3-2, 1-1 if the UE is configured with PMI/RI reporting and number of CSI-RS ports > 1; modes 2-0, 3-0, 1-0 if the UE is configured without PMI/RI reporting or without PMI reporting or number of CSI-RS ports=1 or the number of CSI-RS ports in each of one or more CSI-RS resources in a CSI process is one when eMIMO-Type is set to ‘CLASS B’.

For a BL/CE UE configured with CEModeA, the following reporting modes are supported on PUSCH:

Transmission mode 1	: Mode 2-0

Transmission mode 2	: Mode 2-0

Transmission mode 6	: Mode 2-0

Transmission mode 9 	: Mode 2-0

…

7.2.2	Periodic CSI Reporting using PUCCH

…

For a non-BL/CE UE and for each of the transmission modes defined in subclause 7.1, the following periodic CSI reporting modes are supported on PUCCH:

Transmission mode 1	: Modes 1-0, 2-0

Transmission mode 2	: Modes 1-0, 2-0

Transmission mode 3	: Modes 1-0, 2-0

Transmission mode 4	: Modes 1-1, 2-1

Transmission mode 5	: Modes 1-1, 2-1

Transmission mode 6	: Modes 1-1, 2-1

Transmission mode 7	: Modes 1-0, 2-0

Transmission mode 8	: Modes 1-1, 2-1 if the UE is configured with PMI/RI reporting; modes 1-0, 2-0 if the UE is configured without PMI/RI reporting

Transmission mode 9 	: Modes 1-1, 2-1 if the UE is configured with PMI/RI reporting and number of CSI-RS ports>1; modes 1-0, 2-0 if the UE is configured without PMI/RI reporting or without PMI reporting or number of CSI-RS ports=1 or the number of CSI-RS ports in each of one or more CSI-RS resources in a CSI process is one when eMIMO-Type is set to be ‘CLASS B’.

Transmission mode 10 	: Modes 1-1, 2-1 if the UE is configured with PMI/RI reporting and number of CSI-RS ports>1; modes 1-0, 2-0 if the UE is configured without PMI/RI reporting or without PMI reporting or number of CSI-RS ports=1 or the number of CSI-RS ports in each of one or more CSI-RS resources in a CSI process is one when eMIMO-Type is set to be ‘CLASS B’.

For a BL/CE UE configured with CEModeA, the following periodic CSI reporting modes are supported on PUCCH:

Transmission mode 1	: Mode 1-0

Transmission mode 2	: Mode 1-0

Transmission mode 6	: Mode 1-1

Transmission mode 9 	: Modes 1-1, 1-0.

…

7.2.3	Channel Quality Indicator (CQI) definition

…

For a UE in transmission mode 9 or 10 and for a CSI process, if the UE is configured with parameter eMIMO-Type by higher layers, and eMIMO-Type is set to ‘CLASS A’, and one CSI-RS resource configured, or the UE is configured with parameter eMIMO-Type by higher layers, and eMIMO-Type is set to ‘CLASS B’, and parameter channelMeasRestriction is not configured by higher layers, the UE shall derive the channel measurements for computing the CQI value reported in uplink subframe n and corresponding to the CSI process, based on only the non-zero power CSI-RS (defined in [3]) within a configured CSI-RS resource associated with the CSI process. If the UE is configured with parameter eMIMO-Type by higher layers, and eMIMO-Type is set to ‘CLASS B’ and the number of configured CSI-RS resources is K>1, and parameter channelMeasRestriction is not configured by higher layers, the UE shall derive the channel measurements for computing the CQI value using only the configured CSI-RS resource indicated by the CRI.

For a UE in transmission mode 9 or 10 and for a CSI process, if the UE is configured with parameter eMIMO-Type by higher layers, and eMIMO-Type is set to ‘CLASS B’, and parameter channelMeasRestriction is configured by higher layers, the UE shall derive the channel measurements for computing the CQI value reported in uplink subframe n and corresponding to the CSI process, based on only the most recent, no later than the CSI reference resource, non-zero power CSI-RS (defined in [3]) within a configured CSI-RS resource associated with the CSI process. If the UE is configured with parameter eMIMO-Type by higher layers, and eMIMO-Type is set to ‘CLASS B’ and the number of configured CSI-RS resources is K>1, and parameter channelMeasRestriction is configured by higher layers, the UE shall derive the channel measurements for computing the CQI value using only the most recent, no later than the CSI reference resource, non-zero power CSI-RS within the configured CSI-RS resource indicated by the CRI.

For a UE in transmission mode 10, when parameter eMIMO-Type is not configured by higher layers, the UE shall derive the interference measurements for computing the CQI value reported in uplink subframe n and corresponding to a CSI process, based on only the configured CSI-IM resource associated with the CSI process.

For a UE in transmission mode 10 and for a CSI process, when parameters eMIMO-Type and interferenceMeasRestriction is configured by higher layers, the UE shall derive the interference measurements for computing the CQI value reported in uplink subframe n and corresponding to the CSI process, based on only the most recent, no later than the CSI reference resource, configured CSI-IM resource associated with the CSI process. If the UE is configured with parameter eMIMO-Type by higher layers, and eMIMO-Type is set to ‘CLASS B’ and the number of configured CSI-RS resources is K>1, and interferenceMeasRestriction is configured, the UE shall derive interference measurement for computing the CQI value based on only the most recent, no later than the CSI reference resource, the configured CSI-IM resource associated with the CSI-RS resource indicated by the CRI. If interferenceMeasRestriction is not configured, the UE shall derive the interference measurement for computing the CQI value based on the CSI-IM associated with the CSI-RS resource indicated by the CRI.

…

9.1.4.3	Resource mapping parameters for EPDCCH

For a given serving cell, if the UE is configured via higher layer signalling to receive PDSCH data transmissions according to transmission mode 10, and if the UE is configured to monitor EPDCCH, for each EPDCCH-PRB-set, the UE shall use the parameter set indicated by the higher layer parameter re-MappingQCL-ConfigId-r11 for determining the EPDCCH RE mapping (defined in subclause 6.8A.5 of [3]) and EPDCCH antenna port quasi co-location. The following parameters for determining EPDCCH RE mapping (as described in subclause 6.8A.5 of [3]) and EPDCCH antenna port quasi co-location are included in the parameter set:

-	crs-PortsCount-r11.

-	crs-FreqShift-r11.

-	mbsfn-SubframeConfigList-r11.

-	csi-RS-ConfigZPId-r11.

-	pdsch-Start-r11.

-	qcl-CSI-RS-ConfigNZPId-r11.

-	csi-RS-ConfigZPId2-r12 if the UE is configured with CSI subframe sets  and  by the higher layer parameter csi-SubframePatternConfig-r12 for the serving cell or the UE is configured with higher layer parameter eMIMO-Type for TDD serving cell.