3GPP TSG RAN WG1 Meeting #85		                                                R1-164844

Nanjing, China, May 23-27, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | XXXX | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On MPDCCH AL for 8 EREGs per ECCE in TS 36.211 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core |  |  |  |  |  |  | Date: |  | 2016-05-23 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | For MPDCCH USS configured with 2+4 PRB-pairs or mPDCCH-NumRepetition>1, and MPDCCH common search space, the ALs and the number of monitored MPDCCH candidates are defined suitable for the case when there are 4EREGs per ECCE. There are also some special cases with 8EREGs per ECCE, in which the number of ECCEs per PRB pair is half. When applying the ALs and the number of monitored candidates for 8EREGs per ECCE, there would be the following problems: - For MPDCCH UE-specific search space, the total number of monitored candidates in a subframe would be rather less because: - If the UE is monitoring MPDCCH with mPDCCH-NumRepetition >1 and MPDCCH-PRB-set size is 2, or the UE is monitoring MPDCCH within 2 PRB pairs and MPDCCH-PRB-set size is 2+4, the candidates with AL>4 cannot be monitored. - If the UE is monitoring MPDCCH with mPDCCH-NumRepetition >1 and MPDCCH-PRB-set size is 4, or the UE is monitoring MPDCCH within 4 PRB pairs and MPDCCH-PRB-set size is 2+4, the candidates with AL>8 and the second candidate with AL=8 cannot be monitored. - For CE mode B, MPDCCH-PRB-set size 2 can’t be supported, and PRB set 2 and 4 in 2+4 PRB set can’t be monitored. - For Type0-MPDCCH common search space, MPDCCH-PRB-set size 2 and 4 can’t be supported. - For Type2-MPDCCH common search space, if the most recent coverage enhancement level used for PRACH is coverage enhancement level 2 or 3, PRB set 2 and 4 in 2+4 PRB set can’t be monitored. If the most recent coverage enhancement level used for PRACH is coverage enhancement level 0 or 1, the total number of monitored candidates in a subframe would be reduced. - For the candidate spanning both PRB sets in 2+4 PRB set, AL = 12 is defined as MPDCCH format 5 in Table 6.8B.1-2 of TS36.211, however it is not used in TS36.213. A simple solution is to use smaller ALs with half the values for 8EREGs per ECCE for each MPDCCH format. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For MPDCCH format 0 with 8EREGs per ECCE, AL =1 is used which is half the value for the same format with 4EREGs per ECCE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPDCCH format 0 can't be supported for 8EREGs per ECCE. The total number of monitored candidates in a subframe may be reduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8B.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS 36.213 |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

### 6.8B.1	MPDCCH formats

The MPDCCH formats are defined as in Clause 6.8A.1 with the following exceptions:

-	The term EPDCCH is replaced by MPDCCH.

-	The MTC physical downlink control channel carries downlink control information and is transmitted across  consecutive valid downlink subframes. Within each of the  valid downlink subframes an MPDCCH is transmitted using an aggregation of one or several consecutive enhanced control channel elements (ECCEs) where each ECCE consists of multiple enhanced resource element groups (EREGs), defined in clause 6.2.4A.

-	For frame structure type 2,

-	For a UE configured in CEModeA, if repetition is not configured for the MPDCCH, the number of EREGs per ECCE is given by Table 6.8A.1-1. If repetition is configured for the MPDCCH, the number of EREGs per ECCE is given by Table 6.8B.1-1. The MPDCCH is not supported in special subframe with configurations not listed in Table 6.8B.1-1.

-	For a UE configured in CEModeB, MPDCCH is not supported in the special subframe.

-	For both CEModeA and CEModeB, for those special subframes where the MPDCCH is not supported, these special subframes are considered valid downlink subframes for both MPDCCH and PDSCH transmission, only if they are indicated as valid downlink subframe by higher layer signalling.

-	For an MPDCCH associated with 2 or 4 PRBs, if repetition is not configured for the MPDCCH, the supported MPDCCH formats are given by Table 6.8A.1-2. Otherwise, the supported MPDCCH formats are given by Table 6.8B.1-2. However, for MPDCCH format 5, the equation defining the relation between ECCE index and EREG index does not apply and the number of ECCEs refers to the MPDCCH mapping to the REs of the 2+4 PRB set as defined in section 6.8B.5.

Table 6.8B.1-1: Number of EREGs per ECCE, , for frame structure type 2.

| Normal cyclic prefix |  | Extended cyclic prefix |  |
| --- | --- | --- | --- |
| Normal subframe | Special subframe, configuration 3, 4, 8 | Normal subframe | Special subframe, configuration 1, 2, 3, 5, 6 |
| 4 |  | 8 |  |

Table 6.8B.1-2: Supported MPDCCH formats

| MPDCCH format | Number of ECCEs in a subframe for one MPDCCH, |  |  |  |
| --- | --- | --- | --- | --- |
|  | =4 |  | =8 |  |
|  | Localized transmission | Distributed transmission | Localized transmission | Distributed transmission |
| 0 | 2 | 2 | 1 | 1 |
| 1 | 4 | 4 | 2 | 2 |
| 2 | 8 | 8 | 4 | 4 |
| 3 | 16 | 16 | 8 | 8 |
| 4 | - | - | - |  |
| 5 | 24 | 24 | 12 | 12 |

#### < Unchanged parts are omitted >