3GPP TSG RAN WG1 Meeting #85		R1-164912

Nanjing, China 23rd - 27th May 2016

Source:	Panasonic

Title: 	Evaluation results on processing time reduction in physical layer

Agenda Item:		6.2.10.2

Document for:		Discussion and Decision

Introduction

In our previous evaluation [1], the physical layer delay in TCP ACK and DL HARQ RTT are evaluated as the scalable to sTTI length including the processing time. On the other hand, for longer sTTI like 7 OFDM symbols, to reduce the processing time is possible in UL and DL. The standardization complexity and RS/signalling overheads are different especially between 7 OFDM symbols sTTI and below sTTIs. RAN2 discussed the procedure to reduce the UL latency like contention based PUSCH [2]. To take into account these aspects, we evaluated 7 OFDM symbols sTTI with reduced DL HARQ RTT in DL and TCP ACK delay. According to our evaluation, we found following.

- Seven OFDM symbols sTTI with DL HARQ RTT 4 sTTI and UL TCP ACK delay 7 sTTI can achieve similar or better performance than 2 or 3/4 OFDM symbols sTTI with DL HARQ RTT 8 sTTI and UL TCP ACK delay 13 sTTI.

Therefore, we propose to take following as latency reduction design.

- DL sTTI length is 1 slot (7 OFDM symbols).

- ACK/NACK corresponding to DL sTTI is sent in 2 slots later (DL HARQ RTT is 4 sTTI).

- In order to achieve the same UL latency with 7 OFDM symbols sTTI with TCP ACK delay 7 sTTI, UL design is one of the following options:

- Option 1:	UL sTTI length is 3 or 4 OFDM symbols with conventional PUSCH procedure

- Option 2:	UL sTTI length is 1 slot but to introduce contention based PUSCH procedure.

Discussion

We evaluate user throughput and packet delay on the following case.

| sTTI length | DL HARQ RTT | TCP ACK delay |
| --- | --- | --- |
| 7 OFDM symbols | 4 sTTIs (2 msec) | 7 sTTIs (3.5 msec) |

The timing relation of above is illustrated in Figure 1.

Figure 1. Timing relation of 7 OFDM symbols sTTI with DL 4 sTTIs RTT and ACK delay 7 sTTIs

For comparison, following cases also evaluated.

| sTTI length | DL HARQ RTT | TCP ACK delay |
| --- | --- | --- |
| 14 OFDM symbols | 8 sTTIs (8 msec) | 13 sTTIs (13 msec) |
| 7 OFDM symbols | 8 sTTIs(4 msec) | 13 sTTIs (7.5 msec) |
| 3/4 OFDM symbols | 8 sTTIs (2 msec) | 13 sTTIs (3.25 msec) |
| 2 OFDM symbols | 8 sTTIs (1.14 msec) | 13 sTTIs (1.86 msec) |

The other evaluation assumptions are listed in Annex 1.

The files size =100kbits and 500kbytes with RU 20%, 60 % are shown in Table 1 and Table 2.

Table 1  Evaluation Results for small file size (100kbits/file)

(a) Low load case

| Number of OFDM Symbols | System TP [Mbps] | Mean User TP [Mbps] | 95% User TP [Mbps] | 50% User TP [Mbps] | 5% User TP [Mbps] | Mean Packet Delay [ms] | 95% Packet Delay [ms] | 50% Packet Delay [ms] | 5% Packet delay [ms] | RU[%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 2.35 | 0.76 | 0.84 | 0.77 | 0.648 | 133.73 | 163.00 | 130.01 | 112.99 | 20.37 |
| 7 (RTT=8sTTI, ACK delay =13sTTI) | 2.52 (7.2) | 0.95 (24.8) | 1.05 (25.0) | 0.96 (25.7) | 0.782 (20.7) | 107.66 (-19.5) | 131.52 (-19.3) | 103.00 (-20.8) | 92.51 (-18.1) | 22.30 |
| 7 (RTT=4sTTI, ACK delay =7sTTI) | 2.59 (10.2) | 1.06 (40.2) | 1.18 (40.8) | 1.09 (41.6) | 0.866 (33.6) | 95.89 (-28.3) | 118.98 (-27.0) | 90.97 (-30.0) | 83.50 (-26.1) | 23.08 |
| 3/4 | 2.63 (11.9) | 1.07 (41.5) | 1.19 (42.0) | 1.11 (44.2) | 0.836 (29.0) | 95.62 (-28.5) | 119.97 (-26.4) | 89.72 (-31.0) | 82.46 (-27.0) | 22.68 |
| 2 | 2.66 (13.2) | 1.12 (48.3) | 1.26 (50.5) | 1.17 (52.7) | 0.819 (26.4) | 91.99 (-31.2) | 119.03 (-27.0) | 84.53 (-35.0) | 78.09 (-30.9) | 20.26 |

(b) High load case

| Number of OFDM Symbols | System TP [Mbps] | Mean User TP [Mbps] | 95% User TP [Mbps] | 50% User TP [Mbps] | 5% User TP [Mbps] | Mean Packet Delay [ms] | 95% Packet Delay [ms] | 50% Packet Delay [ms] | 5% Packet delay [ms] | RU[%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 5.85 | 0.71 | 0.83 | 0.74 | 0.509 | 142.03 | 190.03 | 133.97 | 114.98 | 60.19 |
| 7 (RTT=8sTTI, ACK delay =13sTTI) | 6.81 (16.4) | 0.86 (20.5) | 1.03 (25.1) | 0.90 (22.5) | 0.520 (2.2) | 118.46 (-16.6) | 172.01 (-9.5) | 107.49 (-19.8) | 94.51 (-17.8) | 68.69 |
| 7 (RTT=4sTTI, ACK delay =7sTTI) | 7.34 (25.5) | 0.95 (33.7) | 1.17 (41.5) | 1.01 (37.4) | 0.519 (2.0) | 107.20 (-24.5) | 163.99 (-13.7) | 95.00 (-29.1) | 84.50 (-26.5) | 72.72 |
| 3/4 | 7.37 (26.0) | 0.94 (31.3) | 1.17 (41.5) | 1.00 (36.1) | 0.464 (-8.8) | 109.58 (-22.8) | 173.24 (-8.8) | 95.48 (-28.7) | 84.26 (-26.7) | 71.01 |
| 2 | 7.50 (28.2) | 0.94 (32.6) | 1.23 (49.1) | 1.03 (39.5) | 0.379 (-25.5) | 109.49 (-22.9) | 184.14 (-3.1) | 92.02 (-31.3) | 80.13 (-30.3) | 61.36 |

Table 2  Evaluation Results for large file size (500KB/file)

(a) Low load case

| Number of OFDM Symbols | System TP [Mbps] | Mean User TP [Mbps] | 95% User TP [Mbps] | 50% User TP [Mbps] | 5% User TP [Mbps] | Mean Packet Delay [ms] | 95% Packet Delay [ms] | 50% Packet Delay [ms] | 5% Packet delay [ms] | RU[%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 3.06 | 6.68 | 9.57 | 6.99 | 2.684 | 695.23 | 1382.80 | 566.76 | 403.55 | 20.12 |
| 7 (RTT=8sTTI, ACK delay =13sTTI) | 3.09 (1.0) | 7.95 (19.0) | 11.69 (22.2) | 8.39 (20.1) | 2.702 (0.7) | 612.80 (-11.9) | 1311.74 (-5.1) | 472.72 (-16.6) | 332.88 (-17.5) | 20.11 |
| 7 (RTT=4sTTI, ACK delay =7sTTI) | 3.10 (1.3) | 8.76 (31.2) | 13.01 (35.9) | 9.33 (33.5) | 2.893 (7.8) | 568.50 (-18.2) | 1269.83 (-8.2) | 429.58 (-24.2) | 300.89 (-25.4) | 19.72 |
| 3/4 | 3.13 (2.3) | 8.53 (27.7) | 12.91 (34.9) | 9.16 (31.1) | 2.547 (-5.1) | 606.88 (-12.7) | 1417.31 (2.5) | 436.53 (-23.0) | 303.05 (-24.9) | 20.99 |
| 2 | 3.18 (3.9) | 8.63 (29.2) | 13.44 (40.5) | 9.25 (32.4) | 2.280 (-15.1) | 633.35 (-8.9) | 1575.26 (13.9) | 433.69 (-23.5) | 292.03 (-27.6) | 20.17 |

(b) High load case

| Number of OFDM Symbols | System TP [Mbps] | Mean User TP [Mbps] | 95% User TP [Mbps | 50% User TP [Mbps] | 5% User TP [Mbps] | Mean Packet Delay [ms] | 95% Packet Delay [ms] | 50% Packet Delay [ms] | 5% Packet delay [ms] | RU[%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 7.60 | 4.82 | 8.02 | 4.88 | 1.525 | 1038.70 | 2382.20 | 781.62 | 467.22 | 60.94 |
| 7 (RTT=8sTTI, ACK delay =13sTTI) | 7.81 (2.8) | 5.57 (15.7) | 9.61 (19.8) | 5.62 (15.2) | 1.550 (1.6) | 947.10 (-8.8) | 2340.39 (-1.8) | 679.95 (-13.0) | 388.22 (-16.9) | 61.23 |
| 7 (RTT=4sTTI, ACK delay =7sTTI) | 7.89 (3.8) | 6.04 (25.4) | 10.69 (33.3) | 6.06 (24.1) | 1.563 (2.5) | 893.42 (-14.0) | 2235.16 (-6.2) | 631.78 (-19.2) | 346.54 (-25.8) | 61.26 |
| 3/4 | 7.90 (3.9) | 5.71 (18.5) | 10.50 (30.8) | 5.68 (16.3) | 1.231 (-19.3) | 995.43 (-4.2) | 2637.57 (10.7) | 664.27 (-15.0) | 351.22 (-24.8) | 62.15 |
| 2 | 7.95 (4.6) | 5.42 (12.5) | 10.52 (31.1) | 5.26 (7.6) | 0.897 (-41.2) | 1109.45 (6.8) | 3146.95 (32.1) | 699.23 (-10.5) | 344.69 (-26.2) | 56.18 |

We can observe following:

- Seven OFDM symbols sTTI with DL HARQ RTT 4 sTTI and UL TCP ACK delay 7 sTTI can achieve similar or better performance than the other cases.

- The better performance of 7 OFDM symbol TTI with DL HARQ RTT 4 sTTI and UL TCP ACK delay 7 sTTI is especially obtained in the following case because of reduced the overhead of control channel and RS

  - Small file size with low MCS condition (5% user through put and 95% packet delay)

  - Small file size with RU=60% condition

  - Large file size with all evaluation metric

We also note following observations:

- DL 7 OFDM symbol TTI design would allow one-level DCI, which is flexible and reduced overhead.

- DL 7 OFDM symbol TTI design means to align the slot boundary. It has much similarity with LAA design to locate PDCCH in the beginning of each slot. Seven OFDM symbol design can reduce modifications from current LTE and reduce standardization efforts.

- DL HARQ RTT 4 sTTI is a bit challenge but by limiting TBS and TA values, we think it would be worth target.

- RAN1 evaluation has been mainly 13 sTTIs scalable to sTTI length. This is coming from the conventional procedure of SR, grant assignment and transmission of UL used in TR36.881[2]. By using RAN2 proposed contention based PUSCH transmission, 8.5 to 7.5 sTTIs length is achieved. Our understanding is these procedure delay is scalable to sTTI. The UL TCP ACK delay corresponding to 7 OFDM symbol sTTI would be achieved by contention based PUSCH with 7 OFDM symbol sTTI. Another alternative is to realize it with 3/4 OFDM symbol sTTI with conventional PUSCH procedure.

Therefore, we propose to take following as latency reduction design.

- DL sTTI length is 1 slot (7 OFDM symbols).

- ACK/NACK corresponding to DL sTTI is sent in 2 slots later (DL HARQ RTT is 4 sTTI).

- In order to achieve the same UL latency with 7 OFDM symbols sTTI with TCP ACK delay 7 sTTIs, UL design is one of the following options:

- Option 1:	UL sTTI length is 3 or 4 OFDM symbol with conventional PUSCH procedure

- Option 2:	UL sTTI length is 1 slot but to introduce contention based PUSCH procedure

Conclusion

We evaluated 7 OFDM symbol TTI with DL HARQ RTT 4 sTTI and UL TCP ACK delay 7 sTTI and found such condition can achieve similar or better performance than the other conditions. Therefore, we propose following.

- DL sTTI length is 1slot (7 OFDM symbols).

- Ack/Nack corresponding to DL sTTI is sent in 2 slots later.

- In order to achieve the same UL latency with 7 OFDM symbols with TCP ACK delay 7 sTTIs, UL design is one of the following options:

- Option 1:	UL sTTI length is 3 or 4 OFDM symbols with conventional PUSCH procedure

- Option 2:	UL sTTI length is 1 slot but to introduce contention based PUSCH procedure

- Reference

[1] 		R1-162536 	"Evaluation results on shortened TTIs with dynamic overhead of control channel", Panasonic

[2]		R2-163011 	TR 36.881 v0.7.0 “Study on latency reduction techniques for LTE”

[3]		R1-164911	"Discussion on Single level DCI and two-level DCI", Panasonic

- Annex 1

Table 3   Simulation Assumptions

| Parameter | Assumptions |
| --- | --- |
| Layout | 7 Macro eNBs, 3sectors per site |
| System bandwidth per carrier | 10MHz |
| Carrier frequency | 2GHz |
| Inter-site distance | 500m |
| Total BS TX power (Ptotal per carrier) | 46dBm |
| TTI length | 1, 2, 3/4, 7, and 14 OFDM symbols. |
| TCP ACK delay for SR, grant and UL transmission | 13 sTTIs / 7 sTTIs |
| TCP ACK in UL | Error free and only ACK is modelled |
| CSI report period | 10 TTIs |
| CSI report delay | 6 TTIs |
| Fast UL Access schemes | Based on TTI length |
| RS and control signaling overhead | Assumed (see Table 2) |
| TBS determination | Scalable with TTI length as baseline |
| HARQ RTT | 8 TTIs /4TTIs for 7 OFDM symbols |
| Scheduler | Proportional fairness |
| Antenna configuration | 2Tx(eNB), 2Rx(UE), Cross-polarized |
| Number of UEs | 10 UEs per macro cell for FTP model 2 (latency reduction capable UEs only) |
| UE dropping | Randomly and uniformly dropped throughout the macro geographical area. 20% UEs are outdoor and 80% UEs are indoor. |
| Traffic model | FTP model 2, File size [100kbits, 500kB] RU[20%, 40%, 60%] |
| TCP models | TCP Reno model (RFC 2581) - SSThresh 65535 Bytes - Initial window size 1460 Bytes - Max segment size 1460 Bytes 40 Bytes TCP header are added to the initial window size and max segment size The three way handshake; not modeled |
| UE receiver | MMSE-IRC |
| UE speed | 3km/h |
| Network synchronization | Synchronized |
| Core, transport and internet network delay | 10ms |
| CRS | 2 ports |
| PDCCH length | 2 OFDM symbols |
| Overhead in TCP/IP layer | 40bytes (TCP/IP header) |
| Overhead in RAN level | PDCP header: 2 bytes RLC header: 2 bytes MAC header: 2 bytes CRC bits: 24bits |

Table 4  (s)PDCCH overhead based on UE geometry

| SINR region[dB] | (-inf, -2] | (-2, 1] | (1, 7] | (7, +inf] |
| --- | --- | --- | --- | --- |
| Aggregation level | 8 | 4 | 2 | 1 |
| # of REs | 288 | 144 | 72 | 36 |