3GPP TSG RAN Meeting #85	R1-164939

Nanjing, China, 23rd – 27th May 2016

Agenda item:	6.1.6

Source:	NEC

Title:	Clarification on repetition and Pseudo-random hopping of NPRACH

Document for:	Discussion and decision

Introduction

For NPRACH repetition and hopping, following agreements were made [1]:

RAN1#84 agreements:

- Details of hopping:

  - First level single-subcarrier hopping is used between 1st/2nd and between 3rd/4th symbol groups

  - Second level 6-subcarrier hopping is used between 2nd/3rd symbol groups

  - Pseudo-random hopping is used between repetitions of groups of 4 symbol groups:

    - Constrained within max 12 subcarriers, and according to LTE PUSCH type 2 hopping

These agreements are captured by the draft CR [2] as follows:

The preamble consisting of 4 symbol groups transmitted without gaps shall be transmitted  times.

…

In this contribution, we discuss this description of NPRACH hopping and repetition how it may read.

Discussion

We consider the description of the CR may potentially read as:

- Pseudo-random hopping is performed every 4 symbol-group transmissions of NPRACH during NrepNPRACH times transmissions of 4 symbol-groups. In other words, parameter i in hopping definition increases every transmission of symbol group during the period, i.e. i ranges 0 to 4∙NrepNPRACH-1.

On the other hand, our understanding of the agreement “Pseudo-random hopping is used between repetitions of groups of 4 symbol groups” is as follows:

- Step 1: the first preamble consisting of the first 4 symbol-groups (0≤i≤3) of NPRACH is repeated, then

- Step 2: the second preamble consisting of the next 4 symbol-groups (4≤i≤7) of NPRACH, which are Pseudo-randomly hopped from the first preamble, is repeated

Pseudo-random hopping, in our understanding, is always needed to achieve better timing estimation, even when NrepNPRACH=1. So we consider the second preamble (4≤i≤7) should not be counted as repetition. In other words, basic unit of NPRACH is the pair of the first and the second preambles consisting of 8 symbol-groups (0≤i≤7) while unit for repetition is each preamble consisting of 4 symbol-groups (0≤i≤3 and 4≤i≤7).

Taking this into account, our understanding of the agreement is written further:

- Step 1: the first preamble consisting of the first 4 symbol-groups of NPRACH (0≤i≤3) is repeated NrepNPRACH times, then

- Step 2: the second preamble consisting of the next 4 symbol-groups of NPRACH (4≤i≤7), which are Pseudo-randomly hopped from the first preamble, is repeated NrepNPRACH times

As seen above, we consider current description may be interpreted differently from the agreement. Therefore we propose:

Proposal:

RAN1 to check if the current description on NPRACH repetition captures the agreement correctly

Conclusion

In this contribution, we provided our understanding of the agreements and description of NPRACH repetition and hopping, which seem different. Therefore we would recommend:

Proposal:

RAN1 to check if the current description on NPRACH repetition captures the agreement correctly

Reference

[1] R1-163943	List of agreements	Ericsson

[2] R1-163952	Draft CR to 36.211	Ericsson