TSG-RAN WG1 #85	R1-164952

Nanjing, China, May 23 – 27, 2016

Source:	Ericsson

Title:	On CSI for multi-antenna operation in NR

Agenda Item:	7.1.6

Document for:	Discussion and Decision

# Background

In this contribution we discuss our view on the multi-antenna framework for data channels in NR and in particular the framework needed for CSI acquisition in order to perform proper link adaptation. This contribution will not discuss the actual data channel transmissions, as this will be treated in another contribution [1].

It is well known that channel knowledge at the transmitter generally enhances MIMO performance when the number of TXRUs is large as could be the case in NR.  Hence, ways to acquire channel knowledge at the transmitter should be investigated. In TDD, channel reciprocity can sometimes be utilized (see Section 3), and for FDD, high resolution spatial channel feedback can be used (see Section 6).

The possible number of TXRUs used at the transmitter will vary widely in NR, one reason is the large carrier frequency range NR should cover. Hence, there will be a need for traditional, LTE like MIMO solutions and we suggest considering if it is possible to bring in parts of LTE CSI measurement designs but by unifying the existing LTE measurement classes (see Section 4).

Moreover, the NR design should be lean and transmissions self-contained for future compatibility and for energy efficiency and provision of low inter cell interference operation. This also leads to a CSI-RS transmission framework that relies on scheduled CSI measurement signals, to avoid always on signals. This is elaborated a bit in Section 5.

We begin this contribution by making some observation based on evaluations.

# Observation based on simulation results

Better forward channel knowledge at the transmitter side leads in general to better MIMO performance, in particular for multi-user MIMO but as is shown below, also for SU-MIMO.  Forward channel knowledge is beneficial when the channel has rich scattering, i.e. having large angular spread compared to the aperture, as when there are a massive number of steerable antennas at the transmitter.

Channel knowledge at the transmitter side can be based on either high resolution CSI feedback or based on uplink measurements exploiting reciprocity. Using reciprocity may lead to smaller uplink overhead (needs to be studied) and it may also reduce the latency in scheduling. Hence, reciprocity assisted SU-MIMO and SRS design to provide high quality estimates are well-worth investigating for NR in addition to the baseline of well-known closed-loop CSI feedback schemes using implicit CSI derivation.

Table 1 below shows the performance gain of SU-MIMO using transmitter channel knowledge compared to classical coarsely quantized LTE based closed loop codebook feedback for a large number of TXRUs. There is a clear benefit of having channel knowledge but it requires enhancements in channel sounding to benefit also the cell edge users.

Table 1 Performance gain over codebook based CSI feedback for reciprocity based SU-MIMO

| 3D UMa modified with high AS, 2 GHz, Mode 3-2 |  | Cell edge UTP Gain over LTE codebook | Mean UTP Gain over LTE codebook |
| --- | --- | --- | --- |
| 1x16 (32 TXRU) | Reciprocity based CSI, with SRS impairments | -19% | 10 % |
|  | Ideal reciprocity, no SRS impairments | 92% | 21% |

For MU-MIMO scheduling shown in Table 2, we show the corresponding gain of codebook based MU-MIMO over codebook based SU-MIMO and also when using reciprocity assisted MU-MIMO operation, where SRS impairments has been included.

Table 2 Performance gain over codebook based SU-MIMO for codebook based MU-MIMO and reciprocity based MU-MIMO

| With impairments, 3D UMi, 2 GHz, Mode 3-2 |  | Cell edge UTP Gain over SU-MIMO | Mean UTP Gain over SU-MIMO |
| --- | --- | --- | --- |
| 8x4 (32 TXRU) | LTE FD-MIMO codebook based CSI and MU-MIMO | 34% | 10 % |
|  | Reciprocity based CSI and MU-MIMO 23 dBm UE TX | 64% | 17% |

Based on these evaluations, we make the following observation:

Observation: Having detailed channel knowledge at the transmitter shows promising SU as well as MU MIMO performance gain over codebook based operation, for a large number of TXRUs

Hence, as a massive number of TXRUs is target for higher frequencies NR, one of the objectives of the SI should be how to provide channel knowledge to the transmitter in one way or the other.

# Using MIMO channel reciprocity for CSI estimation

One focus of NR multi-antenna operation should be reciprocity based operation, including design of channel sounding reference signals providing adequate channel estimate qualities. Other study points are how to perform rank and CQI estimation and possibly also discussions on whether specified calibration assistance by the UEs is required for reciprocity operation.

The conditions when channel reciprocity can be utilized to make the instantaneous channel available to the transmitter in an efficient manner are

- The MIMO channel for the forward an reverse link are reciprocal (necessity: TDD)

- The RX-TX chains are calibrated at the BS

- The number of RX and TX ports and the antennas are the same at the UE

- The UE supports sounding the channel in the UL of frequencies corresponding to the entire DL system bandwidth

- There is sufficiently good SINR  of the channel sounding signal (i.e., there needs to be coverage for SRS) and short latency compared to channel coherence time

Condition 3 is challenging, and will likely not be fulfilled in general, especially if the UE has a large number of RX antennas it will be difficult to match this with the same number of TX chains. Implementation complexity and RF losses in the switch may make it more challenging to achieve condition 3.

Similarly, condition 4 may not be fulfilled in carrier aggregation scenarios where some carriers have a single transmission direction and for terminals where sounding reference signal switching is not supported.  Moreover, in FDD systems the carrier is not bi-directional. In addition, the uplink may be carried on a lower frequency band and in unlicensed operation a similar situation occurs. Hence, in-complete reciprocity needs to be handled by the NR.

Condition 5 is challenging in coverage limited scenarios. Hence, reciprocity based MIMO operations will at higher carrier frequencies have a maximum useful range and the closed loop CSI feedback is needed to extend coverage of MIMO for terminals. Higher carrier frequencies are likely TDD bands where reciprocity can be used and reciprocity can be used for those terminals close to the node (cell center MSs) that can provide sufficient channel sounding quality and which are uplink capable on that carrier.

The use of closed-loop on the cell-edge also make sense from the perspective that cell-edge UEs are more geared towards SU-MIMO operation, which even classical closed-loop CSI feedback handles in a competitive manner since relaibale uplink channel sounding at cell edge is challenging and requires resources in terms of power.

Figure 1 Joint use of reciprocity and CSI feedback from the same node

The use of reciprocity is not always guaranteed and particularly not for FDD. Hence, when the conditions are not fulfilled, NR needs also to have support for alternative, non-reciprocity based ways for the transmitter to have detailed forward channel knowledge. Reciprocity based operation when conditions are fulfilled are further discussed in [2]. We conclude with these summarizing observations:

Observations:

- Reciprocity based CSI acquisition is a promising approach in some cases

- Closed loop CSI feedback is a promising approach in other cases

- The two CSI acquisition approaches are complementary, even in the same node

# A unified CSI framework

As observed in the previous section, closed loop CSI feedback operation with implicit CSI feedback is necessary in NR MIMO operation, to be able to cover the wide range of sizes of antenna arrays, carrier frequencies and when reciprocity cannot be utilized.

Reflecting on the LTE specifications, feedback classes as well as subclasses exist and there are overlapping functionality by different configurations, which makes the specification unnecessary complicated and hard to understand.

Observation: CSI measurements and feedback in LTE is unnecessarily complicated and contains overlapping functionality.

To simplify the discussion and also specifications, we propose in NR to aim for designing a single, easy to understand core closed loop CSI feedback framework that with a few different parameter settings can achieve the desired CSI feedback functionality for most scenarios, particularly at lower frequencies and for smaller antenna arrays.

Observation: Strive for a unified and simple CSI framework for closed loop feedback in NR to support antenna arrays with small as well as massively large number of antenna elements

If methods are enabled that allows the transmitter to have forward channel knowledge (reciprocity or high resolution feedback), then it is desirable if they also could make use of the same unified closed loop CSI framework, e.g. to acquiring interference feedback and to allow for reciprocity based operation to make benefit of advanced receiver processing (which is unknown to the transmitter and which reciprocity based CSI does not capture).

Some desired requirements on such unified framework is that it

- Handles ,

  - “classical” codebook based closed-loop precoding (non-precoded CSI-RS),

  - beamformed CSI-RS

  - operation when channel reciprocity (see [2]) is utilized

  - operation when high resolution CSI feedback (see section 6) is utilized

- has efficient support for all beamforming implementation architectures: digital, analog, hybrid

- provides low latency in realistic situations

How to meet these requirements needs further studies and discussions, but this should be the mind set in our view.

It is noted that classical codebook based precoding feedback (Class A) can in LTE be seen as a special case of the beamformed CSI-RS feedback approach (Class B). Hybrid approaches are currently discussed in LTE Rel-14, which can be a combination of Class A+B or Class B+B feedback, in a single CSI process. We believe that all these can be described in a single framework where the number of ports, and which ports, the UE shall measure on is more generally described, together with the associated measurement restrictions.

Another reflection is that the availability of channel reciprocity by uplink measurements can be combined with beamformed CSI-RS operation, to ensure robust link adaptation (although whether CSI-RS antenna ports are beamformed or not is transparent to the UE in a unified CSI feedback framework). Using reciprocity together with beamformed CSI-RS operation suits better for large packet transmissions, as there is some latency associated with a closed loop CSI feedback.

For low latency use cases, it may be better to directly use the reciprocity information provided that the network can derive the interference at the UE from a CQI report.

# CSI reference signal transmissions in a lean NR design

A key principle of NR is lean and self-contained design and the consequences of these needs to be reflected in the CSI reference signal procedures and design. Hence, our approach is that the CSI-RS design should avoid transmitting always on, periodic, CSI-RS and SRS signals as in LTE.

There are several reasons to this, firstly, pooling of CSI-RS/SRS resources keeps the CSI-RS/SRS overhead down, which is important, especially when the number of simultaneously served UEs is large. Another reason for a dynamic/scheduled CSI-RS/SRS resource are the same as for lean designs in general; to meet the energy efficiency requirement of the new RAT and to avoid static reference signal configurations eases the introduction of new services in the same band. Hence, scheduling of CSI or SRS measurements with a single/multi-shot/semi-persistent CSI-RS/SRS transmission should be further investigated.

Proposal: Layer 1 measurement signals (e.g. CSI-RS and SRS) in NR should be possibly to dynamically schedule, to avoid and minimize “always on” signals in line with the key NR design principles

# Introducing high spatial resolution channel feedback

As discussed above, reciprocity cannot always be utilized and in these cases, and in particular for FDD, a high spatial resolution (HR) channel feedback should also be specified in parallel or as an “add-on” to the normal closed loop CSI measurement feedback. The grid of beam codebook based on spatially oversampled DFT beams is in this context seen as a “normal” spatial resolution feedback.

A high resolution CSI feedback is important to approach the upper ranges of MIMO performances so that these large spectral efficiencies can be achieved in all scenarios, duplex modes and deployments.

Observation: In addition to the normal spatial resolution feedback provided by a spatially oversampled GoB codebook, there is also a need for an enhanced closed loop CSI feedback resolution to approach that attainable by the use of channel reciprocity, so that MU-MIMO benefits can be obtained also in all the cases when reciprocity cannot be utilized

A HR channel feedback can for example be obtained by using a HR codebook based on DFT beams but where multiple beams is selected, corresponding to the channel multipath. Each beam is associated with amplitude and a phase which is determined per subband.

# Conclusion

To provide multi-antenna operation for data channels in NR, the standard should support the following CSI feedback components and the study item studies can be also categorized into these areas

Proposal: Study these areas for multi-antenna operation in NR

- Implicit, closed loop CSI feedback

  - including configuration of measurement restriction

  - preferably a single unified framework  without classes and sub-classes as in LTE

  - providing normal spatial resolution feedback (spatially oversampled GoB)

- High resolution closed loop feedback (may be categorized as an optional CSI feedback add on, under the implicit feedback)

- Reciprocity-based CSI acquisition

  - Channel acquisition based on uplink  sounding signals

  - Means to capture UE side interference plus noise on the network side and also to benefit from advanced receivers

For all these components, the impact of dynamic scheduling of CSI reference signals should be taken into account, to avoid “always on” signals and to allow CSI resource pooling. This will cater for high energy efficiency at the network side and allow for forward compatibility that ease introduction of new features in NR.

Proposal: Layer 1 measurement signals (e.g. CSI-RS and SRS) in NR should be possibly to dynamically schedule, to avoid and minimize “always on” signals in line with the lean NR design principle

# References

R1-164953, “Multi-antenna transmission principles for data channels in NR”, Ericsson

R1-164955, “Reciprocity based MIMO in NR”, Ericsson