3GPP TSG RAN WG1 Meeting #85	R1-165060

Nanjing, China 23rd – 27th May 2016

Agenda Item:   7.1.5.1

Source:	  InterDigital Communications

Title:	          Performance Evaluation of Channel Coding for eMBB

Document for:	  Discussion

# 1	Introduction

In the RAN1 meeting #84bis, a lot of contributions on the topic of channel coding design for the physical layer of the NR system were discussed. The initial agreements regarding the channel coding for NR include the following [1]:

- Candidates for 5G new RAT data transmission are identified as the following

  - LDPC code

  - Polar code

  - Convolutional code (LTE and/or enhanced convolutional coding)

  - Turbo code (LTE and/or enhanced turbo coding)

  - Note: It is RAN1 common understanding that combination of above codes is not precluded

  - Note: Outer erasure code is not precluded

- Selection of 5G new RAT channel coding scheme(s) will consider,

  - Performance

  - Implementation complexity

  - Latency (Decoding/Encoding)

  - Flexibility (e.g., variable code length, code rate, HARQ (as applicable for particular scenario(s)))

In this contribution, we present our preliminary analysis and simulation results for turbo code, LDPC code and polar code for the enhanced Mobile Broadband (eMBB) use case. The comparison and evaluation of the performance of these codes are provided.

# Discussion

To facilitate unified performance evaluation, the simulation assumptions for channel coding for eMBB use case have been agreed [2] (see Appendix for details). Our simulations are based on these assumptions and our performance evaluation is in terms of the Block Error Rate (BLER) vs. SNR.

## 2.1   Performance Comparison of Channel Coding for eMBB

In our simulations, channel codes as in [3l for polar codes, LDPC codes and turbo codes are used.

We apply the max-log-MAP decoding algorithm for turbo codes with the maximum number of iterations = 8; the sum-product decoding algorithm for LDPC codes with the maximum number of iterations = 20; the CRC-aided successive cancellation list decoding algorithm with list sizes of 1, 4 and 32 for polar codes.

We simulated coding rates 1/3 and 5/6 as specified in Table 4, with information block and coded block lengths details as summarized in Table 1.

Table 1: Block length used in simulations

|  | R = 1/3 |  |  | R=5/6 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | LDPC | Turbo | Polar | LDPC | Turbo | Polar |
| Information block length (bits) | 1356 | 1344 | 1356 | 6825 | 6825 | 6825 |
| Coded block length w/o puncturing (bits) | 4068 | 4032 | 4096 | 8192 | 8192 | 8192 |
| Punctured bits | NA | NA | 28 | NA | NA | 2 |

Figure 1 provides the BLER performance comparison of rate 1/3 codes. It is shown from the figure that the turbo code has the best performance. LDPC code has similar performance as polar code with list 4.

Observation 1: For medium information block length with rate 1/3, turbo code outperforms LDPC code and polar code.

Figure 2 provides the BLER performance comparison of rate 5/6 codes. It is shown from the figure that the LDPC code has the best performance. Turbo code outperforms polar code.

Observation 2: For medium-large information block length with rate 5/6, LDPC code outperforms turbo code and polar code.

Figure 1: Performance comparison for rate 1/3 codes

Figure 2: Performance comparison for rate 5/6 codes

For convenience, we summarize the minimum SNR (in dB) required for turbo, LDPC, and polar codes to achieve the target BLER levels, shown in Table 2. This table is derived from Figures 1 and 2. The numbers in green indicate the best performance, and the numbers in red indicate the worst performance.

Table 2: The minimum SNR required for turbo, LDPC, and polar codes to achieve the target BLER (unit: dB)

| Code Rate | BLER | QPSK |  |  |  |  | 64QAM |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Turbo | LDPC | Polar-1 | Polar-4 | Polar-32 | Turbo | LDPC | Polar-1 | Polar-4 | Polar-32 |
| 1/3 | 1e-2 | -0.76 | -0.02 | 0.26 | -0.36 | -0.73 | 8.09 | 9.11 | >10 | 9.21 | 8.67 |
|  | 1e-3 | -0.56 | 0.41 | 0.57 | -0.07 | -0.41 | 8.30 | 9.52 | >10 | 9.61 | 9.06 |
| 5/6 | 1e-2 | 5.84 | 5.60 | 6.75 | 6.40 | 6.38 | 17.83 | 17.31 | 19.03 | 18.41 | 18.15 |
|  | 1e-3 | 6.01 | 5.79 | 7.19 | 6.97 | 6.88 | 18.09 | 17.49 | 19.26 | 18.89 | 18.57 |

## 2.2 	Complexity Analysis

In this section, we compare the decoding complexity for the codes we have simulated. Specifically, our comparison is in terms of operations count. For turbo codes and LDPC codes, we apply the conclusions in [4], while for polar codes, we utilize the results from [5]. For the rate 1/3 LDPC code, we have the degree distribution dv=3, dc=4.5; For the rate 5/6 LDPC code, we have the degree distribution dv=3, dc=18.

The decoding complexity comparison of turbo, LDPC and polar codes is shown in Table 3. It is seen from the table that polar code with list 1 or 4 always has lower decoding complexity than turbo or LDPC code. LDPC code has lower decoding complexity than turbo code, and the gap is large at high coding rate. Polar code with list 32 has high decoding complexity at high coding rate and large block length.

Table 3: Decoding complexity comparison of turbo, LDPC, polar codes (unit: operations count)

|  | Turbo (log- map) | Turbo (max-log- map) | LDPC (sum- product) | LDPC (min-sum) | Polar (L=1) | Polar (L=4) | Polar (L=32) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R=1/3 | 6,311,424 | 1,838,592 | 3,851,040 | 1,030,560 | 50,508 | 212,880 | 1,833,216 |
| R=5/6 | 34,374,720 | 10,013,760 | 5,581,500 | 1,665,300 | 113,321 | 507,884 | 4,718,272 |

Taking into account both BLER performance and decoding complexity, we propose to consider LDPC code as a candidate channel code for eMBB at high coding rates, and consider either LDPC code or turbo code as a candidate channel code for eMBB at low coding rates.

Proposal 1: Taking into account the BLER performance and decoding complexity, LDPC code could be considered as a candidate channel code for eMBB at high coding rates and medium-large information block lengths; either LDPC code or turbo code could be considered as candidate channel codes for eMBB at low coding rates and medium information block lengths.

# 3	Conclusion

In this contribution, we discussed various coding schemes for eMBB, compared their performance and complexity. Our simulation results show that

Observation 1: For medium information block length with rate 1/3, turbo code outperforms LDPC code and polar code.

Observation 2: For medium-large information block length with rate 5/6, LDPC code outperforms turbo code and polar code.

Hence, we have the following proposal:

Proposal 1: Taking into account the BLER performance and decoding complexity, LDPC code could be considered as a candidate channel code for eMBB at high coding rates and medium-large information block lengths; either LDPC code or turbo code could be considered as candidate channel codes for eMBB at low coding rates and medium information block lengths.

# References

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #84bis, Busan, Korea, Apr. 2016.

- R1-163757, “WF on channel coding evaluation for 5G new radio,” Apr. 2016.

- R1-165059, “Performance evaluation of channel coding for URLLC and mMTC,” May 2016.

- R1-060874, “Complexity comparison of LDPC codes and Turbo codes,” Mar. 2006.

- R1-162162, “High-level comparison of candidate FEC schemes for 5G,” Apr. 2016.

# Appendix: Evaluation Assumptions of Channel Coding for eMBB

Table 4: Simulation assumptions for eMBB [2]

| Channel | AWGN |  |  |
| --- | --- | --- | --- |
| Modulation | QPSK, 64QAM |  |  |
| Coding scheme | Turbo | LDPC | Polar |
| Code rate | 1/5, 1/3, 2/5, ½, 2/3, ¾, 5/6, 8/9 |  |  |
| Decoding algorithm | Max-log-MAP | min-sum | List-X |
| Info. block length (bits w/o CRC) | 100, 400, 1000, 2000, 4000, 6000, 8000 Optional (12K, 16K, 32K, 64K) |  |  |