3GPP TSG RAN WG1 Meeting #85                                                               	           R1-165071

Nanjing, China, 23rd – 27th May 2016

Agenda item:	6.2.1.5

Source: 	Intel Corporation

Title: 	Defer period for UL LBT

Document for:	Discussion and Decision

- Introduction

During Release 13 LAA, the channel access mechanism had been specified [1]. In particular, the eNB may perform transmission on LAA SCell(s) after sensing the channel to be idle during the slot duration of a defer duration Td and after the random backoff counter N is reached zero. The defer duration Td consists of duration Tf=16us immediately followed by mp consecutive slot durations where each slot duration is Tsl=9us. The mp values for each priority class are given in Table 15.1.1-1 in [1], which is also copied below.

| Channel Access Priority Class () |  |  |  |  | allowed sizes |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3 | 7 | 2 ms | {3,7} |
| 2 | 1 | 7 | 15 | 3 ms | {7,15} |
| 3 | 3 | 15 | 63 | 8 or 10 ms | {15,31,63} |
| 4 | 7 | 15 | 1023 | 8 or 10 ms | {15,31,63,127,255,511,1023} |

In this contribution, we discuss and propose mp values for UL LBT in accordance with the behavior of WLAN STAs.

- IEEE 802.11e EDCA

The following is referred from IEEE 802.11e standard [2].

The duration AIFS[AC] is a duration derived from the value AIFSN[AC] by the relation

AIFS[AC] = AIFSN[AC] × aSlotTime + aSIFSTime.

The value of AIFSN[AC] shall be greater than or equal to 2 for non-AP QSTAs and is advertised by the QAP in the EDCA Parameter Set information element in Beacon and Probe Response frames transmitted by the QAP. The value of AIFSN[AC] shall be greater than or equal to 1 for QAPs.

It can be also found from Annex D in [2] that the default configuration for STAs is mp={2, 2, 3, 7} for access priority classes from 1 to 4. The Release 13 design of defer period is in accordance of default configuration for APs as in IEEE 802.11e standard.

- Discussion on the defer period for UL LBT

As discussed in Section 2, the Release 13 defer period is in accordance of the behavior of IEEE 802.11e compliant APs. In honoring the incumbent WLAN systems and to enable fair competition in channel access, it is proposed to clarify the UL LBT parameters for access priority classes in accordance of the behavior of the IEEE 802.11e compliant STAs. That is the mp for UL LBT is 2 when the channel access priority class is 1 or 2.

Proposal: The value of mp in computing the defer period for uplink channel access is 2 for access priority class 1 and 2.

- Conclusions

In this contribution, we discussed the defer period for UL LBT and made the following proposal.

Proposal: The value of mp in computing the defer period for uplink channel access is 2 for access priority class 1 and 2.

References

[1] 3GPP TS 36.213 V13.0.1, Physical layer procedures

[2] IEEE Std 802.11eTM-2005 Amendment 8: Medium Access Control (MAC) Quality of Service Enhancement