3GPP TSG RAN WG1 Meeting #85	R1-165076

Nanjing, China, 23th – 27th May 2016

Agenda item:	6.2.10.3

Source: 	Intel Corporation

Title: 	Link level simulation results for S-PDCCH

Document for:	Discussion and Decision

- Introduction

In this contribution we provide link simulation results for the PDCCH for shortened TTI. The details of the S-PDCCH design are presented in the companion contribution [1].

- Discussion

In [1], multiple design options for shortened PDCCH (S-PDCCH) are presented to achieve sufficient diversity for reliable and robust reception of S-PDCCH. In this contribution, we discuss performance results for a distributed S-PDCCH design consisting of 36 REs based on legacy PDCCH block interleaver with 2 S-REGs per PRB as one of the potential S-PDCCH design choice.  An illustration of the S-PDCCH design is shown in Figure 1.

Figure 1. Illustration of the proposed S-PDCCH structure

- Performance results

This section presents link-level simulation performance for S-PDCCH transmission based on the on evaluation methodology in [2].

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| Short TTI bandwidth | 50 RB (10 MHz) |
| TTI length | 2 or more symbols |
| Channel model | EPA, EVA |
| UE speed | 3km/h (5 Hz), 60 km/h (111 Hz) |
| Antenna configuration | 2Tx(eNB), 2Rx(UE) |
| CP length | Normal |
| Receiver type | MMSE |
| Channel estimation | Practical |

In this contribution performance results for the CRS-based S-PDCCH is presented.

S-PDCCH consisting of multiple CCEs (i.e. each CCE consists of 36 REs) is placed in the first symbol of the S-TTI. Figure 2(a) and Figure 2(b) show the comparison of SNR required to achieve 1% target BLER for S-PDCCH under the EVA and EPA models respectively. Figures 3-7 show BLER performance for S-PDCCH for different payloads and aggregation levels for EVA channel model. Figures 7-10 show BLER performance for S-PDCCH for different payloads for EPA channel model.  The performance of S-PDCCH is compared against legacy PDCCH.

Based on the results following observation is made:

Observation: The proposed S-PDCCH design has comparable decoding performance to legacy PDCCH.

| (a) EVA | (b) EPA |
| --- | --- |

Figure 2. Target SNR required for 1% BLER

Figure 3. BLER for S-PDCCH and PDCCH, EVA 50 km/h, (s)PDCCH uses 1 CCE

Figure 4. BLER for S-PDCCH and PDCCH, EVA 50 km/h, (s)PDCCH uses 2 CCEs

Figure 5. BLER for S-PDCCH and PDCCH, EVA 50 km/h, (s)PDCCH uses 4 CCEs

Figure 6. BLER for S-PDCCH and PDCCH, EVA 50 km/h, (s)PDCCH uses 8 CCEs

Figure 7. BLER for S-PDCCH and PDCCH, EPA 3 km/h, (s)PDCCH uses 1 CCE

Figure 8. BLER for S-PDCCH and PDCCH, EPA 3 km/h, (s)PDCCH uses 2 CCEs

Figure 9. BLER for S-PDCCH and PDCCH, EPA 3 km/h, (s)PDCCH uses 4 CCEs

Figure 10. BLER for S-PDCCH and PDCCH, EPA 3 km/h, (s)PDCCH uses 8 CCEs

Table 1: S-PDCCH SNR coverage for 1% BLER for different payload and number of CCEs (EVA channel)

| Payload including CRC | 1 CCE | 2 CCE | 4 CCE | 8 CCE |
| --- | --- | --- | --- | --- |
| 40 | 3.58 | 0.4 | -3.1 | -5.5 |
| 50 | 5.2 | 0.5 | -2.2 | -4.65 |
| 60 | 7.7 | 1.5 | -1.4 | -3.9 |
| 70 | 12.9 | 2.5 | -0.7 | -3.3 |

Table 2: S-PDCCH SNR coverage for 1% BLER for different payload and number of CCEs (EPA channel)

| Payload including CRC | 1 CCE | 2 CCE | 4 CCE | 8 CCE |
| --- | --- | --- | --- | --- |
| 40 | 3.79 | 0.4 | -2.36 | -4.7 |
| 50 | 5.7 | 1.5 | -2.55 | -3.99 |
| 60 | 7.4 | 2.38 | -0.49 | -3.4 |
| 70 | 11.9 | 3.2 | 0.10 | -2.5 |

- Conclusions

In this contribution, the link level performance of S-PDCCH is presented. Based on the evaluation following observations are made:

Observation: The proposed S-PDCCH design has comparable decoding performance to legacy the PDCCH.

References

[1] R1-164161, Downlink control singling design for shorten TTI, 3GPP TSG RAN WG1 Meeting #85, Intel Corporation, May, 2016

[2] R1-161422, Way forward on simulation results and methodology, Ericsson., RAN1#84, February 2016