3GPP TSG RAN WG1 Meeting #85                                                                   R1-165200

Nanjing, China 23rd - 27th May 2016

Source:	NTT DOCOMO

Title:	UE-Specific Aperiodic CSI-RS with Semi-static Resource Reservation

Agenda Item:	6.2.3.1.2

Document for: 	Discussion and Decision

Introduction

At the RAN1 #84bis meeting, following agreements have been reached about beamformed CSI for Rel. 14 [1].

- Until RAN1#85, down-selection/merge the following alternatives for reducing the overhead for beamformed CSI-RS, considering overhead reduction, performance, backward compatibility, UE implementation complexity:

- Alt.1: Aperiodic CSI-RS

  - E.g., One-shot CSI-RS transmission with transmission on the same subframe as triggering subframe

- Alt.2: Dynamic CSI-RS activation and/or deactivation

  - E.g., One-shot or multi-shot CSI-RS transmission after the triggering subframe

- Alt.3: Reduced frequency density via decimation over part or all of the bandwidth

- Alt.4: MR with dynamic sharing of the beams on CSI-RS resources among the UEs

- Alt.5: Triggered measurement of one of multiple CSI-RS configurations

- Alt.6: Spatially multiplexed beamformed CSI-RS

- Other approaches are not precluded

In this contribution, we present our views on further enhancement of BF CSI-RS achieving efficient CSI-RS transmission.

Discussion on Overhead Reduction for UE-specific BF CSI-RS

Up to six alternatives have been proposed in last meeting and the analysis of the alternatives are list below.

Table 1. Analysis of alternatives

| Alternatives | Main Features | Pros | Cons |
| --- | --- | --- | --- |
| 1: Aperiodic BF CSI-RS | CSI-RS transmitted only when needed REs reused for PDSCH | Overhead reduction; Enhanced flexibility for CSI-RS scheduling | DCI signalling overhead for rate matching |
| 2: Dynamic CSI-RS activation and/or deactivation | One indication of CSI-RS ON/OFF REs reused for PDSCH | Overhead reduction; Less specification impact | DCI signalling overhead for rate matching |
| 3: Low-frequency density | Transmitted every x-th PRBs. Other PRBs can be used for PDSCH or another UE specific BF CSI-RS | Overhead reduction; | Impact to CSI accuracy; Difficult in resource sharing with legacy UE. |
| 4: Dynamic beam sharing CSI-RS | MR with dynamic sharing of the beams on CSI-RS resources among the UEs | Overhead reduction; Enhanced flexibility for CSI-RS scheduling; | Difficult in resource sharing with legacy UE. |
| 5: Triggered measurement of one of multiple CSI-RS configurations | Tiggered measurement of one of multiple CSI-RS configurations | Enhanced flexibility for CSI-RS scheduling; | DCI signalling overhead for trigger. |
| 6: Spatially multiplexed BF CSI-RS | Spatial multiplexing of CSI-RS | Without specificaion impacts |  |

As each of the above alternative solutions has its own pros and cons, combination of some of them may provide the best tradeoff w.r.t. overhead reduction, performance, backward compatibility, UE implementation complexity. A proposed combination scheme is discussed in the next section.

UE-Specific Aperiodic CSI-RS with Semi-static Resource Reservation

Semi-static cell-specific resource pool reservation

In Rel. 13 standardization, a same CSI-RS resource can be shared by multiple UEs by turning on measurement restriction (MR). In that sense, a CSI-RS resource pool is achieved in Rel. 13. In Rel. 14, the resource pool concept can be further extended to supprot more flexibility for aperiodic CSI-RS, e.g., the min. periodicity for CSI-RS resource reservation can be 1 ms and flexible frequency cofigurations can be supported.

Proposal 1: CSI-RS resources are reserved in periodic manner:

- The min. periodicity for CSI-RS resource reservation can be 1 ms.

Dynamic UE-specific Resource Allocation

To improve the utilization effeciency of CSI-RS resources, the time/freq./space sharing within the resource pool shall be supported. As illustrated in the right part of Fig. 1, UE1 and UE2 are configured with the same resource, UE3 and UE4 have some overlapped antenna ports. In general, CSI-RS resource can be shared in port level. In frequency domain, UE can be configured with different density of CSI-RS like the left part of Fig. 1. To summarize, we have the following proposals:

Proposal 2: Dynamic UE-specific resource allocation is supported:

- The configuration can be explicitly/implicitly indicated in DL grant;

- The resource allocation should be confined in CSI-RS resource pool and configurable;

- Different UEs can be configured with overlapped CSI-RS ports;

- The CSI-RS configured to UE can be wideband or subband.

Figure 1: UE-specific resource allocation

One of the extreme case is no extra bit was used to indicate the UE specific CSI-RS configuration, for example, the number of antenna port used for CSI-RS of resource pool is 2, and the periodicity is 1ms. UE measures the whole ports once triggered. With the enhanced dynamic trigger which would be introduced in next section, UE can measure multi-beams.

Enhanced CSI trigger

Whether reserved CSI-RS resource is measured or not is dynamically triggered in DL/UL grant. To improve the accuracy of CSI measurement and the flexibility of CSI-RS configuration, the transmission instances assumed by a single CSI measurment, should be considered, e.g., by introducing a measument instance number indicator, q. Fig. 2 shows the different cases with q=1 and q>1. Another possible implementation is to introduce a reset flag to indicate the start of a new measurement.

Figure 2: Aperiodic CSI-RS transmission instance

Proposal 3: CSI trigger shouled be enhanched:

- Alt. 1 CSI-RS transmission instance number per CSI measurement;

- Alt. 2 Reset flag is indicated.

Rate Matching

Rate matching should be considered if REs can be reused for PDSCH transmission, when CSI-RS is not multiplexed. Two alternatives can be adopted to solve the problem. Alt. 1 is that eNB and UE conduct rate mathcing based on CSI-RS transmission trigger [2]. Considering that resource pool can be configured with longer periodicity and less antenna ports when a smaller number of beams should be transmitted which can partly reduce the overhead, Alt. 2 means REs cannot be reused for PDSCH transmission.

Proposal 4: The following rate matching alternatives are considered:

- Alt. 1: eNB and UE conduct rate mathcing based on the trigger [2];

- Alt. 2: REs reserved for CSI-RS cannot be reused for PDSCH transmission.

Summary

In this contribution, we presented our views on futher enhancedment of BF CSI-RS achieving efficient CSI-RS transmission. Based on the discussion, we made the following proposals.

Proposal 1: CSI-RS resources are reserved in periodic manner:

- The min. periodicity for CSI-RS resource reservation can be 1 ms.

Proposal 2: Dynamic UE-specific resource allocation is supported:

- The configuration can be explicitly/implicitly indicated in DL grant;

- The resource allocation should be confined in CSI-RS resource pool and configurable;

- Different UEs can be configured with overlapped CSI-RS ports;

- The CSI-RS configured to UE can be wideband or subband.

Proposal 3: CSI trigger shouled be enhanched:

- Alt. 1 CSI-RS transmission instance number per CSI measurement;

- Alt. 2 Reset flag is indicated.

Proposal 4: The following rate matching alternatives are considered:

- Alt. 1: eNB and UE conduct rate mathcing based on the trigger [2];

- Alt. 2: REs reserved for CSI-RS cannot be reused for PDSCH transmission.

- References

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #84bis.

- 3GPP, R1-162969, NTT DOCOMO, “Discussion on Aperiodic CSI-RS”, April, 2016.