TSG RAN WG1 #85	R1-165218

Nanjing, China, May 23-27, 2016

Source:	Keysight

Title:	Time–Varying Delays and Angles

Agenda Item:	7.2.2

Document for:	Discussion and Decision

# Introduction

In Ericsson’s contributions [1] and [2] was identified a problem related to generation of radio channel impulse responses in the case of time variant Doppler shift term. Also a proposal to fix the problem was given. In this document we clarify the issue and slightly reformulate the Ericsson’s proposal.

# Problem

As the background for the problem a snapshot is taken from [1]:

Citation starts

----------------------

In the 3D SCM model the following step is given for converting clusters and rays into complex channel coefficients:

Step 11: Generate channel coefficients for each cluster n and each receiver and transmitter element pair u, s.

For the N – 2 weakest clusters, say n = 3, 4,…, N, the channel coefficients are given by:

(7.21)

…

The Doppler frequency component vn,m is calculated from the arrival angles (AOA, ZOA), UT velocity vector with speed v, travel azimuth angle ϕv, elevation angle θv and is given by

,	(7.24)

Observation: The current channel coefficient generation procedure in the 3D SCM model [3] is not compatible with time-varying angles since unphysically high Doppler spreads may be generated.

-------------------------

Citation ends

# Discussion

A plane wave is illustrated in Figure 1. If for example the receiver moves with constant velocity along a route denoted by the white or the black line in Figure 1 it will experience different Doppler shifts on the two routes. Along the white route the Doppler shift has higher absolute value. The motion across or along waves can be illustrated also by a phasor as shown in Figure 2 and Figure 3. In Figure 2 the Doppler shift is constant, as indicated by the flat red curve, and the phase difference between any two consecutive sample points (blue dots) is also constant. In Figure 3 the Doppler shift is first linearly increasing and then linearly decreasing, however being positive all the time. One can observe that the phase increment, i.e. the distance between blue dots, is time variant. Actually, unlike the last term of eq. (7.21) above, the complex phasor should be written as

expj2π0tυn,mtdt.

Luckily, with time invariant Doppler shift term this integral reduces to the form of eq. (7.21), i.e.  expj2πυn,mt. However, in general case with time variant Doppler shift term

expj2π0tυn,mtdt≠expj2πυn,mt t

as noted by [1].

Figure 1. A plane wave with two different trajectories denoted by white and black lines.

Figure 2. A phasor representing a plane wave with constant (time invariant) Doppler shift.

Figure 3. A phasor representing a plane wave with time variant Doppler shift.

For discrete time simulation purposes the complex phasor at time instant t=k∆T can be written as

expj2πk=0Kυn,mk∆T∆T

where ∆T is the time sampling interval, and k is a positive integer. This fixes the problem and gives the same effect as Ericsson’s proposals in [1]:

Citation starts

----------------------

- Proposal 1: Replace the factor expj2πυn,m⋅t with expj2πυn,mt⋅t-tiexpj2πυn,mti⋅ti in the channel coefficient equation

  - The phase shift at time ti is established as an anchor point and an incremental phase shift is applied

  - The anchor point should be updated sufficiently often so that υn,mti+1-υn,mti≪1

- Proposal 2: Proposal 1 needs only to be considered in case the Doppler frequencies are time-varying. This could be due to:

  - Non-linear UE movement

  - Time-varying angles of arrival due to spatial consistency modeling

----------------------

Citation ends

While the effect is same the ambiguity of updating ti is avoided. All computer simulations and possible HW emulations are performed with discrete time in any case, so defining the complex phasor as it is done above doesn’t introduce any restriction.

# Conclusion

In order to maintain physical Doppler frequencies and phase continuity in the presence of time-varying angles, the following is proposed:

- Proposal 1: Replace the factor expj2πυn,m⋅t with expj2πk=0Kυn,mk∆T⋅∆T in the channel coefficient equation

  - where t=k∆T, ∆T is the time sampling interval and k is a positive integer

- Proposal 2: Proposal 1 needs only to be considered in case the Doppler frequencies are time-varying. This could be due to:

  - Non-linear UE movement

  - Time-varying angles of arrival due to spatial consistency modeling

# References

- R1-163246, “Channel modeling with time-varying delays and angles”, Ericsson, RAN1 #84bis, April, 11-16, 2016

- R1-163636 “WF on modeling of time-varying angles”, Ericsson, RAN1 #84bis, April, 11-16, 2016

- 3GPP TR 36.873 “Study on 3D channel model for LTE”, v12.0.0, June 2015