3GPP TSG RAN WG1 Meeting #85	R1-165371

Nanjing, China, 23th - 27th May 2016

Agenda Item:    7.1.4

Source:	Convida Wireless

Title:	Discussion of Numerology for NR

Document for:	Discussion

# Introduction

During RAN1#84b, a scalable numerology had been discussed for supporting diverse use cases and deployment scenarios for NR. There are two agreed way forwards on numerology:

Agreements:

- For NR, it is necessary to support more than one values of subcarrier-spacing

  - Values of subcarrier-spacing are derived from a particular value of subcarrier-spacing multiplied by N where N is an integer

    - Alt.1: Subcarrier-spacing values include 15 kHz subcarrier-spacing (i.e., LTE based numerology)

    - Alt.2: Subcarrier-spacing values include 17.5 kHz subcarrier-spacing with uniform symbol duration including CP length

    - Alt.3: Subcarrier-spacing values include 17.06 kHz subcarrier-spacing with uniform symbol duration including CP length

    - Alt.4: Subcarrier-spacing values 21.33 kHz

    - Note: other alternatives are not precluded

    - FFS: exact value of a particular value and possible values of N

  - The values of possible subcarrier-spacing will be further narrowed-down in RAN1#85

- Companies are encouraged to provide detailed analysis and input the views in the following table.

Agreements:

- RAN1 will continue further study and conclude between following alternatives in the next meeting

- Alt. 1:

- The subcarrier spacing for the NR scalable numerology should scale as

- fsc = f0 * 2m

- where

  - f0 is FFS

  - m is an integer chosen from a set of possible values

- Alt. 2:

- The subcarrier spacing for the NR scalable numerology should scale as

- fsc = f0 * M

- where

  - f0 is FFS

  - M is an integer chosen from a set of possible positive values

- All companies are requested to analyze/evaluate following aspects

      - Realistic phase noise

      - How each alternative allows mixing different numerologies

- All companies are requested to propose exact values of

      - f0, m, and M

In this contribution, we provide our numerology analysis and performance simulation results for further discussions on the scalable numerology for NR.

# Discussions

During RAN1#84b, several issues related to the numerology base values and scaling methods were brought into discussions and it was called that further analysis with more details from companies for RAN1#85 numerology discussions and decision making [1].

## Numerology Design

The following parameters of a scalable numerology need to be properly designed for supporting different use cases and deployment scenarios (as shown in Table 1).

Subcarrier Spacing

- Influence of Doppler Spread: for example, a subcarrier spacing in each deployment scenario may be chosen to be at least 20 times of the Doppler spread

- Influence of phase noise: as described in [3], the phase noise power increases with increasing carrier frequency, and therefore subcarrier spacing selection needs also consider phase noise with high frequency band.

CP Length

- Delay spread: to mitigate ISI, the value of 10 times of delay spreading may be used as reference for deciding the total symbol length (symbol + CP).

TTI Length

- Various latency: if the number of symbols per TTI is kept the same, the TTI length may be scaled accordingly with scalable subcarrier spacing scaling to meet different latency requirements.

Table 1 Doppler and Delay Spreading for eMBB and URLL Use Cases

## Numerology Comparison

In Table 2, we compare the scalable numerology designs with different base values and 2^n scaling method.

Table 2 Numerology Comparison

# Performance results

In this section, we provide our initial BLER (Figure 1) and throughput (Figure 2) simulation results with the parameters listed in Table 3 for comparing performance of the 15kHz numerology and 17.5kHz numerology. As shown in the figures, the performances of different numerologies are very close to each other.

Table 3 Simulation Parameters

| Parameter | Assumed values |
| --- | --- |
| Bandwidth | 10 MHz |
| FFT size | 1024 |
| Numerology (NR) | 1: (LTE 14 symbols, 15 KHz) 2: (NR 16 symbols, 17.5 KHz) |
| Cyclic prefix length (samples) | 1: [80 72 72 72 72 72 72 80 72 72 72 72 72 72] 2:[96 96 96 96 96 96 96 96 96 96 96 96 96 96 96 96] |
| Sampling rate | 1: 15.36 MHz, 2: 17.92 MHz |
| Physical data channel | PDSCH Number of codewords: 1 Transmission layers: 1 Transport block sizes codeword: 14112 bits/TTI Code rate codeword : 0.46102 Channel coding: Turbo Modulation order: 16QAM |
| HARQ processor | 8 |
| Number of downlink resource allocation | 1: 600 REs 2: 525 REs |
| Tx-Rx antenna configuration | 1×2 |
| Channel model | EPA5, ETU70, ETU300, TDL-A Uma Long-delay profile [TR38.900] with Doppler 70 Hz |
| SNR (dB) | [6 8 10 12] |
| Channel estimation method | Ideal |
| Noise estimation method | Ideal |
| Demodulation method | MMSE (equivalent to MRC in 1×2 antenna setup) |

Figure 1 BLER performance

Figure 2 Throughput performance

# Conclusion

In this contribution, we provide the analysis and performance simulation results and discuss some implementation pros and cons that may have an impact on the scalable numerology design for NR, and we have the following observations.

Observation 1: Generally 2^n scaling with low n values produces good numerology values.

Observation 2: Performance of f0 = 15KHz and f0 = 17.5KHz are very close.

# References

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #84b.

- R1-163605, ‘‘WF on NR numerology,’’  NTT DOCOMO

- R1-162386, ‘‘Numerology for new radio interface,’’  Intel Corporation