3GPP TSG RAN WG1 Meeting #85			R1-165410

Nanjing, China, 23th - 27th May 2016

Agenda Item:	7.1.3.2

Source: 	CATR

Title: 	Initial link level simulation results of new multiple access scheme

Document for:	Discussion and Decision

Introduction

A study item of new radio access technology is agreed in RAN1#84bis[1], the study aims to develop an NR access technology to meet a broad range of use cases including enhanced mobile, broadband, massive MTC, critical MTC, and additional requirements defined during the RAN requirements study.

Last meeting in RAN1#84bis, different new multiple access schemes are observed such as MUSA, RSMA, SCMA, PDMA ,NCMA, NOMA and some other low code rate spreading and frequency domain spreading schemes,the following agreements are given:

- Non-orthogonal multiple access should be investigated for diversified NR usage scenarios and use cases

- At least for UL mMTC, autonomous/grant-free/contention based non-orthogonal multiple access should be studied

The detailed link level and system level simulation assumptions have been discussed in the associated email discussion, and the agreed simulation assumptions of link level are given in appendix.

In this contribution, we give initial uplink link level simulation results of some new multiple access schemes mentioned above, and also some observations are given based on the simulation results.

Discussion of New multiple access schemes

From all the new multiple access schemes observed in last meeting, we can see that some schemes have common features like code domain non orthogonal signal superposition based on legacy OFDMA, take Sparse Code Multiple Access (SCMA) [2] and Pattern defined multiple access (PDMA) as example, SCMA utilizes sparce spreading similar as low density spreading, as shown in figure 1, coded bits of a data stream are directly mapped to a codeword from a codebook built based on a multi-dimensional constellation.

Figure 1 Sparce code multiple access

As for PDMA, the principle is that a code is used to define sparse mapping from data to a group of resources. The code could be represented by a binary vector. The dimension of the vector equals to number of resource in a group. Each element in the vector corresponds to a resource in a resource group. A ‘1’ means that data shall be mapped to the corresponding resource. Actually, the number of ‘1’’s in the code is defined as its transmission diversity order. A code matrix is constructed by all codes sharing on the same resource group.

In our link level simulation, the transmitter side change the encoding operation from OFDMA to SCMA and PDMA modulater, and the receiver side adopt message passing algorithm (MPA) receiver algorithm. To be noted here is that we only give the uplink evaluation results.

Evaluation results and analysis

The detailed simulation assumtpions in our simulation are same as the agreed assumption shown in appendix, some other parameters which are not agreed are shown as following:

- The total allocated bandwith for transmission is 12 RB (2.16MHz)

- Target spectral efficiency is 1.5 bit/s/Hz

- The channel mode we used here is EPA channel model

- Equal average SNR (short-term variation remains)

- BS antenna configuration 2 Rx

- Given BLER level: 0.1 for 1 transmission

- Overloading factor: 150%

- Assuming six users are multiplexed on 4 REs

- Distributed resource allocation scheme for OFDMA, SCMA and PDMA

The BLER performance comparison between SCMA and OFDMA in uplink link level simulation is shown in Figure 2, from this figure, we can see that SCMA could get better performance than OFDMA.

Figure 2 performance comparison between OFDMA and SCMA

The BLER performance comparison between PDMA and OFDMA in uplink link level simulation is shown in Figure 3, from this figure, we can see that PDMA could get better performance than OFDMA.

Figure 3 performance comparison between OFDMA and PDMA

The performance gain of SCMA and PDMA both come from the code domain signal superposition,which allows superposition of multiple symbols from different users on each resource element (RE),and the advanced receiver can be adopted to differentiate multiple users on same RE. So based on our observation that code domain non orthogonal signal superposition could get performance gain compared with OFDMA, we propose that code domain spreading non orthognal multiple access scheme should be supported as new multiple access scheme.

Conclusion

In this contribution, we give our simulation results of some new multiple access scheme such as SCMA and PDMA, from the simulation results, we could see the code domain spreading non orthogonal multiple access could get performance gain than OFDMA, and is proposed to be supported as new mutliple access scheme.

References

- RP-160671 New SID Proposal: Study on New Radio Access Technology, NTT DOCOMO ,RAN#71 meeting,2016.3

- R1-162155	Sparse Code Multiple Access (SCMA) for 5G Radio Transmission	Huawei, HiSilicon

- R1-163383	Candidate Solution for New Multiple Access	CATT

Appendix

Table 1 Evaluation parameters – LLS for UL

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 2 GHz |
| Waveform | OFDM /SC-FDMA Other waveform is not precluded |
| Channel coding | LTE Turbo as start point, other coding schemes are not precluded. |
| Numerology | Same as Release 13 |
| System Bandwidth | 10 MHz |
| Total allocated bandwidth for transmission | Companies need to report this value. |
| Overhead | 2 DMRS symbols, no SRS, i.e., 144 available RE per RB for data transmission, or equivalent overhead |
| Target spectral efficiency | Proponents report per UE spectral efficiency and the number of UEs multiplexed if multi-UEs LLS is assumed |
| BS antenna configuration | 2/4 Rx as baseline 8Rx optional |
| UE antenna configuration | 1Tx |
| Transmission mode | TM1 (refer to TS36.213) |
| SNR distribution of Multiple UEs | Proponents report if single-user or multi-user LLS is used, and what SNR distribution is assumed. |
| Suggested SNR distribution of multiple UEs | Equal average SNR (short-term variation remains) Unequal average SNR (the SNR distribution is FFS, e.g., uniformly distributed within a range of 3dB, and proponents should report their assumption) |
| Propagation channel & UE velocity NOTE2 | TDL for in TR38.900 as mandatory EPA, EVA, ETU as optional 3km/h, 30km/h, 120km/h |
| Max number of HARQ transmission | 1, 4 |
| Given BLER level (to calculate sum throughput) | 0.1 for 1 transmission as starting point, other numbers not precluded, e.g., 0.01 for 1 transmission |
| Overloading factor (Optional, definition refers to R1-163881) | Some example values: 100%, 150%, 200%, 300% |

NOTE: Non-ideal effects (e.g., channel estimation, frequency offset) evaluation FFS.

NOTE2: Companies could choose the propagation model for bringing evaluations at RAN1#85 Nanjing meeting, but companies are expected to provide evaluations at least for the channels listed in the table by August 2016.