3GPP TSG RAN WG1 #85		R1-165490

Nanjing, China 23rd - 27th May 2016

Agenda item:	7.1.2

Source:	Samsung, Intel, Ericson, Qualcomm, Xinwei

Title:	Way-forward for TRP antenna modeling

Document for:	Discussion and Decision

# Background

In the agreed WF [1] in RAN1#84b and in the subsequent e-mail discussion [84b-13], evaluation parameters have been agreed for the TRP. The relevant agreements are summarized below.

Number of antenna elements:

- 700MHz: Up to 64 Tx /Rx antenna elements

- 4GHz: Up to 256 Tx /Rx antenna elements

- 30GHz: Up to 256 Tx /Rx antenna elements;

  - [84b-13 agreement] Continue discussion whether to increase the number of antenna elements.

- 70GHz: Up to 256 Tx /Rx antenna elements

  - [84b-13 agreement] Up to 1024 Tx /Rx antenna elements

TRP antenna panel model:

Figure 1 URPA (uniform rectangular panel array) antenna model

As shown in Figure 1, URPA comprises multiple (Mg∙Ng) antenna panels, on each of which, uniform rectangular array is placed, which comprises multiple antenna elements with (M, N, P) and (dH, dV) as defined in [2].

3-sector TRP antenna radiation pattern:

Table 1. 3-sector TRP antenna radiation pattern

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ''=-min12θ''-900θ3dB2,SLAV, θ3dB=650, SLAV=30 |
| Antenna element horizontal radiation pattern (dB) | AE,Hφ''=-min12φ''φ3dB2,Am, φ3dB=650, Am=30 |
| Combining method for 3D antenna element pattern (dB) | A''θ'',φ''=-min-AE,Vθ''+AE,Hφ'',Am |
| Maximum directional gain of an antenna element GE,max | 9 dBi |

# Proposals

Proposal 1: For evaluation, consider the following antenna configurations:

- At 4GHz:

- Dense urban and Urban macro:

–      Baseline: (M,N,P,Mg,Ng) = (8,16,2,1,1), (dV,dH) = (0.8, 0.5)λ.

–      Other configurations to be further discussed for each feature to be evaluated

- Indoor hotspot:

–      FFS

- At 30GHz:

- Dense urban and Urban macro:

–      Baseline: (M,N,P,Mg,Ng) = (4,8,2,2,2). (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = ([4.0], [8.0])λ.

–      Other configurations to be further discussed for each feature to be evaluated

- Indoor hotspot:

–      FFS

- At 70GHz:

- Dense urban:

–      Baseline: (M,N,P,Mg,Ng) = (8,16,2,4,4). (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = ([4.0], [8.0]) λ.

–      Other configurations to be further discussed for each feature to be evaluated

- Indoor hotspot:

–      FFS

Proposal 2: Consider the following a TXRU to antenna elements mapping as examples

- At 4GHz: the same as TR36.897

- At 30GHz and 70GHz:

- Option 1: a single TXRU is mapped per panel per polarization.

- Option 2: a single TXRU is mapped per panel per subarray per polarization,

  - E.g., where a subarray consists of consecutive M/2 vertical antennas and N/2 horizontal antennas with the same polarization.

  - Other subarray configurations are not precluded.

- Option 3: Fully connected TXRU mapping within a panel per polarization.

  - Other Fully connected TXRU mapping is not precluded.

Proposal 3: For evaluating multi beam based approaches at 30GHz and 70GHz, consider the following:

- TXRU to antenna mapping weights are adjustable and used to steer the panel beam direction in multi beam based approaches in time domain.

- Companies should describe TXRU mapping weights for the panel beams

- Consider the following as two examples for constructing mapping weights:

  - Example 1: Kroneker products of two oversampled DFT vectors, with O1 and O2 oversampling factors for the two dimensions.

    - Companies can choose O1, O2 and also beam scanning range reduction along with their results.

  - Example 2: Beam broadening weights

  - Other weights are not precluded. E.g, weights with finite resolution

# References

[1] R1-163885, WF on antenna modelling and related parameters for NR evaluations, Intel, Samsung, CATT

[2] TR36.897, Study on Elevation Beamforming/Full-Dimension (FD) Multiple Input Multiple Output (MIMO) for LTE (Release 13), v13.0.0.

[3] R1-164018, Analog/digital/hybrid beamforming for massive MIMO, Samsung