3GPP TSG-RAN WG1 #85	R1-165706

Nanjing, P.R. China, 23th – 27th May 2016

Source: 	Ericsson

Title:	Updates to System level observations

Agenda Item:	6.2.10.3

Document for:	Discussion and Decision

# Introduction

During RAN1#84, RAN1#84bis and RAN1#85 several companies provided system level evaluation results for latency reduction based on shortened TTI. In this TP, the observations obtained from the results provided according to the agreed format [1] are added to the latency reduction TR [2].

# Proposal

Incorporate the following in the text proposal in the Latency reduction TR.

---------------------------------- Start of Text Proposal ------------------------------------------------------

## 9.4	Findings from system evaluations on TTI reduction and reduced processing time

Findings from the results in Annex B for the scenario described in Annex A with FTP over TCP traffic are summarized below.

The results of all sources reveal that shortening TTI length can have great benefits in terms of user perceived throughput and latency. Source 10 shows gains with shortening TTI for all UEs at all loads and for all file sizes in case of negligible control overhead. In all sources with non-negligible control overhead benefits are not always present but certain situations are found to be favourable to observe gains with TTI shortening. With small file sizes (100kbit, 100kB), shortening TTI length improves significantly the performance of all UEs at low to medium load, while mostly good UEs and cell-center UEs benefit from TTI shortening at high load. With medium file size (500kB), cell-edge UEs have little benefit or may even experience performance degradation in all load points, but good UEs and cell center UEs benefit for most load points. With large file size (1MB), only cell center UEs benefit from shortening TTI length.

All sources with results for different core network delays show that gains when operating with shorter TTI reduce with larger core network delay but are still very high at low load.

Most sources obtained smaller gains from shortening TTI with increased resource utilization (RU) in most simulated scenarios. A less strong correlation between RU and gains with shortened TTI was observed by two sources.

In most cases, in the favourable situations for TTI shortening, gains are larger with TTI lengths shorter than slot based TTI. However, it was found by two sources that slot based TTI may provide gains in more scenarios than even shorter TTI. Source 10 shows better performance with 1 OS TTI than all other TTI lengths in all simulated scenarios in case of negligible control overhead and 0ms core network delay, while four other sources have more contrasted results when using non- negligible control overhead. One of them shows that performance gains with 1 OS TTI are smaller than with 2 OS TTI in most cases. Another source shows that the performance benefit of 1 OS TTI over 2 OS TTI is only visible for cell center UEs. Two other sources show that in most scenarios 1 OS TTI is less beneficial than 2 OS TTI. Three of them show performance gains with 2OS TTI and 3/4OS TTI over other TTI lengths in most cases.

The additional control overhead for shortened TTI and the additional DMRS overhead in case of shortened TTI with DMRS based transmission modes that are more frequent for 1 OS and 2 OS TTI are the limiting factors of very short TTI length. Three sources highlight that in average less than 2 UEs are scheduled in the same shortened TTI at low load in their simulations. One of these sources shows that the required size of the control channel region for scheduling shortened TTI is in average less than 2 CCEs at low load. Another of these sources shows that multiplexing more UEs in a shortened TTI, between 2 and 7 UEs depending on the RU, is beneficial as limiting the number of co-scheduled UEs in a short TTI to one implies performance degradation that increases with RU, i.e. with increasing number of active UEs at the eNB. Source 5 shows that shortened TTI gains increase with larger system BW as the relative control channel overhead for shortened TTI decreases. Two sources showed benefits of a dynamic control region for short TTI, where resources occupied by sPDCCH depends on the number of scheduled UEs per sTTI and their required aggregation level, over a statically reserved control region. Source 8 showed that DMRS optimization is beneficial for 2OS TTI in case of DMRS based demodulation. The studied DMRS optimization enables to bring the performance of 2 OS TTI at a higher level than the one of other TTI lengths at low-medium load.

Four sources evaluated the usage of shortened TTI for obtaining faster update of CSI measurements. One of the source shows minor performance improvement with fast CSI. Two of the sources show some benefit of using shortened TTI for including CSI update, especially at high load and for cell edge UEs. Some benefit of having fast CSI was observed in the fourth source mostly due to the effect of a smaller CQI report delay rather than a smaller CQI report period. Two sources evaluated TTI shortening with fast CSI for different UE speeds. One of the sources shows that short TTI benefits are higher when UE speed increases from 3 km/h to 120 km/h. The other source shows moderate impact on shortened TTI gain when UE speed increases from 3 km/h to 30km/h. Source 10 shows performance improvement with fast CSI, even with the original 14OS TTI length.

Most sources show that short TTI may have a benefit in terms of reduced average system resource utilization.

Source 8 shows benefit of shortened TTI with UL FTP traffic at low load. Larger gains were observed with 2OS TTI and optimization of DMRS overhead than with 3/4 OS TTI and slot based TTI.

Three sources evaluated fast UL access with shortened TTI and shows that the performance of short TTI improves with fast UL access. It is observed that performance difference among various TTI lengths is getting smaller with fast UL access. However, it should be noted that fast UL access has some drawbacks regarding the amount of reserved UL resources that was not considered in all of these studies.

Two sources show that gains of TTI shortening reduce if HARQ RTT and UL access delay cannot be linearly scaled with TTI length. Source 11 (R1-164948) showed that when a DL HARQ RTT of 14 sTTI and a UL access delay of 14 or 21sTTI is required for operating 2OS TTI, the performance gain of 2OS TTI over slot based TTI with DL HARQ RTT of 8 sTTI and UL TCP ACK delay of 12 sTTI is reduced (and can become negative) assuming a 500kB file size. Two sources show that the performance of 14OS TTI length with reduced processing time (i.e. with reduced HARQ RTT and UL access delay) comes close to the one of slot based TTI length. However, source 4 shows that if 14OS TTI follows the same TBS assumption as slot based TTI length, i.e. 14OS TTI UEs are served with correspondingly decreased coding rate, gains of 14OS TTI length with reduced processing time are smaller than the ones of other short TTI lengths. Source 11 (R1-165238) showed that more than half of the short TTI gain comes from processing time reduction, which has benefits in terms of reduced UL access delay and HARQ RTT. Source 2 (R1-164912) shows that slot based TTI with DL HARQ RTT of 4 sTTI and UL TCP ACK delay of 7 sTTI can achieve similar or better performance than 2OS TTI and 3/4 OS TTI with DL HARQ RTT of 8 sTTI and UL TCP ACK delay of 13 sTTI. No particular TBS limitation was assumed by source 2 for slot based TTI and reduced DL HARQ RTT of 4 sTTI.

Source 9 shows that gains of TTI shortening in the evaluated scenario depend on assumptions regarding how long a TCP connection is maintained, which is TCP implementation-specific. More specifically, large gains were observed when TCP slow start is assumed to occur for each file transfer of a UE while smaller gains were observed otherwise. 0ms core network delay was assumed in source 9. Furthermore, source 11 shows that gains of TTI shortening in the evaluated scenario depend on assumptions regarding the setting of the TCP parameter ssthresh (slow start threshold), which is TCP implementation-specific. More specifically, when TCP sender sets slow start threshold according to RFC5681, the gains from TTI shortening are reduced.

Source 5 shows that if additional subframe type (downlink transmission, GP, uplink transmission in a subframe) is introduced and applied for subframe #1, #3, #4 and #6, #7, #8, #9 with TDD DL/UL configuration #1 or #2 assuming those subframes are either MBSFN subframe or uplink subframe or special subframe with special subframe configuration 8 for legacy UEs, and short TTI with 2 OSs is used, considerable gains can be achieved if it is assumed that the same additional subframe type at the same subframe is used for all cells in the network. Three sources show that in the most favourable situation for TTI shortening, introducing short TTI in existing TDD UL/DL configuration without additional subframe type can lead to large user perceived throughput improvement.  Three sources show that in the most favourite situation for TTI shortening, introducing short TTI with additional subframe type within existing UL/DL configuration can lead to improved user perceived throughput compared to introducing short TTI without additional subframe type within existing UL/DL configurations, especially for short TTI length as 2OS. Two sources show that introducing short TTI with additional subframe type within existing UL/DL configuration have marginal user perceived throughput gain compared to introducing short TTI without additional subframe type within existing UL/DL configurations for slot based TTI. Source 4 shows that significant performance loss in 5th percentile user throughput with shorter TTI length like 2OS both with and without additional subframe type, and performance gain with shorter TTI decreases with larger file sizes. In all sources, it is assumed that the same additional subframe type at the same subframe is used for all cells in the network. At least shown by two sources, under a given TDD UL/DL subframe configuration, introducing additional subframe type for a new UE can further reduce the U-plane latency for TDD compared to the case without introducing additional subframe types in the evaluated scenarios.

The above performance gains of introducing additional subframe type in terms of user perceived throughput is observed based on the assumption of no legacy UE exists in the system. There is difficulty to schedule legacy UEs in the additional subframes and therefore the performance of legacy UEs is degraded. How much the degradation for  legacy UEs is has not been evaluated in the study item.

Performance impact on legacy UEs in a carrier using shorter TTI has not been evaluated for all considered scenarios.

----------------------------------------------- End of Text Proposal ------------------------------------------------

# References

- R1-161422, Way forward on simulation results and methodology, Ericsson, RAN1#84, February 2016

- TR36.881v0.6.0, Study on latency reduction techniques for LTE