Annex C:
Link-level evaluation results

In this Annex the link results are provided for sPDSCH, sPDCCH, sPUSCH and sPUCCH. The results are generated by the simulation assumptions available in Annex A if nothing else is described.

## C1	Link-level evaluation sPDSCH

## C1.1	CRS based demodulation

Table C1.1-1 EPA, 3 km/h, rank 1

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 1.02 dB 2.55 Mbps | 0.65 dB 2.86 Mbps |  | 0.59 dB 3.01 Mbps | 0.49 dB 3.06 Mbps | 0.52 dB 3.15 Mbps |
| 16QAM r3/4 | 13.72 dB 12.18 Mbps | 13.50 dB 13.22 Mbps |  | 13.87 dB 13.74 Mbps | 13.71 dB 13.91 Mbps | 13.62 dB 14.24 Mbps |
| 64QAM r5/6 | 22.50 dB 20.44 Mbps | 22.02 dB 22.10 Mbps |  | 22.14 dB 22.93 Mbps | 22.72 dB 23.21 Mbps | 22.38 dB 23.74 Mbps |
| R1-163313 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 4.59 dB 3.73 Mbps |  | 5.18 dB 3.98 Mbps | 3.8 dB 3.88 Mbps | 3.56 dB 3.88 Mbps |
| 16QAM r3/4 |  | 15.52 dB 17.28 Mbps |  | 16.55 dB 18.30 Mbps | 14.86 dB 17.49 Mbps | 14.31 dB 17.58 Mbps |
| 64QAM r5/6 |  | 22.94 dB 28.9 Mbps |  | 23.59 dB 30.46 Mbps | 22.03 dB 29.45 Mbps | 21.24 dB 28.97 Mbps |
| R1-163399/ Source 13 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS included in OFDM symbols within 1 msec before the scheduled TTI is used for channel estimation. |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 4.05 dB 3.79 Mbps |  | 4.53 dB 3.91 Mbps | 3.93 dB 3.90 Mbps | 3.56 dB 3.88 Mbps |
| 16QAM r3/4 |  | 15.07 dB 17.32 Mbps |  | 15.29 dB 17.77 Mbps | 14.85 dB 17.44 Mbps | 14.31 dB 17.58 Mbps |
| 64QAM r5/6 |  | 21.97 dB 29.41 Mbps |  | 22.73 dB 29.92 Mbps | 21.91 dB 29.52 Mbps | 21.24 dB 28.97 Mbps |
| R1-163399/ Source 13 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS from the first OFDM symbol the legacy subframe boundary including the scheduled sTTI to the scheduled TTI is used for channel estimation |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 3.41 dB 3.73 Mbps |  | 3.41 dB 3.95 Mbps | 3.82 dB 3.88 Mbps | 3.56 dB 3.88 Mbps |
| 16QAM r3/4 |  | 14.46 dB 17.42 Mbps |  | 14.43 dB 18.00 Mbps | 14.85 dB 17.44 Mbps | 14.31 dB 17.58 Mbps |
| 64QAM r5/6 |  | 21.37 dB 29.20 Mbps |  | 21.54 dB 30.32 Mbps | 21.91 dB 29.52 Mbps | 21.24 dB 28.97 Mbps |
| R1-163399/ Source 13 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS Scheme if CRS is present within the scheduled TTI, only the CRS is used for channel estimation. Otherwise, CRS within one TTI before the scheduled TTI. |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.4 dB 3.96 Mbps |  |  | 1.3 dB 3.96 Mbps | 0.7 dB 3.96 Mbps |
| 16QAM r3/4 |  | 13.3 dB 17.82 Mbps |  |  | 13.5 dB 17.82 Mbps | 13 dB 17.82 Mbps |
| 64QAM r5/6 |  | 20.3 dB 29.7 Mbps |  |  | 21.1 dB 29.7 Mbps | 20.4 dB 29.7 Mbps |
| R1-163264 / Source 11 50 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | -2.21 dB 3.78 Mbps | -2.29 dB 3.82 Mbps |  | -2.12 dB 3.94 Mbps | -2.35 dB 3.91 Mbps |
| 16QAM r3/4 |  | 9.57 dB 17.37 Mbps | 9.36 dB 17.5 Mbps |  | 9.83 dB 17.64 Mbps | 9.95 dB 17.84 Mbps |
| 64QAM r5/6 |  | 16.91 dB 29.24 Mbps | 17.47 dB 29.21 Mbps |  | 17.21 dB 28.33 Mbps | 17.58 dB 29.35 Mbps |
| R1-164164 / Source 7 50 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |

Table C1.1-2 EPA, 3 km/h, rank 2

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 6.81 dB 5.09 Mbps | 6.32 dB 5.72 Mbps |  | 6.27 dB 6.03 Mbps | 6.17 dB 6.13 Mbps | 6.16 dB 6.30 Mbps |
| 16QAM r3/4 | 23.13 dB 24.36 Mbps | 22.79 dB 26.44 Mbps |  | 23.31 dB 27.48 Mbps | 23.07 dB 27.82 Mbps | 22.93 dB 28.48 Mbps |
| 64QAM r5/6 | 32.81 dB 40.88 Mbps | 32.15 dB 44.20 Mbps |  | 32.39 dB 45.86 Mbps | 33.01 dB 46.41 Mbps | 32.53 dB 47.49 Mbps |
| R1-163313 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 | 7dB 2.7518Mbps | 6.8dB 3.9004Mbs | -- -- | -- -- | 6.8dB 4.4793Mbps | 6.25dB 3.9168Mbps |
| 16QAM r3/4 | 23.5dB 14.541Mbps | 23.2dB 18.631Mbps | -- -- | -- -- | 23.1dB 20.47Mbps | 23.2dB 17.776Mbps |
| 64QAM r5/6 | 34.5dB 24.64Mbps | 33.8dB 31.254Mbps | -- -- | -- -- | 35.5dB 34.175Mbps | 34dB 29.6568Mbps |
| R1-163511 / Source 10 25 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |

Table C1.1-3 EPA, 3 km/h, rank 4

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 3.55 dB 10.21 Mbps | 3.44 dB 11.25 Mbps |  | 3.42 dB 11.77 Mbps | 3.32 dB 11.94 Mbps | 3.46 dB 12.25 Mbps |
| 16QAM r3/4 | 23.47 dB 47.42 Mbps | 23.22 dB 51.36 Mbps |  | 23.53 dB 53.33 Mbps | 23.77 dB 53.98 Mbps | 23.78 dB 55.26 Mbps |
| 64QAM r5/6 | 35.46 dB 79.30 Mbps | 34.78 dB 85.73 Mbps |  | 34.93 dB 88.95 Mbps | 34.75 dB 90.02 Mbps | 35.10 dB 92.13 Mbps |
| R1-163313 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |

Table C1.1-4 EPA, 60 km/h, rank 1

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 1.12 dB 2.55 Mbps | 0.69 dB 2.86 Mbps |  | 0.50 dB 3.01 Mbps | 0.38 dB 3.06 Mbps | 0.15 dB 3.15 Mbps |
| 16QAM r3/4 | 14.21 dB 12.18 Mbps | 13.58 dB 13.22 Mbps |  | 13.29 dB 13.74 Mbps | 13.10 dB 13.91 Mbps | 13.33 dB 14.24 Mbps |
| 64QAM r5/6 | 22.98 dB 20.44 Mbs | 22.04 dB 22.10 Mbps |  | 21.53 dB 22.93 Mbps | 22.19 dB 23.21 Mbps | 22.21 dB 23.74 Mbps |
| R1-163313 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |

Table C1.1-5 EPA, 60 km/h, rank 2

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 6.66 dB 5.09 Mbps | 6.10 dB 5.72 Mbps |  | 5.86 dB 6.03 Mbps | 5.67 dB 6.13 Mbps | 5.45 dB 6.30 Mbps |
| 16QAM r3/4 | 23.14 dB 24.36 Mbps | 22.48 dB 26.44 Mbps |  | 22.68 dB 27.48 Mbps | 22.57 dB 27.82 Mbps | 23.18 dB 28.48 Mbps |
| 64QAM r5/6 | >36 dB | 35.03 dB 44.20 Mbps |  | 32.76 dB 45.86 Mbps | 34.26 dB 46.41 Mbps | 34.68 dB 47.49 Mbps |
| R1-163313 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 | 7.5dB 2.7518Mbps | 7.5dB 3.9004Mbs | -- | -- | 7.5dB 4.4793Mbps | 6.25dB 3.9168Mbps |
| 16QAM r3/4 | >25dB -- | >25dB -- | -- -- | -- -- | >25dB -- | >25dB -- |
| 64QAM r5/6 | -- | -- | -- | -- | -- | -- |
| R1-163511 / Source 10 25 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |

Table C1.1-6 EVA, 3 km/h, rank 1

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | -0.73 dB 2.55 Mbps | -0.99 dB 2.86 Mbps |  | -0.98 dB 3.01 Mbps | -1.06 dB 3.06 Mbps | -1.07 dB 3.15 Mbps |
| 16QAM r3/4 | 12.02 dB 12.18 Mbps | 11.92 dB 13.22 Mbps |  | 12.16 dB 13.74 Mbps | 12.02 dB 13.91 Mbps | 12.06 dB 14.24 Mbps |
| 64QAM r5/6 | 20.76 dB 20.44 Mbps | 20.39 dB 22.10 Mbps |  | 20.45 dB 22.93 Mbps | 20.78 dB 23.21 Mbps | 20.67 dB 23.74 Mbps |
| R1-163313 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |

Table C1.1-7 EVA, 3 km/h, rank 2

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 3.91 dB 5.09 Mbps | 3.52 dB 5.72 Mbps |  | 3.56 dB 6.03 Mbps | 3.45 dB 6.13 Mbps | 3.43 dB 6.30 Mbps |
| 16QAM r3/4 | 20.13 dB 24.36 Mbps | 20.00 dB 26.44 Mbps |  | 20.36 dB 27.48 Mbps | 20.08 dB 27.82 Mbps | 20.08 dB 28.48 Mbps |
| 64QAM r5/6 | 31.99 dB 40.88 Mbps | 31.04 dB 44.20 Mbps |  | 31.25 dB 45.86 Mbps | 31.70 dB 46.41 Mbps | 31.38 dB 47.49 Mbps |
| R1-163313 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |

Table C1.1-8 EVA, 3 km/h, rank 4

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 2.94 dB 10.21 Mbps | 2.90 dB 11.25 Mbps |  | 2.90 dB 11.77 Mbps | 2.80 dB 11.94 Mbps | 2.95 dB 12.25 Mbps |
| 16QAM r3/4 | 22.17 dB 47.42 Mbps | 22.06 dB 51.35 Mbps |  | 22.16 dB 53.33 Mbps | 22.16 dB 53.98 Mbps | 22.10 dB 55.26 Mbps |
| 64QAM r5/6 | >36 | >36 |  | >36 | >36 | >36 |
| R1-163313 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |

Table C1.1-9 EVA, 60 km/h, rank 1

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | -0.06 dB 2.55 Mbps | -0.40 dB 2.86 Mbps |  | -0.55 dB 3.01 Mbps | -0.69 dB 3.06 Mbps | -0.84 dB 3.15 Mbps |
| 16QAM r3/4 | 12.74 dB 12.18 Mbps | 12.41 dB 13.22 Mbps |  | 12.27 dB 13.74 Mbps | 12.09 dB 13.91 Mbps | 12.15 dB 14.24 Mbps |
| 64QAM r5/6 | 22.27 dB 20.44 Mbps | 21.21 dB 22.10 Mbps |  | 20.70 dB 22.93 Mbps | 21.16 dB 23.21 Mbps | 20.83 dB 23.74 Mbps |
| R1-163313 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 3.79 dB 3.74 Mbps |  | 4.03 dB 4.08 Mbps | 2.35 dB 3.80 Mbps | 1.79 dB 3.85 Mbps |
| 16QAM r3/4 |  | 14.61 dB 17.09 Mbps |  | 15.2 dB 17.75 Mbps | 13.41 dB 16.88 Mbps | 13.03 dB 16.78 Mbps |
| 64QAM r5/6 |  | 21.95 dB 29.22 Mbps |  | 22.79 dB 30.08 Mbps | 20.65 dB 28.66 Mbps | 20.28 dB 28.84 Mbps |
| R1-163399 / Source 13 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS included in OFDM symbols within 1 msec before the scheduled TTI is used for channel estimation. |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.72 dB 3.66 Mbps |  | 3.08 dB 3.79 Mbps | 2.17 dB 3.84 Mbps | 1.79 dB 3.85 Mbps |
| 16QAM r3/4 |  | 14.02 dB 17.56 Mbps |  | 14.35 dB 17.51 Mbps | 13.65 dB 17.26 Mbps | 13.03 dB 16.78 Mbps |
| 64QAM r5/6 |  | 21.78 dB 29.01 Mbps |  | 22.76 dB 30.70 Mbps | 20.71 dB 28.50 Mbps | 20.28 dB 28.84 Mbps |
| R1-163399 / Source 13 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS from the first OFDM symbol the legacy subframe boundary including the scheduled sTTI to the scheduled TTI is used for channel estimation |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.93 dB 3.78 Mbps |  | 2.09 dB 3.99 Mbps | 2.17 dB 3.84 Mbps | 1.79 dB 3.85 Mbps |
| 16QAM r3/4 |  | 13.12 dB 16.85 Mbps |  | 13.65 dB 17.26 Mbps | 13.65 dB 17.26 Mbps | 13.03 dB 16.78 Mbps |
| 64QAM r5/6 |  | 21.67 dB 29.01 Mbps |  | 22.64 dB 30.84 Mbps | 20.71 dB 28.50 Mbps | 20.28 dB 28.84 Mbps |
| R1-163399 / Source 13 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS Scheme if CRS is present within the scheduled TTI, only the CRS is used for channel estimation. Otherwise, CRS within one TTI before the scheduled TTI. |  |  |  |  |  |  |

Table C1.1-10 EVA, 60 km/h, rank 2

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 4.56 dB 5.09 Mbps | 4.21 dB 5.72 Mbps |  | 4.10 dB 6.03 Mbps | 3.97 dB 6.13 Mbps | 3.79 dB 6.30 Mbps |
| 16QAM r3/4 | 21.11 dB 24.36 Mbps | 20.59 dB 26.44 Mbps |  | 20.70 dB 27.48 Mbps | 20.42 dB 27.82 Mbps | 20.47 dB 28.48 Mbps |
| 64QAM r5/6 | >36 | 35.40 dB 44.20 Mbps |  | 32.29 dB 45.86 Mbps | 33.55 dB 46.41 Mbps | 33.08 dB 47.49 Mbps |
| R1-163313 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |

Table C1.1-11 ETU, 3 km/h, rank 1

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | -0.71 dB 2.55 Mbps | -1.04 dB 2.86 Mbps |  | -1.06 dB 3.01 Mbps | -1.13 dB 3.06 Mbps | -1.18 dB 3.15 Mbps |
| 16QAM r3/4 | 12.08 dB 12.18 Mbps | 11.90 dB 13.22 Mbps |  | 12.19 dB 13.74 Mbps | 12.00 dB 13.91 Mbps | 12.01 dB 14.24 Mbps |
| 64QAM r5/6 | 23.52 dB 20.44 Mbps | 22.79 dB 22.10 Mbps |  | 22.93 dB 22.93 Mbps | 23.62 dB 23.21 Mbps | 23.30 dB 23.74 Mbps |
| R1-163313 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.26 dB 3.66 Mbps |  | 3.42 dB 3.91 Mbps | 1.71 dB 3.75 Mbps | 1.3 dB 3.64 Mbps |
| 16QAM r3/4 |  | 14.46 dB 17.02 Mbps |  | 15.13 dB 17.82 Mbps | 13.08 dB 16.40 Mbps | 12.9 dB 16.58 Mbps |
| 64QAM r5/6 |  | 21.66 dB 28.57 Mbps |  | 22.36 dB 30.81 Mbps | 20.59 dB 28.43 Mbps | 20.14 dB 29.14 Mbps |
| R1-163399 / Source 13 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS included in OFDM symbols within 1 msec before the scheduled TTI is used for channel estimation. |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.26 dB 3.66 Mbps |  | 2.61 dB 3.70 Mbps | 1.77 dB 3.79 Mbps | 1.3 dB 3.64 Mbps |
| 16QAM r3/4 |  | 13.66 dB 17.09 Mbps |  | 14.16 dB 17.79 Mbps | 13.02 dB 16.28 Mbps | 12.9 dB 16.58 Mbps |
| 64QAM r5/6 |  | 20.59 dB 27.47 Mbps |  | 21.36 dB 28.92 Mbps | 20.5 dB 28.08 Mbps | 20.14 dB 29.14 Mbps |
| R1-163399 / Source 13 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS from the first OFDM symbol the legacy subframe boundary including the scheduled sTTI to the scheduled TTI is used for channel estimation |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.37 dB 3.53 Mbps |  | 1.48 dB 3.78 Mbps | 1.77 dB 3.79 Mbps | 1.3 dB 3.64 Mbps |
| 16QAM r3/4 |  | 12.67 dB 16.70 Mbps |  | 12.64 dB 17.00 Mbps | 13.02 dB 16.28 Mbps | 12.9 dB 16.58 Mbps |
| 64QAM r5/6 |  | 19.94 dB 29.30 Mbps |  | 20.06 dB 30.33 Mbps | 20.5 dB 28.08 Mbps | 20.14 dB 29.14 Mbps |
| R1-163399 / Source 13 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS Scheme if CRS is present within the scheduled TTI, only the CRS is used for channel estimation. Otherwise, CRS within one TTI before the scheduled TTI. |  |  |  |  |  |  |

Table C1.1-12 ETU, 3 km/h, rank 2

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 4.20 dB 5.09 Mbps | 3.80 dB 5.72 Mbps |  | 3.79 dB 6.03 Mbps | 3.65 dB 6.13 Mbps | 3.60 dB 6.30 Mbps |
| 16QAM r3/4 | 23.28 dB 24.36 Mbps | 22.83 dB 26.44 Mbps |  | 23.62 dB 27.48 Mbps | 22.99 dB 27.82 Mbps | 23.00 dB 28.48 Mbps |
| 64QAM r5/6 | >36 | >36 |  | >36 | >36 | >36 |
| R1-163313 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 | 4.4dB 2.7518Mbps | 4.3dB 3.9004Mbps | -- -- | -- -- | 4.2dB 4.4793Mbps | 3.75dB 3.9168Mbps |
| 16QAM r3/4 | 20.2dB 14.541Mbps | 20dB 18.631Mbps | -- -- | -- -- | 19.9dB 20.47Mbps | 19.9dB 17.776Mbps |
| 64QAM r5/6 | 29.8dB 24.64Mbps | 29.8dB 31.254Mbps | -- -- | -- -- | 31dB 34.175Mbps | 30.5dB 29.6568Mbps |
| R1-163511 / Source 10 25 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |

Table C1.1-13 ETU, 3 km/h, rank 4

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 3.11 dB 10.21 Mbps | 3.01 dB 11.25 Mbps |  | 2.99 dB 11.77 Mbps | 2.88 dB 11.94 Mbps | 3.01 dB 12.25 Mbps |
| 16QAM r3/4 | 27.26 dB 47.42 Mbps | 27.08 dB 51.36 Mbs |  | 27.99 dB 53.33 | 28.62 dB 53.98 Mbps | 29.97 dB 55.26 Mbps |
| 64QAM r5/6 | >36 | >36 |  | >36 | >36 | >36 |
| R1-163313 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |

Table C1.1-14 ETU, 60 km/h, rank 1

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 0.08 dB 2.55 Mbps | -0.30 dB 2.86 Mbps |  | -0.47 dB 3.01 Mbps | -0.60 dB 3.06 Mbps | -0.77 dB 3.15 Mbps |
| 16QAM r3/4 | 12.88 dB 12.18 Mbps | 12.57 dB 13.22 Mbps |  | 12.46 dB 13.74 Mbps | 12.22 dB 13.91 Mbps | 12.21 dB 14.24 Mbps |
| 64QAM r5/6 | 24.59 dB 20.44 Mbps | 23.00 dB 22.10 Mbps |  | 22.44 dB 22.93 Mbps | 23.09 dB 23.21 Mbps | 22.27 dB 23.74 Mbps |
| R1-163313 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.84 dB 3.51 Mbps |  | 3.34 dB 3.88 Mbps | 1.24 dB 3.62 Mbps | 1.75 dB 3.77 Mbps |
| 16QAM r3/4 |  | 14.42 dB 17.06 Mbps |  | 14.97 dB 17.63 Mbps | 13.1 dB 16.47 Mbps | 12.85 dB 16.57 Mbps |
| 64QAM r5/6 |  | 21.92 dB 29.14 Mbps |  | 22.56 dB 30.22 Mbps | 20.65 dB 28.47 Mbps | 20.24 dB 28.86 Mbps |
| R1-163399 / Source 13 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS included in OFDM symbols within 1 msec before the scheduled TTI is used for channel estimation. |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.28 dB 3.67 Mbps |  | 2.65 dB 3.78 Mbps | 1.58 dB 3.64 Mbps | 1.75 dB 3.77 Mbps |
| 16QAM r3/4 |  | 13.54 dB 16.85 Mbps |  | 14.24 dB 17.59 Mbps | 13.03 dB 16.36 Mbps | 12.85 dB 16.57 Mbps |
| 64QAM r5/6 |  | 21.42 dB 28.34 Mbps |  | 22.23 dB 30.60 Mbps | 20.54 dB 28.45 Mbps | 20.24 dB 28.86 Mbps |
| R1-163399 / Source 13 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS from the first OFDM symbol the legacy subframe boundary including the scheduled sTTI to the scheduled TTI is used for channel estimation |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.5 dB 4.16 Mbps |  | 1.35 dB 3.68 Mbps | 1.58 dB 3.64 Mbps | 1.75 dB 3.38 Mbps |
| 16QAM r3/4 |  | 12.81 dB 19.24 Mbps |  | 13.06 dB 17.22 Mbps | 13.03 dB 16.36 Mbps | 12.85 dB 16.06 Mbps |
| 64QAM r5/6 |  | 21.23 dB 27.81 Mbps |  | 21.83 dB 30.60 Mbps | 20.54 dB 28.45 Mbps | 20.24 dB 28.84 Mbps |
| R1-163399 / Source 13 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS Scheme if CRS is present within the scheduled TTI, only the CRS is used for channel estimation. Otherwise, CRS within one TTI before the scheduled TTI. |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0 dB 3.96 Mbps |  |  | -0.2 dB 3.96 Mbps | -0.4 dB 3.96 Mbps |
| 16QAM r3/4 |  | 14.5 dB 17.74 Mbps |  |  | 13.5 dB 17.85 Mbps | 13.3 dB 17.82 Mbps |
| 64QAM r5/6 |  |  |  |  | 21.8 dB 29.73 Mbps | 21.5 dB 29.7 Mbps |
| R1-163264 / Source 11 50 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 |  | TTI length=3 |  | TTI length=4 |  | TTI length=7 |  | TTI length=14 |  |
| QPSK r1/3 |  | -0.57 dB 3.69 Mbps |  | -0.73dB 3.68 Mbps |  |  |  | -0.72 dB 3.78Mbps |  | -1.34 dB 3.72 Mbps |  |
| 16QAM r3/4 |  | 12.35 dB 17.31 Mbps |  | 12.24 dB 17.51Mbps |  |  |  | 12.52 dB 17.30 Mbps |  | 11.72 dB 17.48 Mbps |  |
| 64QAM r5/6 |  | 23.77 dB 29.63 Mbps |  | 23.74 dB 29.44 Mbps |  |  |  | 22.90 dB 28.44 Mbps |  | 20.50 dB 28.98 Mbps |  |
| R1—164164 / Source 7 50 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  |  |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  | 22.3 dB 34.65 Mbps |  |  |  | 21.2 dB 35.45 Mbps |
| R1-163714 / Source 5 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS Scheme for sTTI, the most recent four CRS symbols are used for channel estimation for 1ms TTI, CRS symbols in current subframe are used for channel estimation |  |  |  |  |  |  |

Table C1.1-15 ETU, 60 km/h, rank 2

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 4.62 dB 5.09 Mbps | 4.25 dB 5.72 Mbps |  | 4.13 dB 6.03 Mbps | 4.00 dB 6.13 Mbps | 3.72 dB 6.30 Mbps |
| 16QAM r3/4 | 22.71 dB 24.36 Mbps | 21.92 dB 26.44 Mbps |  | 22.02 dB 27.48 Mbps | 21.55 dB 27.82 Mbps | 21.37 dB 28.48 Mbps |
| 64QAM r5/6 | >36 | >36 |  | >36 | >36 | >36 |
| R1-163313 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 | 4.8dB 2.7518Mbps | 4.8dB 3.9004Mbps | -- -- | -- -- | 4.7dB 4.4793Mbps | 3.9dB 3.9168Mbps |
| 16QAM r3/4 | >25dB -- | >25dB -- | -- -- | -- -- | >25dB -- | >25dB -- |
| 64QAM r5/6 | -- | -- | -- | -- | -- | -- |
| R1-163511 / Source 10 25 PRB, 10 MHz system bandwidth, |  |  |  |  |  |  |

## C1.2	DMRS based demodulation

Table C1.2-1 EPA, 3 km/h, rank 1

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 2.04 dB 1.99 Mbps |  | 1.99 dB 2.58 Mbps | 1.91 dB 2.78 Mbps | 1.30 dB 2.86 Mbps |
| 16QAM r3/4 |  | 13.95 dB 9.33 Mbps |  | 13.88 dB 11.79 Mbps | 14.21 dB 12.61 Mbps | 13.95 dB 12.94 Mbps |
| 64QAM r5/6 |  | 23.18 dB 15.62 Mbps |  | 23.06 dB 19.69 Mbps | 22.98 dB 21.05 Mbps | 22.83 dB 21.58 Mbps |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.43 dB 3.05 Mbps |  | 2.53 dB 3.28 Mbps | 2.58 dB 3.44 Mbps | 2.68 dB 3.41 Mbps |
| 16QAM r3/4 |  | 13.81 dB 14.53 Mbps |  | 13.77 dB 15.16 Mbps | 13.81 dB 15.64 Mbps | 14.02 dB 15.84 Mbps |
| 64QAM r5/6 |  | 21.2 dB 24.11 Mbps |  | 20.77 dB 24.97 Mbps | 20.85 dB 26.10 Mbps | 21.19 dB 26.09 Mbps |
| R1-163400 / Source 13 50 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.01 dB 2.43 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 14.00 dB 11.28 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 23.63 dB 18.86 Mbps |  |  |  |  |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.25 dB 1.99 Mbps |  | 1.16 dB 2.58 Mbps | 1.04 dB 2.78 Mbps | 0.66 dB 2.86 Mbps |
| 16QAM r3/4 |  | 13.58 dB 9.33 Mbps |  | 13.56 dB 11.79 Mbps | 13.88 dB 12.61 Mbps | 13.70 dB 12.94 Mbps |
| 64QAM r5/6 |  | 22.62 dB 15.62 Mbps |  | 22.55 dB 19.69 Mbps | 22.45 dB 21.05 Mbps | 22.41 dB 21.58 Mbps |
| R1-163410 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI PRBs bundling over three 3PBRs |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.20 dB 2.43 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 13.63 dB 11.28 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 22.94 dB 18.86 Mbps |  |  |  |  |
| R1-163410/ Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI PRBs bundling over three 3PBRs DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.6 dB 2.9 Mbps |  |  | 1.5 dB 3.6 Mbps | 0.7 dB 3.6 Mbps |
| 16QAM r3/4 |  | 13.4 dB 13 Mbps |  |  | 13.7 dB 16.2 Mbps | 13 dB 16.2 Mbps |
| 64QAM r5/6 |  | 21.5 dB 21.6 Mbps |  |  | 23.8 dB 27 Mbps | 20.2 dB 27 Mbps |
| R1-163264 / Source 11 50 PRB, 10 MHz system bandwidth DM-RS available in every TTI DMRS density 6RE per PRB in sTTI/12RE per PRB in TTI Channel estimation performed with 3PRB granularity |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.5 dB 3.42 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 13.4 dB 15.38 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 21.5 dB 25.64 Mbps |  |  |  |  |
| R1-163264 / Source 11 50 PRB, 10 MHz system bandwidth DMRS density 6RE per PRB in sTTI /12RE per PRB in TTI DMRS only available in every second TTI Channel estimation performed with 3PRB granularity |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.8 dB 3.24 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 13.8 dB 14.58 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 26.8 dB 24.29 Mbps |  |  |  |  |
| R1-163264 / Source 11 50 PRB, 10 MHz system bandwidth Bundled pattern over 3 PRBs (total 12RE per 3PRB) DM-RS available in every TTI Channel estimation performed with 3PRB granularity |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.8 dB 3.6 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 13.8 dB 16.2 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 25.8 dB 27.03 Mbps |  |  |  |  |
| R1-163264 / Source 11 50 PRB, 10 MHz system bandwidth Bundled pattern over 3 PRBs (total 12RE per 3PRB) DM-RS only available in every second TTI Channel estimation performed with 3PRB granularity |  |  |  |  |  |  |
| SNR at 10% BLER Throughput |  | TTI length=1 | TTI length=2 |  | TTI length=3 |  | TTI length=4 |  | TTI length=7 |  | TTI length=14 |
| QPSK r1/3 |  |  | -0.85 dB 2.68 Mbps |  | -1.15dB 3.09 Mbps |  |  |  | -0.98 dB 3.42 Mbps |  | -1.50 dB 3.57 Mbps |
| 16QAM r3/4 |  |  | 10.03 dB 12.65 Mbps |  | 10.13 dB 14.35 Mbps |  |  |  | 10.51 dB 15.92 Mbps |  | 10.24 dB 15.69 Mbps |
| 64QAM r5/6 |  |  | 17.91 dB 21.25 Mbps |  | 18.16 dB 24.08 Mbps |  |  |  | 18.94 dB 26.66 Mbps |  | 18.74 dB 27.23 Mbps |
| R1-164164 / Source 7 50 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | -0.1 dB 2.03 Mbps |  |  |  | 0.0 dB 2.13 Mbps |
| 16QAM r3/4 |  | 12.2 dB 9.59 Mbps |  |  |  | 12.9 dB 9.67 Mbps |
| 64QAM r5/6 |  | 20.9 dB 16.07 Mbps |  |  |  | 20.9 dB 16.12 Mbps |
| R1-163714 / Source 5 30 PRB, 10 MHz system bandwidth RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.1 dB (DMRS pattern1: 4 RE/RB) 0.3 dB (DMRS pattern2: 6 RE/RB) 2.12 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 11.1 dB (DMRS pattern1: 4 RE/RB) 12.4 dB (DMRS pattern2: 6 RE/RB) 10.05 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 18.5 dB (DMRS pattern1: 4 RE/RB) 20.7 dB (DMRS pattern2: 6 RE/RB) 16.86 Mbps |  |  |  |  |
| R1-164062 / Source 5 30 PRB, 10 MHz system bandwidth. RX imperfections and 6% TX EVM, 3 PRBs bundling. DMRS pattern1 represents DMRS CDM in frequency domain, DMRS pattern2 represents DMRS CDM in time domain. Note that code rate for DMRS pattern1 will be lower due to less RS overhead. |  |  |  |  |  |  |

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  |  | -0.5 dB (DMRS: 2RE/RB) -1.3dB (DMRS: 4RE/RB) -1.6 dB (DMRS: 6RE/RB) -1.3 dB (DMRS: 8RE/RB) 1.90 Mbps |  | -0.4 dB (DMRS: 2RE/RB) -1.3 dB (DMRS: 4RE/RB) -1.7 dB (DMRS: 6RE/RB) -1.7 dB (DMRS: 8RE/RB) 2.26 Mbps |  |
| 16QAM r3/4 |  |  | 8.3 dB (DMRS: 2RE/RB) 8.3 dB (DMRS: 4RE/RB) 8.8 dB (DMRS: 6RE/RB) 9.5 dB (DMRS: 8RE/RB) 8.40 Mbps |  | 9.8 dB (DMRS: 2RE/RB) 9.3 dB (DMRS: 4RE/RB) 9.4 dB (DMRS: 6RE/RB) 9.7 dB (DMRS: 8RE/RB) 10.32 Mbps |  |
| 64QAM r5/6 |  |  | 14.2 dB (DMRS: 2RE/RB) 14.4 dB (DMRS: 4RE/RB) 15.0 dB (DMRS: 6RE/RB) 16.4 dB (DMRS: 8RE/RB) 13.93 Mbps |  | 16.3 dB (DMRS: 2RE/RB) 16.1 dB (DMRS: 4RE/RB) 16.3 dB (DMRS: 6RE/RB) 16.8 dB (DMRS: 8RE/RB) 17.01 Mbps |  |
| R1-162505 / Source 4 25 PRB, 10 MHz system bandwidth No short PDCCH region within sTTI Precoding matrix determination based on CSI feedback TB sizes for evaluation are obtained by targeting 1/3, 3/4, and 5/6 code rate when the maximum RS overhead is assumed (i.e., CRS and DMRS with 8 REs/PRB) |  |  |  |  |  |  |

Table C1.2-2 EPA, 3 km/h, rank 2

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 8.27 dB 3.99 Mbps |  | 8.17 dB 5.16 Mbps | 8.01 dB 5.55 Mbps | 7.06 dB 5.73 Mbps |
| 16QAM r3/4 |  | 23.97 dB 18.66 Mbps |  | 23.89 dB 23.59 Mbps | 24.34 dB 25.23 Mbps | 23.98 dB 25.89 Mbps |
| 64QAM r5/6 |  | 34.79 dB 31.24 Mbps |  | 34.68 dB 39.38 Mbps | 34.55 dB 42.09 Mbps | 35.11 dB 43.17 Mbps |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 8.22 dB 4.85 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 24.05 dB 22.55 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 35.36 dB 37.72 Mbps |  |  |  |  |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 7.13 dB 3.99 Mbps |  | 7.00 dB 5.16 Mbps | 6.85 dB 5.55 Mbps | 6.42 dB 5.73 Mbps |
| 16QAM r3/4 |  | 23.30 dB 18.66 Mbps |  | 23.20 dB 23.59 Mbps | 23.72 dB 25.23 Mbps | 23.41 dB 25.89 Mbps |
| 64QAM r5/6 |  | 33.42 dB 31.24 Mbps |  | 33.39 dB 39.38 Mbps | 33.27 dB 42.09 Mbps | 33.45 dB 43.17 Mbps |
| R1-163410/ Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI PRBs bundling over three 3PBRs |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 7.07 dB 4.85 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 23.37 dB 22.55 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 33.92 dB 37.72 Mbps |  |  |  |  |
| R1-163410/ Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI PRBs bundling over three 3PBRs DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 | -- -- | 13.25dB 3.4839Mbps | -- -- | -- -- | 13.25dB 4.1186Mbps | 9dB 3.5568Mbps |
| 16QAM r3/4 | -- -- | -- -- | -- -- | -- -- | -- -- | 25.5dB 16.1568Mbps |
| 64QAM r5/6 | -- -- | -- -- | -- -- | -- -- | -- -- | -- -- |
| R1-163511 / Source 10 25 PRB, 10 MHz system bandwidth, no PRB bundling |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 5.9 dB 4.06 Mbps |  |  |  | 5.6 dB 4.26 Mbps |
| 16QAM r3/4 |  | 22.1 dB 19.18 Mbps |  | 22.1 dB 18.62 Mbps |  | 22.2 dB 19.34 Mbps |
| 64QAM r5/6 |  | 32.5 dB 32.14 Mbps |  |  |  | 32.7 dB 32.24 Mbps |
| R1-163714 / Source 5 30 PRB, 10 MHz system bandwidth RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 5.7 dB (DMRS pattern1: 4 RE/RB) 5.9 dB (DMRS pattern2: 6 RE/RB) 4.24 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 20.6 dB (DMRS pattern1: 4 RE/RB) 22 dB (DMRS pattern2: 6 RE/RB) 20.10 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 29.9 dB (DMRS pattern1: 4 RE/RB) 32.1 dB (DMRS pattern2: 6 RE/RB) 33.72 Mbps |  |  |  |  |
| R1-164062 / Source 5 30 PRB, 10 MHz system bandwidth. RX imperfections and 6% TX EVM, 3 PRBs bundling. DMRS pattern1 represents DMRS CDM in frequency domain, DMRS pattern2 represents DMRS CDM in time domain. Note that code rate for DMRS pattern1 will be lower due to less RS overhead. |  |  |  |  |  |  |

Table C1.2-3 EPA, 60 km/h, rank 1

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 1.46 dB 1.99 Mbps |  | 1.41 dB 2.58 Mbps | 1.32 dB 2.78 Mbps | 0.84 dB 2.86 Mbps |
| 16QAM r3/4 |  | 13.20 dB 9.33 Mbps |  | 13.38 dB 11.79 Mbps | 14.04 dB 12.61 Mbps | 14.10 dB 12.94 Mbps |
| 64QAM r5/6 |  | 22.45 dB 15.62 Mbps |  | 29.25 dB 19.69 Mbps | >36 | 23.67 dB 21.58 Mbps |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.46 dB 2.43 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 13.46 dB 11.28 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 25.74 dB 18.86 Mbps |  |  |  |  |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.2 dB 2.03 Mbps |  |  |  | 0.3 dB 2.13 Mbps |
| 16QAM r3/4 |  | 13.0 dB 9.59 Mbps |  |  |  | 14.2 dB 9.67 Mbps |
| 64QAM r5/6 |  | 24.2 dB 16.07 Mbps |  |  |  | >28 dB |
| R1-163714 / Source 5 30 PRB, 10 MHz system bandwidth RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

Table C1.2-4 EPA, 60 km/h, rank 2

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 7.49 dB 3.99 Mbps |  | 7.49 dB 5.16 Mbps | 7.34 dB 5.55 Mbps | 6.65 dB 5.73 Mbps |
| 16QAM r3/4 |  | 23.11 dB 18.66 Mbps |  | 26.02 dB 23.59 Mbps | >36 | 24.92 dB 25.89 Mbps |
| 64QAM r5/6 |  | >36 |  | >36 | >36 | >36 |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 7.52 dB 4.85 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 25.87 dB 22.55 Mbps |  |  |  |  |
| 64QAM r5/6 |  | >36 |  |  |  |  |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 | -- -- | 13.3dB 3.4839Mbps | -- -- | -- -- | 14dB 4.1186Mbps | 10dB 3.5568Mbps |
| 16QAM r3/4 | -- -- | -- -- | -- -- | -- -- | -- -- | 32.2dB 16.1568Mbps |
| 64QAM r5/6 | -- -- | -- -- | -- -- | -- -- | -- -- | -- -- |
| R1-163511 / Source 10 25 PRB, 10 MHz system bandwidth, no PRB bundling |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 6.1 dB 4.06 Mbps |  |  |  | 6.2 dB 4.26 Mbps |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  | >36 dB |  |  |  | >36 dB |
| R1-163714 / Source 5 30 PRB, 10 MHz system bandwidth RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

Table C1.2-5 EVA, 3 km/h, rank 1

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 0.35 dB 1.99 Mbps |  | 0.37 dB 2.58 Mbps | 0.33 dB 2.78 Mbps | -0.35 dB 2.86 Mbps |
| 16QAM r3/4 |  | 12.37 dB 9.33 Mbps |  | 12.39 dB 11.79 Mbps | 12.70 dB 12.61 Mbps | 12.27 dB 12.94 Mbps |
| 64QAM r5/6 |  | 21.72 dB 15.62 Mbps |  | 21.48 dB 19.69 Mbps | 21.52 dB 21.05 Mbps | 21.51 dB 21.58 Mbps |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.34 dB 2.43 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 12.43 dB 11.28 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 22.06 dB 18.86 Mbps |  |  |  |  |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | -0.3 dB 2.03 Mbps |  |  |  | -0.7 dB 2.13 Mbps |
| 16QAM r3/4 |  | 11.8 dB 9.59 Mbps |  |  |  | 11.6 dB 9.67 Mbps |
| 64QAM r5/6 |  | 20.6 dB 16.07 Mbps |  |  |  | 20.0 dB 16.12 Mbps |
| R1-163714 / Source 5 30 PRB, 10 MHz system bandwidth RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

Table C1.2-6 EVA, 3 km/h, rank 2

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 5.33 dB 3.99 Mbps |  | 5.34 dB 5.16 Mbps | 5.23 dB 5.55 Mbps | 4.43 dB 5.73 Mbps |
| 16QAM r3/4 |  | 21.35 dB 18.66 Mbps |  | 21.41 dB 23.59 Mbps | 21.91 dB 25.23 Mbps | 21.41 dB 25.89 Mbps |
| 64QAM r5/6 |  | >36 |  | 34.95 dB 39.38 Mbps | 35.11 dB 42.09 Mbps | >36 |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 5.29 dB 4.85 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 21.49 dB 22.55 Mbps |  |  |  |  |
| 64QAM r5/6 |  | >36 |  |  |  |  |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |

Table C1.2-7 EVA, 60 km/h, rank 1

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 0.25 dB 1.99 Mbps |  | 0.27 dB 2.58 Mbps | 0.21 dB 2.78 Mbps | -0.35 dB 2.86 Mbps |
| 16QAM r3/4 |  | 12.30 dB 9.33 Mbps |  | 12.46 dB 11.79 Mbps | 12.97 dB 12.61 Mbps | 12.92 dB 12.94 Mbps |
| 64QAM r5/6 |  | 22.07 dB 15.62 Mbps |  | 25.65 dB 19.69 Mbps | >36 | 22.98 dB 21.58 Mbps |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.86 dB 2.92 Mbps |  | 0.97 dB 3.11 Mbps | 0.92 dB 3.20 Mbps | 0.96 dB 3.31 Mbps |
| 16QAM r3/4 |  | 12.48 dB 2.92 Mbps |  | 12.5 dB 3.11 Mbps | 12.98 dB 3.20 Mbps | 12.59 dB 3.31 Mbps |
| 64QAM r5/6 |  | 19.87 dB 24.26 Mbps |  | 20.71 dB 24.66 Mbps | - dB 0 Mbps | 19.85 dB 26.04 Mbps |
| R1-163400 / Source 13 50 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.24 dB 2.43 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 12.51 dB 11.28 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 25.34 dB 18.86 Mbps |  |  |  |  |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | -0.4 dB 2.03 Mbps |  |  |  | -0.4 dB 2.13 Mbps |
| 16QAM r3/4 |  | 12.1 dB 9.59 Mbps |  |  |  | 13.0 dB 9.67 Mbps |
| 64QAM r5/6 |  | 23.6 dB 16.07 Mbps |  |  |  | >28 dB |
| R1-163714 / Source 5 30 PRB, 10 MHz system bandwidth RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

Table C1.2-8 EVA, 60 km/h, rank 2

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 5.54 dB 3.99 Mbps |  | 5.64 dB 5.16 Mbps | 5.52 dB 5.55 Mbps | 4.77 dB 5.73 Mbps |
| 16QAM r3/4 |  | 21.71 dB 18.66 Mbps |  | 23.32 dB 23.59 Mbps | 26.84 dB 25.23 Mbps | 22.89 dB 25.89 Mbps |
| 64QAM r5/6 |  | >36 |  | >36 | >36 | >36 |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 5.55 dB 4.85 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 23.61 dB 22.55 Mbps |  |  |  |  |
| 64QAM r5/6 |  | >36 |  |  |  |  |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |

Table C1.2-9 ETU, 3 km/h, rank 1

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 0.42 dB 1.99 Mbps |  | 0.39 dB 2.58 Mbps | 0.31 dB 2.78 Mbps | -0.51 dB 2.86 Mbps |
| 16QAM r3/4 |  | 12.98 dB 9.33 Mbps |  | 12.95 dB 11.79 Mbps | 13.27 dB 12.61 Mbps | 12.65 dB 12.94 Mbps |
| 64QAM r5/6 |  | 25.86 dB 15.62 Mbps |  | 25.33 dB 19.69 Mbps | 25.35 dB 21.05 Mbps | 24.52 dB 21.58 Mbps |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.38 dB 2.96 Mbps |  | 0.41 dB 3.18 Mbps | 0.44 dB 3.32 Mbps | 0.56 dB 3.35 Mbps |
| 16QAM r3/4 |  | 12.1 dB 14.42 Mbps |  | 12.07 dB 15.01 Mbps | 12.1 dB 15.68 Mbps | 12.3 dB 15.44 Mbps |
| 64QAM r5/6 |  | 19.22 dB 22.76 Mbps |  | 19.27 dB 23.97 Mbps | 19.34 dB 24.51 Mbps | 19.5 dB 24.91 Mbps |
| R1-163400/ Source 13 50 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.39 dB 2.43 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 13.04 dB 11.28 Mbps |  |  |  |  |
| 64QAM r5/6 |  | >36 |  |  |  |  |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | -0.5 dB 2.03 Mbps |  |  |  | -0.9 dB 2.13 Mbps |
| 16QAM r3/4 |  | 11.8 dB 9.59 Mbps |  | 11.8 9.31 Mbps |  | 11.5 dB 9.67 Mbps |
| 64QAM r5/6 |  | 20.0 dB 16.07 Mbps |  |  |  | 19.8 dB 16.12 Mbps |
| R1-163714 / Source 5 30 PRB, 10 MHz system bandwidth RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

Table C1.2-10 ETU, 3 km/h, rank 2

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 6.14 dB 3.99 Mbps |  | 6.12 dB 5.16 Mbps | 6.03 dB 5.55 Mbps | 4.75 dB 5.73 Mbps |
| 16QAM r3/4 |  | 25.46 dB 18.66 Mbps |  | 25.17 dB 23.59 Mbps | 25.87 dB 25.23 Mbps | 24.79 dB 25.89 Mbps |
| 64QAM r5/6 |  | >36 |  | >36 | >36 | >36 |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 6.09 dB 4.85 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 26.45 dB 22.55 Mbps |  |  |  |  |
| 64QAM r5/6 |  | >36 |  |  |  |  |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 | -- -- | 8.5dB 3.4839Mbps | -- -- | -- -- | 8.5dB 4.1186Mbps | 6.25dB 3.5568Mbps |
| 16QAM r3/4 | -- -- | 27dB 16.738Mbps | -- -- | -- -- | 27dB 18.848Mbps | 22.5dB 16.1568Mbps |
| 64QAM r5/6 | -- -- | -- -- | -- -- | -- -- | -- -- | -- -- |
| R1-163511 / Source 10 25 PRB, 10 MHz system bandwidth, no PRB bundling |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  |  |  |  |
| 16QAM r3/4 |  | 20.9 dB 19.18 Mbps |  | 20.9 dB 18.62 Mbps |  | 20.0 dB 19.34 Mbps |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163714 / Source 5 30 PRB, 10 MHz system bandwidth RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

Table C1.2-11 ETU, 60 km/h, rank 1

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 0.39 dB 1.99 Mbps |  | 0.39 dB 2.58 Mbps | 0.34 dB 2.78 Mbps | -0.29 dB 2.86 Mbps |
| 16QAM r3/4 |  | 12.93 dB 9.33 Mbps |  | 13.10 dB 11.79 Mbps | 13.66 dB 12.61 Mbps | 13.44 dB 12.94 Mbps |
| 64QAM r5/6 |  | 26.17 dB 15.62 Mbps |  | >36 | >36 | 28.17 dB 21.58 Mbps |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.4 dB 2.99 Mbps |  | 0.41 dB 3.14 Mbps | 0.64 dB 3.28 Mbps | 0.43 dB 3.26 Mbps |
| 16QAM r3/4 |  | 12 dB 14.57 Mbps |  | 12.13 dB 15.00 Mbps | 12.52 dB 15.16 Mbps | 12.13 dB 15.51 Mbps |
| 64QAM r5/6 |  | 19.38 dB 23.23 Mbps |  | 20.03 dB 25.11 Mbps | - dB 0 Mbps | 19.53 dB 24.94 Mbps |
| R1-163400/ Source 13 50 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.38 dB 2.43 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 13.19 dB 11.28 Mbps |  |  |  |  |
| 64QAM r5/6 |  | >36 |  |  |  |  |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughtpu | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0 dB 2.9 Mbps |  |  | -0.2 dB 3.6 Mbps | -0.4 dB 3.6 Mbps |
| 16QAM r3/4 |  | 11.9 dB 12.96 Mbps |  |  | 13.1 dB 16.1 Mbps | 13.3 dB 16.2 Mbps |
| 64QAM r5/6 |  | 19.8 dB 25 Mbps |  |  |  | 21.5 dB 27 Mbps |
| R1-163264 / Source 11 50 PRB, 10 MHz system bandwidth DM-RS available in every TTI DMRS density 6RE per PRB in sTTI/12RE per PRB in TTI Channel estimation performed with 3PRB granularity |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0 dB 3.42 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 12.5 dB 15.25 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 23.5 dB 25 Mbps |  |  |  |  |
| R1-163264 / Source 11 50 PRB, 10 MHz system bandwidth DMRS only available in every second TTI DMRS density 6RE per PRB in sTTI/12RE per PRB in TTI Channel estimation performed with 3PRB granularity |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.5 dB 3.24 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 12.5 dB 14.59 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 21.9 dB 24.3 Mbps |  |  |  |  |
| R1-163264 / Source 11 50 PRB, 10 MHz system bandwidth Bundled pattern over 3 PRBs (total 12RE per 3PRB) DMRS available in every TTI Channel estimation performed with 3PRB granularity |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.5 dB 3.6 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 13 dB 16.2 Mbps |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163264 / Source 11 50 PRB, 10 MHz system bandwidth Bundled pattern over 3 PRBs (total 12RE per 3PRB) DM-RS only available in every second TTI Channel estimation performed with 3PRB granularity |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | -0.6 dB 2.03 Mbps |  |  |  | -0.6 dB 2.13 Mbps |
| 16QAM r3/4 |  | 11.6 dB 9.59 Mbps |  |  |  | 12.3 dB 9.67 Mbps |
| 64QAM r5/6 |  | 23 dB 16.07 Mbps |  |  |  | >28 dB |
| R1-163714 / Source 5 30 PRB, 10 MHz system bandwidth RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | -0.7 dB (DMRS pattern1: 4 RE/RB) -0.6 dB (DMRS pattern2: 6 RE/RB) 2.12 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 10.6 dB (DMRS pattern1: 4 RE/RB) 11.6 dB (DMRS pattern2: 6 RE/RB) 10.05 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 18.1 dB (DMRS pattern1: 4 RE/RB) 19.9 dB (DMRS pattern2: 6 RE/RB) 16.86 Mbps |  |  |  |  |
| R1-164062 / Source 5 30 PRB, 10 MHz system bandwidth. RX imperfections and 6% TX EVM, 3 PRBs bundling. DMRS pattern1 represents DMRS CDM in frequency domain, DMRS pattern2 represents DMRS CDM in time domain. Note that code rate for DMRS pattern1 will be lower due to less RS overhead. |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  | 2.1 dB (DMRS: 2RE/RB) 0.5 dB (DMRS: 4RE/RB) 0.1 dB (DMRS: 6RE/RB) 0.1 dB (DMRS: 8RE/RB) 1.90 Mbps |  | 2.6 dB (DMRS: 2RE/RB) 0.8 dB (DMRS: 4RE/RB) 0.2 dB (DMRS: 6RE/RB) 0.1 dB (DMRS: 8RE/RB) 2.26 Mbps |  |
| 16QAM r3/4 |  |  | >25.0 dB (DMRS: 2RE/RB) 10.5 dB (DMRS: 4RE/RB) 10.6 dB (DMRS: 6RE/RB) 11.5 dB (DMRS: 8RE/RB) 8.40 Mbps |  | >25.0 dB (DMRS: 2RE/RB) 12.2 dB (DMRS: 4RE/RB) 12.0 dB (DMRS: 6RE/RB) 12.1 dB (DMRS: 8RE/RB) 10.32 Mbps |  |
| 64QAM r5/6 |  |  | >30.0 dB (DMRS: 2RE/RB) 17.3 dB (DMRS: 4RE/RB) 17.3 dB (DMRS: 6RE/RB) 20.0 dB (DMRS: 8RE/RB) 13.93 Mbps |  | >30.0 dB (DMRS: 2RE/RB) 24.6 dB (DMRS: 4RE/RB) 22.8 dB (DMRS: 6RE/RB) 25.4 dB (DMRS: 8RE/RB) 17.01 Mbps |  |
| R1-162505 / Source 4 25 PRB, 10 MHz system bandwidth No short PDCCH region within sTTI Precoding matrix determination based on CSI feedback TB sizes for evaluation are obtained by targeting 1/3, 3/4, and 5/6 code rate when the maximum RS overhead is assumed (i.e., CRS and DMRS with 8 REs/PRB) |  |  |  |  |  |  |

Table C1.2-12 ETU, 60 km/h, rank 2

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 6.03 dB 3.99 Mbps |  | 6.14 dB 5.16 Mbps | 6.03 dB 5.55 Mbps | 6.07 dB 5.73 Mbps |
| 16QAM r3/4 |  | 25.38 dB 18.66 Mbps |  | 28.61 dB 23.59 Mbps | >36 | >36 |
| 64QAM r5/6 |  | >36 |  | >36 | >36 | >36 |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 6.03 dB 4.85 Mbps |  |  |  |  |
| 16QAM r3/4 |  | >36 |  |  |  |  |
| 64QAM r5/6 |  | >36 |  |  |  |  |
| R1-163314 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 | -- -- | 8.7dB 3.4839Mbps | -- -- | -- -- | 8.85dB 4.1186Mbps | 7.75dB 3.5568Mbps |
| 16QAM r3/4 | -- -- | 27.75dB 16.738Mbps | -- -- | -- -- | >30dB -- | 25.75dB 16.1568Mbps |
| 64QAM r5/6 | -- -- | -- -- | -- -- | -- -- | -- -- | -- -- |
| R1-163511 / Source 10 25 PRB, 10 MHz system bandwidth, no PRB bundling |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 5 dB 4.06 Mbps |  |  |  | 4.8 dB 4.26 Mbps |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  | >36 dB |  |  |  | >36 dB |
| R1-163714 / Source 5 30 PRB, 10 MHz system bandwidth RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 4.6 dB (DMRS pattern1: 4 RE/RB) 4.5 dB (DMRS pattern2: 6 RE/RB) 4.24 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 20.7 dB (DMRS pattern1: 4 RE/RB) 22.1 dB (DMRS pattern2: 6 RE/RB) 20.1 Mbps |  |  |  |  |
| 64QAM r5/6 |  | >36 dB |  |  |  |  |
| R1-164062 / Source 5 30 PRB, 10 MHz system bandwidth. RX imperfections and 6% TX EVM, 3 PRBs bundling. DMRS pattern1 represents DMRS CDM in frequency domain, DMRS pattern2 represents DMRS CDM in time domain. Note that code rate for DMRS pattern1 will be lower due to less RS overhead. |  |  |  |  |  |  |

Table C1.2-13 EPA XPOL, 3 km/h, rank 1

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  |  |  |  |  |  |
| 16QAM r3/4 |  | 4.36 dB 9.33 Mbps |  | 4.33 dB 11.79 Mbps | 4.57 dB 12.61 Mbps | 4.44 dB 12.94 Mbps |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-165304 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI PRBs bundling over three 3PBRs |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  |  |  |  |
| 16QAM r3/4 |  | 4.38 dB 11.28 Mbps |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-165304 / Source 8 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI PRBs bundling over three 3PBRs DM-RS only available in every second TTI |  |  |  |  |  |  |

Table C1.2-14 EPA, 3 km/h, rank 4

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  |  |  |  |  |  |
| 16QAM r3/4 |  | 12.4 dB 19.18 Mbps |  | 12.4 dB 18.62 Mbps |  | 12.7 dB 19.34 Mbps |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163714 / Source 5 30 PRB, 10 MHz system bandwidth RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. Note that for RANK 4 transmission, the same TBS of RANK 2 for each codeword is assumed. |  |  |  |  |  |  |

Table C1.2-15 ETU, 3 km/h, AMC, with interference UEs

| SNR | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| 5 dB |  | 2 Mbps |  |  |  | 1 Mbps |
| 10 dB |  | 3.7 Mbps |  |  |  | 2 Mbps |
| 15 dB |  | 5.6 Mbps |  |  |  | 3.5 Mbps |
| 20 dB |  | 8.2 Mbps |  |  |  | 6 Mbps |
| 25 dB |  | 10.8 Mbps |  |  |  | 8.5 Mbps |
| 30 dB |  | 12.8 Mbps |  |  |  | 10.7 Mbps |
| R1-164821 / Source 5 12 PRB, 10 MHz system bandwidth Aggressive TBS selection and 16 RVs for 2 OS sTTI; TBS selection with 10% initial BLER and 4 RVs for 1ms TTI. |  |  |  |  |  |  |

## C2	Link-level evaluation sPDCCH

Table C2-1 EPA 3km/h, 50 PRBs, CRS

|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| --- | --- | --- | --- | --- |
| 40 bits | 4.0844 | 0.24589 | -2.963 |  |
| 70 bits | 11.9942 | 3.3731 | 0.003019 |  |
| R1-163305 / Source 8 |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 4.6 | 1.2 | -1.8 | -4.7 |
| 70 bits | 12.6 | 4 | 1.4 | -2.1 |
| R1-162401 / SOURCE 1 |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 3.0 | -0.7 | -3.6 | -6.3 |
| R1-162506 / Source 4 distributed PRBs, 1 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 5.2 | 1.6 | -1.8 | -5.5 |
| R1-162506 / Source 4 localized PRBs, 1 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 5.3 | 1.8 | -1.3 | -4.6 |
| R1-162506 / Source 4 localized PRBs, 2 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 2.9 | -0.7 | -3.6 | -6.3 |
| R1-162506 / Source 4 distributed PRBs, 2 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 5.4 | 1.8 | -1.0 | -4.0 |
| R1-162506 / Source 4 localized PRBs, 3 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 3.0 | -0.7 | -3.7 | -6.3 |
| R1-162506 / Source 4 distributed PRBs, 3 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 5.5 | 2.1 | -0.8 | -3.4 |
| R1-162506 / Source 4 localized PRBs, 7 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 2.9 | -0.7 | -3.6 | -6.2 |
| R1-162506 / Source 4 distributed PRBs, 7 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits |  |  | 3.79 | 0.4 |  |  | -2.36 |  | -4.7 |
| 70 bits |  |  | 11.9 | 3.2 |  |  | 0.10 |  | -2.5 |
| R1-165076 / Source 7 |  |  |  |  |

Table C2-2 EVA 60km/h, 25 PRBs, CRS

|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| --- | --- | --- | --- | --- |
| 40 bits | 2.8008 | -0.96088 | -3.5526 |  |
| 70 bits | 12.1822 | 2.0945 | -1.3566 |  |
| R1-163305 / Source 8 |  |  |  |  |

Table C2-3 EVAU 60km/h, 50 PRBs, CRS

|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| --- | --- | --- | --- | --- |
| 40 bits | 3.58 | 0.4 | -3.1 | -5.5 |
| 70 bits | 12.9 | 2.5 | -0.7 | -3.3 |
| R1-165076 / Source 7 |  |  |  |  |

Table C2-4 ETU 60km/h, 50 PRBs, CRS

|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| --- | --- | --- | --- | --- |
| 40 bits | 4.6 | 1 | -2 | -5.1 |
| 70 bits | 13.8 | 3.6 | 0.9 | -2.5 |
| R1-162401 / SOURCE 1 |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 5.2 | 1.2 | -1.8 | -4.6 |
| R1-162506 / Source 4 localized PRBs, 1 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 3.8 | 0.2 | -2.5 | -4.8 |
| R1-162506 / Source 4 distributed PRBs, 1 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 5.8 | 1.8 | -0.9 | -4.0 |
| R1-162506 / Source 4 localized PRBs, 2 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 3.8 | 0.3 | -2.3 | -4.6 |
| R1-162506 / Source 4 distributed PRBs, 2 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 6.4 | 2.4 | -0.5 | -3.4 |
| R1-162506 / Source 4 localized PRBs, 3 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 3.8 | 0.2 | -2.5 | -4.8 |
| R1-162506 / Source 4 distributed PRBs, 3 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 6.3 | 3.2 | 0.2 | -2.5 |
| R1-162506 / Source 4 localized PRBs, 7 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 3.8 | 0.5 | -1.8 | -3.9 |
| R1-162506 / Source 4 distributed PRBs, 7 symbol sPDCCH |  |  |  |  |

Table C2-5 EPA 3km/h, 50 PRBs, DMRS

|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| --- | --- | --- | --- | --- |
| 40 bits | 6.6 (PRB bundle: 1 PRB) 5.9 (PRB bundle: 3 PRBs) | 2.9 (PRB bundle: 1 PRB) 2.2 (PRB bundle: 3 PRBs) | 0.5 (PRB bundle: 1 PRB) -0.3 (PRB bundle: 3 PRBs) | -2.2 (PRB bundle: 1 PRB) -3.2 (PRB bundle: 3 PRBs) |
| R1-162504 / Source 4 sPDCCH transmission using 36xAL REs except CRS resource (including DMRS resource) localized PRBs, 2 symbol sPDCCH, DMRS: 4RE/RB |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 5.3 (PRB bundle: 1 PRB) 4.7 (PRB bundle: 3 PRBs) | 0.4 (PRB bundle: 1 PRB) -0.3 (PRB bundle: 3 PRBs) | -1.9 (PRB bundle: 1 PRB) -2.8 (PRB bundle: 3 PRBs) | -4.1 (PRB bundle: 1 PRB) -5.0 (PRB bundle: 3 PRBs) |
| R1-162504 / Source 4 sPDCCH transmission using 36xAL REs except CRS resource (including DMRS resource) distributed PRBs, 2 symbol sPDCCH, DMRS: 4RE/RB |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 6.0 (DMRS: 2RE/RB) >30 (DMRS: 4RE/RB) >30 (DMRS: 6RE/RB) | 0.6 (DMRS: 2RE/RB) 1.2 (DMRS: 4RE/RB) >30 (DMRS: 6RE/RB) | -1.7 (DMRS: 2RE/RB) -1.8 (DMRS: 4RE/RB) 0.6 (DMRS: 6RE/RB) | -3.8 (DMRS: 2RE/RB) -4.2 (DMRS: 4RE/RB) -2.6 (DMRS: 6RE/RB) |
| R1-162504 / Source 4 sPDCCH transmission using 36xAL REs except CRS resource (including DMRS resource) distributed PRBs, 3 PRBs PRB bundling, 1 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 3.6 (DMRS: 2RE/RB) 4.7 (DMRS: 4RE/RB) 6.5 (DMRS: 6RE/RB) | 0.4 (DMRS: 2RE/RB) -0.3 (DMRS: 4RE/RB) 0.1 (DMRS: 6RE/RB) | -1.8 (DMRS: 2RE/RB) -2.8 (DMRS: 4RE/RB) -2.8 (DMRS: 6RE/RB) | -3.9 (DMRS: 2RE/RB) -5.0 (DMRS: 4RE/RB) -5.3 (DMRS: 6RE/RB) |
| R1-162504 / Source 4 sPDCCH transmission using 36xAL REs except CRS resource (including DMRS resource) distributed PRBs, 3 PRBs PRB bundling, 2 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 3.8 (DMRS: 2RE/RB) 4.1 (DMRS: 4RE/RB) 5.4 (DMRS: 6RE/RB) | 0.4 (DMRS: 2RE/RB) -0.4 (DMRS: 4RE/RB) -0.4 (DMRS: 6RE/RB) | -1.8 (DMRS: 2RE/RB) -2.9 (DMRS: 4RE/RB) -3.3 (DMRS: 6RE/RB) | -3.8 (DMRS: 2RE/RB) -5.2 (DMRS: 4RE/RB) -5.6 (DMRS: 6RE/RB) |
| R1-162504 / Source 4 sPDCCH transmission using 36xAL REs except CRS resource (including DMRS resource) distributed PRBs, 3 PRBs PRB bundling, 3 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 3.0 (DMRS: 2RE/RB) 1.6 (DMRS: 4RE/RB) 2.1 (DMRS: 6RE/RB) | 0.2 (DMRS: 2RE/RB) -1.0 (DMRS: 4RE/RB) -1.4 (DMRS: 6RE/RB) | -1.6 (DMRS: 2RE/RB) -3.0 (DMRS: 4RE/RB) -3.6 (DMRS: 6RE/RB) | -3.6 (DMRS: 2RE/RB) -5.2 (DMRS: 4RE/RB) -5.9 (DMRS: 6RE/RB) |
| R1-162504 / Source 4 sPDCCH transmission using 36xAL REs except CRS resource (including DMRS resource) distributed PRBs, 3 PRBs PRB bundling, 7 symbol sPDCCH |  |  |  |  |

Table C2-6 ETU 60km/h, 50 PRBs, DMRS

|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| --- | --- | --- | --- | --- |
| 40 bits | >10 (DMRS: 2RE/RB) >30 (DMRS: 4RE/RB) >30 (DMRS: 6RE/RB) | 7.0 (DMRS: 2RE/RB) 4.5 (DMRS: 4RE/RB) >30 (DMRS: 6RE/RB) | 3.8 (DMRS: 2RE/RB) 0.9 (DMRS: 4RE/RB) 4.0 (DMRS: 6RE/RB) | 1.0 (DMRS: 2RE/RB) -1.8 (DMRS: 4RE/RB) 0.2 (DMRS: 6RE/RB) |
| R1-162504 / Source 4 sPDCCH transmission using 36xAL REs except CRS resource (including DMRS resource) distributed PRBs, 3 PRBs PRB bundling, 1 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | >10 (DMRS: 2RE/RB) 9.6 (DMRS: 4RE/RB) >10 (DMRS: 6RE/RB) | 5.6 (DMRS: 2RE/RB) 3.2 (DMRS: 4RE/RB) 3.4 (DMRS: 6RE/RB) | 2.4 (DMRS: 2RE/RB) 0.5 (DMRS: 4RE/RB) 0.1 (DMRS: 6RE/RB) | -0.6 (DMRS: 2RE/RB) -2.2 (DMRS: 4RE/RB) -2.8 (DMRS: 6RE/RB) |
| R1-162504 / Source 4 sPDCCH transmission using 36xAL REs except CRS resource (including DMRS resource) distributed PRBs, 3 PRBs PRB bundling, 2 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | >12 (DMRS: 2RE/RB) 8.7 (DMRS: 4RE/RB) 10.1 (DMRS: 6RE/RB) | 5.9 (DMRS: 2RE/RB) 3.2 (DMRS: 4RE/RB) 3.0 (DMRS: 6RE/RB) | 2.6 (DMRS: 2RE/RB) 0.4 (DMRS: 4RE/RB) -0.2 (DMRS: 6RE/RB) | -0.2 (DMRS: 2RE/RB) -2.2 (DMRS: 4RE/RB) -3.9 (DMRS: 6RE/RB) |
| R1-162504 / Source 4 sPDCCH transmission using 36xAL REs except CRS resource (including DMRS resource) distributed PRBs, 3 PRBs PRB bundling, 3 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 11 (DMRS: 2RE/RB) 6.3 (DMRS: 4RE/RB) 5.6 (DMRS: 6RE/RB) | 5.4 (DMRS: 2RE/RB) 2.8 (DMRS: 4RE/RB) 2.1 (DMRS: 6RE/RB) | 3.0 (DMRS: 2RE/RB) 0.7 (DMRS: 4RE/RB) 0.1 (DMRS: 6RE/RB) | 0.6 (DMRS: 2RE/RB) -1.7 (DMRS: 4RE/RB) -2.6 (DMRS: 6RE/RB) |
| R1-162504 / Source 4 sPDCCH transmission using 36xAL REs except CRS resource (including DMRS resource) distributed PRBs, 3 PRBs PRB bundling, 7 symbol sPDCCH |  |  |  |  |

## C3	Link-level evaluation sPUSCH

Table C3-1 EPA, 3 km/h, 25 PRB

| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 2.69 dB 1.26 Mbps | 2.06 dB 1.68 Mbps | 1.89 dB 1.89 Mbps | 1.84 dB 2.16 Mbps | 1.61 dB 2.16 Mbps |
| 16QAM r3/4 |  | 13.97 dB 5.70 Mbps | 13.79 dB 7.56 Mbps | 13.86 dB 8.49 Mbps | 13.66 dB 9.72 Mbps | 13.63 dB 9.72 Mbps |
| 64QAM r5/6 |  | 20.94 dB 9.42 Mbps | 20.81 dB 12.60 Mbps | 20.80 dB 14.16 Mbps | 20.94 dB 16.20 Mbps | 20.78 dB 16.20 Mbps |
| R1-163316 / Source 8 DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.43 dB 1.90 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 13.84 dB 8.52 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 20.88 dB 14.18 Mbps |  |  |  |  |
| R1-163316 / Source 8 DM-RS included in every second sTTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.29 dB 2.11 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 13.83 dB 9.46 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 20.88 dB 15.75 Mbps |  |  |  |  |
| R1-163316 / Source 8 DM-RS included in third sTTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.05 dB 1.09 Mbps |  | 1.95 dB 1.82 Mbps | 2.27 dB 2.09 Mbps | 2.19 dB 2.13 Mbps |
| 16QAM r3/4 |  | 13.72 dB 5.39 Mbps |  | 13.72 dB 8.05 Mbps | 13.69 dB 9.52 Mbps | 13.53 dB 9.57 Mbps |
| 64QAM r5/6 |  |  |  |  |  |  |
| 163401/ Source 13 |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 4.67 dB 1.68 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 24.10 dB 7. 60 Mbps |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163411 / Source 8 Two UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 4.67 dB 1.89 Mbps |  |  |  |  |
| 16QAM r3/4 |  | - - |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163411 / Source 8 Three UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 5.26 dB 2. 02Mbps |  |  |  |  |
| 16QAM r3/4 |  | - - |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163411 / Source 8 Four UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.35 dB 0.90 Mbps | 0.56 dB 1.37 Mbps | 0.57 dB 1.58 Mbps | 0.56 dB 1.85 Mbps | 0.78 dB 1.99 Mbps |
| 16QAM r3/4 |  | 13.11 dB 5.28 Mbps | 13.30 dB 7.27 Mbps | 13.25 dB 8.17 Mbps | 13.12 dB 9.27 Mbps | 13.60 dB 9.58 Mbps |
| 64QAM r5/6 |  | 21.02 dB 9.32 Mbps | 21.00 dB 12.53 Mbps | 20.69 dB 13.81 Mbps | 20.84 dB 15.75 Mbps | 21.65 dB 16.47 Mbps |
| R1-163820 / SOURCE 4 DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  | 0.58 dB 1.58 Mbps |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163820 / SOURCE 4 Two UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.37 dB 1.02 Mbps | 1.36 dB 1.45 Mbps | 1.36 dB 1.67 Mbps | 1.37 dB 1.98 Mbps | 1.35 dB 2.01 Mbps |
| 16QAM r3/4 |  | 13.6 dB 5.43 Mbps | 13.7 dB 7.35 Mbps | 14.1 dB 8.32 Mbps | 14.3 dB 9.51 Mbps | 13.9 dB 9.57 Mbps |
| 64QAM r5/6 |  | 21.25 dB 9.47 Mbps | 21.5 dB 12.77 Mbps | 21.5 dB 14.43 Mbps | 21.75 dB 16.49 Mbps | 21.75 dB 16.5 Mbps |
| R1-163465 / Source 12 DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 | 2dB 2.22Mbps | 1.75dB 2.37Mbps |  |  |  |  |  | 1.5dB 2.12Mbps |  | 1.5dB 2.14Mbps |  |
| 16QAM r3/4 | 13.2dB 11.05Mbps | 13dB 11.2Mbps |  |  |  |  |  | 12.8dB 9.69Mbps |  | 12.8dB 9.71Mbps |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-164464 / Source 10 DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.05 dB sTTI 0: 1.11 Mbps sTTI 1~3: 2.37 Mbps |  | 2.05 dB 1.82 Mbps |  |  |
| 16QAM r3/4 |  | 13.50 dB sTTI 0: 5.52 Mbps sTTI 1~3: 11.19 Mbps |  | 13.50 dB 8.43 Mbps |  | 13.9 dB 9.70 Mbps |
| 64QAM r5/6 |  | 20.55 dB sTTI 0: 9.30 Mbps sTTI 1~3: 18.75 Mbps |  | 20.55 dB 14.10 Mbps |  |  |
| R1-164063, R1-162114 / Source 5 25 PRB, 10 MHz system bandwidth. Shared DMRS for 4-symbol sTTI. DMRS occupies the 1st symbol in sTTI 0, and is shared by four 2-symbol sTTIs, (i.e. sTTI 0~3, which are consecutive deployed in time domain) |  |  |  |  |  |  |

Table C3-2 EPA, 3 km/h, 50 PRB

| SNR at 10% BLER Throughtput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 3.9 dB 2.52 Mbps |  |  | 4.2 dB 4.32 Mbps | 3.7 dB 4.32 Mbps |
| 16QAM r3/4 |  | 15.4 dB 11.34 Mbps |  |  | 15.9 dB 19.44 Mbps | 15.7 dB 19.44 Mbps |
| 64QAM r5/6 |  | 22.3 dB 18.9 Mbps |  |  | 23.1 dB 32.4 Mbps | 22.9 dB 32.4 Mbps |
| R1-160779 / Source 11 DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 3.9 dB 4.2 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 15.5 dB 11.34 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 22.3 dB 31.38 Mbps |  |  |  |  |
| R1-160779 / Source 11 DM-RS included in every third sTTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput |  |  |  |  | TTI length=1 |  |  |  |  | TTI length=2 |  |  |  | TTI length=3 |  |  |  | TTI length=4(*) |  |  | TTI length=7 |  |  |  |  |  |  | TTI length=14 |
| QPSK r1/3 |  |  |  |  |  |  | 1.28 dB 2.52 Mbps |  |  |  | 1.23 dB 3.36 Mbps |  |  |  |  |  |  |  | 1.25 dB 4.32 Mbps |  |  | 1.12 dB 4.32 Mbps |  |  |  |
| 16QAM r3/4 |  |  |  |  |  |  | 13.24 dB 11.3 Mbps |  |  |  | 13.28 dB 15.1 Mbps |  |  |  | 13.39 dB 19.4 Mbps |  |  |  | 13.36 dB 19.4 Mbps |  |  | 13.43 dB 19.4 Mbps |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  | 20.19 dB 18.9 Mbps |  |  |  | 20.21 dB 25.2 Mbps |  |  |  |  |  |  |  | 20.39 dB 32.4 Mbps |  |  | 20.43 dB 32.4 Mbps |  |  |  |
| R1-164115 / Source 14 DM-RS included in every TTI Full symbols (*) shared symbol 4(3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.93 dB 3.78 Mbps | 1.90 dB 4.20 Mbps |  | 1.48 dB 4.32 Mbps |  |
| 16QAM r3/4 |  | 13.63 dB 17.0 Mbps | 13.58 dB 18.9 Mbps | 13.75 dB 19.8 Mbps | 13.4 dB 21.1 Mbps |  |
| 64QAM r5/6 |  | 20.55 dB 28.3 Mbps | 20.67 dB 31.5 Mbps | 20.25 dB 27.88 Mbps | 20.52 dB 32.4 Mbps |  |
| R1-164115 / Source 14 DM-RS included in every TTI Hybrid symbols TTI length=2,3,4,7: one hybrid symbol |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER |  |  |  |  | TTI length=1 |  |  |  |  | TTI length=2 |  |  |  | TTI length=3 |  |  |  | TTI length=4 |  |  | TTI length=7 |  |  | TTI length=14 |  |  |  |
| QPSK r1/3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 13.1 dB 17.0 Mbps |  |  | 13.4 dB 19.4 Mbps |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R1-164115 / Source 14 DM-RS included in every TTI Hybrid symbols TTI length=4,7: two hybrid symbols |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table C3-3 EVA, 60 km/h, 25 PRB

| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 1.48 dB 1.26 Mbps | 1.20 dB 1.68 Mbps | 1.10 dB 1.89 Mbps | 0.94 dB 2.16 Mbps | 0.46 dB 2.16 Mbps |
| 16QAM r3/4 |  | 13.64 dB 5.70 Mbps | 13.47 dB 7.56 Mbps | 13.67 dB 8.49 Mbps | 13.63 dB 9.72 Mbps | 13.47 dB 9.72 Mbps |
| 64QAM r5/6 |  | 21.22 dB 9.42 Mbps | 21.92 dB 12.60 Mbps | 24.30 dB 14.16 Mbps | 25.07 dB 16.20 Mbps | 24.60 dB 16.20 Mbps |
| R1-163316 / Source 8 |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.33 dB 1.08 Mbps |  | 0.39 dB 1.77 Mbps | 0.3 dB 2.10 Mbps | 0.51 dB 2.07 Mbps |
| 16QAM r3/4 |  | 12.87 dB 5.28 Mbps |  | 13.42 dB 7.76 Mbps | 13.19 dB 8.82 Mbps | 12.66 dB 9.43 Mbps |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163401/ Source 13 |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | -0.25 dB 1.03 Mbps | -0.25 dB 1.48 Mbps | -0.1 dB 1.68 Mbps | -0.1 dB 1.96 Mbps | -0.2 dB 2.01 Mbps |
| 16QAM r3/4 |  | 13.3 dB 5.48 Mbps | 13 dB 7.34 Mbps | 13.3 dB 8.35 Mbps | 13.4 dB 9.56 Mbps | 13.78 dB 9.6 Mbps |
| 64QAM r5/6 |  | 21.1 dB 9.47 Mbps | 21.2 dB 12.79 Mbps | 21.5 dB 14.34 Mbps | 25 dB 16.47 Mbps | 21.75 dB 16.52 Mbps |
| R1-163465 / Source 12 DM-RS included in every TTI |  |  |  |  |  |  |

Table C3-4 ETU, 3 km/h, 25 PRB

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 0.06 dB 1.09 Mbps |  | 0.11 dB 1.80 Mbps | - 0.12 dB 2.15 Mbps | 0.22 dB 2.10 Mbps |
| 16QAM r3/4 |  | 12.68 dB 5.18 Mbps |  | 12.59 dB 8.13 Mbps | 12.58 dB 9.35 Mbps | 12.56 dB 9.39 Mbps |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163401 / Source 13 |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  | -0.69 dB 1.57 Mbps |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163820 / SOURCE 4 DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  | -0.68 dB 1.57 Mbps |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163820 / SOURCE 4 Two UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 | 0.3dB 2.22Mbps | 0dB 2.37Mbps |  |  | <0dB 2.12Mbps | <0dB 2.14Mbps |
| 16QAM r3/4 | 12.25dB 11.05Mbps | 12dB 11.2Mbps |  |  | 11.9dB 9.69Mbps | 12.1dB 9.71Mbps |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-164464 / Source 10 DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  |  |  |  |
| 16QAM r3/4 |  |  |  | 12.50 dB 8.43 Mbps |  | 13.0 dB 9.70 Mbps |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-162114 / Source 5 25 PRB, 10 MHz system bandwidth. Shared DMRS for 4-symbol sTTI. |  |  |  |  |  |  |

Table C3-5 ETU, 60 km/h, 25 PRB

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 0.2 dB 1.06 Mbps |  | 0.17 dB 1.77 Mbps | 0.16 dB 2.09 Mbps | 0.17 dB 2.11 Mbps |
| 16QAM r3/4 |  | 13.03 dB 5.13 Mbps |  | 13.29 dB 7.61 Mbps | 13.01 dB 8.91 Mbps | 12.65 dB 9.32 Mbps |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163401 / Source 13 |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 | 0.5dB 2.22Mbps | 0.15dB 2.37Mbps | -- | -- | <0dB 2.12Mbps | <0dB 2.14Mbps |
| 16QAM r3/4 | 15.25dB 11.05Mbps | 15.25dB 11.2Mbps | -- | -- | <14dB 9.69Mbps | <14dB 9.71Mbps |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-164464 / Source 10 DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | sTTI 0: 0.05 dB 1.11 Mbps sTTI 1: 0.05 dB 2.37 Mbps sTTI 2: 0.15 dB 2.37 Mbps sTTI 3: 0.35 dB 2.37 Mbps |  | 0.00 dB 1.82 Mbps |  |  |
| 16QAM r3/4 |  | sTTI 0: 12.25 dB 5.52 Mbps sTTI 1: 12.70 dB 11.19 Mbps sTTI 2: 14.70 dB 11.19 Mbps sTTI 3: -- |  | 12.55 dB 8.43 Mbps |  | 12.50 dB 9.70 Mbps |
| 64QAM r5/6 |  | sTTI 0: 20.05 dB 9.30 Mbps sTTI 1: 25.15 dB 18.75 Mbps sTTI 2: -- sTTI 3: -- |  | 22.75 dB 14.10 Mbps |  |  |
| R1-164063, R1-162114 / Source 5 25 PRB, 10 MHz system bandwidth. Shared DMRS for 4-symbol. DMRS occupies the 1st symbol in sTTI 0, and is shared by four 2-symbol sTTIs, (i.e. sTTI 0~3, which are consecutive deployed in time domain) |  |  |  |  |  |  |

Table C3-6 ETU, 60 km/h, 50 PRB

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 3.5 dB 2.52 Mbps |  |  | 3.3 dB 4.32 Mbps | 3.2 dB 4.32 Mbps |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-160779 / Source 11 DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 3.5 dB 4.19 Mbps |  |  |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-160779 / Source 11 DM-RS included in every third sTTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER |  | TTI length=1 |  | TTI length=2 |  | TTI length=3 |  | TTI length=4 |  | TTI length=7 |  | TTI length=14 |  |
| QPSK r1/3 |  |  |  | 0.61 dB 2.52 Mbps |  | 0.48 dB 3.36 Mbps |  |  |  | 0.51 dB 4.32 Mbps |  | 0.23 dB 4.32 Mbps |  |
| 16QAM r3/4 |  |  |  | 12.59 dB 11.3 Mbps |  | 12.52 dB 15.1 Mbps |  |  |  | 13.05 dB 19.4 Mbps |  | 13.29 dB 19.4 Mbps |  |
| 64QAM r5/6 |  |  |  | 19.99 dB 18.9 Mbps |  | 20.02 dB 25.2 Mbps |  | 21.27 dB 28.4 Mbps |  | 26.25 dB 32.4 Mbps |  | 28.21 dB 32.4 Mbps |  |
| R1-164115 / Source 14 DM-RS included in every TTI Full symbols |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.08 dB 3.78 Mbps | 2.04 dB 4.20 Mbps |  | 1.33 dB 4.32 Mbps |  |
| 16QAM r3/4 |  | 13.66 dB 17.0 Mbps | 13.58 dB 18.9 Mbps | 13.92 dB 19.8 Mbps | 14.16 dB 21.1 Mbps |  |
| 64QAM r5/6 |  | 20.97 dB 28.3 Mbps | 21.22 dB 31.5 Mbps | 22.19 dB 33.1 Mbps |  |  |
| R1-164115 / Source 14 DM-RS included in every TTI Hybrid symbols TTI length=2,3,4,7: one hybrid symbol |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  |  |  |  |
| 16QAM r3/4 |  |  |  |  | 13.15 dB 19.4 Mbps |  |
| 64QAM r5/6 |  |  |  |  | 20.52 dB 32.4 Mbps |  |
| R1-164115 / Source 14 DM-RS included in every TTI Hybrid symbols TTI length=4,7: two hybrid symbols |  |  |  |  |  |  |  |  |  |  |  |  |

Table C3-7 ETU, 120 km/h, 25 PRB

| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 1.84 dB 1.26 Mbps | 1.67 dB 1.68 Mbps | 1.65 dB 1.89 Mbps | 1.53 dB 2.16 Mbps | 1.15 dB 2.16 Mbps |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163316 / Source 8 |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  | -0.57 dB 1.57 Mbps |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163820 / SOURCE 4 DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  | -0.56 dB 1.57 Mbps |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163820 / SOURCE 4 Two UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.4 dB 1.03 Mbps | 0.35 dB 1.48 Mbps | 0.4 dB 1.7 Mbps | 0.5 dB 1.97 Mbps | 0.4 dB 2.01 Mbps |
| 16QAM r3/4 |  | 13.5 dB 5.45 Mbps | 13.5 dB 7.4 Mbps | 13.5 dB 8.28 Mbps | 15.1 dB 9.59 Mbps | 14 dB 9.62 Mbps |
| 64QAM r5/6 |  | 23.5 dB 9.47 Mbps | 23.5 dB 12.53 Mbps | 30 dB 14.26 Mbps |  | 23.4 dB 16.49 Mbps |
| R1-163465 / Source 12 DM-RS included in every TTI |  |  |  |  |  |  |

Table C3-8 EPA, 3 km/h, 4 PRB

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  |  |  | 2.54 dB 0.23 Mbps |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163820 / SOURCE 4 DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  | 2.58 dB 0.23 Mbps |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163820 / SOURCE 4 Two UEs share the same DM-RS |  |  |  |  |  |  |

Table C3-9 ETU, 3 km/h, 4 PRB

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  |  |  | 1.50 dB 0.23 Mbps |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163820 / SOURCE 4 DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  | 1.63 dB 0.23 Mbps |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163820 / SOURCE 4 Two UEs share the same DM-RS |  |  |  |  |  |  |

Table C3-10 ETU, 120 km/h, 4 PRB

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  |  |  | 1.84 dB 0.23 Mbps |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163820 / SOURCE 4 DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  | 1.95 dB 0.23 Mbps |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163820 / SOURCE 4 Two UEs share the same DM-RS |  |  |  |  |  |  |

Table C3-11 ETU, 120 km/h, 50 PRB

| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 0.64 dB 2.52 Mbps | 0.52 dB 3.36 Mbps |  | 0.66 dB 4.32 Mbps | 0.20 dB 4.32 Mbps |
| 16QAM r3/4 |  | 12.83 dB 11.3 Mbps | 12.80 dB 15.1 Mbps |  | 15.41 dB 19.4 Mbps | 15.98 dB 19.4 Mbps |
| 64QAM r5/6 |  | 21.85 dB 18.9 Mbps | 21.88 dB 25.2 Mbps | ∞ dB 0 Mbps | ∞ dB 0 Mbps | ∞ dB 0 Mbps |
| R1-164115 / Source 14 DM-RS included in every TTI Full symbols |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.10 dB 3.78 Mbps | 2.07 dB 4.20 Mbps |  | 1.32 dB 4.32 Mbps |  |
| 16QAM r3/4 |  | 13.82 dB 17.0 Mbps | 13.80 dB 18.9 Mbps | 14.76 dB 19.8 Mbps | 16.36 dB 21.1 Mbps |  |
| 64QAM r5/6 |  | 22.12 dB 28.3 Mbps | 22.80 dB 31.5 Mbps | ∞ dB 0 Mbps |  |  |
| R1-164115 / Source 14 DM-RS included in every TTI Hybrid symbols TTI length=2,3,4,7: one hybrid symbol |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  |  |  |  |
| 16QAM r3/4 |  |  |  |  | 13.24 dB 19.4 Mbps |  |
| 64QAM r5/6 |  |  |  | 19.75 dB 26.6 Mbps | 20.71 dB 32.4 Mbps |  |
| R1-164115 / Source 14 DM-RS included in every TTI Hybrid symbols TTI length=4,7: two hybrid symbols |  |  |  |  |  |  |

Table C3-12 EVA, 60 km/h, 50 PRB

| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4(*) | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 0.57 dB 2.52 Mbps | 0.47 dB 3.36 Mbps |  | 0.51 dB 4.32 Mbps | 0.25 dB 4.32 Mbps |
| 16QAM r3/4 |  | 12.68 dB 11.3 Mbps | 12.64 dB 15.1 Mbps | 13.14 dB 19.4 Mbps | 13.33 dB 19.4 Mbps | 13.65 dB 19.4 Mbps |
| 64QAM r5/6 |  | 20.16 dB 18.9 Mbps | 20.19 dB 25.2 Mbps | 21.54 dB 28.4 Mbps | 28.18 dB 32.4 Mbps | 31.96 dB 32.4 Mbps |
| R1-164115 / Source 14 DM-RS included in every TTI Full symbols (*) shared symbol 4(3) for 16QAM r3/4, no shared symbol for 64QAM r5/6 |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.39 dB 3.78 Mbps | 1.32 dB 4.20 Mbps |  | 0.77 dB 4.32 Mbps |  |
| 16QAM r3/4 |  | 13.11 dB 17.0 Mbps | 13.10 dB 18.9 Mbps | 13.50 dB 19.8 Mbps | 13.83 dB 21.1 Mbps |  |
| 64QAM r5/6 |  | 20.51 dB 28.3 Mbps | 20.75 dB 31.5 Mbps | 21.78 dB 33.1 Mbps |  |  |
| R1-164115 / Source 14 DM-RS included in every TTI Hybrid symbols TTI length=2,3,4,7: one hybrid symbol |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  |  |  |  |
| 16QAM r3/4 |  |  |  | 12.84 dB 17.0 Mbps | 12.93 dB 19.4 Mbps |  |
| 64QAM r5/6 |  |  |  |  | 20.27dB 32.4Mbps |  |
| R1-164115 / Source 14 DM-RS included in every TTI Hybrid symbols TTI length=4,7: two hybrid symbols |  |  |  |  |  |  |

Table C3-13 EPA, 3 km/h, 12 PRB

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  |  |  | 2.24 dB 0.453 Mbps |  |  |  |  |  |  |  |  |  |
| 16QAM r3/4 |  |  |  | 14.12 dB 2.57 Mbps |  |  |  |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  | 21.57 dB 4.48 Mbps |  |  |  |  |  |  |  |  |  |
| R1-164549 / SOURCE 4 DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.25 dB 0.302 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 14.21 dB 1.71 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 21.95 dB 2.99 Mbps |  |  |  |  |
| R1-164549 / SOURCE 4 Two UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.25 dB 0.227 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 14.22 dB 1.28 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 22.15 dB 2.24 Mbps |  |  |  |  |
| R1-164549 / SOURCE 4 Three UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.27 dB 0.181 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 14.27 dB 1.03 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 22.28 dB 1.79 Mbps |  |  |  |  |
| R1-164549 / SOURCE 4 Four UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.28 dB 0.151 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 14.31 dB 0.855 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 22.89 dB 1.49 Mbps |  |  |  |  |
| R1-164549 / SOURCE 4 Five UEs share the same DM-RS |  |  |  |  |  |  |

Table C3-14 ETU, 60 km/h, 12 PRB

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 0.55 dB 0.450 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 13.02 dB 2.57 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 21.56 dB 4.47 Mbps |  |  |  |  |
| R1-164549 / SOURCE 4 DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.55 dB 0.300 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 13.14 dB 1.71 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 22.64 dB 2.99 Mbps |  |  |  |  |
| R1-164549 / SOURCE 4 Two UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.60 dB 0.225 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 13.66 dB 1.28 Mbps |  |  |  |  |
| 64QAM r5/6 |  | N/A |  |  |  |  |
| R1-164549 / SOURCE 4 Three UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.64 dB 0.180 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 13.78 dB 1.02 Mbps |  |  |  |  |
| 64QAM r5/6 |  | N/A |  |  |  |  |
| R1-164549 / SOURCE 4 Four UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.71 dB 0.150 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 14.77 dB 0.855 Mbps |  |  |  |  |
| 64QAM r5/6 |  | N/A |  |  |  |  |
| R1-164549 / SOURCE 4 Five UEs share the same DM-RS |  |  |  |  |  |  |

Table C3-15 ETU, 120 km/h, 12 PRB

| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 0.56 dB 0.450 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 13.32 dB 2.56 Mbps |  |  |  |  |
| 64QAM r5/6 |  | 25.40 dB 4.48 Mbps |  |  |  |  |
| R1-164549 / SOURCE 4 DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.57 dB 0.300 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 13.50 dB 1.70 Mbps |  |  |  |  |
| 64QAM r5/6 |  | N/A |  |  |  |  |
| R1-164549 / SOURCE 4 Two UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.73 dB 0.226 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 15.72 dB 1.28 Mbps |  |  |  |  |
| 64QAM r5/6 |  | N/A |  |  |  |  |
| R1-164549 / SOURCE 4 Three UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.74 dB 0.180 Mbps |  |  |  |  |
| 16QAM r3/4 |  | 15.98 dB 1.03 Mbps |  |  |  |  |
| 64QAM r5/6 |  | N/A |  |  |  |  |
| R1-164549 / SOURCE 4 Four UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER Throughput | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.08 dB 0.151 Mbps |  |  |  |  |
| 16QAM r3/4 |  | N/A |  |  |  |  |
| 64QAM r5/6 |  | N/A |  |  |  |  |
| R1-164549 / SOURCE 4 Five UEs share the same DM-RS |  |  |  |  |  |  |

Table C3-15 EPA, 60 km/h, 25 PRB

| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 2.5dB 2.22Mbps | 2.2dB 2.37Mbps | -- | -- | 1.75dB 2.12Mbps | 1.7dB 2.14Mbps |
| 16QAM r3/4 | 16.25dB 11.05Mbps | 16.25dB 11.2Mbps | -- | -- | 14.8dB 9.69Mbps | 14.2dB 9.71Mbps |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-164464 / Source 10 DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  |  |  |  |
| 16QAM r3/4 |  |  |  | 11.9 dB 8.43 Mbps |  | 12.3 dB 9.70 Mbps |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-162114 / Source 5 25 PRB, 10 MHz system bandwidth. Shared DMRS for 4-symbol sTTI. |  |  |  |  |  |  |

Table C3-16 ETU, 3 km/h, AMC, with interference UEs

| SNR | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 dB |  |  |  | 0.9 Mbps |  | 0.38 Mbps |
| 5 dB |  |  |  | 1.85 Mbps |  | 1.2 Mbps |
| 10 dB |  |  |  | 3.4 Mbps |  | 2.3 Mbps |
| 15 dB |  |  |  | 5.1 Mbps |  | 3.9 Mbps |
| 20 dB |  |  |  | 7.1 Mbps |  | 5.8 Mbps |
| 25 dB |  |  |  | 8.35 Mbps |  | 7.65 Mbps |
| R1-164820 / Source 5 12 PRB, 10 MHz system bandwidth Aggressive TBS selection and 16 RVs for 4-symbol sTTI; TBS selection with 10% initial BLER and 4 RVs for 1ms TTI. |  |  |  |  |  |  |

Table C3-17 EVA, 120 km/h, 50 PRB

| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4(*) | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 0.59 dB 2.52 Mbps | 0.49 dB 3.36 Mbps |  | 0.68 dB 4.32 Mbps | 0.21 dB 4.32 Mbps |
| 16QAM r3/4 |  | 12.94 dB 11.3 Mbps | 12.95 dB 15.1 Mbps | 15.67 dB 19.4 Mbps | 16.10 dB 19.4 Mbps | 17.08 dB 19.4 Mbps |
| 64QAM r5/6 |  | 22.24 dB 18.9 Mbps | 22.27 dB 25.2 Mbps | ∞ dB 0 Mbps | ∞ dB 0 Mbps | ∞ dB 0 Mbps |
| R1-164115 / Source 14 DM-RS included in every TTI Full symbols (*) shared symbol 4(3) for 16QAM r3/4, no shared symbol for 64QAM r5/6 |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.42 dB 3.78 Mbps | 1.35 dB 4.20 Mbps |  | 0.78 dB 4.32 Mbps |  |
| 16QAM r3/4 |  | 13.36 dB 17.0 Mbps | 13.36 dB 18.9 Mbps | 14.41 dB 19.8 Mbps | 16.54 dB 21.1 Mbps |  |
| 64QAM r5/6 |  | 22.77 dB 28.3 Mbps | 22.55 dB 31.5 Mbps | ∞ dB 0 Mbps |  |  |
| R1-164115 / Source 14 DM-RS included in every TTI Hybrid symbols TTI length=2,3,4,7: one hybrid symbol |  |  |  |  |  |  |
| SNR at 10% BLER Throughput at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  |  |  |  |
| 16QAM r3/4 |  |  |  | 12.82 dB 17.0 Mbps | 13.09 dB 19.4 Mbps |  |
| 64QAM r5/6 |  |  |  |  | 20.50 dB 32.4 Mbps |  |
| R1-164115 / Source 14 DM-RS included in every TTI Hybrid symbols TTI length=4,7: two hybrid symbols |  |  |  |  |  |  |

## C4	Link-level evaluation sPUCCH

Table C4-1 EPA, 1bit, 3km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 0 |  | -3 | -2.3 | -8 |
| R1-162399 and R1-164640 / SOURCE 1 2 symbols and 4/4 symbols PUCCH details in R1-164640 7 symbols PUCCH format 1a/1b without frequency hopping |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
|  | 4.4 |  |  |  |  |
| R1-165305 / Source 8 Without frequency hopping Reuse legacy PUCCH 1a structure |  |  |  |  |  |

Table C4-2 EPA, 1bit, 60 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 0 |  | -3 | -2.3 | -8 |
| R1-162399 and R1-164640 / SOURCE 1 2 symbols and 4/4 symbols PUCCH details in R1-164640 7 symbols PUCCH format 1a/1b without frequency hopping |  |  |  |  |  |

Table C4-3 EPA, 2bit, 3km/h

| 1 symbol |  | 2 symbols |  | 3 symbols |  | 4 symbols |  | 7 symbols | 14 symbols |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0.5 |  |  |  | -2.5 |  | -0.3 |  |  |
| R1-162399 and R1-164640 / SOURCE 1 2 symbols and 4/4 symbols PUCCH details in R1-164640 7 symbols PUCCH format 1a/1b without frequency hopping |  |  |  |  |  |  |  |  |  |  |
| 1 symbol |  | 2 symbols |  | 3 symbols |  | 4 symbols |  | 7 symbols | 14 symbols |  |
|  |  | 5.88 |  |  |  | 2.92 |  | 0.14 | -5.46 |  |
| R1-163402 / Source 13 Without frequency hopping Reuse legacy PUCCH structure |  |  |  |  |  |  |  |  |  |  |
| 1 symbol |  | 2 symbols |  | 3 symbols |  | 4 symbols |  | 7 symbols | 14 symbols |  |
|  |  |  |  |  |  | 0.19 |  | -2.32 | -5.46 |  |
| R1-163402 / Source 13 With frequency hopping Reuse legacy PUCCH structure |  |  |  |  |  |  |  |  |  |  |
| 1 symbol |  | 2 symbols |  | 3 symbols |  | 4 symbols |  | 7 symbols | 14 symbols |  |
|  |  | 4.9 |  |  |  |  |  |  |  |  |
| R1-165305 / Source 8 Without frequency hopping Reuse legacy PUCCH 1b structure |  |  |  |  |  |  |  |  |  |  |
| 1 symbol | 2 symbols |  | 3 symbols |  | 4 symbols |  | 7 symbols |  |  | 14 symbols |
|  |  |  |  |  | -1.0 |  | -2.9 |  |  | -8.8 |
| R1-165212 / Source 15 Without frequency hopping Reuse legacy PUCCH 1b structure |  |  |  |  |  |  |  |  |  |  |
| 1 symbol | 2 symbols |  | 3 symbols |  | 4 symbols |  | 7 symbols |  |  | 14 symbols |
|  |  |  |  |  | -3.7 |  | -5.8 |  |  | -8.8 |
| R1-165212 / Source 15 With frequency hopping Reuse legacy PUCCH 1b structure |  |  |  |  |  |  |  |  |  |  |

Table C4-4 EPA, 2bit, 60 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 0.5 |  | -2.5 | -0.3 |  |
| R1-162399 and R1-164640 / SOURCE 1 2 symbols and 4/4 symbols PUCCH details in R1-164640 7 symbols PUCCH format 1a/1b without frequency hopping |  |  |  |  |  |

Table C4-5 ETU, 1bit, 3km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 0 |  | -2.9 | -2.9 | -7.8 |
| R1-162399 and R1-164640 / SOURCE 1 2 symbols and 4/4 symbols PUCCH details in R1-164640 7 symbols PUCCH format 1a/1b without frequency hopping |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| 6.1 |  | -0.6 | -2.5 |  |  |
| R1-163520 / Source 4 Sequence-based PUCCH details in R1-163520 |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
|  |  | 1.7 | -2.5 |  |  |
| R1-163520 / Source 4 sPUCCH (similar to legacy) details in R1-163520 |  |  |  |  |  |

Table C4-6 ETU, 1bit, 60 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 0 |  | -2.9 | -2.9 | -7.8 |
| R1-162399 and R1-164640 / SOURCE 1 2 symbols and 4/4 symbols PUCCH details in R1-164640 7 symbols PUCCH format 1a/1b without frequency hopping |  |  |  |  |  |

Table C4-7 ETU, 2bit, 3km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 0.5 |  | -2.4 | -0.7 |  |
| R1-162399 and R1-164640 / SOURCE 1 2 symbols and 4/4 symbols PUCCH details in R1-164640 7 symbols PUCCH format 1a/1b without frequency hopping |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
|  | 4.31 |  | 1.23 | -1.4 |  |
| R1-163402 / Source 13 Without frequency hopping Reuse legacy PUCCH structure |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
|  |  |  | -0.56 | -2.98 | -6.34 |
| R1-163402 / Source 13 With frequency hopping Reuse legacy PUCCH structure |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| 6.5 |  | -0.4 | -2.1 |  |  |
| R1-164545 / Source 4 Sequence-based PUCCH details in R1-164545 |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
|  |  | 2.4 | -1.8 |  |  |
| R1-164545 / Source 4 sPUCCH (similar to legacy) details in R1-164545 |  |  |  |  |  |

Table C4-8 ETU, 2bit, 60 km/h

| 1 symbol |  | 2 symbols |  | 3 symbols |  | 4 symbols |  | 7 symbols | 14 symbols |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0.5 |  |  |  | -2.4 |  | -0.4 |  |  |
| R1-162399 and R1-164640 / SOURCE 1 2 symbols and 4/4 symbols PUCCH details in R1-164640 7 symbols PUCCH format 1a/1b without frequency hopping |  |  |  |  |  |  |  |  |  |  |
| 1 symbol |  | 2 symbols |  | 3 symbols |  | 4 symbols |  | 7 symbols | 14 symbols |  |
|  |  | 4.17 |  |  |  | 1.13 |  | -1.5 |  |  |
| R1-163402 / Source 13 Without frequency hopping Reuse legacy PUCCH structure |  |  |  |  |  |  |  |  |  |  |
| 1 symbol |  | 2 symbols |  | 3 symbols |  | 4 symbols |  | 7 symbols | 14 symbols |  |
|  |  |  |  |  |  | -0.6 |  | -2.28 | -5.96 |  |
| R1-163402 / Source 13 With frequency hopping Reuse legacy PUCCH structure |  |  |  |  |  |  |  |  |  |  |
| 1 symbol | 2 symbols |  | 3 symbols |  | 4 symbols |  | 7 symbols |  |  | 14 symbols |
|  |  |  |  |  | 0 |  | -3.0 |  |  | -8.6 |
| R1-165212 / Source 15 Without frequency hopping Reuse legacy PUCCH 1b structure |  |  |  |  |  |  |  |  |  |  |
| 1 symbol | 2 symbols |  | 3 symbols |  | 4 symbols |  | 7 symbols |  |  | 14 symbols |
|  |  |  |  |  | -3.2 |  | -5.6 |  |  | -8.6 |
| R1-165212 / Source 15 With frequency hopping Reuse legacy PUCCH 1b structure |  |  |  |  |  |  |  |  |  |  |

Table C4-9 EVA, 2bit, 60 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 5.38 |  | 2.53 | -0.37 |  |
| R1-163402 / Source 13 Without frequency hopping Reuse legacy PUCCH structure |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
|  |  |  | 0.24 | -2.32 | -5.76 |
| R1-163402 / Source 13 With frequency hopping Reuse legacy PUCCH structure |  |  |  |  |  |

Table C4-10 EPA, 3 bits, 3 km/h

| 1 symbol |  | 2 symbols |  | 3 symbols |  | 4 symbols |  | 7 symbols | 14 symbols |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | -3.1 | -6.6 |  |
| R1-165421/ Source 8 Frequency hopping Reuse legacy PUCCH 3 structure |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1.3 (2 RS) 2 (1 RS) |  | -1.0 |  |  |  |
| R1-165421 / Source 8 Without frequency hopping Reuse legacy PUCCH 3 structure |  |  |  |  |  |  |  |  |  |  |

Table C4-11 EPA, 5 bits, 3 km/h

| 1 symbol |  | 2 symbols |  | 3 symbols |  | 4 symbols |  | 7 symbols | 14 symbols |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | -2.5 | -6.0 |  |
| R1-165421/ Source 8 Frequency hopping Reuse legacy PUCCH 3 structure |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 2.1 (2 RS, no OCC) 2.6 (2 RS, 2xOCC) 2.9 (1 RS) |  | -0.2 |  |  |  |
| R1-165421 / Source 8 Without frequency hopping Reuse legacy PUCCH 3 structure |  |  |  |  |  |  |  |  |  |  |

Table C4-12 EPA, 10 bits, 3 km/h

| 1 symbol |  | 2 symbols |  | 3 symbols |  | 4 symbols |  | 7 symbols | 14 symbols |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | -0.2 | -3.4 |  |
| R1-165421 / Source 8 Frequency hopping Reuse legacy PUCCH 3 structure |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 4.6 (1RS, no OCC) 5.4 (1 RS, 3xocc) 6.9 (2 RS, no OCC) 7.4 (2 RS, 2x OCC ) |  | 3.5 (5 x OCC) 2.7 (no OCC) |  |  |  |
| R1-165421 / Source 8 Without frequency hopping Reuse legacy PUCCH 3 structure |  |  |  |  |  |  |  |  |  |  |

Table C4-13 EPA, 30 bits, 3 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 3.8 | -0.1 |
| R1-165421 / Source 8 Frequency hopping Reuse legacy PUCCH 4 structure |  |  |  |  |  |

Table C4-14 EPA, 40 bits, 3 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5.3 | 1.0 |
| R1-165421 / Source 8 Frequency hopping Reuse legacy PUCCH 4 structure |  |  |  |  |  |

Table C4-15 ETU, 1 bits, 3 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
| 1.1 dB | <- 1dB |  |  |  |  |
| R1-164464 / Source 10 BLER at 1 % SNR |  |  |  |  |  |

Table C4-16 ETU, 2 bits, 3 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 0.9 | -0.5 |  |  |  |
| R1-162508 / Source 4 sPUCCH for CSI details in R1-162508 |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| 3.8 dB | -0.6 dB |  |  |  |  |
| R1-164464 / Source 10 BLER at 1 % SNR |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| 2.25 |  |  |  |  |  |
| R1-164464 / Source 10 Disjoint allocation of 2 PRBs BLER at 1 % SNR |  |  |  |  |  |

Table C4-17 ETU, 4 bits, 3 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 3.1 | 1.1 |  |  |  |
| R1-162508 / Source 4 sPUCCH for CSI details in R1-162508 |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| 7.5 dB | 1.4 dB |  |  | 1.5 dB |  |
| R1-164464 / Source 10 BLER at 1 % SNR |  |  |  |  |  |

Table C4-18 ETU, 7 bits, 3 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 5 | 2.8 |  |  |  |
| R1-162508 / Source 4 sPUCCH for CSI details in R1-162508 |  |  |  |  |  |

Table C4-19 ETU, 8 bits, 3 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 3.75 dB | -2 dB |
| R1-164464 / Source 10 BLER at 1 % SNR |  |  |  |  |  |

Table C4-20 ETU, 11 bits, 3 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 7.6 | 4.5 |  |  | -1.9 |
| R1-162508 / Source 4 sPUCCH for CSI details in R1-162508 |  |  |  |  |  |

Table C4-21 EPA, 1 bits, 3 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
| 2 | <-1dB |  |  |  |  |
| R1-164464 / Source 10 BLER at 1 % SNR |  |  |  |  |  |

Table C4-22 EPA, 2 bits, 3 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
| 4.25 dB | -0.6dB |  |  |  |  |
| R1-164464 / Source 10 BLER at 1 % SNR |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| 2.25 dB |  |  |  |  |  |
| R1-164464 / Source 10 Disjoint allocation of 2 PRBs BLER at 1 % SNR |  |  |  |  |  |

Table C4-23 EPA, 4 bits, 3 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
| 6 dB | 0.6 dB |  |  | 1.25 dB |  |
| R1-164464 / Source 10 BLER at 1 % SNR |  |  |  |  |  |

Table C4-24 EPA, 8 bits, 3 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 3dB |  | -2.5 dB |  |
| R1-164464 / Source 10 BLER at 1 % SNR |  |  |  |  |  |