RAN WG1 Meeting #85					R1-165979

- Nanjing, China, 23rd – 27th May 2016

_______________________________________________________________Agenda item: 7.1.2

Source: Mitsubishi

Title: Evaluation assumptions for the high speed train scenario for around 30GHz (RRH + relay) for NR

| RRH |  |
| --- | --- |
| Layout | As agreed in R1-165484 |
| Inter-RRH distance | As agreed in R1-165484 |
| Carrier frequency | 30GHz |
| Aggregated system bandwidth | Up to 1GHz (DL+UL) |
| Simulation bandwidth | 80 MHz Other values greater than 80MHz can be considered |
| Channel model | CDL-D, with DS=10 ns and K-factor = {7, 13.3}* dB with the following scaling angle spreads Parameter set #; ASD; ASA; ZSA; ZSD. 1; 5; 5; 1; 1. 2; 5; 15; 5; 1. TDL-D with DS=10ns and K-factor = {7, 13.3}* dB * Other values are not precluded. “K-factor” here refers to K_desired in Section 7.7.6 in TR38.900, or “per-channel K factor” in R1-165440. For other candidate values for K-factor, the values in Table 7.5-6 in TR38.900 can also be used as a reference. NOTE: - Companies need to report the Doppler spread or shift model used in the evaluation - If considered necessary, companies should report the intracell delayed signal model considered in the evaluation. For an illustrative example of an intracell delayed signal model, refer to the figure below. - Other channel models are not precluded. If a company proposes a new channel model or alternative set of parameters, the company must provide justification for the proposal. Whether we need a new channel model for the high speed train scenario is FFS |
| RRH and Relay power EIRP NOTE1 | RRH Tx power: 30dBm Relay Tx power： 27dBm System bandwidth: 100 MHz or more -EIRP for RRH should not exceed 69 dBm** -EIRP for relay should not exceed 43 dBm **This value is tentative. Once more concrete inputs are made, this value can be revisited |
| RRH antenna elements | Up to 256 TX/RX antenna elements |
| RRH antenna architecture (M,N,P,Mg,Ng), dH,dV,dH,g,dV,g | dH=dV=0.5 -For unidirectional beam: (8, 16, 2, 1, 1), Other configurations such as (8,8,2,1,1) or (4,8,2,2,2) with dH,g=8.0,dV,g=4.0 are not precluded for unidirectional beam -For bidirectional beam: (4,8,2,2,2) with dH,g=8.0,dV,g=4.0 Other configurations are not precluded for bidirectional beam |
| RRH Antenna element radiation pattern | AE,Vθ''=-min12θ''-900θ3dB2,SLAV, θ3dB=650, SLAV=30 AE,Hφ''=-min12φ''φ3dB2,Am, φ3dB=650, Am=30 A''θ'',φ''=-min-AE,Vθ''+AE,Hφ'',Am GE,max= 8 dBi Other values are not precluded |
| RRH antenna height | 2.5 m |
| RRH receiver noise figure NOTE1 | 7dB |
| Relay |  |
| Relay antenna elements | Up to 256 TX/RX antenna elements |
| Relay antenna architecture (M,N,P,Mg,Ng), dH,dV,dH,g,dV,g | dH=dV=0.5 -For unidirectional beam: (8, 16, 2, 1, 1), Other configurations such as (8,8,2,1,1) or (4,8,2,2,2) with dH,g=8.0,dV,g=4.0 are not precluded for unidirectional beam -For bidirectional beam: (4,8,2,2,2) with dH,g=8.0,dV,g=4.0 Other configurations are not precluded for bidirectional beam |
| Relay antenna element radiation pattern | AE,Vθ''=-min12θ''-900θ3dB2,SLAV, θ3dB=650, SLAV=30 AE,Hφ''=-min12φ''φ3dB2,Am, φ3dB=650, Am=30 A''θ'',φ''=-min-AE,Vθ''+AE,Hφ'',Am GE,max=8 dBi Optional set of parameters AE,Vθ''=-min12θ''-900θ3dB2,SLAV, θ3dB=900, SLAV=25 AE,Hφ''=-min12φ''φ3dB2,Am, φ3dB=900, Am=25 A''θ'',φ''=-min-AE,Vθ''+AE,Hφ'',Am GE,max=5dBi Other values are not precluded |
| Relay antenna height NOTE1 | 2.5 m |
| Relay receiver noise figure NOTE1 | 7dB |
| Channel estimation | Realistic |
| Maximum speed | 500 km/hr |
| Phase noise model, MCS and channel coding |  |
| Phase noise model | Follow the agreement in R1-165685 |
| MCS, channel coding | Companies report coding or modulation schemes with the parameters used in the evaluations. The modulation schemes under study in RAN1 and coding schemes studied in RAN1#84b and RAN1#85 can be considered. For turbo code, QPSK(pi/4) 1/3, QPSK: 1/2, QPSK: 1/3, 16QAM: 1/2 or 2/3; 64QAM: 1/2 or 3/4; 256QAM ¾ are recommended |

Other evaluation assumptions

- Following the working assumption of baseline subcarrier spacing of 15 kHz and scaling factor of 2n) can be considered. Other values are not precluded. Each company should provide the corresponding FFT size, CP length, and TTI.

- Each company should specify whether TDD or FDD is assumed.

- Each company should specify which MIMO mode is used (i.e., open-loop, closed-loop, etc)

NOTE1: The parameter can be used as a system-level or link-level simulation parameter. Link-level simulation is recommended (As agreed in R1-165484) here.