3GPP TSG RAN WG1 Meeting #85		                                                       R1-164208

Nanjing, China 23rd - 27th May 2016

Source:	CATT

Title:	Resource pool enhancement in PC5-based V2V

Agenda Item:	6.2.2.4

Document for:	Discussion and Decision

Introduction

In RAN1 #84bis meeting, following agreements were achieved [1]：

- In V2V, SA resource and data resource are always FDMed from system perspective(Working assumption)

  - If significant issues are found, can consider further supporting TDM

- SA and the associated data are transmitted in the same TTI,

  - When SA and the associated data are transmitted in the same TTI, they can be transmitted in non-adjacent RBs.

- SA and the associated data are transmitted in different TTIs

  - The scheduling timing between SA and associated data is variable

    - In UE-autonomous resource selection mode, the timing is chosen by the transmitting UE from a configurable range

    - In eNB-scheduling mode, the timing is determined by eNB

    - SA includes information about the scheduling timing

- Note: the association does not necessarily include the case of intention of using the resources for a different TB, if any (which is FFS)

In this contribution, we mainly discuss the further enhancement of resource pools on the basis of the above agreements.

Discussion

  - Whether to need a fixed time window in SA and Data pools

Based on reference [4], V2V service needs to be supported with a maximum latency of 20ms. In R12 D2D, if a single SA and Data resource pool is configured, the following problems will be raised:

1)  When the service data reaching at the end of the SA resource pool or the time period of the Data pool, the terminal must wait for a period of time to send a SA message to the next SA pool. This may not be able to guarantee the delay requirements of the emergency services.

2)  For the normal service its available resources being reduced, it is not conducive to enhance the utilization of resources.

For multiple SA and Data resource pool, more signaling costs will be used and the system resources into a number of resource pools will lead to a decline in resource utilization.

When SA and service data are FDM, but SA and Data are still artificially divided by period. It can reduce the problems of high delay and low efficiency in R12 D2D, but can’t completely solve the mentioned two problems.

Figure 1: The PSCCH pools and the PSSCH pools are TDM and FDM

In Figure 2, the SA and service data are FDM, but there is no fix Resource pool in time for SA and Data. When terminals has service data to send, the time-frequency resources for SA are selected from the subsequent SA resource pool, and the time-frequency resources for data are selected in a relatively wide range to meet the delay requirements. The data transmission delay can be effectively reduced, and the efficiency of the system can be improved.

Figure 2: The PSCCH pools and the PSSCH pools are FDM

Proposal1: No fixed time window in SA and Data pools.

Conclusion

In this contribution, we mainly discuss the resource pool enhancement in PC5-based V2V and give the corresponding proposals.

Proposal1: No fixed time window in SA and Data pools.

References

  - Draft Minutes 3GPP TSG RAN WG1 Meeting #84bis, RAN1 Chairman’s Notes final

  - R1-156607, “Physical layer channel design enhancement to support resource allocation in PC5-based V2V”, CATT

  - R1-162274, “Resource pool enhancement in PC5-based V2V”, CATT

  - 3GPP TR 22.885 V14.0.0 (2015-12) " Technical Specification Group Services and System Aspects;Study on LTE Support for V2X Services"