3GPP TSG RAN WG1 meeting #85	R1-164588

Nanjing, China, 23th - 27th May, 2016

Agenda Item:	7.1.3.1

Source:	Spreadtrum Communications

Title:	Initial LLS evaluation results of NR waveforms under case 1a/1b/2

Document for:	Discussion

Introduction

At the RAN#71 meeting, the new SI of  “Study on NR New Radio Access Technology” [1] was approved with fundamental physical layer signal structure for new RAT as one of the targeting areas, including

- Waveform based on OFDM, with potential support of non-orthogonal waveform and multiple access

  - FFS: other waveforms if they demonstrate justifiable gain

Waveform evaluation assumptions have reached some consensus on evaluation method, evaluation cases, metrics and parameters.

In this contribution, we provide the link level simulation results on CP-OFDM and FBMC under case 1a/1b/2, with BLER-vs-SNR as the metric. Due to the time limitation, not all parameter combinations are visited.

Case 1a Simulation results

Figure 1 shows the BLER-SNR curves under case 1a for CP-OFDM and FBMC, respectively. Three types of channels, combined with different delay spreads (DS’s), including TDL-A with 10ns and 30ns DS, TDL-B with 100ns DS, and TDL-C with 300ns and 1000ns DS are used. It is assumed that CP-OFDM and FBMC use the same 50 PRBs for transmission. The guard band for potential transmission due to the good spectrum shape of FBMC is reserved before future assumptions, e.g., PA non-linearity, power allocation, and etc., are made. Other simulation parameters are summarized in Appendix.

Figure 1 BLER-SNR for CP-OFDM and FBMC under case 1a

From the figure, it is shown that there is not much difference between CP-OFDM and FBMC. As FBMC may outperform CP-OFDM in combating ICI and there is ICI introduced by Doppler spread, we can see that there is minor gain with FBMC over CP-OFDM in some cases; while, the minor performance loss with FBMC would come from the CP-less mechanism which would suffer a bit from the multi-path propagation environment and the equalizer simplification with constant channel response within a subcarrier. However, as FBMC is CP-less, with FBMC we can put 15 symbols into 1 SF instead of 14 symbols with CP-OFDM. Then, compared to CP-OFDM, there would be 7% SE gain with FBMC although there is minor BLER performance loss as shown in Figure 1.

As for different channel types, since we think that TDL-A, TDL-B and TDL-C share close performance, the difference between curves reflects the impact from the delay spread. In the simulation, the channel gain are normalized in the transmission band. Therefore, the instant SNR averaged over the band is the evaluated SNR, and time domain selectivity is not simulated. A larger DS allows more distinct paths because of the rounding operation in sampling. The distinct paths’ number contributes to the frequency diversity, which is further reflected in the negative slope in the figure.

The following observations are made

Observation 1: FBMC shows a close BLER compared to CP-OFDM, under case 1a.

Observation 2: Larger delay spread shows higher frequency diversity order for wide-band transmission, under case 1a.

Case 1b simulation results

Figure 2 shows the BLER-SNR curves under case 1b for CP-OFDM and FBMC, respectively. Three types of channels, combined with different DS’s, including TDL-A with 10ns and 30ns DS, TDL-B with 100ns DS, and TDL-C with 300ns and 1000ns DS are used. It is assumed that all the three waveforms use the same 4 PRBs for transmission. As case 1b is an uplink scenario, an extra DFT precoder is performed on the subband subcarriers after the layer mapper. Other simulation parameters are summarized in Appendix.

Figure 2 shows that there is not much difference between CP-OFDM and FBMC.

As for different channel types, a larger delay spread tends to generate more severe frequency selectivity, to which the single-carrier mode is sensitive. Meanwhile, the channel gain in the subband is normalized, so a channel with a short delay spread behaves more as an AWGN channel under the target SNR. As delay spread increases, the frequency selectivity degrades the performance.

Figure 2 BLER-SNR for CP-OFDM and FBMC under case 1b

The following observations are made

Observation 3: FBMC shows a close BLER compared to CP-OFDM, under case 1b.

Observation 4: Larger delay spread shows more severe frequency selectivity and further degrades the performance, under case 1b.

Case 2 simulation results

Figure 3 BLER-SNR for CP-OFDM and FBMC under case 2

Figure 3 shows the BLER-SNR curves under case 2 for CP-OFDM and FBMC, respectively, with interference from a different numerology and the guard subcarrier number being 0, denoted as GS0 OFDM, and GS0 FBMC, respectively. CP-OFDM without interference, denoted as OFDM, is also shown in the figure as a reference. Three types of channels, combined with different DS’s, including TDL-A with 10ns and 30ns DS, TDL-B with 100ns DS, and TDL-C with 300ns and 1000ns DS are used. It is assumed that all the three waveforms use the same 4 PRBs for transmission. The interfering numerology takes the same bandwidth adjacent to the target transmission. Other simulation parameters are summarized in Appendix.

From the figure, it shown that the interference from a different numerology brings the conventional CP-OFDM around 0.3dB loss, while FBMC achieves a close performance to non-interfered CP-OFDM.

As for different channel types, a larger delay spread tends to generates more severe frequency selectivity, which affects multi-carrier mode as well, but not as much as single-carrier mode.

The following observations are made

Observation 5: Interference from a different numerology brings CP-OFDM around 0.3dB loss, under case 2.

Observation 6: FBMC shows a close performance compared to non-interfered CP-OFDM, under case 2.

Conclusions

In the contribution, we put forward the following observations on waveform evaluation for new RAT:

Observation 1: FBMC shows a close BLER compared to CP-OFDM, under case 1a.

Observation 2: Larger delay spread shows higher frequency diversity order for wide-band transmission, under case 1a.

Observation 3: FBMC shows a close BLER compared to CP-OFDM, under case 1b.

Observation 4: Larger delay spread shows more severe frequency selectivity and further degrades the performance, under case 1b.

Observation 5: Interference from a different numerology brings CP-OFDM around 0.3dB loss, under case 2.

Observation 6: FBMC shows a close performance compared to non-interfered CP-OFDM, under case 2.

References

RP-160671, “New SID Proposal: Study on New Radio Access Technology”, NTT DOCOMO, 3GPP TSG RAN Meeting #71, Göteborg, Sweden, 7.-10. March, 2016

R1-163558, “Way forward on assumptions for waveform evaluation”, Huawei, HiSilicon.

R1-163889, “Way forward on assumptions for waveform evaluation”, Huawei, HiSilicon, InterDigital.

R1-163935, “Way forward on evaluation assumptions for NR waveform”, Huawei, HiSilicon, InterDigital.

Appendix

The simulation parameters are listed in

| Assumptions | Value |
| --- | --- |
| Carrier Frequency | 4GHz |
| System Bandwidth | 10 MHz |
| Subcarrier Spacing | 15kHz |
| Guard Time Interval | 4.7us (interval of LTE normal CP) for CP-OFDM 0 us for FBMC |
| FFT Size | 1024 |
| Data Transmission Bandwidth | Case 1a: 50 PRBs Case 1b: 4 PRBs Case 2a: 4 PRBs |
| Interference for Case 2 | 4 PRB with FFT size 512, subcarrier spacing 30kHz, time domain symbol duration including CP scaled by 2. |
| Antenna Configuration | 1T1R |
| Rank per UE | 1 |
| MCS | 16QAM: 1/2 |
| Control Overhead | Zero |
| Channel Estimation | Ideal |
| Channel Equalization | Case 1a/2: frequency domain ZF Case 1b: frequency domain linear MMSE |
| Channel Model | TDL-A with 10/30 ns DS, TDL-B with 100 ns DS, and TDL-C with 300/1000 ns DS Mobility: 3km/h |
| Channel Normalization | Channel gain normalized in the transmission band. Instant channel gain averaged over the transmission band is equal to 1. |
| FBMC Parameter | OQAM with Phydyas prototype Overlap factor 4 Transceiver structure with PPN and channel response assumed constant within a subcarrier |