3GPP TSG RAN WG1 Meeting #85	R1-164818

Nanjing, China, May 23-27, 2016

Agenda Item:	6.2.2.3

Source:	Huawei, HiSilicon

Title:	SPS enhancement for V2V

Document for:	Discussion and decision

# Introduction

At the RAN1 #85 meeting, the following agreements on network control were reached [1]:

- For SPS of V2V traffic for mode-1 SPS on PC5:

  - The eNB may configure multiple SPS configurations for a given UE

    - At least SPS-configuration-specific MCS (if MCS is part of the SPS-configuration) and SPS-configuration-specific periodicity can be configured

      - FFS if/which other SPS parameters can differ across the SPS-configurations

    - The eNB can dynamically trigger/release the different SPS-configurations by use of (E)PDCCH

      - Details of the trigger/release are FFS

  - Working assumption: The UE can indicate to the eNB that is does not intend to transmit data before a transmission associated to an SPS configuration

    - FFS any details of the signaling protocol

    - FFS whether eNB acknowledgment of the UE indication is needed

In this contribution, details of Semi-Persistent Scheduling (SPS) and fast Scheduling Request (SR) for V2V, e.g. process, signaling are presented.

# SPS

In [2], SPS was introduced to reduce the mode 1 signaling overhead. In this section, more details, e.g. process, signaling, DCI contents are provided.

## SPS in LTE

The SPS process in LTE mainly includes some operations, e.g. configuration, activation, update and release. A brief description of them is provided below. Only uplink SPS is described since it is more related to V2V compared with downlink SPS.

Configuration:

The eNB provides some SPS parameters, e.g. SPS C-RNTI, interval, to the UE.

Activation:

A DCI scrambled by SPS C-RNTI is sent to UE to activate the SPS by eNB. In addition, the resources for SPS in frequency domain are indicated by the DCI and the resources for SPS in time domain are decided by when the DCI is received and the interval.

Update:

A DCI scrambled by SPS C-RNTI is sent to UE to update the SPS. In addition, some new resources for SPS in frequency domain are decided according to the DCI.

Release:

There are different methods to release the SPS. For instance, a DCI scrambled by SPS C-RNTI can be sent to UE to release the SPS explicitly by eNB. In another way, for UL SPS only, the parameter implicitReleaseAfter is configured to UE which indicates the number of empty transmissions before implicit release by the configuration operation.

## SPS over PC5 for V2V

When the SPS mechanism is used over PC5, some modifications are necessary:

Configuration:

An SPS C-RNTI equivalent, which can be denoted as SPS SL-RNTI, is necessary to indicate the SPS over PC5. In addition, the interval in original LTE stands for the number of subframes between the subsequent two SPS TBs’ initial transmissions, the same meaning can be used here. In another way, the interval here stands for the number of periods between the subsequent two periods in which SPS transmissions occur.

Proposal 1: A SPS SL-RNTI for V2V over PC5 is necessary.

Activation:

A DCI for V2V PC5 scrambled by SPS SL-RNTI is sent by the eNB to the UE to activate the SPS over PC5. In addition, the resources for SPS transmissions in time and frequency domain in a period are indicated in the DCI. When the UE receives the DCI, it can transmit SPS data within the next available period.

Update:

A DCI for V2V PC5 scrambled by SPS SL-RNTI is sent to UE to update the SPS over PC5 by eNB. In addition, some new resources for SPS in time and frequency domain are allocated in the DCI.

Proposal 2: A DCI for V2V scrambled by the SPS SL-RNTI for V2V is used to activate or update the SPS.

Release:

Since the eNB may not be aware of when transmission over PC5 ends, it is unable to release the SPS via a DCI. This process is different from what is done for SPS for a cellular communication. A simple solution is to have the SPS transmission activated for a fixed duration (e.g., 5s or so), with automatic release once the duration expires. Or some explicit signaling (e.g. RRC signaling, MAC CE or UCI) could be sent by the UE to the eNB to release resources earlier. Furthermore, this method can also be used for activation/update operation.

Proposal 3: The release mechanism for SPS over PC5 is implicit (e.g., automatic release after a given duration).

- Explicit signaling can be used for activation/update/release.

## Variable period and message size

For V2V communication, both message periodicity and message size can be variable, and needs to be accommodated by the signaling.

### Variable period

When the period of messages is changed, the SPS configuration needs to be modified. The UE needs to notify the eNB with, e.g. the new period, to adjust the configuration of SPS. According to the existing procedure, the eNB can reconfigure the SPS via higher layer signaling and can indicate the new periodicity. For V2X, multiple SPS configurations with different periods can be preconfigured for a UE by eNB and one will be activated. When the period is changed, eNB will choose a suitable one and configure it for UE via DCI.

Proposal 4: When the period is changed, the UE notifies the eNB with the new period.

Proposal 5: The period information is indicated in the DCI to address the variable period of messages.

Note that in practice, the traffic periodicity can vary from 100ms to 1s, with a periodicity of 100ms. Thus, only 10 values (4 bits) are needed to communicate the new periodicity.

When multiple SPS are configured for a UE, it is necessary to differentiate them so that the UE can active/update/release the correct SPS process. One possibility is to use different SPS SL-RNTIs. Another possibility is to include a new field with an SPS process index in the DCI.

Proposal 6: SPS index in DCI and/or dedicated SPS SL-RNTI is used to differentiate SPS processes.

### Variable message size

For V2X, there could still be some packet size variations. For instance, a packet may not always be 190 bytes, and the size may slightly vary. This can be taken care of by either padding, or by changing the MCS to adapt the coded payload size to the semi-persistent allocated resources. For instance, if a higher MCS order is used to fit a 300-byte packet in a resource allocation for 190 bytes, there is little system performance difference, as seen in Appendix.

Note that, when the difference between message sizes is not big, a variable MCS can be used. However, when the difference is large, the SPS resources are inadequate in size even with variable MCS. An alternate approach is that the eNB can configure multiple SPS processes to accommodate the variable message size. For example, let assume that an X-byte message is followed by four Y-byte messages (X > Y) and that messages are generated with a 100ms interval. The eNB can configure two sets of SPS resources: one with a periodicity of 100ms for the Y-byte packet, and one with a periodicity of 500ms for (X-Y) bytes. The resource allocation must ensure the two resources are adjacent in order to preserve the single carrier property as shown in Figure 1. If the message size to be transmitted has a small variation, variable MCS can also be used with multiple SPS.

Figure 1. Multiple SPS processes

Proposal 7: The variable message size is addressed by the following mechanisms:

- Configuration of 2 SPS processes (e.g., when the security certificate is transmitted vs. 'regular' transmissions)

- Adapting the MCS(e.g.,  when the variation in packet size are more limited)

- Up to the eNB implementation to decide when to use which mechanism

If a message of unexpected size and/or at an unexpected time needs to be sent by the UE, the eNB would first send a DCI providing resources to the UE.

In addition, for Rel.12 D2D, the MCS is configured via RRC signaling. For V2X, different MCS can be configured for different SPS processes to accommodate for e.g., different reliability requirements. Thus, the MCS field should be sent via the DCI.

Proposal 8: The DCI explicitly contains the MCS.

# Fast SR

Although SPS is introduced, dynamic scheduling is also necessary in certain scenarios. For instance, when the interval between two messages is changed (e.g., a vehicle accelerating after a traffic light), it is better to wait until the vehicle has reached a relatively steady speed before allocating semi-persistent resources.

As was described in [2], “fast SR” is an attractive way to reduce the mode 1 signaling overhead and latency for dynamic scheduling. In addition, for the UE-triggered SPS activation/update/release according to Proposal 4, the mechanism of fast SR can also be used. In this section, details on fast SR are provided.

## SR process in LTE

The SR related process in LTE mainly includes some operations, e.g. configuration, transmission and reception. A brief description of them is provided below.

Configuration:

Some SR related parameters are sent to the UE by the eNB to allocate the resource for SR in frequency domain, the cyclic shift and orthogonal code sequence and the resources in time domain (period and offset).

Transmission:

When the UE has no UL resources to transmit its data, it can send an SR according to the related parameters configured by eNB to be granted some UL resources.

Reception:

When the eNB detects an SR, it allocates resources to the UE with the corresponding cyclic shift and orthogonal code sequence. The eNB only allocates a few resources so that the UE can transmit at least its Buffer Status Report (BSR). Then more resources will be allocated by the eNB according to the BSR.

## Fast SR for V2V

The V2V message size is largely predictable (e.g. 190 bytes and 300 bytes), and periodic. This can be used to reduce the signalling overhead and latency: for instance, the BSR does not need to be transmitted all the time. Similarly, the SR can be modified to take advantage of the V2V traffic characteristics (fast SR).

Configuration:

For fast SR, multiple SR resources (e.g. PRBs) can be configured to indicate different typical message sizes (e.g., one for 190 bytes and one for 300 bytes) by using either a predefined or eNB-configured mapping between the SR and the typical message sizes.

Transmission:

When the UE has no UL resources to transmit its data, an SR can be chosen from the configured/predefined ones according to the amount of data and be sent to the eNB.

Reception:

When the eNB detects an SR with the corresponding cyclic shift and orthogonal code sequence, it knows that the UE has data to transmit (just like a regular SR), and also the amount of data to transmit. Thus some resources for sidelink can be immediately allocated to the UE, thus eliminating the need to transmit the BSR, reducing both signaling overhead and latency.

Proposal 9: Fast SR is used to reduce overhead of signaling and latency by eliminating the need to transmit the BSR.

# Conclusion

In this contribution, it was observed that mode-1 outperforms mode-2. SPS and fast SR are discussed to enhance mode 1 performance and reduce overhead. We propose:

Proposal 1: A SPS SL-RNTI for V2V over PC5 is necessary.

Proposal 2: A DCI for V2V scrambled by the SPS SL-RNTI for V2V is used to activate or update the SPS.

Proposal 3: The release mechanism for SPS over PC5 is implicit (e.g., automatic release after a given duration).

- Explicit signaling can be used for activation/update/release.

Proposal 4: When the period is changed, the UE notifies the eNB with the new period.

Proposal 5: The period information is indicated in the DCI to address the variable period of messages.

Proposal 6: SPS index in DCI and/or dedicated SPS SL-RNTI is used to differentiate SPS processes.

Proposal 7: The variable message size is addressed by the following mechanisms:

- Configuration of 2 SPS processes (e.g., when the security certificate is transmitted vs. 'regular' transmissions)

- Adapting the MCS(e.g.,  when the variation in packet size are more limited)

- Up to the eNB implementation to decide when to use which mechanism

Proposal 8: The DCI explicitly contains the MCS.

Proposal 9: Fast SR is used to reduce overhead of signaling and latency by eliminating the need to transmit the BSR.

# References

“RAN1 Chairman’s Notes”, RAN1 #85, Busan, Korea, Apr. 11-15, 2016.

R1-160282, “Enhancement on mode 1”, Huawei, HiSilicon, RAN1 #84, St Julian’s, Malta, Feb. 15-19, 2016.

# Appendix: Simulation results of AMC

Figure  shows the performance of adaptive MCS on PRR in mode 1 and Figure  shows that in mode 2. The PRR with 16QAM 1/2 coding (12RB, legend “AMC”) of 300 byte packet size is a little better than that with QPSK 1/2 coding (16RB, legend “noAMC”).

Figure 2. The effects of MCS on PRR in the in-coverage scenario.

Figure 3. The effects of MCS on PRR in the out-of-coverage scenario.