3GPP TSG RAN WG1 Meeting #86	R1- 166178

Gothenburg, Sweden, August 22-26, 2016

Agenda Item:	7.2.11.2

Source:	Huawei, HiSilicon

Title:	Physical layer design for multicast in NB-IoT

Document for:	Discussion and decision

# Introduction

In RAN Meeting #72, a new work item of the enhancement for NB-IoT has been approved [1]. One of the objectives for the work item is the narrowband support for multi-cast.

Multicast:

Extend Rel-13 SC-PTM to support multi-cast downlink transmission (e.g. firmware or software updates, group message delivery) for NB-IoT [RAN2 lead, RAN1, RAN4, RAN3]

-	Introduction of necessary enhancements to support narrowband operation, e.g. support of NPDCCH, and coverage enhancement, e.g. repetitions

In this contribution, the consideration on basic physical layer design is provided to support multi-cast in NB-IoT based on the Rel-13 SC-PTM mechanism.

# Background

The basic procedure of Rel-13 SC-PTM is illustrated in Figure 1. As shown in Figure 1, two types of logical channels, SC-MCCH and SC-MTCH, are introduced to support multi-cast. The data of one multi-cast session is transmitted by one SC-MTCH, which is conveyed by PDSCH in the physical layer. The configuration parameters for each SC-MTCH transmission such as the group RNTI of SC-MTCH and the DRX pattern of the SC-MTCH, is transmitted as the IE SC-MTCH-Info in the SC-MCCH. SC-MCCH is used to broadcast a list of the configuration of SC-MTCHs of all multi-cast sessions.

Figure 1 Procedure of Rel-13 SC-PTM

The SC-MCCH is transmitted periodically with a repetition period (RP) and the contents in SC-MCCH is only allowed to be updated on the boundary of a pre-configured modification period (MP), when new multi-cast sessions need to be added or any on-going multi-cast session needs to be reconfigured or removed.

# Basic design of SC-PTM for NB-IoT

## SC-MCCH transmission in NB-IoT

In Rel-13 SC-PTM, a large number of multi-cast sessions need to be supported in LTE. The adding of new sessions and reconfiguring or removing of on-going sessions would happen frequently. Therefore, good flexibility is needed in Rel-13 SC-PTM and dynamic scheduling of SC-MCCH content is introduced.

However, this is quite different in NB-IoT multi-cast, where the software update application is the major target scenario. The software should not be updated frequently and in typical case should be updated perhaps every couple of months. Hence, the update of the SC-PTM configuration information, e.g. adding of new sessions and removing of on-going session, will not happen frequently.

Furthermore, the coverage of multi-cast service needs to be extended in NB-IoT and hence the transmission duration for each multi-cast session would be quite long. For example, for one software update package of ~200kBytes, and it consists of 2500 TBs assuming maximum physical layer TBS (680bits) is used for each TB. To cover MCL of 164dB, the transmission duration for each TB is about one second and the whole multi-cast session would need about one hour.

According to the above aspects, the multi-cast in NB-IoT does not need frequent and flexible update of configuration information for multi-cast sessions.

Observation 1: Configuration parameters of NB-IoT multi-cast do not need to be updated frequently.

On the other hand, the downlink overhead will increase if dynamic scheduling is applied for SC-MCCH. Some further investigation is provided by comparing dynamically scheduled SC-MCCH and SIB based multicast configuration information transmission, as listed in Table 1.

Assuming the same 680 bits size TB in physical layer is used for the transmission of SC-PTM configuration, by using dynamically scheduled SC-MCCH, 40ms absolute time duration, i.e. 40 PRBs resource, is needed for SI to broadcast the SC-MCCH transmission pattern to UEs with 164 dB MCL. For DCI that schedules SC-MCCH and the NPDSCH that conveys SC-MCCH, the resource used is 256 PRBs and 1280 PRBs, giving a total of 1576 PRBs of resource. If the 680 bits SC-PTM configuration information is carried on SIB, then the resource used is 640 PRBs, i.e. corresponds to 640ms duration of 180 kHz transmission. The details can be found in Annex.

Table 1 Resource consumption for configuration information transmission

| Candidate solutions | Resource used (absolute duration in time domain * 180kHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | SI | DCI | NPDSCH | Total |
| SIB based solution | 640 PRBs | 0 | 0 | 640 PRBs |
| Reuse Rel-13 SC-MCCH transmission | 40 PRBs | 256 PRBs | 1280 PRBs | 1576 PRBs |

It can be observed from Table 1 that the downlink resource consumed by multi-cast configuration information transmission is much less (about 60% less) in the SIB based transmission than that in reusing Rel-13 SC-MCCH mechanism. This is firstly due to no DCI overhead any more. Also, this is because of the larger time diversity gains when SI is used to convey the same amount of configuration information.

As discussed in the companion paper, if legacy Rel-13 SC-PTM is fully reused, it needs introduce the DCIs that schedules SC-MTCH, the DCIs for notification change and the DCIs that schedules SC-MCCH. This would increase the complexity on the UE or have difficult scheduling restrictions to define how to monitor these search spaces. Furthermore, the allocation on downlink resource would be more complicated if too many search spaces are introduced. More details can be found in [2].

Another impact of having dynamic scheduling of SC-MCCH would be regarding the collision handling, as shown in figure 2. It is known that NB-IoT UEs’ bandwidth is one PRB. If the dynamic scheduling is introduced for both SC-MTCH and SC-MCCH, the collision handling between SC-MTCH and SC-MCCH need to be designed. More details can be found in [2].

Figure 2 Example of collision between SC-MCCH and SC-MTCH

Observation 2: Dynamic scheduling of SC-MCCH may increase the downlink overhead due to DCI transmission, and introduce more UE complexity due to additional search space detection and complicated collision handling.

Proposal 1: Do not use dynamic scheduling of SC-MCCH in NB-IoT multi-cast.

Based on the above analysis, the SIB based transmission of multi-cast configuration information fits the requirement of the NB-IoT multi-cast requirement, because as we analyzed the configuration of multi-cast in NB-IoT does not need to be updated frequently.

Proposal 2: Send LS to RAN2 to request using SIB to convey the configuration information of multi-cast sessions.

## SC-MTCH transmission in NB-IoT

Multi-cast traffic in NB-IoT needs to reach the NB-IoT UEs under the worst coverage in the multi-cast group. So, multi-cast traffic in NB-IoT should be repeated for several times to get sufficient transmission reliability.

Repetition at transport block level should be used, which is similar as the mechanism used in Rel-13 NB-IoT, together with the repetition of a larger unit, i.e. session level or sub-session level, should be also provided to support the possibility for some UEs to finish the reception of the multi-cast packet with a shorter latency. More details can be found in [3].

Due to the 680 bits maximum size of physical layer TB in NB-IoT and the fact that one software update packet for NB-IoT may be in the order of hundreds of kB or even several MB, one multi-cast traffic packet may be divided into a large number of TBs on NPDSCH, which will decrease the transmission reliability and efficiency, and add the latency for NB-IoT multi-cast. So it is necessary to support a larger maximum TBS for NB-IoT downlink transmission, like 1000 bits etc.

Proposal 3: Increase the maximum TBS for NB-IoT downlink from 680 bits to 1000 bits.

## Feedback after multi-cast processing in NB-IoT

Considering a software update packet would be 200kB and the maximum TB size for downlink is 80Bytes, the number of TBs of one multi-cast packet is about 2500. So for one UE, there could be always some TBs that the UE does not receive successfully. If there is no feedback mechanism, eNBs will not know which TBs the UE didn’t receive successfully and must re-send the whole multi-cast packet to the UE by unicast. The network load will be seriously increased considering the number of such UEs is large.

So some kind of feedback would be beneficial in NB-IoT multi-cast, which enables eNBs to send only the necessary traffic to UE by unicast after the whole multi-cast session. But it would be complicated to apply physical layer feedback because of the large number of TBs for a multi-cast traffic which are received by a large number of UEs. So, higher layer feedback design, like RLC AM etc, is considered to be more suitable.

Observation 3: Higher layer feedback technique is needed to facilitate unicast re-transmission after multi-cast processing.

# Conclusion

In this contribution, open issues regarding the SC-PTM design for NB-IoT are discussed. And several observations and proposals have been made.

Observation 1: Configuration parameters of NB-IoT multi-cast do not need to be updated frequently.

Observation 2: Dynamic scheduling of SC-MCCH may increase the downlink overhead due to DCI transmission, and introduce more UE complexity due to additional search space detection and complicated collision handling.

Observation 3: Higher layer feedback technique is needed to facilitate unicast re-transmission after multi-cast processing.

Proposal 1: Do not use dynamic scheduling of SC-MCCH in NB-IoT multi-cast.

Proposal 2: Send LS to RAN2 to request using SIB to convey the configure information of multi-cast sessions.

Proposal 3: Increase the maximum TBS for NB-IoT downlink from 680 bits to 1000 bits.

# References

RP-161324, “3GPP Work Item Description: Enhancements of NB-IoT”, RAN #72, Busan, Korea, June 2016.

R1-166199, “Search space and collision handling for multi-cast in NB-IoT”, Huawei, HiSilicon, RAN1#86, Gothenburg, Sweden, August 2016

R1-167165, “Physical layer processing for multi-segments transmission in NB-IoT”, Huawei, HiSilicon, RAN1#86, Gothenburg, Sweden, August, 2016

# Annex

For example, to transmit a 680bit-TBS packet under 164 dB MCL, it will occupy about 640ms if transmitting as SI but need about 1280ms if transmitting as downlink data, as figure 4 shows.

Table 2: Simulation assumptions

| Simulation assumptions |  |
| --- | --- |
| TBS | 680 bits |
| Antenna | 2T1R |
| Modulation | QPSK |
| Resource Mapping | SI: 8 RUs NPDSCH: 10 RUs |
| Repetition Number | SI: 80 repetitions NPDSCH: 128 repetitions |
| Channel | TU 1 |
| Operation Mode | In-band |

Figure 4 Comparison of resource needed between using SIB and dynamically-scheduled SC-MCCH to carry SC-PTM configuration information in NB-IoT.