3GPP TSG RAN WG1 Meeting #86		                                                  R1-166181

Gothenburg, Sweden, August 22-26, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0254 | rev | - | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on NPRACH in TS 36.211 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IoT |  |  |  |  |  |  | Date: |  | 2016-08-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | According to Table 10.1.2.1-1, the subcarrier spacing for NPUSCH, , can be 15 kHz or 3.75 kHz. However, in the description for NPRACH baseband signal generation, is unnecessarily restricted to be only 15 kHz. (Note that the current NPRACH baseband signal generation equation is correct for equal to both 15 kHz and 3.75 kHz.) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the restriction of for NPUSCH in the description for NPRACH baseband signal generation. The range of t for the baseband signal generation expression for NPRACH is also clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is an inappropriate restriction in specification, leading to K being undefined when for NPUSCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

#### 10.1.6.2	Baseband signal generation

The time-continuous random access signal  is defined by

where ,  is an amplitude scaling factor in order to conform to the transmit power  specified in clause  16.3.1 in 3GPP TS 36.213 [4], ,  accounts for the difference in subcarrier spacing between the random access preamble and uplink data transmission,  and the location in the frequency domain controlled by the parameter  is derived from clause 10.1.6.1. The variable is given by Table 10.1.6.2-1.

Table 10.1.6.2-1: Random access baseband parameters

| Preamble format |  |
| --- | --- |
| 0, 1 | 3.75 kHz |

#### < Unchanged parts are omitted >