3GPP TSG RAN WG1 Meeting #86	R1-166201

Gothenburg, Sweden 22nd - 26th August 2016

Agenda Item:	7.2.2.7

Source:	Huawei, HiSilicon

Title:	PSBCH contents and SLSS transmission procedure

Document for:	Discussion and decision

# Introduction

In this contribution, we discuss the following remaining issues for PSBCH contents and SLSS transmission procedure:

- PSBCH contents

- SLSS transmission procedure.

For other synchronization-related issues, please refer to our companion contributions on OOC synchronization priority rule [1], timing alignment [2], and co-existence issues [3].

# Discussion on the remaining issues for PSBCH and SLSS transmission

## PSBCH content

Table 1 lists the existing content of D2D PSBCH.

Table 1. Existing contents of D2D PC5 PSBCH

| Indication content | Number of bits |
| --- | --- |
| Sidelink system bandwidth | 3 |
| TDD UL-DL configuration | 3 |
| Direct frame number | 10 |
| Direct subframe number | 4 |
| In-coverage indicator | 1 |
| Reserved bits | 19 |
| Total | 40 bits |

Since support for the shared carrier for V2V PC5 has been agreed, the information in Table 1 is also needed for V2V PC5 communication. Hence all the existing fields in D2D PSBCH should be supported for the V2V PSBCH.

Proposal 1: the existing content of the D2D PSBCH is kept for V2V.

Some additional information as below can be further considered for the PSBCH:

- eNB timing alignment to GNSS timing. If the eNB is not aligned with GNSS for the shared carrier, cellular traffic needs to be protected from out-of-coverage transmissions. This information can be used to select the priority rule as given in [1].

- If V2V operation is enabled. This bit is used by the eNB to enable UE transmissions on V2V PC5 for out-of-coverage UEs [3]. We denote the transmission indicator as ‘inTx’.

Proposal 2: the following additional information is needed for the V2V PC5 link and is transmitted using reserved bits of the D2D PSBCH:

- Aligned: indicates if PC5 link uses the shared carrier

- inTx: indicates if V2V operation is enabled

## SLSS transmission procedure

### For in-coverage UEs:

For D2D, the eNB can instruct the UE to transmit SLSS or stop SLSS transmission. When the RSRP threshold is below a configured threshold, the UE transmits the SLSS.

We can reuse the existing D2D mechanism.

- If the eNB instructs the UEs to synchronize to eNB timing, the procedure is the same as for D2D.

- If the eNB instructs the UEs synchronize to GNSS, and the GNSS signal is available, there are several options:

  - Option 1: reuse the D2D mechanism and let the eNB instruct the UE to transmit the SLSS.

  - Option 2: UE always transmits PSBCH and SLSS.

  - Option 3: reuse the D2D mechanism of when the RSRP quality is below a configured threshold, the UE transmits the SLSS.

  - Option 4: when the detected S-RSRP quality is below a configured threshold, the UE transmits the SLSS.

- If the GNSS signal becomes unavailable, the UE can either:

- Option 1: report the GNSS signal is lost to the eNB.

- Option 2: switch the V2V PC5 synchronization source to eNB and use eNB based resource pool.

In order to reduce the possible standard effort, we propose to reuse the Rel-12 D2D mechanism:

Proposal 3: for the in-coverage UEs:

- If the eNB instructs the UEs synchronize to GNSS when available:

  - Reuse the D2D mechanism and let the eNB instruct the UE to transmit the SLSS.

- If the eNB instructs the UEs synchronize to GNSS but the GNSS is unavailable, the UEs can report the GNSS signal is lost to the eNB.

### For Out-of-Coverage UEs:

When UE is OOC, the UE uses the priority rules [1] to select the synchronization source. If the detected synchronization sources have the same priority, the UE should first select the highest signal quality synchronization reference source. If the detected S-RSRP is lower than a pre-configured threshold, the UE that synchronizes to GNSS should transmit SLSS.

Proposal 4: for the out of coverage UEs,

- The UE selects the synchronization reference source according to the defined OOC synchronization priority rules.

  - If the detected S-RSRP is lower than a pre-configured threshold, then the UE that synchronizes to GNSS should transmit SLSS.

# Conclusions

In this contribution, we discussed some remaining issues for PSBCH content and SLSS transmission procedure, and we have the following conclusions:

Proposal 1: the existing content of the D2D PSBCH is kept for V2V.

Proposal 2: the following additional information is needed for the V2V PC5 link and is transmitted using reserved bits of the D2D PSBCH:

- Aligned: indicates if PC5 link uses the shared carrier

- inTx: indicates if V2V operation is enabled

Proposal 3: for the in-coverage UEs:

- If the eNB instructs the UEs synchronize to GNSS when available:

  - Reuse the D2D mechanism and let the eNB instruct the UE to transmit the SLSS.

- If the eNB instructs the UEs synchronize to GNSS but the GNSS is unavailable, the UEs can report the GNSS signal is lost to the eNB.

Proposal 4: for the out of coverage UEs,

- The UE selects the synchronization reference source according to the defined OOC synchronization priority rules.

  - If the detected S-RSRP is lower than a pre-configured threshold, then the UE that synchronizes to GNSS should transmit SLSS.

# References

R1-166200, “OOC synchronization priority and SLSS ID set definition”, Huawei, HiSilicon, 3GPP RAN1 #86, Aug. 22-26, 2016.

R1-166168, “Timing alignment of different synchronization sources for V2V”, Huawei, HiSilicon, 3GPP RAN1 #86, Aug. 22-26, 2016.

R1-166125, “Coexistence of cellular and V2V services on the same carrier”, Huawei, HiSilicon, 3GPP RAN1 #86, Aug. 22-26, 2016.