3GPP TSG-RAN WG1 #86	R1-166275

22nd – 26th Aug 2016

Gothenburg, Sweden

Agenda item:	7.2.4.3

Source: 	Qualcomm Incorporated

Title: 	Discussion on DMRS based OL and Semi-OL Transmission

Document for:	Discussion/Decision

# Introduction

In 3GPP RAN1#85, it was agreed to specify the enhancements to support DMRS based semi-open-loop transmission with the existing numbers of CSI-RS ports as well as the newly supported number of CSI-RS ports. The scope for enhancements were provided in [1].

Agreements:

- Specify enhancement to support DMRS-based semi-open-loop transmission with the existing numbers of CSI-RS ports as well as the newly supported number of CSI-RS ports, in the following areas

  - CSI reporting scheme with partial PMI feedback, and/or

  - Transmission method with transmit diversity and/or precoder cycling, with possible enhancements to precoder bundling

  - Associated enhancements to DMRS if required

- Further system level simulations (e.g. considering wideband scheduling and BLER targets at high speed) will be conducted in order to identify the exact scheme to be specified.

In this contribution, we further discuss the required specification enhancements for supporting DMRS based semi-open-loop transmission schemes.

# CSI reporting enhancements

DMRS based open-loop transmission can be implemented in a standards transparent manner by using random beamforming on DMRS on a per RB- or RB bundling basis. However, such a transmission scheme may not perform well without supporting feedback from the UE. In Rel-13, a UE in transmission mode 9 or 10 can be configured without PMI feedback for class B FD-MIMO type if the parameter pmi-RI-Report is not configured by higher layer signalling. This CSI feedback scheme can be reused for DM-RS based open-loop MIMO. However, the determination of RI and CQI for CSI reporting without PMI feedback is based on a selected precoder in the codebook set although the selected precoder is not reported. For open-loop beamforming, it is desirable that RI and CQI are determined based on random precoder cycling. The aim is to sweep many different beam directions over multiple allocated RBs to make the channel appear ergodic. Therefore, a modified CSI feedback scheme to support efficient open-loop transmission is needed.

Class A and Class B K=1

For Rel-10 8Tx, Rel-12 4TX or Rel-13 Class A FD-MIMO, a precoder can be written as a product of two precoding matrices. The first precoding matrix W1 is a long-term and wideband precoding matrix denoting a set of DFT beams. The second precoding matrix W2 is used to select one or multiple beams from the set of the DFT beams of W1 and also perform polarization co-phasing. Therefore, W2 is required to feedback more frequently based on instantaneous channel measurement.

W=W1W2

Therefore, for semi-open-loop transmission for class A and Class B K=1, UE can report a long-term wideband PMI and also RI and CQI using the assumption of cycling for all the W2 codebook.  The benefits of this scheme is to utilize the long-term PMI feedback to improve the beamforming accuracy so that the transmitter energy is focused into the subspace which is correlated to long-term DL channel statistics thus avoiding wasting energy by transmitting along the direction where the channel energy is weak. The reported CQI shall be averaged CQI not dependent on any specific precoder.

Proposal 1: For semi-open-loop transmission with Class A and Class B K=1, UE shall report RI/CQI and optional W1. The reported CSI is based on the assumption of cycling over all the W2 codebook.

Class B K>1 CSI-RS resources

The semi-open-loop transmission can also be supported for Class B with K>1. Two CSI reporting schemes can be considered. The first is that UE will not report CRI but only RI/CQI by assuming cycling over all the configured CSI-RS resources and all the possible precoder for each resource, and the second is that UE may report one or multiple CRIs together with one joint RI/CQI with optional W1. For the second option, CSI reporting is based on cycling over a subset of CSI-RS resources instead of all the resources.  The size of the subset can be semi-statically configured by higher layer signaling or also reported by UE. If the size is equal to K then the second option is same as the first option. Therefore, option 2 is preferred.

Proposal 2: CSI reporting for semi-open-loop transmission with Class B K>1 shall allow cycling over a subset of CSI-RS resources.

# Transmission scheme enhancements

Similar to close-loop beamforming, transparent DMRS precoding can be applied to semi-OL transmission, e.g., using same precoder for both DMRS and PDSCH data symbol. The eNB has full flexibility in choosing a different precoder for each RB to attain maximum diversity gain. To improve channel estimation performance, precoding bundling can also be applied so that the precoder is changed across the RB bundling instead of one RB. To increase diversity gain for small and medium RB allocation, the subcarrier group level bundling is preferred. For example, as shown in Figure 1, the 12 tones per RB can be divided into 3 subcarrier groups each associated with a set of 4 DMRS tones with same subcarrier index. By applying different precoders for each subcarrier group, the increased frequency selectivity can be achieved. However, it is noted that the channel estimation is degraded due to no frequency domain interpolation.

Figure 1. Subcarrier-group level bundling for DMRS based OL transmission

Proposal 3: If transparent DMRS precoding is supported for OL transmission the adaptive bundling size based on BW allocation shall be applied to tradeoff between channel estimation performance and increased frequency selectivity.

The non-transparent DMRS precoding can also be used for semi-OL transmission [2], such as SFBC for rank 1 transmission and LD-CDD or layer permutation for rank 2 transmission. For these schemes, a unified precoding framework can be formulated as the following:

W=W1iRBW2(i)

Where W1iRB denotes DMRS port virtualization matrix which maps P DMRS ports to Nt transmission antennas, the matrix W2(i) denotes P x L precoding matrix mapping L layer PDSCH to P DMRS ports. The matrix W1iRB is used to precode DMRS and W1iRB and W2(i) are jointly used to precode PDSCH.

The precoder cycling can also be applied to W1iRB with a different precoder selected from a set of precoding matrices for each RB or RB bundling. For semi-open-loop transmission with first PMI feedback, the W1iRB can be cycled through the associated W2 codebook.

There are several choices for W2(i), such as SFBC precoding matrix for rank 1, and LD-CDD precoding matrix, e.g., W2i=DiU or RE-level layer permutation matrix, e.g., W2i=U(i) for rank 2. It is noted that the matrix W2(i) may change every PDSCH symbol to achieve more diversity gain.

For PDSCH with large RB allocation, using the per-RE precoder W2(i) is not necessary since the RB or RB bundling level precoder WiRB already introduces sufficient diversity gain. The additional per-RE precoder such as LD-CDD will increase complexity for PDSCH channel estimation. For small or medium RB allocation, it may be beneficial to use the per-RE precoder for increasing frequency selectivity, but the performance shall be compared with the non-transparent DMRS precoding with subcarrier group bundling in order to justify the increase of channel estimation complexity.

Link level simulations are provided to evaluate the performance difference for transparent and non-transparent semi-OL transmission schemes. In the simulation, a 16-TX with port layout (2, 4, 2) virtualized from (8, 4, 2) antenna array is considered. Rel-13 Class A codebook with Config2 is used for PMI feedback. We evaluate the link level throughput at different SNR points. Only SU are considered in the simulation. Other simulation assumption are based on [3].

Figure 2-4 compares the throughput performance of the rank-1 transmission in 30km/h, 60km/h and 120km/h for 3D-UMa channel and 20RB allocation. We observe that non-transparent semi-OL transmission using cycling+SFBC provides a moderate 1-2 dB gains over the transparent transmission using only the cycling.

Figure 5-7 show the performance of the rank 2 transmission for 30km/h, 60km/h and 120 km/h. Different from rank-1, the transparent DMRS precoding outperforms the non-transparent precoding using cycling+LD-CDD. However, the difference is small especially for high speed. It can be concluded that the additional frequency diversity by LD-CDD precoding only provides marginal gain compared to RB-level precoding cycling for rank 2 transmission.

| Figure 2: Link level throughput comparison (16-ports, 30km/h, 3D-UMa, 20 RBs PDSCH, Rank-1) | Figure 3: Link level throughput comparison (16-ports, 60km/h, 3D-UMa, 20 RBs PDSCH, Rank-1) |
| --- | --- |
| Figure 4: Link level throughput comparison (16-ports, 120km/h, 3D-UMa, 20 RBs PDSCH, Rank-1) | Figure 5: Link level throughput comparison (16-ports, 30km/h, 3D-UMa, 20 RBs PDSCH, Rank-2) |
| Figure 6: Link level throughput comparison (16-ports, 60km/h, 3D-UMa, 20 RBs PDSCH, Rank-2) | Figure 7: Link level throughput comparison (16-ports, 120km/h, 3D-UMa, 20 RBs PDSCH, Rank-2) |

Based on the above analysis and link level evaluation results we thus propose to consider the following transmission schemes for the DMRS-based semi-open-loop

Proposal 4: Consider the following DMRS based precoding schemes for semi-OL transmission:

- Rank 1: non-transparent precoding using a combination of cycling and SFBC

- Rank 2: transparent precoding using only cycling

# Conclusions

In this contribution, we discuss DM-RS based open-loop and semi-open-loop transmission. We propose:

Proposal 1: For semi-open-loop transmission with Class A and Class B K=1, UE shall report RI/CQI and optional W1. The reported CSI is based on the assumption of cycling over all the W2 codebook.

Proposal 2: CSI reporting for semi-open-loop transmission with Class B K>1 shall allow cycling over a subset of CSI-RS resources.

Proposal 3: If transparent DMRS precoding is supported for OL transmission the adaptive bundling size based on BW allocation shall be applied to tradeoff between channel estimation performance and increased frequency selectivity.

Proposal 4: Consider the following DMRS based precoding schemes for semi-OL transmission:

- Rank 1: non-transparent precoding using a combination of cycling and SFBC

- Rank 2: transparent precoding using only cycling

# References

- RP-160623, “New WID Proposal: Enhancements on Full-Dimension (FD) MIMO for LTE”, Samsung

- R1-164433, “Discussion on DMRS based OL and semi-OL transmissions”, Qualcomm Inc.