3GPP TSG RAN WG1 #86	R1-166277

August 22nd – 26th, 2016

Gothenburg, Sweden

Agenda item:	7.2.5.1

Source: 	Qualcomm Incorporated

Title: 	Power ratio design for MUST

Document for:	Discussion and Decision

# 1	Introduction

In RAN1 #84b, it is agreed that

- One or more transmission power ratios for each constellation combination are supported

  - FFS: The number of multiple power ratios is down-selected from 1 to 8

In this contribution, we study this problem from receiver complexity perspective.

# 2	Discussion

In the HW implementation of R-ML demapper, a typical operation is c×H, where c is a constellation point, and H is estimated channel, both complex numbers. When the constellation is 256QAM, the I and Q components of c take value from {-15, -13,…,-1,1,…,13,15}. The representation of I and Q of c is a 5 bit signed value. In the case that an arbitrary power ratio is chosen, the bitwidth of c will increase accordingly, and the chip area to implement the multiplier increases proportionally with respect to the bitwidth.

Proposal 1. The number of power ratios to support for each modulation order combination should be kept to the minimum.

Some system simulation results show gain with more than one power ratio. However, as we described above, arbitrary power ratio implies higher implementation complexity in the demapper and translate to larger chip area and/or higher power consumption. To resolve this dilemma, we propose to generate the combined non-uniform constellation using an extended underlying uniform constellation.

An example is given is Figure 1. In this example, we define a combined constellation with QPSK base layer and 16QAM enhancement layer. If the combined constellation is uniform 64QAM, the power ratio will be fixed to be 0.762. In order to have a different power ratio, we can start with a 256QAM constellation (only one quadrant shown in Figure 1). Out of the 64 points in the quadrant, we pick 16 uniform points form a 16QAM constellation. Here in the example, out of I/Q={±1, ±3, ±5, ±7, ±9, ±11, ±13, ±15}, we pick I/Q={±7, ±9, ±11, ±13}. Across all 4 quadrants, 64 points are selected to form a non-uniform 64QAM combined constellation, and Gray mapping is used. The 2 MSB are assigned to base layer and the 4 LSB are assigned to enhancement layer. The resulting power ratio is 0.928. In the near-UE receiver, the demapper can operate as if the combined constellation is the underlying uniform constellation, restricted to the selected constellation points.

By selecting different subsets of points in the underlying uniform constellation, different power ratios can be achieved. Some possible choices are listed in Table 1.

Figure 1. Example of non-uniform combined constellation design using an extended underlying uniform constellation (Selecting QPSK BL + 16QAM EL within 256QAM underlying constellation)

Note that the underlying uniform constellation is not used in modulation and demodulation process. It is only used as an intermediate step to derive the power ratio. After the power ratio is selected, the modulation and demodulation directly work on the non-uniform combined constellation as shown in

Figure 2. Example of resulting non-uniform combined constellation design using an extended underlying uniform constellation (Selecting QPSK BL + 16QAM EL within 256QAM underlying constellation)

Table 1. Example of power ratios with different underlying uniform constellation and combined constellation

| Modulation order pair (BL+EL) | Underlying uniform constellation | Subset selection (I/Q in Figure 1) | Power ratio |
| --- | --- | --- | --- |
| QPSK+QPSK | 16QAM | {±1, ±3} | 4/5=0.8 (uniform case) |
| QPSK+QPSK | 64QAM | {±1, ±5} | 9/13=0.692 |
| QPSK+QPSK | 64QAM | {±3, ±7} | 25/29=0.862 |
| QPSK+QPSK | 64QAM | {±3, ±5} | 16/17=0.941 |
| QPSK+QPSK | 64QAM | {±5, ±7} | 36/37=0.973 |
| QPSK+16QAM | 64QAM | {±1, ±3, ±5, ±7} | 16/21=0.762 (uniform case) |
| QPSK+16QAM | 256QAM | {±1, ±5, ±9, ±13} | 49/69=0.71 |
| QPSK+16QAM | 256QAM | {±3, ±7, ±11, ±15} | 81/101=0.802 |
| QPSK+16QAM | 256QAM | {±3, ±5, ±7, ±9} | 36/41=0.878 |
| QPSK+16QAM | 256QAM | {±5, ±7, ±9, ±11} | 64/69=0.928 |
| QPSK+16QAM | 256QAM | {±7, ±9, ±11, ±13} | 100/105=0.952 |
| QPSK+16QAM | 256QAM | {±9, ±11, ±13, ±15} | 144/149=0.966 |
| QPSK+64QAM | 256QAM | {±1, ±3, ±5, ±7, ±9, ±11, ±13, ±15} | 100/121=0.826 (uniform case) |

Note that the Table 1 contains entries with up to 256QAM underlying uniform constellation. If 1024QAM underlying constellation is supported by the demapper, more power ratios for QPSK+64QAM can be generate as well. For QPSK+QPSK, Table 1 contains entries with 64QAM as underlying uniform constellation. If we start with 256QAM underlying uniform constellation, more power ratios can be generation, though it may not be necessary to have so many choices.

Proposal 2. If more than one power ratio is absolutely necessary, the set of power ratios should be down-selected from Table 1 or its extensions for each modulation order combination.

Since supporting multiple power ratios has hardware design implications, a UE capability should be added to indicate the list of modulation order combination and power ratios it supports to allow different implementation complexity and performance tradeoff.

Proposal 3. UE capability should be defined to indicate the list of modulation order combinations and power ratios it can support.

# 3	Conclusions

Based on the discussion presented in the paper, we have the following proposals.

Proposal 1. The number of power ratios to support for each modulation order combination should be kept to the minimum.

Proposal 2. If more than one power ratio is absolutely necessary, the set of power ratios should be down-selected from Table 1 or its extensions for each modulation order combination.

Proposal 3. UE capability should be defined to indicate the list of modulation order combinations and power ratios it can support.

# 4			References

[1]	R1-163046, “Mechanisms for efficient operation for MUST”, Qualcomm, RAN1 #84bis.