3GPP TSG RAN WG1 #86	R1-166294

August 22nd – 26th, 2016

Gothenburg, Sweden

Agenda item:	7.2.9

Source: 	Qualcomm Incorporated

Title: 	PUSCH in UpPTS

Document for:	Discussion and Decision

# 1	Introduction

In Rel-13, a 6-symbol UpPTS was introduced, primarily to provide more SRS transmission opportunities due to the increased need for 3D-MIMO feature. However, this 6-symbol UpPTS does not support PUSCH. In Rel-14, a new work item was approved to support PUSCH in the 6-symbol UpPTS.

# 2	Discussion

This paper covers the following aspect of supporting PUSCH in UpPTS special subframes:

- Subframe structure

- HARQ timing

## 2.1	Subframe structure

The intention to add PUSCH support in UpPTS is to increase UL data rate by opening up new resources for UL data transmission. However, we should avoid very complex design to achieve this goal to limit the changes to the spec and to the UE and eNB implementation. Therefore, we propose to keep the design simple and do not change the PUCCH or UCI piggy back design. In other words, it is preferable not to add PUCCH in the UpPTS or support UCI piggy back in the PUSCH defined in UpPTS

Proposal 1. Do not add PUCCH, and do not support UCI piggy back in PUSCH in UpPTS.

For the payload of the PUSCH in UpPTS, there are two options, either using a separate TTI, or bundle the TTI with the next U subframe. For the separate TTI case, the scheduling of the PUSCH in each subframe and special subframe are independent. New HARQ timeline is to be defined. However, we have the full flexibility in scheduling UEs. For the bundled TTI case, it is possible to use the same grant to schedule the U subframe after the UpPTS to grant the PUSCH in UpPTS as well. However, the TBS sizes increases and there might be implementation implications. Comparing the two options, we propose to use separate TTIs for better flexibility. The TBS in the PUSCH in UpPTS should scale with the number of OFDM symbols in the PUSCH.

Proposal 2. Separate TTIs in UpPTS and the following uplink subframe. The TBS in the PUSCH of UpPTS scales with the number of OFDM symbols.

For DMRS, a few options are available:

Figure 1. DMRS location options for PUSCH in UpPTS

In options A and B, single DMRS symbols are placed in symbol 10 and symbol 11 respectively, where option A is similar to shortened PUSCH in the first slot (where the first symbol is lost) while option B is similar to shortened PUSCH in the second slot (where the last symbol is lost). For option C, two DRMS symbols are placed in symbol 9 and symbol 12. Option C will be more friendly to high Doppler and MIMO operation, at the cost of higher DMRS overhead.

Proposal 3. Number of DMRS symbol and the location of DMRS symbols are to be studied.

For SRS support, we propose, from a UE perspective, to allow a single SRS symbol when the PUSCH is transmitted in UpPTS, follow PUSCH in a normal U subframe.

Figure 2. SRS location PUSCH in UpPTS

Proposal 4. Follow normal PUSCH to allow one SRS symbol at the last OFDM symbol in the subframe.

## 2.2	HARQ-ACK timeline

The TDD UL/DL configurations are listed in Table 1.

Even though the PUSCH in UpPTS is of only 6 OFDM symbols, and is shorter than one slot, we still define the timeline HARQ timeline assuming 1ms TTI length, decoupling this topic from another ongoing work item on shortened TTIs. The design of PUSCH in UpPTS assuming 1-slot or 2-symbol sTTI lengths should be separately considered in the ULL discussion.

Proposal 5. The HARQ design of PUSCH in UpPTS assumes 1ms TTI length.

The current PUSCH uses synchronous HARQ. It is natural to define the HARQ timeline for PUSCH in UpPTS with the same synchronous principle as well, even though this may involve some changes to the interpretation of parameters such as IPHICH, and fields in DCI such as DAI and UL index.

Proposal 6. Use synchronous HARQ for PUSCH in UpPTS.

Table 1. Uplink-downlink configurations

| Uplink-downlink configuration | Downlink-to-Uplink Switch-point periodicity | Subframe number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 5 ms | D | S | U | U | U | D | S | U | U | U |
| 1 | 5 ms | D | S | U | U | D | D | S | U | U | D |
| 2 | 5 ms | D | S | U | D | D | D | S | U | D | D |
| 3 | 10 ms | D | S | U | U | U | D | D | D | D | D |
| 4 | 10 ms | D | S | U | U | D | D | D | D | D | D |
| 5 | 10 ms | D | S | U | D | D | D | D | D | D | D |
| 6 | 5 ms | D | S | U | U | U | D | S | U | U | D |

Note that PHICH is not available in all subframes. The current PHICH availability is summarized in Table 2. To maintain backward compatibility, we do not want to change this table. Instead, we can use additional PHICH resource in the same resource pool to carry the HARQ ACK/NAK information to support the PUSCH in UpPTS.

Table 2: The factor  for frame structure type 2

| Uplink-downlink configuration | Subframe number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 2 | 1 | 0 | 0 | 0 | 2 | 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 |
| 2 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| 3 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 6 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 |

The PHICH resource is identified by the index pairwhere is the PHICH group number and is the orthogonal sequence index within the group as defined by:

To support PUSCH transmission in special subframes, we will need to define the timeline from UL grant or PHICH to PUSCH transmission. To support the HARQ for PUSCH in UpPTS while maintain backward compatibility, we will need to add one more PHICH resource to provide HARQ ACK/NAK for the new PUSCH. One way to do that is to introduce another index pair (nPHICHgroup,nPHICHseq,1) such that

nPHICHseq,1=IPRB_RANPHICHgroup+nDMRS+noffset mod 2NSFPHICH

With noffset=0 or 1. Effectively, noffset=0 will give us the original PHICH resource, while  noffset=1 will give us the next PHICH resource. The scheduler needs to select the IPRB_RA and nDMRS for different UL grants carefully to avoid PHICH resource collision across UEs.

Alternatively, instead of introducing the noffset concept, the scheduler can always select IPRB_RA and/or nDMRS carefully to avoid collisions. In the following discussion, we still use noffset=1 to identify the PHICH resource for PUSCH in UpPTS. In the case no noffset is introduced, this will be replaced by specific IPRBRA,nDMRS combination for the UL grant for the PUSCH in UpPTS.

Proposal 7. Use extra PHICH resource in the legacy PHICH resource pool to support PUSCH in UpPTS.

To support PUSCH in UpPTS, we will need to define:

- When will the UL grant or the PHICH be transmitted when PUSCH is transmitted in UpPTS.

- When a PUSCH is transmitted, when will the UE expect to receive the corresponding PHICH.

- When transmitting PUSCH, which power control bit will be used.

### 2.2.1	Grant or PHICH to PUSCH transmission timeline

In the current design, the UL grant or PHICH to PUSCH transmission timeline is defined as follows:

For TDD, if the UE is not configured with EIMTA-MainConfigServCell-r12 for any serving cell, and if a UE is configured with one serving cell, or if the UE is configured with more than one serving cell and the TDD UL/DL configuration of all the configured serving cells is the same,

-	For TDD UL/DL configurations 1-6 and normal HARQ operation, the UE shall upon detection of a PDCCH/EPDCCH with uplink DCI format and/or a PHICH transmission in subframe n intended for the UE, adjust the corresponding PUSCH transmission in subframe n+k, with k given in Table 3, according to the PDCCH/EPDCCH and PHICH information.

-	For TDD UL/DL configuration 0 and normal HARQ operation the UE shall upon detection of a PDCCH/EPDCCH with uplink DCI format and/or a PHICH transmission in subframe n intended for the UE, adjust the corresponding PUSCH transmission in subframe n+k if the MSB of the UL index in the PDCCH/EPDCCH with uplink DCI format is set to 1 or PHICH is received in subframe n=0 or 5 in the resource corresponding to , with k given in Table 3.

-	If, for TDD UL/DL configuration 0 and normal HARQ operation, the LSB of the UL index in the DCI format 0/4 is set to 1 in subframe n or a PHICH is received in subframe n=0 or 5 in the resource corresponding to , or PHICH is received in subframe n=1 or 6, the UE shall adjust the corresponding PUSCH transmission in subframe n+7.

-	If, for TDD UL/DL configuration 0, both the MSB and LSB of the UL index in the PDCCH/EPDCCH with uplink DCI format are set in subframe n, the UE shall adjust the corresponding PUSCH transmission in both subframes n+ k and n+7, with k given in Table 3.

Table 3: k for TDD configurations 0-6

| TDD UL/DL Configuration | subframe number n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 4 | 6 |  |  |  | 4 | 6 |  |  |  |
| 1 |  | 6 |  |  | 4 |  | 6 |  |  | 4 |
| 2 |  |  |  | 4 |  |  |  |  | 4 |  |
| 3 | 4 |  |  |  |  |  |  |  | 4 | 4 |
| 4 |  |  |  |  |  |  |  |  | 4 | 4 |
| 5 |  |  |  |  |  |  |  |  | 4 |  |
| 6 | 7 | 7 |  |  |  | 7 | 7 |  |  | 5 |

The description above can be summarized in Table 4.

Table 4: k for TDD configurations 0-6, summarized

| TDD UL/DL Configuration | subframe number n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 (UL grant w/ UL index MSB=1) | 4 | 6 |  |  |  | 4 | 6 |  |  |  |
| 0 (UL grant w/ UL index LSB=1) | 7 | 7 |  |  |  | 7 | 7 |  |  |  |
| 0 (UL grant w/ UL index LSB&MSB=1) | 4&7 | 6&7 |  |  |  | 4&7 | 6&7 |  |  |  |
| 0 (PHICH w/ n=0 or 5 and IPHICH=0, or PHICH w/ n=1 or 6) | 4 | 7 |  |  |  | 4 | 7 |  |  |  |
| 0 (PHICH w/ n=0 or 5 and IPHICH=1) | 7 |  |  |  |  | 7 |  |  |  |  |
| 1 (UL grant or PHICH) |  | 6 |  |  | 4 |  | 6 |  |  | 4 |
| 2 (UL grant or PHICH) |  |  |  | 4 |  |  |  |  | 4 |  |
| 3 (UL grant or PHICH) | 4 |  |  |  |  |  |  |  | 4 | 4 |
| 4 (UL grant or PHICH) |  |  |  |  |  |  |  |  | 4 | 4 |
| 5 (UL grant or PHICH) |  |  |  |  |  |  |  |  | 4 |  |
| 6 (UL grant or PHICH) | 7 | 7 |  |  |  | 7 | 7 |  |  | 5 |

One such design is summarized in Table 5, with changes in red.

- 	For TDD UL/DL configuration 0, use the reserved UL index MSB/LSB=0 to grant the PUSCH transmission in subframe 1 and 6 as shown in Table 5.

-	For TDD UL/DL configuration 0, use PHICH received in subframe 1 and 6 with noffset=1 to trigger synchronous transmission of PUSCH in UpPTS in subframe 6 and 1 respectively, as shown in Table 5.

-	For TDD UL/DL configuration 1~5, for UL grant triggered PUSCH transmission in UpPTS, add UL grant in different DL subframes to distinguish from the existing UL grants. Note these subframes previously do not have UL grant transmission.

-	For TDD UL/DL configuration 1~5, for PHICH triggered PUSCH transmission in UpPTS, use PHICH resource with noffset=1 in the same subframe to distinguish from the existing PHICH.

-	For TDD UL/DL configuration 6, there is no UL index to use in the UL grant DCI. However, for this UL/DL configuration, the DAI field in the DCI is always 0 though we have 2 bits. We can use a reserved combination (DAI=1) to indicate a PUSCH grant in UpPTS.

-	For TDD UL/DL configuration 6, for PHICH triggered retransmission, use PHICH resource with noffset=1 in the same subframe to distinguish from the existing PHICH.

Table 5: k for TDD configurations 0-6 for PUSCH in UpPTS

| TDD UL/DL Configuration | subframe number n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 (UL grant w/ UL index LSB&MSB=0) |  | 5 |  |  |  |  | 5 |  |  |  |
| 1 (UL grant) | 6 |  |  |  |  | 6 |  |  |  |  |
| 2 (UL grant) | 6 |  |  |  |  | 6 |  |  |  |  |
| 3 (UL grant) |  |  |  |  |  |  |  | 4 |  |  |
| 4 (UL grant) |  |  |  |  |  |  |  | 4 |  |  |
| 5 (UL grant) |  |  |  |  |  |  |  | 4 |  |  |
| 6 (UL grant with DAI=1) | 6 |  |  |  |  | 6 |  |  |  |  |
| 0 (PHICH w/ noffset=1) |  | 5 |  |  |  |  | 5 |  |  |  |
| 1 (PHICH) |  | 5 |  |  |  |  | 5 |  |  |  |
| 2 (PHICH) |  |  |  | 8 |  |  |  |  | 8 |  |
| 3 (PHICH) | 11 |  |  |  |  |  |  |  |  |  |
| 4 (PHICH) |  |  |  |  |  |  |  |  |  | 12 |
| 5 (PHICH) |  |  |  |  |  |  |  |  | 13 |  |
| 6 (PHICH w/ noffset=1) |  | 5 |  |  |  |  | 5 |  |  |  |

Proposal 8. Adopt Table 5 for PUSCH to PHICH timeline.

### 2.2.2	PUSCH to PHICH timeline

Currently, for PHICH received in subframe i, it corresponds to a PUSCH transmitted in subframe i-k, with k in Table 6.

Table 6. k for TDD configurations 0-6

| TDD UL/DL Configuration | subframe number i |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 (IPHICH=0) | 7 | 4 |  |  |  | 7 | 4 |  |  |  |
| 0 (IPHICH=1) | 6 |  |  |  |  | 6 |  |  |  |  |
| 1 |  | 4 |  |  | 6 |  | 4 |  |  | 6 |
| 2 |  |  |  | 6 |  |  |  |  | 6 |  |
| 3 | 6 |  |  |  |  |  |  |  | 6 | 6 |
| 4 |  |  |  |  |  |  |  |  | 6 | 6 |
| 5 |  |  |  |  |  |  |  |  | 6 |  |
| 6 (IPHICH=0) | 6 | 4 |  |  |  | 7 | 4 |  |  | 6 |

For TDD UL/DL configuration 0, since there are more UL subframes than DL subframes, special design is needed. For PUSCH transmitted in subframe 2/3/7/8, IPHICH=0 and an HARQ-ACK received in subframe i is associated with the PUSCH transmission in the subframe i-k as indicated in Table 6. For PUSCH transmitted in subframe 4 and subframe 9, IPHICH=1 and an HARQ-ACK received in subframe i is associated with the PUSCH transmission in the subframe i-k as indicated in Table 6.

Now we need to define such k for a PUSCH transmitted in special subframe. One potential design is shown in Table 7:

-	For backward compatibility purpose, we do not want to change the PHICH availability in subframe. We can use one more PHICH resource in existing PHICH by using noffset=1, as described in Section 2.2.1.

Table 7. k for TDD configurations 0-6 for PUSCH in UpPTS

| TDD UL/DL Configuration | subframe number i |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 (noffset=1) |  | 5 |  |  |  |  | 5 |  |  |  |
| 1 (noffset=1) |  | 5 |  |  |  |  | 5 |  |  |  |
| 2 (noffset=1) |  |  |  | 7 |  |  |  |  | 7 |  |
| 3 (noffset=1) | 9 |  |  |  |  |  |  |  |  |  |
| 4 (noffset=1) |  |  |  |  |  |  |  |  |  | 8 |
| 5 (noffset=1) |  |  |  |  |  |  |  |  | 7 |  |
| 6 (noffset=1) |  | 5 |  |  |  |  | 5 |  |  |  |

Note that as a result of the design in Table 5 and Table 7, the retransmission of PUSCH in UpPTS will happen in another UpPTS. This has the benefit of maintain the same number of REs across retransmission, and we propose to keep this property in the design.

Proposal 9. Adopt Table 7 for PUSCH to PHICH timeline.

Proposal 10. The HARQ timeline for PUSCH in UpPTS should satisfy the property that a retransmission of this PUSCH happens in another UpPTS subframe.

### 2.2.3	PUSCH power control timeline

For PUSCH transmission in subframe i, the power control command in subframe i-k will be followed with k defined in Table 8.

Table 8. Current  for TDD configuration 0-6

| TDD UL/DL Configuration | subframe number i |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 (LSB of UL index=0) | - | - | 6 | 7 | 4 | - | - | 6 | 7 | 4 |
| 0 (LSB of UL index=1) | - | - | 7 | - | - | - | - | 7 | - | - |
| 1 | - | - | 6 | 4 | - | - | - | 6 | 4 | - |
| 2 | - | - | 4 | - | - | - | - | 4 | - | - |
| 3 | - | - | 4 | 4 | 4 | - | - | - | - | - |
| 4 | - | - | 4 | 4 | - | - | - | - | - | - |
| 5 | - | - | 4 | - | - | - | - | - | - | - |
| 6 | - | - | 7 | 7 | 5 | - | - | 7 | 7 | - |

To support PUSCH in UpPTS, we will need to define the power control timeline for S subframes as well. One such design in given in

Table 9. New for TDD configuration 0-6 for PUSCH in UpPTS

| TDD UL/DL Configuration | subframe number i |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 (LSB/MSB of UL index=1) | - | 5 | - | - | - | - | 5 | - | - | - |
| 1 | - | 6 | - | - | - | - | 6 | - | - | - |
| 2 | - | 6 | - | - | - | - | 6 | - | - | - |
| 3 | - | 4 | - | - | - | - | - | - | - | - |
| 4 | - | 4 | - | - | - | - | - | - | - | - |
| 5 | - | 4 | - | - | - | - | - | - | - | - |
| 6 (DAI=1) | - | 6 | - | - | - | - | 6 | - | - | - |

Proposal 11. Adopt Table 9 for PUSCH power control timeline.

### 2.2.4 Number of synchronous HARQ processes

The current number of synchronous UL HARQ processes for TDD is defined in Table 10. To support PUSCH in UpPTS, more HARQ processes will be needed.

Table 10: Number of synchronous UL HARQ processes for TDD

| TDD UL/DL configuration | Number of HARQ processes for normal HARQ operation | Number of HARQ processes for subframe bundling operation |
| --- | --- | --- |
| 0 | 7 | 3 |
| 1 | 4 | 2 |
| 2 | 2 | N/A |
| 3 | 3 | N/A |
| 4 | 2 | N/A |
| 5 | 1 | N/A |
| 6 | 6 | 3 |

For TDD UL/DL configuration 0, 1, and 6, one more HARQ process needs to be added, while for TDD UL/DL configuration 2-5, two more HARQ processes need to be added, as shown in Table 11.

Table 11: Number of synchronous UL HARQ processes for TDD when PUSCH in UpPTS is supported

| TDD UL/DL configuration | Number of HARQ processes for normal HARQ operation | Number of HARQ processes for subframe bundling operation |
| --- | --- | --- |
| 0 | 8 | 3 |
| 1 | 5 | 2 |
| 2 | 4 | N/A |
| 3 | 5 | N/A |
| 4 | 4 | N/A |
| 5 | 3 | N/A |
| 6 | 7 | 3 |

# 3	Conclusions

We have the following proposals for PUSCH support in UpPTS:

Proposal 1. Do not add PUCCH, and do not support UCI piggy back in PUSCH in UpPTS.

Proposal 2. Separate TTIs in UpPTS and the following uplink subframe. The TBS in the PUSCH of UpPTS scales with the number of OFDM symbols.

Proposal 3. Number of DMRS symbol and the location of DMRS symbols are to be studied.

Proposal 4. Follow normal PUSCH to allow one SRS symbol at the last OFDM symbol in the subframe.

Proposal 5. The HARQ design of PUSCH in UpPTS assumes 1ms TTI length.

Proposal 6. Use synchronous HARQ for PUSCH in UpPTS.

Proposal 7. Use extra PHICH resource in the legacy PHICH resource pool to support PUSCH in UpPTS.

Proposal 8. Adopt Table 5 for PUSCH to PHICH timeline.

Proposal 9. Adopt Table 7 for PUSCH to PHICH timeline.

Proposal 10. The HARQ timeline for PUSCH in UpPTS should satisfy the property that a retransmission of this PUSCH happens in another UpPTS subframe.

Proposal 11. Adopt Table 9 for PUSCH power control timeline.