3GPP TSG-RAN WG1 #86 	R1-166369

22th – 26th August 2016

Gothenburg, Sweden

Agenda item:	8.1.4.1

Source: 	Qualcomm Incorporated

Title: 	LDPC HARQ

Document for:	Discussion/Decision

# Introduction

For the New Radio Access Technology (NR), there is potential to improve the channel coding across performance and computational complexity while efficiently addressing both blocklength scaling and rate compatibility, including incremental redundancy (IR) HARQ. Details of the code constructions and design are provided in [2].

# Evaluation Assumptions

Proposed LDPC design provided in [1] has been studied for IR HARQ. The first simulation scenario is mentioned in the table below.

Table 1: IR HARQ Study 1

| Channel* | AWGN |
| --- | --- |
| Modulation | QPSK, 64 QAM |
| Coding Scheme | LDPC |
| 1st Tx rate | 1/5, 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9 (r) |
| 2nd Tx rate | 1/10, 1/6, 1/5, 1/4, 1/3, 3/8, 5/12, 8/18 (r/2) |
| Decoding algorithm | SP (flooding; max_iter = 50) |
| Info. block length (bits w/o CRC) | 2000, 6000 |

In the second simulation, the proposed LDPC design is demonstrated to provide fine IR HARQ capability, by considering multiple re-transmissions.

Table 2: IR HARQ Study 2

| Channel* | AWGN |
| --- | --- |
| Modulation | QPSK |
| Coding Scheme | LDPC |
| 1st Tx 17th Tx rate | 30/35,30/36,30/37,30/38,30/39,30/40,30/42,30/44,30/48,30/50, 30/60,30/70,30/80,30/90,30/100,30/120,30/150 |
| Decoding algorithm | SP (flooding; max_iter = 50) |
| Info. block length (bits w/o CRC) | 960 |

# Simulation Results

## K=2000 and K=6000 HARQ with 1st TX R and 2nd TX R/2

The proposed LDPC design in [2] is studied for IR HARQ performance. The simulation set-up is given in table 1. In the curves below the 1st transmission curves are in grey and the 2nd transmission curves are in magenta. Also provided in the legend are the codes used to generate these curves. The notation is as follows: Ckb_nb_K_N_ZX,. The description of this full notation is provided in the appendix in [2]. For the purpose of this study it is only important to focus on K and N which correspond to the given information and code blocklength, respectively. For the second transmission N doubles to indicate the rate of  R/2.

In some plots there are multiple curves corresponding to that particular blocklength and 1st transmission rate. Note that the proposed LDPC design provides multiple codes for a given K and N (cf. [2] for details). All those solutions are illustrated in the plots. The results here lead to the following observations.

Observation 1: The proposed LDPC design in [2] is capable of providing IR HARQ

Observation 2: The IR HARQ gains are sizeable especially at higher rates. Over 6dB gain for high 1st transmission rates (greater than 0.5), around 5 dB of gain for intermediate 1st transmission rates of 0.4 and 0.5. Around 4.5 dB gain for 1st transmission rate of 0.33 and around 4 dB gain for the lowest 1st transmission rate of 0.2.

Observation 3: IR HARQ gains provided by the proposed design in [2], over all prescribed eMBB 1st transmission rates, are large compared to Chase combining (gain of 3 dB) when considering 1st transmission rate of R and 2nd transmission rate of R/2.

### 4-QAM Results

### 64-QAM Results

## Fine IR HARQ

The proposed LDPC design (cf. [2]) is studied for fine IR HARQ capability. In principle, starting at the core base graph (see [2] for details) IR HARQ re-transmission corresponding to one parity-check row is possible through extension. The figure below demonstrates that the fine extension maintains good performance starting at rate 8/9 and all the way down to rate 1/5.

Observation 4: The proposed LDPC design in [2] can also provide fine IR HARQ re-transmissions and the performance is good at each fine extension.

Observation 5: The proposed LDPC design in [2] is also fairly robust to variation in code selection from the family of codes provided by the design.

Proposal 1: LDPC codes can efficiently support incremental redundancy HARQ.

# Conclusions

Observation 1: The proposed LDPC design in [2] is capable of providing IR HARQ

Observation 2: The IR HARQ gains are sizeable especially at higher rates. Over 6dB gain for high 1st transmission rates (greater than 0.5), around 5 dB of gain for intermediate 1st transmission rates of 0.4 and 0.5. Around 4.5 dB gain for 1st transmission rate of 0.33 and around 4 dB gain for the lowest 1st transmission rate of 0.2.

Observation 3: IR HARQ gains provided by the proposed design in [2], over all prescribed eMBB 1st transmission rates, are large compared to Chase combining (gain of 3 dB) when considering 1st transmission rate of R and 2nd transmission rate of R/2.

Observation 4: The proposed LDPC design in [2] can also provide fine IR HARQ re-transmissions and the performance is good at each fine extension.

Observation 5: The proposed LDPC design in [2] is also fairly robust to variation in code selection from the family of codes provided by the design.

Proposal 1: LDPC codes can efficiently support incremental redundancy HARQ.

# References

- RP-160671, New SID Proposal: Study on New Radio Access Technology

- R1-166370 LDPC rate compatible design, RAN1 #86, Gothenburg Sweden