3GPP TSG RAN WG1 #86	R1-166390

August 22nd – 26th, 2016

Gothenburg, Sweden

Agenda item:	8.1.7

Source: 	Qualcomm Incorporated

Title: 	Updated Sub6 DL Full-buffer KPI evaluation for eMBB

Document for:	Discussion and Decision

# 1	Introduction

In RAN1 #84b, the evaluation assumptions for NR test was agreed in [1] including following test scenarios: indoor hotspot, dense urban, rural macro, urban macro and high speed. In RAN1 #85 ([2]), we had provided the initial DL simulation results for the dense urban and rural macro scenarios, assuming V-pol antennas at the eNB.

In this contribution, we update the DL simulation results with more realistic X-pol antennas at the eNB. MU-MIMO enables us to meet IMT-A requirements for the cell-edge and mean cell spectral efficiency.

# 2	Simulation Assumptions

In this contribution, we focus on a single layer NR network with sub-6GHz carrier frequency.  Following simulation assumptions have been used based on [1] with modified parameters marked in bold.

Table 1: System Simulation Assumptions

In the simulations, the following DL centric frame structure is used.  We assume 5% Guard band, 8.6% for CP, 2 symbols for DMRS, 2 symbols in 20 MHz for PDCCH and 1 symbol on the UL for SRS and ACK transmission and 1 symbol for Guard period. We effectively have 9.75 symbols for PDSCH. Within the PDSCH part of the subframe, we assume CSI-RS tones are sent spanning the allocated bandwidth to the user, taking up an additional 4% of overhead. The total overhead turns out to be 42%.

For Rural scenario, we increased the number of SRS symbols to 2, thereby increasing the overhead, but improving UL channel estimation performance.

Figure 1: DL Centric Subframe

SRS Modelling

SRS is used at the BS for channel estimation and reciprocal beamforming for DL. The UE’s transmit power for SRS is controlled through open loop power control. SRS is assumed to be transmitted every subframe, and is transmitted non-orthogonally. SRS is combined across subframes, and a processing gain is derived from link simulations for SRS SNR. This processing gain is a function of both the SRS SINR on the UL and the Doppler. The estimated channel is generated through Gaussian perturbation of the true channel. The estimated channel is then subjected to beam-pruning based on UL SINR and is then used for reciprocal beam-forming on the DL.

Scheduling

We have considered multiple scheduler algorithms for DL simulation. The simulation results in the next section are based on a greedy scheduler algorithm for user-grouping. Scheduling decisions are based on the SRS processing at the BS and periodic filtered Rnn feedback from the UE. Prospective beams are derived from SVD of the whitened channel for each user and are ranked based on the proportional fair metric derived using the expected rate for that beam and the throughput served for that user. The beam with the highest PF metric is then added to the scheduled-set. The next beam in the PF list is added to the scheduled set provided the leakage from this beam on to those in the scheduled-set is below a certain threshold.

The scheduling procedure is as follows. Based on SRS and Rnn feedback available at the end of a subframe, the scheduler picks a user group for scheduling and sends CSI-RS to these users. Based on the CQI feedback, MCS selection is performed and data is scheduled in the following DL centric subframe.

Channel Estimation Error and Calibration Error

Channel estimation on the DL is modeled based on link simulations. As on the UL, the channel perturbation on DL also depends on the true Rnn. A processing gain is applied depending upon the number of DMRS tones (CSIRS tones in the case of CQI computation), and Doppler frequency. For Rnn perturbation, we are using the Wishart method ([4]).

We have also modeled calibration errors at the BS, to perturb the DL beam. At each antenna, the amplitude is perturbed using U[-1, 1] dB and the phase by U[-π/32, π/32].

# 3	Simulation Results

Figure 2 shows the Geometry plot for Dense Urban and Rural scenarios. Figure 3 shows the Path Loss distribution. A significant proportion of UEs in Rural scenario are likely to be power-limited on UL.

Figure 2: Geometry CDF (Dense-Urban and Rural)

Figure 3: Path Loss CDF (Dense-Urban and Rural)

Table 2 below shows the DL spectral efficiency for Dense Urban and Rural Scenarios for 10 UEs/Cell. Simulations have been run over a 20 MHz component carrier and scaled to 200 MHz (dense urban) and 80 MHz (rural). UE and BS powers have also been scaled appropriately. All throughput values and plots capture the impact of DL and UL channel estimation modelling, and DL Rnn estimation modelling.

Observation: NR DL massive MIMO could meet the 3x IMT-Advanced spectral efficiency requirements based on the initial evaluation results. Further improvement of results are possible.

Table 2: DL Spectral Efficiency

|  | Dense Urban |  | Rural |  |
| --- | --- | --- | --- | --- |
|  | Target | Simulations | Target | Simulations |
| Cell Spectral Efficiency [bps/Hz/TRP] | 7.80 | 11.16 | 3.30 | 4.18 |
| Edge User Spectral Efficiency [bps/Hz] | 0.225 | 0.43 | 0.12 | 0.14 |

Figure 4 shows the CDF of the UE SRS transmit power on the UL. SRS power is set through OLPC with a SNR target of -5 dB. None of the UEs is power limited in the Dense Urban scenario. Since we have run the simulations over a 20 MHz CC, we have scaled Max UE power to 17 dBm. Roughly 40% of the UEs are power-limited in the Rural case. In the simulations that follow, all the UEs are sounding over the wideband. In the Dense-urban case, the above spectral efficiency is also valid over a BW of 200 MHz. Figure 5 shows the IOT CCDF on the UL for SRS.

Figure 4: CDF of UE SRS Tx Power over 20 MHz CC (SRS Target SNR (OLPC) = -5 dB)

Figure 5: IOT CCDF

Figure 6 below shows the MU-MIMO performance for the dense-urban scenario, along with the impact of DL and UL estimation errors on the performance, over 200 MHz bandwidth. Figure 7 shows the rank distribution. The median rank chosen for MU-scenario is 7.

Figure 6: UE Throughput CDFs (Dense-Urban) scaled to 200 MHz

Figure 7: CDF of Total Rank at BS (Dense-Urban) (200 MHz)

Similarly, Figure 8 below shows the throughput distribution for the Rural scenario. Due to high Doppler for the outdoor UEs, and power-limited UL for indoor UEs, significant loss is observed due to UL and DL estimation errors.

Figure 9 shows the CDF of the total rank used at the BS. The throughput and rank chosen are both lower than in the dense-urban case.

Figure 8: UE Throughput CDFs (Rural) scaled to 80 MHz

Figure 9: CDF of Total BS Rank (Rural) (80 MHz)

Proposal 1: Consider MU-MIMO for DL spectral efficiency evaluation in NR.

Proposal 2: Consider SRS-based CSI feedback for NR massive MIMO system.

# 4	Conclusions

In this contribution, we evaluated the DL spectral efficiency at 4GHz for NR “dense urban” and “rural macro” evaluation scenarios. It was observed that NR could achieve 3x LTE-advanced spectral efficiency requirement. Based on our evaluation, we recommend the group to consider following proposals:

Proposal 1: Consider MU-MIMO for DL spectral efficiency evaluation in NR.

Proposal 2: Consider SRS-based CSI feedback for NR massive MIMO system.

# 5.	References

- R1-163887,	“Updated summary of evaluation assumptions for NR”, CMCC

- R1-164675, “Initial DL KPI evaluation for eMBB”, Qualcomm Incorporated

- RP-160671, WID, “Study on NR New Radio Access Technology”, NTT DOCOMO

- R1-111031, “On advanced UE MMSE receiver modelling in system simulations”.