3GPP TSG RAN WG1 Meeting #86	    R1-166401

Gothenburg, Swiden, 22nd - 27th August 2016

# Source: 	ZTE, ZTE Microelectronics

# Title:	Simulation results of new waveform in case 2, 3 and 4 (multi-subband cases)

# Agenda Item:	8.1.2.1

# Document for:	Discussion

# Introduction

A study item on Next Generation New Radio Access Technology was agreed in RAN Plenary #71 meeting [1]. In this SID, waveform design based on OFDM is one of fundamental areas that need to focus on. In RAN1#85 meeting, the evaluation assumption was discussed and some agreements were made in [2][3]. Besides, after email discussion, it was also agreed that the RAPP PA model for Downlink and polynomial PA model for Uplink should be used in the waveform simulation. Appendix 1 gives the details of PA models for both Downlink and Uplink.

This contribution gives the simulation results and comparisons between CP-OFDM with WOLA (W-OFDM) and FB-OFDM in multi-subband cases, e.g., case 2, 3 and 4.

# Evaluation case and simulation parameters

The case 2, 3 and in [2] is briefly described as following::

  - Case 2: DL mixed numerology case

Fig.1 Case 2: Downlink mixed numerology case

In case 2 as shown by Fig. 1, the target user, which is either at the edge or within the full-band of the “target” subband (using numerology 1), is interfered by the interfering subband (using numerology 2). Gsubband is the width of guard band between neighbouring subbands and its value depends on the OOB performance of the waveform.

  - Case 3: UL single numerology case (asynchronous reception between UEs)

Fig.2 Case 3: UL single numerology case (asynchronous reception between UEs)

In case 3 as shown by Fig. 2, the target user, which is interfered by two asynchronous UEs located in neighbor subbands. GUE is the width of guard band between neighbouring subbands.

  - Case 4: UL mixed numerology case (synchronous reception between UEs)

Fig.3 Case 4: UL mixed numerology case (synchronous transmission)

In case 4 as shown by Fig. 3, the target user, which is interfered by two UEs (using numerology 2) located in neighbor subbands. GUE is the width of guard band between neighbouring subbands.

The simulation parameters used in this contribution are given in Appendix 2 for case 2 and Appendix 3 for case 3 and 4.

# BLER and spectrum efficiency

The spectrum efficiency is calculated as [2], where  denotes the number of correctly received bits by target user, T is the transmission time of the target user. For case 2, 3 and 4,  is the data bandwidth of the target UE plus guard tones.

The simulations use the waveform transmission/reception as described in [5] for CP-OFDM with WOLA, and the waveform transmission/reception as described in [4] for FB-OFDM. In the simulations of FB-OFDM, zero-forcing is used in polyphase filter on receiver side to suppress inter-symbol interference, and a subframe-level cutting window as shown in [4].

Particularly worth mentioning is that PA effects are considered in all the simulation results given in these contribution.

## Case 2

Fig. 4 and Fig. 5 show the spectrum efficiency and BLER performance for case 2.

Fig.4 Case 2 Spectrum efficiency and BLER comparison: 16QAM 1/2, TDL-C 1000 ns 3 kmh

Fig.5 Case 2 Spectrum efficiency and BLER comparison: 256QAM 3/4, TDL-A 30 ns 3kmh

Fig 5 show that, with 256QAM modulation and 3/4 code rate in TDL-A with 30 ns delay spread channel, the BLER performance and spectrum efficiency of FB-OFDM are much better than those of W-OFDM in case 2.

## Case 3

Fig. 6~Fig. 10 show the spectrum efficiency and BLER performance for case 3.

Fig.6 Case 3: Spectrum efficiency and BLER comparison with neighbour subband of 0 dB power offset and 128 Ts time offset: 64QAM 1/2, TDL-C 300 ns 3 kmh

Fig.7 Case 3: Spectrum efficiency and BLER comparison with neighbour subband of 5 dB power offset and 128 Ts time offset: 64QAM 1/2, TDL-C 300 ns 3 kmh

Fig 8 Case 3: Spectrum efficiency and BLER comparison with neighbour subband of 10 dB power offset and 128 Ts time offset: 64QAM 1/2, TDL-C 300 ns 3 kmh

Fig 9 Case 3: Spectrum efficiency and BLER comparison with neighbour subband of 5 dB power offset and 128 Ts time offset: 256QAM 3/4, TDL-A 30 ns 3 kmh

Fig 10 Case 3: Spectrum efficiency and BLER comparison with neighbour subband of 10 dB power offset and 128 Ts time offset: 256QAM 3/4, TDL-A 30 ns 3 kmh

Fig. 6~Fig. 8 show that, with 64QAM modulation and 1/2 code rate in TDL-C with delay spread 300 ns channel, the BLER performance and spectrum efficiency of FB-OFDM are better than those of W-OFDM in case 3, especially when a small number (e.g., 0 or 2).

Fig. 9 and Fig. 10 show that, with 256QAM modulation and 3/4 code rate in TDL-A with delay spread 30ns channel, the BLER performance and spectrum efficiency of FB-OFDM are remarkable better than those of W-OFDM in case 3, and FB-OFDM can achieve good performance even with only one or two guard subcarriers.

It is noted that the higher the power offset of interfering UE is, the larger SE gain FB-OFDM achieved compared with W-OFDM.

## Case 4

The BLER and SE performance are shown in Fig. 11 ~ Fig. 14 for case 4.

Fig.11 Case 4 Config1: Spectrum efficiency and BLER comparison with neighbour subband of 0 dB power offset: 64QAM 1/2, TDL-C for {300 ns} 3 kmh

Fig.12 Case 4 Config1: Spectrum efficiency and BLER comparison with neighbour subband of 5 dB power offset: 64QAM 1/2, TDL-C for {300 ns} 3 kmh

Fig.13 Case 4 Config1: Spectrum efficiency and BLER comparison with neighbour subband of 10 dB power offset: 64QAM 1/2, TDL-C for {300 ns} 3 kmh

Fig.14  Case 4 Config2: Spectrum efficiency and BLER comparison with neighbour subband of 10 dB power offset: 64QAM 1/2, TDL-C for {300 ns} 3 kmh

Fig. 11~Fig. 14 show that, with 64QAM modulation and 1/2 code rate in TDL-C with delay spread 300 ns channel, the BLER performance and spectrum efficiency of FB-OFDM are remarkably better than those of W-OFDM, especially when a small number (e.g., 0,1 or 2) of guard-tones are used between neighbouring subbands. And it is also observed that the higher the power offset is, the larger SE gain FB-OFDM achieved compared with W-OFDM.

Observation 1: Under the selected simulation assumptions, the BLER performance of FB-OFDM is remarkable better than W-OFDM in case 2, 3 and 4, especially when a small number of guard tones are used.

Observation 2: FB-OFDM achieves much larger gain compared with W-OFDM when the power offset of interfering UE get higher. Even with interfering UEs of 10dB power offset located in neighbour subbands, FB-OFDM only need no more than 2 subcarriers as guard tones.

Observation 3: FB-OFDM is very suit for mixed numerology scenarios and asynchronous-subbands cases due to its highly localized power distribution in frequency domain.

Proposal 1: FB-OFDM should be taken as a waveform candidate in NR.

# Conclusion

This contribution concludes with the following observations and proposal:

Observation 1: Under the selected simulation assumptions, the BLER performance of FB-OFDM is remarkable better than W-OFDM in case 2, 3 and 4, especially when a small number of guard tones are used.

Observation 2: FB-OFDM achieves much larger gain compared with W-OFDM when the power offset of interfering UE get higher. Even with interfering UEs of 10dB power offset located in neighbour subbands, FB-OFDM only need no more than 2 subcarriers as guard tones.

Observation 3: FB-OFDM is very suit for mixed numerology scenarios and asynchronous-subbands cases due to its highly localized power distribution in frequency domain.

Proposal 1: FB-OFDM should be taken as a waveform candidate in NR.

# References

- 3GPP RP-160671, New SID Proposal: Study on New Radio Access Technology, NTT DOCOMO

- 3GPP R1-163558, WF on evaluation assumptions for waveform, Huawei, HiSilicon

- 3GPP R1-165859, WF on further evaluation assumptions for NR waveform, Huawei, HiSilicon, InterDigital, NTT DOCOMO, LGE

- 3GPP R1-166399, Tx&Rx structure of FB-OFDM, ZTE, ZTE Microelectronics

- 3GPP R1-162199, Waveform candidates, Qualcomm

# Appendix 1

- Rapp PA model for Downlink

The following modified Rapp model with AM/AM and AM/PM distortion models for Downlink:

AM/AM:

AM/PM:

x is an amplitude of the input signal, and G = 1.

| Parameter for Rapp model < 6GHz DL |  |  |
| --- | --- | --- |
| Target output power [dBm] | 46 |  |
| Saturation output power [dBm] | 57.6 |  |
| Smoothness factor p | 3 |  |
| Smoothness factor q | 5 |  |
| Fitting parameter A | -0.14 |  |
| Fitting parameter B | 1.2 |  |

polynomial PA model for Uplink

The PA model using AM/AM and AM/PM polynomial approximation is shown in the next figure.

Figure 8: AM/AM and AM/PM curves for the proposed PA model. Measured data in blue, polynomial approximation in red.

The polynomial data in dBm domain is also given below. The format can be directly copied to Matlab. The coefficients are organized as follows: [p9  p8  p7  …  p0] and the PA output y(t) can be computed from input x(t) using the formula
y(t) = p0 + p1∙x(t) + p2∙x(t)2 + p3∙x(t)3 + …

p_am = [7.9726e-12  1.2771e-9  8.2526e-8  2.6615e-6  3.9727e-5  2.7715e-5  -7.1100e-3  -7.9183e-2  8.2921e-1  27.3535];

p_pm = [9.8591e-11  1.3544e-8  7.2970e-7  1.8757e-5  1.9730e-4  -7.5352e-4  -3.6477e-2  -2.7752e-1  -1.6672e-2  79.1553];

The validity of the polynomial model deteriorates below input power -35 to -30 dBm, and it is suggested that fully linear model is applied below such input levels. Also extremely deep in compression, e.g. at input power above 9 dBm, the model does not provide realistic results. This should be fine since the signal peaks would anyway be limited to a reasonable value before entering the PA.

# Appendix 2

Table A1 Parameters for case 2

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| Duplex | FDD/TDD (FDD and TDD have the same performance in these simulation results in this contribution) |
| System Bandwidth | 10 MHz |
| TTI length | 1 ms |
| Subcarrier spacing | 15KHz for target UE; 30KHz for interfering subband. |
| Guard time interval | 4.7us (interval of LTE normal CP). The symbol interval of FB-OFDM is 1/14ms just the same as LTE |
| FFT size | 1024 |
| Data transmission bandwidth | 1RB for target UE; 4RB for interfering subband. |
| Guard tone number (Gsubband) | 0, 2 and 4 subcarriers with 15KHz spacing. |
| Antenna configuration | 1T1R |
| MIMO mode | SISO |
| Rank per UE | Fixed single rank |
| MCS | Fixed. 16QAM: 1/2; 64QAM: 1/2 or 3/4; 256 QAM: 3/4 |
| Control Overhead | Zero |
| Channel estimation | Ideal (without RS) |
| Channel Model | TDL-A for DS {30}ns, TDL-C for DS {300※,1000}ns |
| Others | The window process of CP-OFDM with WOLA in transmitter and receiver is the same as that described in [5]. |

# Appendix 3

Table A2 Parameters for case 3 and case 4

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| Duplex | FDD /TDD |
| Subframe duration | 1 ms as baseline, other duration is FFS (short duration could be considered) |
| Subcarrier spacing | Case 3: 15KHz as baseline, Case 4: Target UE: 15KHz; Interferer pair: {30KHz※, 30KHz※}, {7.5KHz, 7.5KHz} other value for interferers is not precluded. |
| Guard time interval | 6.7% overhead as baseline, other interval is FFS (depend on numerology progress ) |
| System bandwidth & FFT size | 10 MHz, 1024 for 15KHz subcarrier spacing |
| UE bandwidth (data transmission bandwidth plus guard tone bandwidth of the desired UE) | Case3: -720 KHz (48 Subcarriers per user allocated for both target UE and interferer UEs) Case4: Config1※: Target UE: - 720KHz (48 Subcarrier allocated) Interferer users: - 720KHz (per UE) Config2: Target UE: - 2880KHz (192 subcarrier allocated) Interferer users: -2880KHz (per UE) |
| Bandwidth of guard tones between neighboring UEs | {0, 15, 30, 60※, 90, 180}KHz |
| Number of uplink users | 3 (1 target user and 2 interferer users) |
| Power offset of the interferer user | 0 dB, 5 dB, 10 dB※ |
| Antenna configuration | 1T1R※ |
| MIMO mode | SISO |
| MCS | Fixed. 16QAM: 1/2 or 2/3; 64QAM: 1/2※ or 3/4; other is not precluded |
| Control overhead | Zero |
| Time offset of interfering user | Case 3: fixed offset {0, 128※, 512} samples (for 15 KHz subcarrier spacing with 1024 FFT size) Case 4: 0 |
| Channel estimation * | Ideal※ (without RS) |
| Channel model ** | TDL model - All values of DS {10, 30, 100, 300, 1000} ns are evaluated with the selected TDL-DS combinations, i.e. TDL-A for DS {10,30}ns, TDL-B for DS {100 }ns, TDL-C for DS {300※,1000}ns. Companies are allowed to choose additional combination(s) of other DS values and TDL–A and/or TDL-C in TR38.900. ETU/EVA/EPA are optional. Mobility: 3km/h※ or 30 km/h or 120 km/h, higher speed is not precluded. |

NOTE: * For realistic channel estimation, the current LTE DL/UL DMRS is recommended,  and the applied DMRS pattern as well as CE implementation have to be described. Pilot overheads should be  considered into the calculation of user spectrum efficiency.

* *   Power-delay profiles of TDL-{A,B,C} are scenario agnostic.

※     All marked values are  recommended for calibration purpose.