3GPP TSG RAN WG1 Meeting #86                                                            R1-166427

Gothenburg, Sweden, 22nd - 26th August 2016

Source:	ZTE, ZTE Microelectronics

Title:	Discussion and Evaluation for MUST Case 3

Agenda Item:	7.2.5.3

Document for: 	Discussion and Decision

# Introduction

In RAN1# 85, the following items about MUST Case 3 were agreed [1][2].

- For MUST Case 1 and Case 2, multiple power ratios are supported at least for some combinations of MUST-near UE and MUST-far UE modulation orders

  - For case 3, FFS

- For MUST case 1/case 2/case 3, dynamic switching between MUST and non-MUST operation is supported.

- Maximum number of spatial layers for MUST

  - For MUST case 1 and case 2, up to 2 spatial layers for each UE are used.

  - For MUST case 3, the maximum number of spatial layers for a UE should be limited, with details FFS.

In this contribution, we discuss MUST case 3 with some evaluation results. The mechanism for MUST case 3 is proposed. Scheduler implementation for case 3 and corresponding system level results are presented.

# Mechanism for MUST Case 3

The superposition with the same precoding vector is used in MUST Case 1. As a further work, the superposition with different precoding vectors is allowed in MUST Case 3. Although the second kind of superposition increases the probability of pairing, it may increase the implementation complexity of receiver.

For Case 3, when the superposition with mutually orthogonal precoding vectors is used, multi-user signal can be demodulated by MMSE receiver. When the superposition with different and non-orthogonal precoding vectors is used, multi-user signal can be demodulated by ML receiver instead of MMSE receiver. If the number of interfering layers is more than 1, the demodulation operation is more complicated for ML receiver. The efficiency of interference cancellation of ML receiver becomes lower.

When the superposition with mutually orthogonal precoding vectors is used, the power difference on multi-user signal does not contribute to demodulation. When the superposition with different and non-orthogonal precoding vectors is used, the power difference on multi-user signal needs the receiver with high computation complexity.

To reduce the implementation complexity of the receiver for Case 3, our proposals are summarized below.

- The superposition with up to two users is supported by Case 3.

- If the superposition with different and non-orthogonal precoding vectors is used, the rank of each user signal is limited to 1.

- Equal power allocation between the spatial layers should be used.

# Scheduler Implementation for Case 3

Scheduler implementation for MUST Case 3 is discussed in this section.

MUST Case 1 is a special case of Case 3 [3], since Case 1 assumes the same precoding vectors. It is proposed for the scheduler that Case 3 can fall back to Case 1 when Case 1 is more helpful in improving system performance than Case 3. Similarly, SU-MIMO is a special case of Case 1. Case 1 can fall back to SU-MIMO, which has been implemented in the scheduler for Case 1.

In order to facilitate the dynamic switching between Case 3 and Case 1, unified signalling format between Case 3 and Case 1 is needed.

To improve the system performance, we have the following proposals:

- Case 3 can fall back to Case 1.

- Unified signalling design between Case 3 and Case 1 is needed.

Above all, the superposition with different precoding vectors, MUST Case 1 and SU-MIMO are offered as alternative operation modes in the scheduler for Case 3. The scheduling policy can make MUST Case 3 achieve better system performance than SU-MIMO or MUST Case 1.

# System Simulation Results for Case 3

Comparison between the system results on Full-Buffer of MUST Case 3 and Case 1 is given in this section. The deep analysis on the system results is provided.

Performance comparison of MUST Case 3 and Case 1 with 2Tx is shown as Table 1.

Table 1 Performance comparison of MUST Case 3 and Case 1 with 2Tx

| Case | User Throughput (bps) | Baseline (SU-MIMO) | MUST Case 3 | Gain | MUST Case 1 | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| 2Tx | Cell average | 1.3240 | 1.6002 | 20.85% | 1.3886 | 4.87% |
|  | Cell edge | 0.0241 | 0.0268 | 11.31% | 0.0287 | 19.19% |

Performance comparison of MUST Case 3 and Case 1 with 4Tx is shown as Table 2.

Table 2 Performance comparison of MUST Case 3 and Case 1 with 4Tx

| Case | Spectral Efficiency (b/s/Hz/Sec) | Baseline (SU-MIMO) | MUST Case 3 | Gain | MUST Case 1 | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| 4Tx | Cell average | 1.5722 | 2.1235 | 35.06% | 1.6091 | 2.35% |
|  | Cell edge | 0.0337 | 0.0358 | 6.24% | 0.0394 | 17.07% |

As the tables above showed, the gain on cell average spectral efficiency of MUST Case 3 over SU-MIMO is more significant than that of Case 1. On the other hand, the gain on cell edge spectral efficiency of MUST Case 3 over SU-MIMO is lower than that of Case 1.

The proportion of user operation modes in MUST Case 3 and Case 1 with 2Tx is shown as Table 3. These user operation modes include SU-MIMO, multi-user superposition with different precoding vectors and multi-user superposition with the same precoding vector.

Table 3 Proportion of user operation modes in MUST Case 3 and Case 1 with 2Tx

| Case | Operation mode | MUST Case 3 | MUST Case 1 |
| --- | --- | --- | --- |
| 2Tx | SU-MIMO | 5.88% | 40.00% |
|  | MUST with different precoding vectors | 64.05% | - |
|  | MUST with same precoding vector | 30.07% | 60.00% |

The proportion of user operation modes in MUST Case 3 and Case 1 with 4Tx is shown as Table 4.

Table 4 Proportion of user operation modes in MUST Case 3 and Case 1 with 4Tx

| Case | Operation mode | MUST Case 3 | MUST Case 1 |
| --- | --- | --- | --- |
| 4Tx | SU-MIMO | 2.13% | 57.68% |
|  | MUST with different precoding vectors | 80.76% | - |
|  | MUST with same precoding vector | 17.12% | 42.32% |

As seen in Tables 3 and 4, for Case 3, the proportion of SU-MIMO is very small. Most of the user operation modes is MUST with different precoding vectors. A small part of the user operation modes is MUST with the same precoding vector. For Case 1, the superposition with different precoding vectors does not exist. The proportion of SU-MIMO is roughly the same as that of MUST with the same precoding vector. When 2Tx is used, the proportion of MUST with the same precoding vector is somewhat bigger. When 4Tx is used, the proportion of SU-MIMO is slightly bigger.

Based on the performance and proportion of user operation modes, it is believed that the gain on cell average spectral efficiency of MUST Case 3 over SU-MIMO mostly comes from using different precoding vectors. The superposition with the same precoding vector contributes certain gain also.

In addition, the gain on cell edge spectral efficiency of MUST Case 1 over SU-MIMO mostly comes from the superposition with the same precoding vector. The contribution of the superposition with the same precoding vector on the cell average spectral efficiency of MUST Case 1 is limited.

# Conclusions

Firstly, this paper gives some discussion on the mechanism and scheduler implementation for Case 3. To reduce the implementation complexity of the receiver for Case 3, our proposals are summarized below.

Proposal 1: The superposition with up to two users is supported by Case 3.

Proposal 2: If the superposition with different and non-orthogonal precoding vectors is used, the rank of two user signal is limited to 1.

Proposal 3: Equal power allocation between the spatial layers should be used.

Secondly, comparison between the system results of MUST Case 3 and Case 1 is presented. Case 3 has large advantage over Case 1 in improving the cell average spectral efficiency. However, Case 1 has some advantage over Case 3 in improving the cell edge spectral efficiency.

To improve the performance, we have the following proposals:

Proposal 4: Case 3 can fall back to Case 1.

Proposal 5: The unified signaling design between Case 3 and Case 1 is needed.

# References

- 3GPP R1-165796, Way Forward on MUST Power Ratio. MediaTek, China Unicom, CHTTL, Huawei, HiSilicon, HTC, III, Intel, ITRI, LG, NTU, Sharp, Sony, ZTE, RAN1 #85.

- 3GPP R1-165793, Way forward on transmission mode for MUST. ZTE, NEC, MediaTek, Sequans. RAN1 #85.

- 3GPP R1-164434, Superposition transmissions for MUST. Qualcomm Incorporated. RAN1 #85.

# Annex

Table A1:  System-level simulation assumptions of MUST

| Cellular Layout | Hexagonal grid, 3 sectors per site, 7 macro sites |
| --- | --- |
| Inter-site distance | 500 m |
| Minimum distance between BS and UE | 25m |
| Distance-dependent path loss | ITU UMa, with 3D distance between an eNB and a UE applied |
| Penetration loss | For outdoor UEs: 0 dB For indoor UEs: (20+0.5din) dB (din: independent uniform random value between [0, 25] for each link) |
| Shadowing | ITU UMa |
| Fast fading channel between eNB and UE | ITU UMa |
| eNB antenna gain + connector loss | 17 dBi |
| UE antenna gain | 0 dBi |
| Channel Estimation | realistic |
| Channel Measurement | realistic |
| UE speed | 3 km/h |
| Carrier Frequency | 2.0 GHz |
| System bandwidth | 10 MHz |
| Antenna configuration | eNB: 2Tx or 4Tx, 0.5 lambda, cross-polarized UE: 2Rx, 0.5 lambda, cross-polarized |
| Number of UEs per cell | 10 |
| Maximum number of multiplexed UE | 2 1ayer/per UE |
| Delay time of scheduling | 5 ms |
| Traffic model | Wideband scheduling |
| UE dropping | 20% UEs are outdoor; 80% UEs are indoor |
| Total BS TX power (total per carrier) | 46 dBm |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| Antenna Height | 25 m |
| UE antenna Height | 1.5 m |
| Codebook | LTE Rel. 8 |
| OLLA | Yes |
| EVM | EVM is modeled, 8% TX, 4% RX |
| Duration of the simulation | 40s for FTP1, 5s for full buffer |