3GPP TSG RAN WG1 Meeting #86	                                                                                R1-166434

Gothenburg, Sweden 22nd - 26th August 2016

Source:	CATT

Title:	Discussion on contents of scheduling assignment

Agenda Item:	7.2.2.5

Document for:	Discussion and Decision

Introduction

In this contribution, contents of SA are analyzed in detail and some proposals are given.

Discussion

The following fields need to be taken into account in SA.

  - Time frequency location of Data resources

According to the working assumption of last meeting, 2 consecutive PRB pairs in a subframe are used for each PSCCH if the number of SA bits is less than 64 including CRC. And the number of data transmission may vary with some parameters, e.g. traffic priority, message size, UE absolute speed. We think retransmission info should be indicated in SA to improve reliability and maximum of 4 transmission is sufficient for V2V traffic, then the total transmission number for a TB may be {2,3,4}, considering the limitation of SA payload and a variable total transmission number, two transmission resources are suggested to be included in SA.

Here we give two options:

- Option1 (non-wraparound): current transmission resources and the next transmission resources is indicated in SA, and in the last SA (e.g. 2nd of 2, or 3rd of 3), the information field of the next transmission resources is invalid/not needed.

- Option2 (wraparound): current transmission resources and the next transmission resources is indicated in SA, but in the last SA (e.g. 2nd of 2, or 3rd of 3), the initial transmission resource is also indicated. This approach could avoid bit wastage as well as get another chance to indicate resource of the initial transmission and lead to more robust sensing.

Table 1 list the time/frequency content of each options when total transmission number= 2/3/4. Here resource1 denotes initial resource, resource2 denotes the 1st retransmission resource, resource3 denotes the 2nd retransmission resource, resource4 denotes the 3rd retransmission resource.

Table 1 Time/frequency info in SA content

|  |  | Time/frequency info in SA content |  |
| --- | --- | --- | --- |
| total transmission number |  | Option1 | Option2 |
| 2 | 1st SA(prior SA) | (resource1, resource2) | (resource1, resource2) |
|  | 2nd SA(last SA) | (resource2) | (resource2, resource 1) |
| 3 | 1rd SA(prior SA) | (resource1, resource2) | (resource1, resource 2) |
|  | 2nd SA(prior SA) | (resource2, resource3) | (resource2, resource 3) |
|  | 3rd SA(last SA) | (resource3) | (resource3, resource 1) |
| 4 | 1rd SA(prior SA) | (resource1, resource2) | (resource1, resource 2) |
|  | 2nd SA(prior SA) | (resource2, resource3) | (resource2, resource 3) |
|  | 3rd SA(prior SA) | (resource3, resource4) | (resource3, resource 4) |
|  | 4th SA(last SA) | (resource4) | (resource4, resource 1) |

The current transmission times and total transmission number of the TB should be included in SA, by which receiving UE is be able to identify whether the SA transmission is the last one or not. 4 bits is used for this purpose.

Proposal 1: SA contains time/frequency resource resources of 2 times.

Proposal 2: Irrespective the way of SA& associated data transmission, the number of data transmission and SA transmission should always be same for each TB.

Proposal 3: The current transmission times and total transmission number should be indicated, using 4 bits (maximum transmission number=4).

Also note that, from HARQ combing point of view, the receiver could retrieve the RV info from current transmission times and total transmission number.

In the following, we give the detailed analysis on time/frequency indication.

- Time domain resource indication

In V2V resource selection, resource selection (re-selection) is based on the sensing, every transmission resource is based on the sensing of every PRB / contiguous PRBs, and there is no fixed transmitting time window in data pools, restricting a time pattern (the subframe indexes of the retransmission resources are identified by that of the initial resource) may deteriorate the sensing performance. Further, due to the variable transmission times and a larger data window length compared with Rel-12/13 D2D, It is difficult to define a certain resource pattern. So the time domain resources used for each transmission should be indicated independent.

The time domain resource can be indicated by the sub-frame offset for each transmission. That is, for current resource, the sub-frame offset 1 between the SA and the current data transmission resource should be indicated in SA; and for the next transmission resource, the sub-frame offset 2 between the current data transmission resource and the next data transmission resource should be indicated in SA. Further, to decrease the SA payload, we could limit the window length for time scheduling.

The value of time domain indication is given in the following Table 2:

Table 2 the value of time domain indication for data resource

|  |  | SA & associated data in same sub-frames |  | SA & associated data in different sub-frames |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Offset1 | Offset2 | Offset1 | Offset2 |
| prior SA | Option 1 | Not needed(note*) | >0 | >0 | >0 |
|  | Option 2 | Not needed(note*) | >0 | >0 | >0 |
| last SA(nth of n) | Option 1 | Not needed(note*) | =0(note**) | >0 | =0(note**) |
|  | Option 2 | Not needed(note*) | <0 (note***) | >0 | <0 (note***) |

Some notes:

- * For SA & associated data in same sub-frames, the offset 1 is not needed for all cases, only offset 2 needs indication. For SA & associated data in different sub-frames, both offset1 and offset 2 need indication.

- ** With option 1, in last SA, since the information of the next transmission resources is not needed, a default offset zero is used to indicate the invalid offset 2.

- *** With option 2, in last SA, the sub-frame offset 2 between the current data transmission resource and the initial transmission resource is negative, here the absolute value of the offset is indicated in SA. Since the current transmission times and total transmission number are included in SA, then receiving UE can identify whether the data transmission is the last SA or not and can understand the (negative) meaning of this special handle.

So, for SA & associated data in same sub-frames, 5 bits is needed. For SA & associated data in different sub-frames, 10 bits is needed.

Proposal 4: The time domain resources used for each transmission should be indicated independently.

Proposal 5: UE learn the relationship between SA and its associated data from resource pool configuration, not from SA content.

Proposal 6: For SA & associated data in same subframe, one offset value is indicated; for SA & associated data in different subframe, two offset values are indicated.

- The frequency domain resource indication

In V2V resource selection, resource selection (re-selection) is based on the sensing, every transmission resource is based on the sensing of every PRB/sub-band, restricting a frequency hopping rule (the frequency starting location of the retransmission resources are somehow constricted by the initial resource) may deteriorate the sensing performance. Thus frequency hopping cannot be applied, and the frequency domain resources used for transmission and retransmission should be indicated independent,

Considering resource length may vary with some parameters, e.g. message size and so on. Here, both sub-band start location and occupied sub-band number should be explicitly signaled in the SA. The resource allocation granularity is sub-band (see our company contribution [2]), and then the combination of starting sub-band and resource length is determined. For example, if there is totally M sub-bands, then the cost of frequency domain indication for a transmission resource is ceil (log2 (M (M + 1) / 2)).

The indications of the current transmission resource and next transmission resource are given in the following Table 3: for the last transmission of the SA, the “next resource” means “initial transmission resource”.

Table 3 the value of frequency domain indication for data resource

|  |  | SA & associated data contiguous in frequency |  | SA & associated data in different sub-frames |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Current resource | Next resource | Current resource | Next resource |
| prior SA | Option 1 | Not needed(note*) | Starting 2 + length 2 | Starting 1 + length 1 | Starting 2(note**) |
|  | Option 2 | Not needed(note*) | Starting 2 + length 2 | Starting 1 + length 1 | Starting 2(note**) |
| last SA(nth of n) | Option 1 | Not needed(note*) | Starting 2(predefined) + Length 1(note***) | Starting 1 + length 1 | Not needed(note**) |
|  | Option 2 | Not needed(note*) | Starting 2 + length 2 | Starting 1 + length 1 | Starting 2(note**) |

Some notes:

- * For SA & associated data contiguous in frequency, SA location can implicitly indicate starting sub-band location of current transmission resource. Because the resource lengths of all transmission resources for a TB are the same, the resource length of the current transmission resource can be implicitly indicated by the resource length of next transmission resource.

- ** For SA & associated data in different sub-frames, Because the resource lengths of all transmission resources for a TB are the same, frequency domain indication of the next resource need to be indicate by the sub-band start location, which needs ceil (log2 (M)) bits.

- *** With option 1, in the last SA, since the information of the next transmission resources is not needed, for SA & associated data contiguous in frequency, the field is used to indicate the sub-band number of the current transmission; here the starting info use a predefined value(e.g. =0) and omitted by receiving UE. Since the current transmission times and total transmission number are included in SA, then receiving UE can identify whether the data transmission is the last SA or not and can understand the meaning of special handling.

So, for SA & associated data contiguous in frequency, (ceil (log2 (M (M + 1) / 2))) bits is needed. For SA & associated data in different sub-frames, (ceil (log2 (M (M + 1) / 2)) +ceil (log2 (M)) bits is needed.

Proposal 7: The frequency domain resources used for each data transmission should be indicated independently.

Proposal 8: Same number of frequency resource should be used for each data transmission carrying same TB.

Proposal 9: For SA & associated data contiguous in frequency,  full frequency info (sub-band start location and occupied sub-band number) is indicated; for SA & associated data different sub-frames,  full frequency info(sub-band start location and occupied sub-band number) plus “starting” are indicated.

From above analysis, option 1 (non-wraparound) and option2 (wraparound) are using same number of bit, while option2 may get some robust gain for sensing.

Proposal 10: The indication of time/frequency resources in SA can use either option1 (non-wraparound) or option2 (wraparound), while option2 (wraparound) is preferred.

  - Other fields

- Resource reservation

- 4bits can be used to indicate “i” value, details see our company contribution [3].

- Transmit Block Temporary ID

- We think the destination id in R12 D2D is not useful for PC5 V2V. That 8 bit could be used as Transmit Block Temporary ID, with which the receiver could identify the SAs scheduling the same TB. The transmitter should randomly change this ID for each TB.

Proposal 11: Transmit Block Temporary ID (8bits) should be indicated in SA.

- Priority

- According to the working assumption of last meeting, SCI explicitly includes priority information. The bit num is depending on the priority level definition .if PPPP is used to identify the priority level, 3 bits can be used to indicate 8 stratum levels of service, while the priority level in 802.11p is reused for PC5 based V2V,2 bits can be used .

- 3 bits can be used.

- MCS

- Reused the filed in the Rel-12/13 D2D, it is valid for current transmission resource and next transmission resource.

- 5 bits can be used.

- TPC

- Since SA transmission cannot obtain the soft combination gain, the power boosting for SA transmission can be considerate in V2V communication

- 1 bit can be used.

- Reserved bits

- As long as the total bit number < 64 bit, some reserve bits could be explicitly defined in SA. This will make it possible to have some future extension without defining new SCI format.

- 3 reserve bits can be considered.

Proposal 12: Explicitly define reserved bits (e.g. 3bits) in SA for future usage.

  - Summary of SA contents

According to the above discussion, the SA contents can be summarized as Table 4.

Table 4: summary of SA contents in V2V communication

| Field |  | SA&data in Same subframe | SA&data in Different subframe |
| --- | --- | --- | --- |
| Time frequency location of Data resources | Time offset1 between SA and current transmission | 0 | 5 bits |
|  | Time offset2 between current transmission and the next transmission | 5 bits | 5 bits |
|  | Frequency of transmission resource (starting and length) | 4 bits (5 sub-bands) 6 bits | 4 bits (5 sub-bands) 6 bits (10 sub-bands) |
|  | The frequency starting of the other resource | 0 | 3 bits (5 sub-bands) 4 bits (10 sub-bands) |
| Resource reservation | SPS period (i)*100ms | 4 bits | 4bits |
| Transmit Block Temporary ID |  | 8 bits | 8 bits |
| current transmission times and total transmission number |  | 4 bits | 4 bits |
| Priority |  | 3 bits | 3 bits |
| MCS |  | 5 bits | 5 bits |
| TPC |  | 1 bit | 1 bit |
| CRC |  | 16 bits | 16 bits |
| Reserved for future usage |  | 3bits | 3 bits |
| Total |  | 53bits (5 sub-bands) 55bits (10 sub-bands) | 61bits (5 sub-bands) 64bits (10 sub-bands) |

Proposal 13: Table 4 can be used as SA content.

Conclusion

In this contribution, the signaling design aspects for SA contents were discussed. Particularly, we have following proposal:

Proposal 1: SA contains time/frequency resource resources of 2 times of transmissions.

Proposal 2: Irrespective the way of SA& associated data transmission, the number of data transmission and SA transmission should always be same for each TB.

Proposal 3: The current transmission times and total transmission number should be indicated, using 4 bits (maximum transmission number=4).

Proposal 4: The time domain resources used for each transmission should be indicated independently.

Proposal 5: UE learn the relationship between SA and its associated data from resource pool configuration, not from SA content.

Proposal 6: For SA & associated data in same subframe, one offset value is indicated; for SA & associated data in different subframe, two offset values are indicated.

Proposal 7: The frequency domain resources used for each data transmission should be indicated independently.

Proposal 8: Same number of frequency resource should be used for each data transmission carrying same TB.

Proposal 9: For SA & associated data contiguous in frequency,  full frequency info (sub-band start location and occupied sub-band number) is indicated; for SA & associated data different sub-frames,  full frequency info(sub-band start location and occupied sub-band number) plus “starting” are indicated.

Proposal 10: The indication of time/frequency resources in SA can use either option1 (non-wraparound) or option2 (wraparound), while option2 (wraparound) is preferred.

Proposal 11: Transmit Block Temporary ID (8bits) should be indicated in SA.

Proposal 12: Explicitly define reserved bits (e.g. 3bits) in SA for future usage.

Proposal 13: Table 4 can be used as SA content.

References

  - 3GPP TSG RAN WG1 Meeting RAN1 #85 chairman notes, Nanjing, China 23rd - 27th May 2016

  - R1-166436 , Resource pool enhancement in PC5-based V2V, CATT

  - R1-166435, Discussion on details of resource reselection and reservation, CATT