3GPP TSG RAN WG1 Meeting #86		                                                       R1-166484

Gothenburg, Sweden, 22nd - 26th August 2016

Source:	CATT

Title:	Discussions on KPI and evaluation configuration for URLLC

Agenda Item:	8.1.7

Document for:	Discussion and Decision

Introduction

In RAN#72, usage scenario URLLC and its KPIs of latency and reliability were discussed again. The related conclusions are as follows [1]:

For URLLC the target for user plane latency should be 0.5ms for UL, and 0.5ms for DL. Furthermore, if possible, the latency should also be low enough to support the use of the next generation access technologies as a wireless transport technology that can be used within the next generation access architecture.

NOTE1:	The reliability KPI also provides a latency value with an associated reliability requirement. The value above should be considered an average value and does not have an associated high reliability requirement.

Reliability can be evaluated by the success probability of transmitting X bytes NOTE1 within 1 ms, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface, at a certain channel quality (e.g., coverage-edge).

The target for reliability should be 1-10-5 within 1ms.

A general URLLC reliability requirement for one transmission of a packet is 1-10-5 for X bytes (e.g., 20 bytes) with a user plane latency of 1ms.

Based on the requirements from 38.913 [1], this contribution further provides analysis on KPIs for URLLC and general evaluation configurations to facilitate further evaluation of this usage scenario.

Discussion

2.1	KPI and performance metrics for URLLC

In RAN1#85 meeting, besides KPI required by RAN, two additional performance metrics were proposed to reflect URLLC capacity and URLLC/eMBB multiplexing capacity. However, it is more important to design the system of 5G NR to meet the required KPI in this usage scenario: latency and reliability, which were not clarified very clear in the last RAN1 meeting and even the last RAN meeting, such as the matching size of packet, methodologies of evaluation.

As stated in 38.913, latency requirement of 0.5ms does not have an associated high reliability requirement. It is considered for DL/UL/SL, as long as user plane latency can be met in any case, e.g. by a special configuration of new designed frame structure, HARQ mechanism, the target is reached. It needs an analysis method for evaluation including latency metrics, which is suggested in the companion contribution [2].

One the other hand, the general URLLC reliability requirement also depends on the condition of user plane latency. Additionally, data rate which can be supported in URLLC scenario should also be considered, which can provide operator a clearer impression what services can be provided. The supported data rate can be reflected by the size of packet within a certain bandwidth and latency. Detailed parameters can be found in the following section. Link level simulatin is suggested as a starting point for evaluation of reliability, which is detailed in [2].

Proposal 1. Evaluation methodology of latency for URLLC is analysis and design target is 0.5ms.

Proposal 2. Evaluation methodology of reliability for URLLC is link level simulation (as starting point) and design target is 1-10-5 within 1ms.

2.2 	Detailed evaluation configuration

Following the conclusion from #85 RAN1 meeting [3]: aim for a single set of assumptions for evaluations of URLLC, the following Table 1. shows the suggested values of evaluation configuration for URLLC, which is mainly for link level simulation of reliability.

It is noted that transmission bandwidth per packet is not provided because it can be calculated based on MCS level, coding rate, packet size and numerology. For example, when there are 2 symbols as one TTI, subcarrier spacing is 15KHz, QPSK and 1/3 coding rate, about 3MHz bandwidth is needed for simulation.

Table 1. Evaluation parameters for URLLC

| Attributes | Values or assumptions |
| --- | --- |
| Carrier frequency | 4GHz |
| Modulation and coding rate | QPSK, 16QAM 1/12, 1/6, 1/3 |
| Packet size/TBS | 20 bytes as mandatory (160kbps) Other values may not be precluded but considering different KPI value |
| Speed | 3km/h as mandatory Other values may not be precluded but considering different KPI value |
| Antenna configuration | BS: up to 256 Tx/Rx antenna elements UE: up to 8 Tx/Rx antenna elements |
| SNR | Option 1. SNR distribution within [-5, 10] Option 2. SNR value = 50% value of Geometry CDF of one matched and existing eMBB deployment scenario, e.g. Urban Macro, Indoor |
| Channel modeling | EPA or CDL if applicable |
| Numerology and frame structure | Companies provide their schemes applied in the simulation |
| Scheduling time | 0ms |
| HARQ duration (ACK/NACK waiting time) | Companies provide the values depending on their schemes |
| Processing latency | Option 1: linear scaling according to TTI length Option 2: fixed BS/UE processing latency, e.g. 100us. |
| Waiting time for data transmission | Average value depending on frame structure design |

Proposal 3. To take Table 1 evaluation parameter for URLLC into account in evaluation

2.3 	Potential PHY techniques to support URLLC

To evaluate performance metric under URLLC, there are several physical layer techniques which can be applied. From the perspective of latency, frame structure design for 5G can effectively reduce transmission latency and support of grant-free schemes can decrease scheduling time. With companies’ proposed frame structure design and any possible schemes to further reduce latency, reliability can be evaluated considering contributions at least from enhancement of channel coding, HARQ, MIMO and MA schemes.

Proposal 4. To support URLLC, the following techniques can be further evaluated:

- Latency: Frame structure design, Grant-free, HARQ enhancement

- Reliability: Channel coding, HARQ, MIMO and MA

Conclusion

In this contribution, further analysis of KPI for URLLC, suggestions on corresponding evaluation configuration, and potential physical techniques to support URLLC are provided. Proposals are as following:

Proposal 1. Evaluation methodology of latency for URLLC is analysis and design target is 0.5ms.

Proposal 2. Evaluation methodology of reliability for URLLC is link level simulation (as starting point) and design target is 1-10-5 within 1ms.

Proposal 3. To take Table 1 evaluation parameter for URLLC into account in evaluation

Proposal 4. To support URLLC, the following techniques can be further evaluated:

- Latency: Frame structure design, Grant-free, HARQ enhancement

- Reliability: Channel coding, HARQ, MIMO and MA

References

[1] RP-161334, 3GPP TR 38.913 v0.4.0 (2016-06)

[2] R1-166485, Discussions on evaluation methodology for URLLC, CATT, Aug. 2016

[3] RAN1 #85 Chairman notes, May 2016