3GPP TSG RAN WG1 Meeting #86                                                               	           R1-166505

Gothenburg, Sweden, 22nd – 26th August, 2016

Agenda item:	7.2.1.1

Source: 	Intel Corporation

Title: 	Indication of the starting PUSCH DFT-S-OFDM symbols

Document for:	Discussion and Decision

- Introduction

During the email discussion following the RAN WG1 Meeting #85, the following agreements were made on the various scheduling aspects for eLAA:

Agreements on scheduling aspects for eLAA

- Confirm the WA: For LAA uplink with single codeword, maximum number of HARQ processes = 16 are supported

- Timing offset is counted from subframe N+4+k, and k is signalled with 4 bits ([0….15] SFs)  (FFS in case of 2-step scheduling)

- DCI format 0B/4B can schedule PUSCH transmission in a single subframe

- For DCI format 0B/4B, bit width of number of scheduled subframes is 1 bit when N_sf is configured as 2 and 2 bits when N_sf is configured as larger than 2

- CSI request in DCI format 0B/4B applies to last subframe if number of scheduled subframes is smaller than 3, and otherwise to the second last scheduled subframe

- Rel-14 eLAA UE is required to decode UL grant PDCCHs as well as the common PDCCH in the ending partial subframe even if the number of occupied OFDM symbols for the ending partial subframe is not indicated from the previous subframe

- FFS special handling of MCS/TBS for PUSCH containing UCI and/or SRS and/or blanked symbol(s)

  - Note: Common MCS value for all the scheduled subframes is already agreed

- The UE is not expected to process a DCI format 0B/4B indicating scheduling in a single subframe and triggering SRS in a different subframe

Notes as basis for further discussion: FFS on the indication of starting PUSCH DFT-S-OFDM symbol

- option 1) Starting PUSCH DFT-S-OFDM symbol in a DCI format 0B/4B applies only to the first scheduled subframe.

  - A single UL grant for an LAA SCell can only schedule subframes which composes a same UL transmission burst of a same UE

- option 2) DCI format 0B/4B indicates starting PUSCH DFS-S-OFDM symbol in one or multiples of the scheduled subframes

  - option 2-1) 2-bit joint indication for the starting PUSCH DFT-S-OFDM symbol, the 4 states define the starting in symbol #1 as: {in none of SFs, in all SFs, only in the 1st subframe, in 1st and 3rd subframe}

  - option 2-2) N_sf size bitmap to indicate the starting PUSCH DFT-S-OFDM symbol position for multiple scheduled subframes. The bitmap size changes according to the configuration of N_sf for MSF, i.e., 2~4.

  - option 2-3) One bit to indicate the starting PUSCH DFT-S-OFDM symbol for the 1st subframe, x bit to indicate starting PUSCH DFT-S-OFDM symbol position in one of the following subframes if N_max >=2. The size of x changes according to the configuration of N_max for MSF.

In this contribution, we discuss the remaining FFS on the indication of starting PUSCH DFT-S-OFDM symbol.

- Discussion on the resource allocation method

Currently, the following options are under discussion as an option for indication of starting PUSCH DFT-S-OFDM symbol.

- option 1) Starting PUSCH DFT-S-OFDM symbol in a DCI format 0B/4B applies only to the first scheduled subframe.

  - A single UL grant for an LAA SCell can only schedule subframes which composes a same UL transmission burst of a same UE

- option 2) DCI format 0B/4B indicates starting PUSCH DFS-S-OFDM symbol in one or multiples of the scheduled subframes

  - option 2-1) 2-bit joint indication for the starting PUSCH DFT-S-OFDM symbol, the 4 states define the starting in symbol #1 as: {in none of SFs, in all SFs, only in the 1st subframe, in 1st and 3rd subframe}

  - option 2-2) N_sf size bitmap to indicate the starting PUSCH DFT-S-OFDM symbol position for multiple scheduled subframes. The bitmap size changes according to the configuration of N_sf for MSF, i.e., 2~4.

  - option 2-3) One bit to indicate the starting PUSCH DFT-S-OFDM symbol for the 1st subframe, x bit to indicate starting PUSCH DFT-S-OFDM symbol position in one of the following subframes if N_max >=2. The size of x changes according to the configuration of N_max for MSF.

Discussion on need for flexible UE multiplexing:

The motivation of creating LBT gaps in between multiple consecutively scheduled subframes is to allow flexible multi-user multiplexing as depicted in the following figure. The option 1, which allows the LBT gap only in the first scheduled subframe, significantly prohibits scheduling flexibility. If four subframes are scheduled using one MSF DCI, then option 1 implies either single UE transmission or multiple UEs should have to be scheduled for all the four subframes. This significantly depreciates the power of MSF DCIs. It is an agreement that the number of consecutively scheduled subframes, N_sf, is UE-specific RRC configuration. Then, option 1 implies that the UEs only having the same N_sf configuration can be multiplexed together, unless the N_sf subframes are not to be fully utilized. This is a significant constraint to eNB scheduler unless the UE-specific N_sf values are actually set equally to all the UEs in the cell.

Observation 1: Option 1 significantly depreciates the value of multi-subframe scheduling, given that number of consecutively scheduled subframes, N_sf, is UE-specific RRC configuration.

Figure 1. Illustration of UL LBT gap creation for UE multiplexing

Figure 2 illustrates the need for flexible multi-user multiplexing. A cell-edge UE, or a UE experiencing high interference signal level, may only be allocated with only one interlace in order to maintain the SINR to the degree that some low MCS levels can be usable. Due to the row rate, the MSF scheduling is extremely valuable for such UEs. On the other hand, a cell-center UE may suffice to have one scheduled subframe to transmit its data using SSF DCI.

Proposal 1: Flexible UE multiplexing should be allowed for multi-subframe scheduling, especially considering multiplexing cell-edge UEs with other UEs.

Figure 2. Illustration of UE multiplexing (green interlace is assigned to cell-edge UE)

Let us now discuss amongst options allowing the creation of UL LBT gap in between consecutively scheduled subframes.

Option 2-1: Note that option 2-1 fundamentally assumes that all the UEs are configured with N_sf with 4. Again, N_sf is UE-specific RRC configuration. This implies that, regardless of the value of UE-specific N_sf, the same predefined four states will be applies.

Option 2-2: This option allows the full flexibility of UE multiplexing. Note that the number of bits needed is dependent on the UE-specific N_sf configuration. The N_sf is not always configured to its maximal value, i.e., 4. Thus, it is expected that Option 2-2 provides full flexibility with actually small number of bits than 4.

Option 2-3: Note that this option allows the indication for the first subframe plus one additional subframe. For this, the required number of bits is 3 for N_sf=4, which is inefficient.

Proposal 2: Option 2-2, i.e., bitmap based approach, should be adopted to indicate the starting PUSCH DFT-S-OFDM symbol.

- Conclusions

In this contribution, we discuss the remaining FFS on the indication of starting PUSCH DFT-S-OFDM symbol and made the following proposals and observation.

Observation 1: Option 1 significantly depreciates the value of multi-subframe scheduling, given that number of consecutively scheduled subframes, N_sf, is UE-specific RRC configuration.

Proposal 1: Flexible UE multiplexing should be allowed for multi-subframe scheduling, especially considering multiplexing cell-edge UEs with other UEs.

Proposal 2: Option 2-2, i.e., bitmap based approach, should be adopted to indicate the starting PUSCH DFT-S-OFDM symbol.