3GPP TSG-RAN WG1 #86  													                           R1-166524

# Gothenburg, Sweden, 22nd – 26th August, 2016

Source: 	Intel Corporation

Title:	Further study of advanced CSI feedback

Agenda item:	    7.2.4.2.3

Document for:	Discussion and Decision

# Introduction

In RAN1 #85, the following agreement about advanced CSI reporting to improve the performance of FD-MIMO was achieved [1]:

- Specify enhancement on CSI reporting to improve eNB precoding. The specified enhancement is to be selected from the following categories:

  - Enhancements to Rel-13 feedback codebooks (FFS which numbers of antenna ports from the set {8, 12, 16, 20, 24, 28, 32}) that increase CSI resolution through improved beam selection / construction in W1 and/or improved beam/port selection / combining / weighting mechanisms in W2

  - Parameters representing channel coefficients, or some reduced space representation thereof including beam combining / weighting with coefficient quantization or channel quantization or channel covariance matrix quantization

    - Uplink physical channel enhancements to carry the representation of channel coefficients can be included if selected

- Also, interference measurement enhancement can be considered

The detailed proposal for explicit CSI reporting using quantized Eigen vectors of channel covariance matrix is discussed in this contribution.

# Explicit CSI report

# 2.1 The description of the proposed explicit CSI feedback

The FD-MIMO system can be described as:

y=HWx+n=HW1W2x+n                                                  (1)

where y, H, W, W1, W2, x, and n represent Nr×1 receiving vector, Nr×Nt channel matrix, Nt×Nl precoding matrix, Nt×Nd vitalization matrix, Nd×Nl precoding matrix, Nl×1 transmitting vector, and Nr×1 noise vector, respectively. The matrix dimension number Nr, Nt, Nd and Nl represent the number of receiving antennas, the number of transmitting antennas, the number of virtualized antenna ports and the number of layers, respectively.

Firstly, the virtualization matrix W1, containing the dominant signal subspaces of all UE signals, is calculated through Eigenvalue decomposition of composite channel covariance matrix. The covariance matrix is calculated according to estimated channel on sounding RS as:

W1=EigenVector(1NUEi=1NUEHi,ULHi,ULH)1:Nd                                                    (2)

where Hi,UL is Nt×Nr the estimated uplink channel of UEi, and NUE denotes number of UEs in the cell served by this given eNB. Then Nl columns of Eigen matrix of the composite channel covariance matrix, corresponding to Nl strongest Eigen values, are adopted as W2. The elements of W2 are quantized and reported by UE to improve the CQI measurement accuracy.

# Discussion

The proposed scheme can be implemented in the LTE system, where the beam weight for beam formed CSI-RS is equal to virtualization matrix W1, and the quantized elements are reported through PUSCH mode 3-1. The larger number antenna elements are virtualized into the smaller antenna ports, e. g. 8 ports, and then UE can derive and report the channel state information based on beamforming the CSI-RS, which can maintain the complexity and overhead in irrespective to the number of antenna elements. The beam formed CSI-RS ports can be shared by multiple UEs, which has reduced CSI-RS overhead, compared with UE-specific beam formed CSI-RS.  Based on beam formed CSI-RS, the channel can be estimated more accurately.

Compared with the implicit CSI feedback mode supported in LTE system, the reporting overhead of explicit CSI may be higher and, therefore, should be carefully studied. More specifically, in the proposed scheme the channel covariance matrix is derived based on the long-term averaged measurements, so it provides more stable channel state information comparing to the instantaneous measurements and therefore avoids frequent CSI reporting updates.

In [2], two quantization schemes, Givens Rotation (GR) and Householder (HH) transformation, are studied for element quantization. For Nd beam formed CSI-RS antenna ports and rank Nl, total Nl(2Nd-Nl-1) real elements are required to be reported, e.g. 26/14 real elements need to be quantized and reported for rank 2/rank 1 respectively based on 8 beam formed CSI-RS antenna ports. To further reduce the CSI reporting overhead, the uneven quantization can be considered. For example in Type-I approach different distribution range of the elements can be utilized, as illustrated in the Figure 3 and Figure 4 of [2], where the elements with wider distribution range can be quantized with more bits, e.g. 4 bits, while the elements with smaller distribution range can be quantized with smaller number of bits, e.g. 3 bits. In this case the reporting overhead can be reduced to 97 bits and 52 bits for rank 2 and rank 1. On the other hand, in Type-II approach the Eigen vector corresponding to strong Eigen value are quantized by using more bits, e.g. 4 bits, while the Eigen vector corresponding to weak Eigen value by smaller number of bits, e.g. 3 bits. In this case the reporting overhead can be also reduced to 92 bits for rank 2.

Proposal 1: For beam formed CSI-RS ports, UE should explicitly report precoding matrix corresponding to the strongest Nl Eigen vectors of the beam formed channel covariance matrix, where Nl is the number of layers.

Proposal 2: Uneven quantization for GR and HH quantization schemes can be utilized to reduce the CSI reporting overhead.

# 2.3 The system-level evaluation results

In order to evaluate the performance of the proposed algorithm, the system-level simulation is performed, where the simulation parameters are listed in the appendix. The FTP results are summarized in the Table 1 for λ = 1.8, and Table 2 for λ = 2.8. For benchmarking implicit CSI reporting based 32 ports CLASS A codebook [3] with Config=1, O1=4 and O2=4 is adopted.

From the presented result, it can be seen that the proposed explicit CSI reporting scheme can achieve better spectral efficiency comparing to the implicit CLASS A reporting scheme. From Table 1, it can be seen that about 21% gain for 5% UPT for low load scenario, and the proposed uneven quantization can further reduce the overhead with negligible performance loss. The report overhead of the proposed scheme is also listed, which is smaller than 100 bits, and can be supported by the existing CSI reporting modes

Table 1: Gain over Rel.13 class A codebook feedback with λ = 1.8

| (N1, N2, O1, O2) = (4,4,4,4) | Edge SE | Mean SE | Overhead |
| --- | --- | --- | --- |
| Class A | 0% | 0% | 16 bits |
| Non-quantization, Nd=8 | 26.8% | 2.4% | +∞ |
| GR, Nd=8, even quantization, 4bits | 21.5% | 1.9% | ~83bits |
| GR, Nd=8 Type-I uneven quantization, 4 bits+3 bits | 21.1% | 1.7% | ~78bits |
| GR, Nd=8 Type-II uneven quantization, 4 bits+3 bits | 21.5% | 1.2% | ~80bits |
| GR, Nd=8, even quantization, 3bits | 19.3% | -0.7% | ~63bits |
| HH, Nd=8, even quantization, 4bits | 21.9% | 1.9% | ~83bits |
| HH, Nd=8 Type-I uneven quantization, 4 bits+3 bits | 16.0% | 0% | ~78bits |
| HH, Nd=8 Type-II uneven quantization, 4 bits+3 bits | 22.0% | 0.5% | ~80bits |
| HH, Nd=8, even quantization, 3bits | 17.7% | -2.3% | ~62bits |

As the load increase, more UEs are active at the same time, then the probability for MU scheduling is increased, owning to abundant MU pairing. The MU transmission mode requires more accurate CSI feedback to suppress the mutual interference. Table 2 illustrates that about 12% gain for 5% UPT and 10% gain for average UPT.

Table 2: Gain over Rel.13 class A codebook feedback with λ = 2.8

| (N1, N2, O1, O2) = (4,4,4,4) | Edge SE | Mean SE | Overhead |
| --- | --- | --- | --- |
| Class A | 0% | 0% | 16 bits |
| Non-quantization, Nd=8 | 14.53% | 12.81% | +∞ |
| GR, Nd=8, even quantization, 4bits | 12.84% | 10.50% | ~88bits |
| GR, Nd=8 type-I uneven quantization, 4 bits+3 bits | 3.72% | 10.97% | ~69bits |
| GR, Nd=8, even quantization, 3bits | 1.69% | 5.81% | ~62bits |

# Summary

In this contribution, the explicit CSI feedback is proposed. The evaluation results are also provided showing the benefits comparing to implicit Class A schemes. Based on the discussion the following proposals were made:

Proposal 1: For beam formed CSI-RS ports, UE should explicitly report precoding matrix corresponding to the strongest Nl Eigen vectors of the beam formed channel covariance matrix, where Nl is the number of layers.

Proposal 2: Uneven quantization for GR and HH quantization schemes can be utilized to reduce the CSI reporting overhead.

# References

[1] R1-165790, WF on the three “evaluate and, if needed, specify” Items	Nokia, Alcatel-Lucent Shanghai Bell, Samsung, CATT

[2] R1-164150	Explicit CSI reporting for FD-MIMO	Intel Corporation

[3] TS 36.213, Technical Specification Group Radio Access Network, Evolved Universal Terrestrial Radio Access (E-UTRA), Physical layer procedures (Release 13)

Simulation assumption

| Parameter | Value |
| --- | --- |
| Scenario | UMa with 500 ISD and 2GHz |
| eNB Antenna Configuration | X-polarized: 45/-45 degrees |
|  | horizontal spacing; vertical spacing |
|  | 2D antenna pattern defined in TR36.873, (4,4,2) |
| UE Antenna Configuration | + polarized: 0/+90 degrees, 2Rxs, |
| UE Configurations | Speed: 3km/h |
|  | UE attachment: Based on RSRP from CRS port 0 |
|  | UE distribution: 80% indoor and 20% outdoor only distributed on floor |
| SRS Configuration | 2Tx at UE |
| System Bandwidth | 10MHz (50RBs) |
| Scheduler | PF |
| Number of UEs per cell | 10 |
| traffic model | FTP, Model 1, 0.5MByte |
| Transmit Mode | Dynamic SU/MU: rank adaptation Max of 2 paired UEs |
| Receiver | Ideal channel estimation |
|  | MMSE-IRC receiver |
| Hybrid ARQ | Maximum 4 transmissions |
| Feedback | PUSCH 3-1 |
|  | CQI and PMI reporting triggered per 10ms |
|  | Feedback delay is 10ms |
| Overhead | 3 symbols for DL CCHs, 2 CRS ports and DM-RS with 12 REs per PRB |