3GPP TSG RAN WG1 Meeting #86                                                              R1-166601

Gothenburg, Sweden, 22th – 26th August 2016

Source:               ZTE

Title:                    Considerations on PUSCH design in special subframe

Agenda item:      7.2.9

Document for:    Discussion and Decision

# Introduction

In RAN#71, the WI on Uplink capacity enhancements for LTE was approved[1]. One of the objectives in the WI is to specify support of UL PUSCH in special subframe. The detailed work starts from Q3 2016 the objectives are:

- UL support of PUSCH transmission in special subframe [RAN1]

  - Specify mechanism for supporting PUSCH transmission in special subframe with DwPTS of 6 OFDM symbols, GP of 2 OFDM symbols. [RAN1, RAN2]

  - Introduce applicable RF requirements [RAN4]

  - Backward compatibility with legacy UEs is maintained

In this contribution, we give our considerations on PUSCH design in special subframe.

# Discussion

To support UL PUSCH transmission in special subframe, two alternatives can be considered:

Alt-1: separate TB is transmitted in special subframe and the adjacent uplink subframe respectively.

Alt-2: a single TB is tranmitted on both special subframe and the adjacent uplink subframe.

Considering the transmission duration of PUSCH in special subframe is 6 OFDM sybmbols, Alt-1 may be more appropriate from the  specification efforts on determining the TB size. In addition, the scheduling is more flexible for Alt-1 than Alt-2.

Proposal-1: A seperate UL TB is transmitted in special subframe.

## HARQ related issues

In LTE, synchronous HARQ is adopted for UL transmission. The HARQ timing for PUSCH transmission includes:

- Timing between UL grant/PHICH and PUSCH.

- Timing between PUSCH and the corresponding PHICH.

Considering the backward compatibility, no additional PHICH resource can be added in PDCCH region. For TDD UL/DL configuration 1-5, some of the DL subframes may not have PHICH region. And as describe below, in order to reuse the current UL DCI to schedule UpPTS, it should avoid the case that one DL subframe schedules multiple PUSCHs. Otherwise UL index should also be introduced for TDD UL/DL configuration 1-5 to support multiple subframes scheduling, And for UL/DL configuration 0 and 6, PHICH resource collision may occure. Although such PHICH resource collision can be avoided by implementation, i.e., the eNB can allocate different PRBs or different CS for the PUSCH transmitted in different uplink subframes, it may bring some restrictions on eNB scheduling. Therefore, PHICH-less operation may be more appropriate for PUSCH retransmission in special subframe. And the transmission instant for UL grant triggering PUSCH retransmission should be defined.

## Proposal-2: PHICH-less operation should be applied for PUSCH retransmission in special subframe.Timing between UL grant and PUSCH

For the scheduling of PUSCH in special subframe, two options can be taken into account.

Opt-1: PUSCH transmission in special subframe is always scheduled together with the following uplink subframe.

Opt-2: PUSCH transmission in special subframe is scheduled by a single DCI.

For Opt-1, no additional scheduling timing is needed. Considering that special subframe is in front of normal uplink subframe, the processing time for PUSCH in special subframe may be less than 4ms if the timing between UL grant and PUSCH in normal subframe is 4ms, as illustrated in Figure 1. Another problem for Opt-1 is that UpPTS is always scheduled with the following uplink subframe, which bring some restriction on scheduling flexibility if legacy DCI is reused. Otherwise, new DCI format need to be introduced to support 2 subframes scheduling. New DCI format means additional blind decoding.

Figure-1. example of scheduling timing for PUSCH in special subframe (Opt-1)

For Opt-2, special consideration should be taken for TDD UL/DL configuration 0 and 6, as the number of DL subframes is smaller than the number of UL subframe (including special subframe) transmitting PUSCH.

For TDD UL/DL configuration 0, the reserved value of “00” in UL index can be used to schedule the PUSCH in special subframe. Figure-2 gives such an example.

Figure-2. Scheduling timing for PUSCH in special subframe for TDD UL/DL configuration 0

Proposal -3: The reserved value “00” in UL index can be used to indicate the timing of UL grant and PUSCH for TDD UL/DL configuration 0.

And for TDD UL/DL configuration 6, there is no UL index in the UL grant since the control field is used as UL DAI to indicate the number of scheduled DL subframes in a bundling window. But according to the timing between PDSCH and HARQ-ACK, the size of bundling window is always 1.The UL DAI in configuration 6 may be useless.  Therefore, UL DAI can be reinterpreted as UL index to support scheduling PUSCH in special subframe. Figure-3 illustrates an example of support multiple subframes scheduling in subframe #0 and #5. The UL index design follows the same principle as in TDD UL/DL configuration 0.

Figure-3. Scheduling timing for PUSCH in special subframe for TDD UL/DL configuration 6

Proposal -4: For TDD UL/DL configuration 6, UL DAI in DCI format 0/4 should be reinterpreted as UL index to support scheduling PUSCH in special subframe.

For other TDD UL/DL configurations, in order to avoid one DL subframe schedules multiple PUSCHs, modifications of Table 8-2 in [2] should be done to support the scheduling of PUSCH in special subframe, which is shown in the Table-1. The modifications is marked with red color.

Table -1: k for TDD configurations 0-6

| TDD UL/DL Configuration | subframe number n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 4 | 6 |  |  |  | 4 | 6 |  |  |  |
| 1 | 6 | 6 |  |  | 4 | 6 | 6 |  |  | 4 |
| 2 |  | 5 |  | 4 |  |  | 5 |  | 4 |  |
| 3 | 4 |  |  |  |  |  |  | 4 | 4 | 4 |
| 4 |  |  |  |  |  |  |  | 4 | 4 | 4 |
| 5 |  |  |  |  |  |  |  | 4 | 4 |  |
| 6 | 7 | 7 |  |  |  | 7 | 7 |  |  | 5 |

## Timing between PUSCH and retransmission triggering

As mentioned above, no additional PHICH resource can be added in the PDCCH region. The retransmission of PUSCH in special subframe shoud be triggered by a UL grant with NDI not toggled. In LTE, the timing between PUSCH and PHICH is defined by kPHICH in [2].  For describe convienent, we also use kPHICH to defined the timing between PUSCH in special subframe and its retransmission UL grant triggering, as shown in the below Table-2.

Table-2:  for TDD

| TDD UL/DL Configuration | subframe index n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 |  | 5 | 4 | 7 | 6 |  | 5 | 4 | 7 | 6 |
| 1 |  | 4 | 4 | 6 |  |  | 4 | 4 | 6 |  |
| 2 |  | 5 | 6 |  |  |  | 5 | 6 |  |  |
| 3 |  | 6 | 6 | 6 | 6 |  |  |  |  |  |
| 4 |  | 6 | 6 | 6 |  |  |  |  |  |  |
| 5 |  | 6 | 6 |  |  |  |  |  |  |  |
| 6 |  | 4 | 4 | 6 | 6 |  | 4 | 4 | 7 |  |

## UL HARQ Process

Considering the supportable number of OFDM symbols in PUSCH is different for special subframe and normal UL subframe, the UL HARQ process may be handled separately. Table-3 gives the number of UL HARQ process for different TDD UL/DL configuration.

Table-3: Number of synchronous UL HARQ processes for TDD

| TDD UL/DL configuration | Number of HARQ processes for normal HARQ operation in normal uplink subframe | Number of HARQ processes for normal HARQ operation in special subframe |
| --- | --- | --- |
| 0 | 7 | 2 |
| 1 | 4 | 2 |
| 2 | 2 | 1 |
| 3 | 3 | 1 |
| 4 | 2 | 1 |
| 5 | 1 | 1 |
| 6 | 6 | 2 |

## TBS determination

Similar to the TBS determination of PDSCH transmission in DwPTS, the TBS determination of PUSCH transmission in UpPTS can be calculated by using the effective allocated PRB numbers with a scaling factor: , where  =0.5 may be a good choice since there are 6 OFDM symbols for the PUSCH transmission.

Proposal-5: TBS for PUSCH transmission in special subframe can be calculated by using the effectively allocated PRB , where =0.5.

## DMRS design

As suggested above, the PUSCH transmitted in special subframe is individual scheduled. Therefore, DMRS should be self-contained in special subframe for PUSCH demodulation. Considering there are 6 SC-FDMA symbols in special subframe used for PUSCH transmission, the DMRS can be located in the 3th SC-FDMA symbol within the UpPTS.

Figure-4. DMRS for PUSCH in UpPTS

Proposal-6: The DMRS is located in the 3rd  SC-FDMA symbol within the UpPTS.

# Conclusion

In this contribution, we discuss the issues related to PUSCH transmission in speicail subframe. In summ:

Proposal-1: A seperate UL TB is transmitted in special subframe.

Proposal-2: PHICH-less operation should be applied for PUSCH retransmission in special subframe.

Proposal -3: The reserved value “00” in UL index can be used to indicate the timing of UL grant and PUSCH for TDD UL/DL configuration 0.

Proposal -4: For TDD UL/DL configuration 6, UL DAI in DCI format 0/4 should be reinterpreted as UL index to support scheduling PUSCH in special subframe.

Proposal-5: TBS for PUSCH transmission in special subframe can be calculated by using the effectively allocated PRB , where =0.5.

Proposal-6: The DMRS is located in the 3rd  SC-FDMA symbol within the UpPTS.

# References

- RP-160644, New Work Item on Uplink Capacity Enhancements for LTE, Ericsson, CMCC.

- 3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

- 3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

- R1-164884,  Discusssion on  PUSCH transmission on UpPTS, CMCC

- R1-164851, Initial discussion on UL PUSCH in TDD special subframes, Huawei, Hisilicon