3GPP TSG RAN WG1 Meeting #86	R1-166612

Gothenburg, Sweden, 22nd - 26th August 2016

Source:	OPPO

Title:	Pairing/Combining the two types of self-contained subframes

Agenda Item:	8.1.3.2

Document for:	Discussion and Decision

Introduction

According to the discussion in RAN1#85 on frame structure for NR, some agreements and preliminary assumptions for self-contained subframes start coming into form, e.g.:

- NR design should strive at least to enable the possibility for

  - Corresponding acknowledgement reporting shortly (in the order of X µs) after the end of the DL data transmission

  - Corresponding uplink data transmission shortly (in the order of Y µs) after reception of UL assignment

  - Note: may depend on e.g. UE capability/category, payload size, etc

  - FFS: X and Y in the order of a few tens of or hundreds of micro sec is feasible

- Other mechanisms/configurations in addition to fast/short corresponding acknowledgement are needed

  - For example to provide coverage or enable TD-LTE coexistence

- Note: RAN1 will continue investigations about UE complexity, implementation processing time, interleaving applicability

The time interval X and Y may in the order of a few tens of or hundreds of s depending on the UE processing capability and design of interval. Hence a key issue is how to arrange the two intervals. In this contribution, we discuss this issue and propose a potential solution.

How to utilize processing interval in a self-contained subframe?

An example of the self-contained subframes is illustrated in Fig.1 [1]. In the DL-dominant subframe, a time interval is needed between the DL part (including DL data) and the UL part (including the corresponding ACK/NAK for DL data). The time interval is a few tens of s or even several hundreds of s, depending on the UE processing delay for the DL data. In the UL-dominant subframe, a time interval is also needed between the DL (including UL grant) and the UL part (including the corresponding UL data). The time interval is a few tens of s depending on the UE processing delay for the UL grant.

Fig.1: An example of the two types of self-contained subframes

The time interval between DL part and UL part of a self-contained subframe could be just filled with a GP (Guard Period) if it is in the order of tens of s. However, if the interval is in order of hundreds of s, we need to consider how to utilize the interval to transmit other signals. Two options can be considered:

- Option 1: Transmit signals not requiring self-contained in a subframe (e.g. delay-tolerant signal such as DL data not requiring immediate ACK/NAK or UL data corresponding to a UL grant in a previous subframe or uplink control information (UCI) such as PMI/CQI).

- Option 2: Transmit signals in another self-contained subframe, i.e. paring or combining two self-contained subframe.

Pairing/combining two types of self-contained subframes

An example of paired self-contained subframes is illustrated in Fig.2: The DL part of a UL-dominant subframe is accommodated in the time interval of a DL-dominant subframe. Meanwhile the UL part of the DL-dominant subframe covers a portion of processing delay of the UL-dominant subframe. The GP is placed between the DL part of the UL-dominant subframe and the UL part of the DL-dominant subframe.

Fig.2: Paired self-conatined subframes

Pairing the two types of self-contained subframes brings following advantages:

- Taking full use of the processing delay period between the DL and UL parts of a self-contained subframe;

- Supporting low-latency services in DL and UL simultaneously;

- The DL-dominant subframe and the UL-dominant subframe share the same GP, thus reduce the overhead due to DL-to-UL switching.

The self-contained pair shown in Fig.2 can also be regarded as a “combined self-contained subframe” including two DL parts and two UL parts.

The paired/combined self-contained subframe is similar to the special subframe in a LTE frame struature type 2. A paired/combined self-contained subframe can be placed surrounding each DL-to-UL switching point, including a multi-functional DL part, a GP and a multi-functional UL part, to support the low-latency services in DL and UL simultaneously.

Fig.3: Use of the paired/combined self-contained subframe surrounding each DL-to-UL switching point

Observation: Compared with the separate DL-dominant and UL-dominant self-contained subframes, the paired/combined self-contained subframe (as shown in Fig.2) can take full use of the processing delay period between each pair of DL and UL parts in the self-contained subframe, support low-latency services in DL and UL simultaneously, and reduce the overhead of DL-to-UL GP.

Proposal: Study the paired/combined self-contained subframe structure shown in Fig.2 to support simultaneous DL/UL low-latency services for the NR system.

Conclusions

Observation: Compared with the separate DL-dominant and UL-dominant self-contained subframes, the paired/combined self-contained subframe (as shown in Fig.2) can take full use of the processing delay period between each pair of DL and UL parts in the self-contained subframe, support low-latency services in DL and UL simultaneously, and reduce the overhead of DL-to-UL GP.

Proposal: Study the paired/combined self-contained subframe structure shown in Fig.2 to support simultaneous DL/UL low-latency services for the NR system.

References

- R1-165739, NR Frame Structure Timing Relations, Ericsson