3GPP TSG RAN WG1 Meeting #86                                  R1-166730

# Gothenburg, Sweden 22nd - 26th August 2016

# Source:        Samsung

Title:           Hybrid PMI codebook based CSI reporting and simulation

results

Agenda Item:   7.2.4.2.2

Document for:  Discussion and Decision

- Introduction

The following working assumption and conclusion about hybrid CSI were made in RAN1#85.

Working assumption:

- Mechanism 1: Hybrid CSI is realized by with one CSI process, support at least CLASS A for the 1st eMIMO-Type and CLASS B with K=1 CSI-RS resource for the 2nd eMIMO-Type

  - i1 is reported while CQI and i2 are not reported for the 1st eMIMO-Type (CLASS A)

    - FFS: whether RI is reported for CLASS A

  - CQI/PMI/RI are reported for the 2nd eMIMO-Type (CLASS B K=1)

- At least one more mechanism is supported, to be discussed in RAN1#86

Conclusion:

- Candidate mechanisms to consider in RAN1#86

- Mechanism 2A: for hybrid CSI with one CSI process, support CLASS B with K≥1 CSI-RS resources for the 1st eMIMO-Type and CLASS B with K=1 CSI-RS resources for the 2nd eMIMO-Type

  - For the 1st eMIMO-Type, depending on the value of K

    - K=1: CQI/RI are reported. In addition, i1 is reported for Rel.12 dual-stage codebooks

    - K>1: two options

      - Option 1: CRI is reported

      - Option 2: PMI/RI for each CSI-RS resource are reported

  - For the 2nd eMIMO-Type

    - CQI/PMI/RI are reported

  - ….

This contribution provides simulation results in support of Mechanism 1 and Mechanism 2A with Option 2 (highlighted in yellow).

- Mechanism 1

We will use notation RI(1) and RI(2) for RI reporting in 1st and 2nd eMIMO-Types, respectively. According to the working assumption about Mechanism 1, the CSI for the 1st eMIMO-Type comprises of (i1,1,i1,2), that for 2nd eMIMO-Type comprises of PMI/CQI/RI(2), and the two CSIs are derived using Rel. 13 Class A and Class B codebooks, respectively. The RI(1) reporting in the 1st eMIMO-Type is currently FFS. For Np = 2 Class B ports in the 2nd eMIMO-Type, RI(1) reporting may not be necessary and RI(1) can be fixed to rank 1 for example. For Np = 4 and 8 Class B ports, however, RI(1) reporting may bring performance gain. In this section, we study the following alternatives for RI(1) reporting.

- Hyb 0: Both RI(1) and RI(2) are reported without any restrictions.

- Hyb 1: RI(1) is fixed to rank 1 and RI(2) is reported without any restrictions.

- Hyb 2: 1-bit RI(1) is reported for rank 1 and rank 3 and RI(2) is reported without any restrictions.

The reason rank 1 and rank 3 are considered for 1-bit RI(1) reporting in Hyb 2 is that Rel. 13 Class A codebook has nested property in the sense that rank 1 and rank 2 beam groups (indicated by (i1,1,i1,2)) are identical (except N1 < N2 and Codebook-Config = 2), and similarly rank 3 and rank 4 beam groups (indicated by (i1,1,i1,2)) are also identical. Therefore, for 1-bit RI(1) reporting, rank 1 and rank 3 are considered so that the reported beam groups are different depending on the rank. Assuming up to rank 4 hybrid CSI reporting, RI(1) and RI(2) reporting overheads can be summarized as in Table 1.

Table 1: RI(1) and RI(2) reporting overhead

| Scheme | RI(1) | RI(2) |
| --- | --- | --- |
| Class A (Ref) | Unrestricted (2-bit) | Unrestricted (2-bit) |
| Hyb 0 | Unrestricted (2-bit) | Unrestricted (2-bit) |
| Hyb 1 | Rank 1 (0-bit) | Unrestricted (2-bit) |
| Hyb 2 | Rank 1, Rank 3 (1-bit) | Unrestricted (2-bit) |

The non-full-buffer system-level evaluation is carried out for UMi-2GHz channel model in low (20% target RU) traffic loading scenarios with up to rank 4 CSI. The detailed results can be found in Table 4 in the Appendix. The results are provided for 32 antenna ports with (N1,N2) = (4,4) and Np = 8 Class B ports in the 2nd eMIMO-Type. The periodicity of 1st eMIMO-Type is 100 ms and that of the 2nd eMIMO-Type is 5ms. For comparison, Class A CSI reported every 5ms is considered in this evaluation. The relevant simulation parameters are enlisted in Table 3. The rest of the simulation assumption is according to [1].

The performance gains with “Class A only” as reference are summarized in Figure 1, Figure 2, and Figure 3 for Codebook-Config = 2, 3, and 4, respectively in the 1st eMIMO-Type. From these results, we can make the following observation.

Observation 1:

- When compared with Hyb 0 (full RI) and Hyb 2 (1-bit RI(1), ~8-10% performance loss in avg. UPT with Hyb 1 (when no RI(1) is reported and (i1,1,i1,2) corresponds to rank 1 in the 1st eMIMO-Type).

- 1-bit RI(1) reporting (Hyb 2) shows small (~2-3%) performance loss when compared with 2-bit RI(1) reporting (Hyb 0).

Proposal 1: Confirm the working assumption for Mechanism 1 with 1-bit RI(1) reporting in the 1st eMIMO-Type.

Figure 1: Performance gain: Codebook-Config = 2

Figure 2: Performance gain: Codebook-Config = 3

Figure 3: Performance gain: Codebook-Config = 4

- Mechanism 2A, Option 2

Figure 4: Illustration of hybrid CSI reporting

An example of Mechanism 2A, Option 2 of hybrid CSI scheme is shown in Figure 4. As shown,

- The first CSI-RS corresponds to “Class B K = 2 eMIMO-Type” in which NP CSI-RS is transmitted from a subset (one row or one column) of antenna ports. There are two components to this CSI-RS: first CSI-RS 1 is for the row and first CSI-RS 2 is for the column.

- The second CSI-RS corresponds to “Class B K = 1 eMIMO-Type” in which BF CSI-RS is transmitted from two beam-formed ports which are beam-formed using the beams associated with the first PMI.

- The UE derives

  - 1st CSI: i1,1 of the first PMI pair (i1,1,i1,2) using the NP CSI-RS corresponding to the row and a 16 port codebook, and i1,2 of the first PMI pair (i1,1,i1,2) using the NP CSI-RS corresponding to the column and a 4 port codebook; and

  - 2nd CSI: the second PMI i2, CQI and RI using BF CSI-RS and a 2 port codebook.

To demonstrate the performance of the proposed hybrid PMI codebook, simulation-level simulation results are provided for following four schemes:

- Class A only: For reference, the performance of Class A eMIMO-Type with Codebook-Config = 1 is considered.

- Mechanism 1: 1st CSI-RS is transmitted from full 2N1N2 ports with a periodicity of 100 ms;

- Mechanism 2A-a: Mechanism 2A, Option 2 with a periodicity of 1st CSI-RS of 100 ms; and

- Mechanism 2A-b: Mechanism 2A, Option 2 with a periodicity of 1st CSI-RS of 50 ms.

The 2nd CSI-RS is transmitted periodically every 5 ms. The non-full-buffer system-level evaluation is carried out for UMa-200m and UMi-2GHz channel models in heavy (70% target RU) traffic loading scenarios. The detailed results can be found in Table 6 in the Appendix. The relevant simulation parameters are enlisted in Table 5. The rest of the simulation assumption is according to [1]. The performance gains with “Class A only” as reference are summarized in Figure 5 and Figure 6. From these results, we can make the following observation.

Observation 2:

- Mechanism 1 shows performance gain over Class A only - 5-6% gain in avg. UPT for both UMa-200m and UMi-2GHz scenarios; and 9% gain in 5% UPT for UMi-2GHz.

- Overall, mechanism 2A performs similarly to Mechanism 1. In some cases, it exhibits slight performance loss over Mechanism 1. Such performance loss can be reduced by reducing the periodicity of 1st CSI-RS transmission.

Since the performance gap between Mechanism 1 and Mechanism 2A is not significantly large and the main motivation of Mechanism 2A, as explained in the companion contribution [2], is to support UEs that are incapable of reporting Class A CSI, which is required in Mechanism 1, we have the following proposal.

Proposal 2: Mechanism 2A, Option 2 is supported.

Figure 5: Performance for (N1,N2) = (4,4), UMa-200m

Figure 6: Performance for (N1,N2) = (4,4), UMi-2GHz

- Conclusion

This document provides SLS results for Mechanism 1 and Mechanism 2A, Option 2 of hybrid CSI reporting. The proposals and observations made can be summarized as follows.

Observation 1:

- When compared with Hyb 0 (full RI) and Hyb 2 (1-bit RI(1), ~8-10% performance loss in avg. UPT with Hyb 1 (when no RI(1) is reported and (i1,1,i1,2) corresponds to rank 1 in the 1st eMIMO-Type).

- 1-bit RI(1) reporting (Hyb 2) shows small (~2-3%) performance loss when compared with 2-bit RI(1) reporting (Hyb 0).

Proposal 1: Confirm the working assumption for Mechanism 1 with 1-bit RI(1) reporting in the 1st eMIMO-Type.

Observation 2:

- Mechanism 1 shows performance gain over Class A only - 5-6% gain in avg. UPT for both UMa-200m and UMi-2GHz scenarios; and 9% gain in 5% UPT for UMi-2GHz.

- Overall, mechanism 2A performs similarly to Mechanism 1. In some cases, it exhibits slight performance loss over Mechanism 1. Such performance loss can be reduced by reducing the periodicity of 1st CSI-RS transmission.

Proposal 2: Mechanism 2A, Option 2 is supported.

References

[1] 3GPP, TR36.897v1.0.1, “Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE”

[2] R1-166731, “Supported schemes for hybrid CSI,” Samsung

Appendix: Simulation Assumptions and Results

Table 3: Simulation Parameters: Mechanism 1

| Parameters | Values |
| --- | --- |
| Simulation Type | Non-full-buffer (Low load 20% Target RU, Lambda = 2) |
| Channel model | UMi-2GHz |
| Number of BS (H,V) antenna elements | (8,8), x-polarized, subarray partition |
| (N1,N2, P) | 32 ports: (4,4,2) |
| BS (H,V) antenna spacing | (0.5, 0.8)λ |
| BS and MS antenna polarizations | BS: (+45°,-45°); MS: (0°, 90°) |
| Number of UE antennas | 4 |
| SU/MU pre-coding | CB |
| Scheduling | SU, Proportional fair |
| Channel estimation | Ideal |
| Transmission rank | 1,2,3,4 |
| Receiver | MMSE-IRC |
| Codebook | Class A: (O1,O2) = (8,8), Codebook-Config = 2,3,4 Class B, K = 1: Np = 8 ports |

Table 4: Non-full buffer simulation results: Mechanism 1

| Codebook-Config | Scheme | Avg UPT | 50% UPT | 5% UPT | RU | Avg UPT gain | 50% UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Class A | 51.05 | 46.02 | 18.59 | 16.3% | 100.0% | 100.0% | 100.0% |
|  | Hyb 0 | 54.92 | 48.82 | 20.56 | 15.2% | 107.6% | 106.1% | 110.6% |
|  | Hyb 1 | 49.04 | 44.47 | 20.35 | 16.5% | 96.1% | 96.6% | 109.4% |
|  | Hyb 2 | 53.46 | 48.28 | 21.08 | 15.6% | 104.7% | 104.9% | 113.4% |
| 3 | Class A | 48.47 | 42.47 | 18.45 | 16.9% | 100.0% | 100.0% | 100.0% |
|  | Hyb 0 | 52.79 | 46.50 | 19.70 | 15.5% | 108.9% | 109.5% | 106.8% |
|  | Hyb 1 | 47.44 | 42.23 | 19.31 | 16.8% | 97.9% | 99.4% | 104.7% |
|  | Hyb 2 | 51.52 | 45.80 | 19.44 | 15.9% | 106.3% | 107.9% | 105.3% |
| 4 | Class A | 48.26 | 42.60 | 18.58 | 16.9% | 100.0% | 100.0% | 100.0% |
|  | Hyb 0 | 52.15 | 45.79 | 20.53 | 15.7% | 108.1% | 107.5% | 110.5% |
|  | Hyb 1 | 46.88 | 42.27 | 19.18 | 16.9% | 97.1% | 99.2% | 103.2% |
|  | Hyb 2 | 51.28 | 45.43 | 20.06 | 15.9% | 106.3% | 106.6% | 108.0% |

Table 5: Simulation Parameters: Mechanism 2A, Option 2

| Parameters | Values |
| --- | --- |
| Simulation Type | Non-full-buffer (Medium load 50% Target RU) |
| Channel model | UMi-2GHz, UMa-200m |
| Number of BS (H,V) antenna elements | (8,8), x-polarized, subarray partition |
| (N1,N2, P) | 32 ports: (4,4,2) |
| BS (H,V) antenna spacing | (0.5, 0.8)λ |
| BS and MS antenna polarizations | BS: (+45°,-45°); MS: (0°, 90°) |
| Number of UE antennas | 2 |
| SU/MU pre-coding | SLNR |
| Scheduling | MU, Proportional fair, up to 4 layers |
| Channel estimation | Ideal |
| Transmission rank | 1,2 |
| Receiver | MMSE-IRC |
| Codebook | Class A: (O1,O2) = (8,8), Codebook-Config = 1 Hybrid codebook: - CNP = Rel. 10 8-Tx rank 1 for both horizontal and vertical; and - CBF = Rel. 8 2-Tx |

Table 6: Non-full buffer simulation results: Mechanism 2A, Option 2

| Channel | Lambda | 1st CSI-RS periodicity (ms) | Scheme | RU | Avg UPT | 50% UPT | 5% UPT | Avg UPT gain | 50% UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UMa-200m | 3.1 | 100 | Class A | 40.5% | 24.58 | 21.39 | 8.31 | 100.00% | 100.00% | 100.00% |
|  |  | 100 | Mech 1 | 39.3% | 25.87 | 22.43 | 8.17 | 105.25% | 104.84% | 98.30% |
|  |  | 100 | Mech 2A-a | 40.8% | 24.87 | 21.78 | 7.92 | 101.15% | 101.80% | 95.25% |
|  |  | 50 | Mech 2A-b | 40.3% | 24.98 | 21.86 | 8.23 | 101.61% | 102.19% | 99.04% |
| UMi-2GHz | 3.3 | 100 | Class A | 43.2% | 24.50 | 21.37 | 8.76 | 100.00% | 100.00% | 100.00% |
|  |  | 100 | Mech 1 | 41.6% | 26.00 | 22.61 | 9.50 | 106.10% | 105.78% | 108.53% |
|  |  | 100 | Mech 2A-a | 43.7% | 24.87 | 21.66 | 7.98 | 101.48% | 101.33% | 91.16% |
|  |  | 50 | Mech 2A-b | 43.4% | 24.98 | 22.10 | 8.43 | 101.93% | 103.39% | 96.27% |