3GPP TSG RAN WG1 Meeting #86                                       R1-166752

Gothenburg, Sweden 22nd - 26th August 2016

Agenda Item:   8.1.2.2

# Source:        Samsung

Title:           Discussion on grant-free/contention-based non-orthogonal multiple access

Document for:  Discussion and decision

- Introduction

In RAN1#85, non-orthogonal multiple access scheme was discussed and the following agreement was reached [1]:

Agreements:

- Autonomous/grant-free/contention based UL non-orthogonal multiple access has the following characteristics

  - A transmission from UE does not need the dynamic and explicit scheduling grant from eNB

  - Multiple UEs can share the same time and frequency resources

- For autonomous/grant-free/contention based UL non-orthogonal multiple access, the following should be studied

  - Collision of time/frequency resources from different UEs, solutions potentially including

    - E.g., code, sequence, interleaver pattern

  - UL synchronization (DL synchronization assumed)

    - Case 1: Timing offsets between UEs are within a cyclic prefix

    - Case 2: Timing offsets between UEs can be greater than a cyclic prefix, FFS the exact model of timing offsets

  - Requirement for power control

    - Case 1: Perfect open-loop power control, i.e., equal average SNR between UEs for potentially link level calibration

    - Case 2: Realistic open-loop power control with certain alpha and P0 values

    - Case 3: Close-loop power control

  - Receiver impact

This contribution considers aspects related to system operation with autonomous/grant-free/contention based multiple access.

- Discussion

All three terms, ‘autonomous’, ‘grant-free’, and ‘contention-based’ were included in the above agreements and were mixed up during the discussion. Although these terms can have same characteristics, differences may also exist. Therefore it would be beneficial to clarify their meanings for non-orthogonal multiple access.

  - Alternatives for supporting grant-free non-orthogonal multiple access

Generally, two alternatives can be envisaged for supporting grant-free non-orthogonal multiple access transmission.

- Alt.1: Grant-free transmission can be viewed as being similar to SPS transmission, where multiple access (MA) resources are pre-assigned/pre-configured to UEs and the MA resources pool shared by UEs is transparent to UE [2]. This allows the eNB to configure the MA resources for each UE based on traffic conditions and accordingly it is up to the scheduler whether to configure contention-based or non-contention based transmission for a UE. For example, when the network load is light, the eNB can make the assigned MA resources UE unique and avoid contention. When the network load is heavy, the eNB can assign one MA resource to two or more UEs these are tolerant to delay/collision and contention happens when those UEs transmit data at the same time. In this manner, the eNB can configure the MA resources according to e.g. the status of network load and UE service types, and accordingly improve the resource utilization. However, this alternative can be enabled only for UEs in the connected mode as a UE needs to be configured the available MA resource by signalling from eNB. Moreover, the assignment of MA resources will require certain level of overhead, even though which is already reduced compared with grant based transmission.

- Alt.2: A UE can autonomously/randomly choose MA resources in a pool when it has data transmission and collisions can occur as different UEs can choose same MA resource at the same time. The eNB may successfully detect some or none of the transmissions. A UE can determine an available resource pool by broadcast signalling, e.g., UE-specific signalling or in predefined manner. This alternative can be applicable even in idle mode (subject to synchronicity issue). However, as this contention based manner is not controlled by the eNB, the complexity at eNB can increase and overall performance may not be guaranteed as unknown collisions need to be handled and resource utilization efficiency cannot be optimized.

Observation 1: Grant-free does not imply contention. MA resources for grant-free transmission can be pre-configured/pre-assigned under the control of base station or selected in UE- autonomous manner.

  - Use case for grant-free transmission

Grant-free transmission can apply for both of URLLC and mMTC. For mMTC, one justification of non-orthogonal multiple access is the ability of having grant-free and contention based UL transmission, due to the characteristics of massive devices but sporadic data transmissions. However, careful study on coverage extension scenario is needed because it may be difficult for the eNB to detect and combine repeated transmissions reliably, especially with unexpected collisions. For URLLC, grant-free transmission can be also beneficial for latency reduction and, as long as the eNB can configure the multiple access (MA) resource to be non-contention based, the reliability would not suffer from any collision.

Observation 2: Grant free transmission can be applied for mMTC and URLLC when, in practice, the eNB can configure the MA resource to be non-contention based.

  - Synchronization

One possibility of grant-free/contention based multiple access for mMTC is to perform transmission without close-loop time alignment signaling or a random access process after DRX. This can provide latency reduction and signaling overhead reduction. However, a UE may not transmit at any time, e.g. on an “arrive and go” manner. Instead, the UE still needs to detect synchronization signals and MIB/SIBs to obtain the necessary information in order to obtain DL synchronization and transmit. Given DL synchronization, a UE can adjust its UL transmit timing and, in many cases [2], achieve UL synchronization (time-offsets between UEs within CP length) without close-loop timing advance command. Even in LTE, absolute UL synchronization is not guaranteed by normal CP. Hence, synchronous UL transmissions within CP can often be feasible for mMTC. Further, for a symbol length that is sufficiently long for mMTC, inter-UE ISI is then trivial. For grant-free URLLC transmission, close-loop timing alignment can be still considered. To avoid increasing the reception complexity and operation difficulty, time misalignment should be limited by using proper CP length and/or symbol duration (numerology configuration) instead of supporting relaxed synchronicity.

Observation 3: UL synchronization among UEs within CP length can be typically feasible based on DL synchronization, and/or proper numerology.

  - Power control

For grant-free/contention based non-orthogonal multiple access, there is no dynamic UL grant and it is not suitable to use a UE-group closed-loop power control mechanism (like LTE DCI format 3/3A) for contention based UL transmission. Therefore, open loop power control is a natural choice. In [3], the long-term statistically average equal SNR was assumed and then the BLER impact of short-term power fluctuation due to fading can be ignored. Further evaluations can be beneficial to conclude on the open-loop power control.

  - Collision issue

For Alt.1 (Section 2.1) of grant free transmission, an eNB can avoid collisions by assigning different MA resources to different UEs, for example according to network load and status of resources, or at least the eNB can control the collision level depending on how many UEs are assigned the same MA resources. On the other hand, the total autonomous/random selection of MA resources from the configured pool (e.g., Alt. 2) can lead to uncontrolled collisions. Even though in general the average collision probability can be lower if the MA resource pool size is larger, the eNB cannot control the collision probability, e.g., in a lightly loaded network many MA resources will be wasted while in a heavily loaded network collisions will happen too frequently and significantly impact system performance.

Observation 4: Grant free transmission under the control of eNB (Alt.1 in Section 2.1) provides more flexibility in terms of controlling collisions.

In addition, the collision issue of grant-free/contention based non-orthogonal multiple access can boil down to DMRS collision issue, concerning that an eNB receiver can only recover UEs’ data conditioned on respective un/less-correlated and accurate channel information. This brings two requirements on DMRS setting: 1) DMRS signals in the same time-frequency resource should preferably be orthogonal and 2) the collision of DMRS should be minimized and DMRS multiplexing capacity should be enhanced. Moreover, considering the DMRS resource is usually more limited than the MA signature (e.g., interleaver, grid mapping pattern etc.), the DMRS can be the limiting factor, which should be studied and designed carefully

Observation 5: DMRS collision should be minimized to efficiently support contention-based non-orthogonal multiple access.

  - Switch between grant-based and grant-free

Dynamic grant to override grant-free transmissions can enable switching between grant-based and grant-free transmission and provide flexibility to the eNB scheduler for handling urgent events or reconfiguring resources. For example, UEs may be able to access different type of services, e.g., URLLC and eMBB, and switching between grant based and grant free may be useful.

  - Support of HARQ

HARQ retransmission is one of important scheme to guarantee the reliability of transmission and/or coverage. In addition to potentially poor channel conditions, collisions among UEs and increased interference level by contention based non-orthogonal multiple access for mMTC may also lead to incorrect data detection. Therefore, HARQ retransmission should be studied for contention-based multiple access. To support HARQ retransmission, one issue is how to identify the first transmission and the retransmissions for a HARQ process. One way is for the eNB to explicitly schedule a retransmission. Another way is to divide the available MA resources into several groups, e.g. depending on the maximal number of retransmissions and their respective probabilities. The groups can be mapped to the first transmission and the re-transmission(s) correspondingly to identify the transmission order for the same HARQ process (initial or the xth retransmission). By mapping the resource partitions to different redundancy versions, the eNB can also identify whether a RV is used or missed and perform HARQ combining accordingly. Furthermore, due to the collision nature, the HARQ procedure can be different from LTE scheduled HARQ procedure.

Observation 7: HARQ retransmission can be supported for contention based non-orthogonal multiple access, e.g. by explicit signalling or by resource partitioning and mapping, for identifying initial transmission and associated retransmission(s).

  - Link adaptation

For grant-free UL transmission, there is no dynamic grant to indicate the MCS. However, link adaptation is beneficial to improve performance is also desirable for grant-free transmission case. Aside from semi-static link adaptation by the eNB, the UE may choose the MCS based on the available power after OL power control, channel quality, and interference measurement etc. and signal the selected MCS to the eNB explicitly or implicitly. Implicit signaling can be by MA resource partitioning or by code/interleaver resource partitioning where different partitions indicate different MCS selections.

Observation 8: Link adaptation with explicit/implicit MCS signalling can be supported by, e.g. resource partitioning, for grant-free non-orthogonal multiple access.

As there can be plenty of MA resource available, resource grouping/partitioning for link adaptation and/or retransmission identification is simple for some non-orthogonal multiple access schemes, e.g. IGMA in [3].

- Conclusion

In this contribution, various aspects of grant-free/contention based multiple access were discussed, and in particular, based on the analysis and observations, the proposals are as follows:

Proposal 1: The definition of autonomous, grant-free, and contention-based in the non-orthogonal multiple access should be clearly clarified.

Proposal 2: UL synchronous transmission is prioritized for the initial study of grant-free/contention based non-orthogonal multiple access.

Proposal 3: Open loop power control is supported for grant-free/contention based non-orthogonal multiple access.

Proposal 4: DMRS collision should be carefully studied with higher priority for contention based non-orthogonal multiple access.

Proposal 5: Means to support HARQ re-transmission should be studied for grant-free/contention based non-orthogonal multiple access.

Proposal 6: Means to support link adaptation should be studied for grant-free/contention based non-orthogonal multiple access.

- References

[1] RAN1 Chairman’s notes, 3GPP RAN1#85, Nanjing, China, May, 2016

[2] R1-163993, “Non-orthogonal Multiple Access Considerations for NR”, Samsung, Nanjing

[3] R1-163992, “Non-orthogonal Multiple access candidate for NR”, Samsung

[4] R1-166750, “Link level performance evaluation for IGMA”, Samsung