3GPP TSG RAN WG1 #86	R1-166792

# Gothenburg, Sweden, 22nd – 26th Aug 2016

Agenda item:	8.1.5

Source:	Samsung

Title:	Discussion on spatial multiplexing for NR

Document for:	Discussion and decision

- Introduction

In RAN1#85, the following is agreed for DL multi-antenna transmissions.

Agreements:

- The following DL multi-antenna transmissions to be studied for NR

  - Closed-loop/(Semi)Open-loop spatial multiplexing

  - Single/Multi-point transmissions

  - SU/MU-MIMO

  - Transmit diversity,

    - e.g., Single/Multi panel spatial diversity

  - Combination of above techniques

- Other DL multi-antenna transmissions and related techniques are not precluded

- This does not imply that used transmission technique needs to be known to the UE

This contribution discusses different spatial multiplexing schemes for NR.

- Discussion on Spatial Multiplexing for NR

MIMO spatial multiplexing can be divided into vertical, horizontal, and diagonal multiplexing schemes on a large scale depending on how to map codewords to layers. It is necessary to study these multiplexing schemes for NR MIMO, since there is a tradeoff for the performances between the number of the utilized resource blocks (RBs) and the employed MCS in different scenarios, in addition to different signaling overheads.

- Vertical multiplexing spreads one codeword to all layers. Thus, it has a benefit from antenna diversity, while it treats other layers as noise, at a receiver. One CQI is required for feedback.

- Horizontal multiplexing transmits a different codeword at each layer. Thus, it has a benefit from successive interference cancellation (SIC) effect, while it does not have antenna diversity. Two CQIs are required for feedback and rate control.

- Diagonal multiplexing spreads a codeword diagonally across antenna and time/frequency, which enables antenna diversity in a codeword. Known blocks are utilized in the beginning or at the end of the transmission in order to help decode the first or the last codeword without errors. This enables the SIC effect. The information bits per diagonal layer are recovered sequentially by performing SCD for multiple codewords. It has a benefit of natural rate control from diagonal coding, while it suffers from the known block rate loss. One CQI is required for feedback.

Link-level performance simulations are carried out for 2x2 MIMO transmission and reception in Ped-B environments. MCS 7, 15, 21 in 50 and 100 RBs are considered to show the tradeoff for the link-level performances of vertical, horizontal, and diagonal multiplexing, as shown in Figs. 1 and 2.

(a) MCS 7, Modulation: QPSK     (b) MCS 15, Modulation: 16-QAM    (c) MCS 21, Modulation: 64-QAM

Fig. 1. Link-level performance for vertical, horizontal, and diagonal multiplexing with RI=2 and 50 RBs in the 2x2 MIMO Ped-B channels.

(a) MCS 7, Modulation: QPSK     (b) MCS 15, Modulation: 16-QAM    (c) MCS 21, Modulation: 64-QAM

Fig. 2. Link-level performance for vertical, horizontal, and diagonal multiplexing with RI=2 and 100 RBs in the 2x2 MIMO Ped-B channels.

There is not a single best spatial multiplexing scheme for all MCS and RBs. Another observation is that diagonal multiplexing tends to have relatively better performances for higher number of RBs. Different spatial multiplexing schemes may need to be revisited, since NR MIMO should be able to support diverse deployment scenarios and use cases unlike LTE.

Proposal 1: Different spatial multiplexing for NR MIMO should be studied by taking performance, signaling overhead, and impact on overall design into account.

- Conclusion

This contribution discussed the spatial multiplexing schemes for NR MIMO from the aspects of performance, signaling overhead and transceiver design. Based on the discussion, our proposal is as follows:

Proposal 1: Different spatial multiplexing for NR MIMO should be studied by taking performance, signaling overhead, and impact on overall design into account.