3GPP TSG RAN WG1 #86		R1-166801

Gothenburg, Sweden 22nd - 26th August 2016

Agenda item:	8.1.7

Source:	Samsung

Title:	TRP density and antenna modeling for indoor open office scenario

Document for:	Discussion

# Introduction

In RAN1#85 [1], antenna placement in Indoor scenarios for system-level evaluation assumption was agreed. This contribution analyzes the further details of the system-level evaluation assumptions and deployment scenarios for indoor open office.

# Deployment scenarios for Open-Office scenario

Four TRP antenna modeling are considered indoor open office scenarios:

- Option #1: single-sector.

- Option #2: Three sector.

- Option #3:Wall mounted.

- Option #4: ceiling mount.

The details of each antenna modeling are described in following sections.

Option #1: single-sector

In single-sector based, omni in azimuth dimension and 130 degree 3dB beamwidth in zenith dimension. Detailed antenna element radiation pattern according to below table. Electrical tilting is 90 degree. The parameters given in Table 1

Table 1

| Antenna element radiation pattern in θ" dim (dB) | AE,Vθ"=-min12θ"-90°θ3dB2,SLAV,θ3dB=130°, SLAV=25 |
| --- | --- |
| Antenna element radiation pattern in ϕ" dim (dB) | AE,Hφ"=0 |
| Combining method for 3D antenna element pattern (dB) | A"θ",φ"=AE,Vθ" |
| Maximum directional gain of an antenna element | [5dBi] |

Option #2: Three sector

Each sector has 65 degree 3dB beam width in both zenith and azimuth dimensions. Detailed antenna element radiation pattern according to below table. [110] degree electrical tilting. The parameters are given in table 2.

Table 2

| Antenna element radiation pattern in θ" dim (dB) | AE,Vθ"=-min12θ"-90°θ3dB2,SLAV,θ3dB=65°, SLAV=25 |
| --- | --- |
| Antenna element radiation pattern in ϕ" dim (dB) | AE,Hφ"=-min12φ"φ3dB2,Am,φ3dB=65°, Am=25 |
| Combining method for 3D antenna element pattern (dB) | A"θ",ϕ"=-min-AE,Vθ"+AE,Hϕ",Am |
| Maximum directional gain of an antenna element | [8dBi] |

Option #3: Ceiling mount

TRP is placed below the ceiling.  Sector with 130 degree 3dB beam width in both zenith and azimuth dimensions. Antenna orientation is towards floor. The parameters of antenna are given in Table 3.

Table 3

| Antenna element radiation pattern in θ" dim (dB) | AE,Vθ"=-min12θ"-90°θ3dB2,SLAV,θ3dB=130°, SLAV=25 |
| --- | --- |
| Antenna element radiation pattern in ϕ" dim (dB) | AE,Hφ"=-min12φ"φ3dB2,Am,φ3dB=130°, Am=25 |
| Combining method for 3D antenna element pattern (dB) | A"θ",ϕ"=-min-AE,Vθ"+AE,Hϕ",Am |
| Maximum directional gain of an antenna element | [5dBi] |

Option #4: wall mounted

Antenna is wall mounted. Single sector with 90 degree 3dB beam width in both zenith and azimuth dimensions. Antenna orientation is opposite to the wall. Detailed TRP positions FFS. Electrical tilting is 90 degree. The parameters are given in Table 4

Table 4

| Antenna element radiation pattern in θ" dim (dB) | AE,Vθ"=-min12θ"-90°θ3dB2,SLAV,θ3dB=90°, SLAV=25 |
| --- | --- |
| Antenna element radiation pattern in ϕ" dim (dB) | AE,Hφ"=-min12φ"φ3dB2,Am,φ3dB=90°, Am=25 |
| Combining method for 3D antenna element pattern (dB) | A"θ",ϕ"=-min-AE,Vθ"+AE,Hϕ",Am |
| Maximum directional gain of an antenna element | 5dBi |

There are three deployment options for indoor open office scenario:

- Deployment option 1: 12 TRPs,

  - The deployment of TRPs for Antenna option #1, #2 and #4 is shown in Figure 1-(a),

  - The deployment of TRPs for antenna option #3 is shown in Figure 1-(b)

- Deployment option 2: 6 TRPs

  - The deployment of TRPs for Antenna option #1, #2 and #4 is shown in Figure 2-(a),

  - The deployment of TRPs for antenna option #3 is shown in Figure 2-(b)

- Deployment option 3: 3 TRPs

  - The deployment of TRPs for Antenna option #1, #2 and #4 is shown in Figure 3-(a),

  - The deployment of TRPs for antenna option #3 is shown in Figure 4-(b)

(a)                                 (b)

Figure 1: 12 TRPs.

(a)                                 (b)

Figure 2: 6 TRPs.

(a)                                 (b)

Figure 3: 3 TRPs.

# Simulation Results

In the following, the LOS angle-based geometry SINR and SIR with the above deployment options and antenna options are evaluated. Three carrier frequencies are simulated: 4 GHz, 30 GHz and 70 GHz. The detailed results can be found in the following sections.

Table 5: Cell-edge performance: interf. vs noise limited

| Number of TRPs | Cell-edge performance: interf. limited vs. noise limited |
| --- | --- |
| 12 | @ 4, 30, 70 GHz, antenna option 1,2,3,4: interference-limited |
| 6 | @ 4 GHz, antenna option 1, 2, 3, 4: interference-limited @ 30 GHz, antenna option 1, 2, 3, 4: interference-limited @ 70 GHz, antenna option 1, 2, 3, 4: noise-limited |
| 3 | @ 4 GHz, antenna option 1, 2, 3, 4: interference-limited @ 30 GHz, antenna option 1, 2, 3, 4: noise-limited @ 70 GHz, antenna option 1, 2, 3, 4: noise-limited |

Table 1 summarizes the simulation results in terms of whether the cell-edge user (<0dB SINR) is noise limited or interference limited, to understand whether the current TRP density of 12 TRPs is good for the performance evaluations. According to the table, it is clear that 12 TRPs create too much interference at all the carrier frequencies. At 4 GHz, in particular, 3 TRPs seem to be sufficient and perform the best. On the other hand, at 30 and 70 GHz, it is not clear whether 3 TRPs or 6 TRPs are the best among those 3 cases. Other charts are provided below in Figures 4-a and 4b to help understand the situation here.

Figure 4-a. Percentage of UEs less than 0 dB SINR

Figure 4-b. Percentage of UEs less than 10 dB SINR

In Figures 4-a and 4-b, percentages of UEs less than 0 dB SINR and 10 dB SINR are respectively shown, as a function of carrier frequency, antenna option and number of TRPs.

The following observations can be made based on Figures 4-a and 4-b.

Observations:

- At all the 3 carrier frequencies, the best geometry is achieved with 3 TRPs.

- The best antenna options are different at different carrier frequencies.

  - At 4/30/70 GHz, the best antenna option is option 3 (ceiling mount) at both 0 dB and 10 dB SINR.

Based on the above observations and potential MIMO deployment for satisfying the eMBB KPIs with 8 MIMO layers, the following is proposed for the indoor open-office scenario.

Proposals:

- At all the carrier frequency, adopt 3 TRP based deployment shown in Figure 3-(a).

- Adopt the antenna patterns as in the following table:

|  | 4 GHz | 30 GHz | 70 GHz |
| --- | --- | --- | --- |
| Antenna patterns | ceiling | ceiling | ceiling |

  - Pattern 1: Ceiling mounted antenna. Each TRP has 1 sector. Antenna has θ3dB=φ3dB=130°, SLAV=Am=25 and GE,max=4 dBi.

  - Pattern 2: Ceiling mounted antenna. Each TRP has 1 sector. Omni antenna with G = 5 dBi.

- Adopt the (M,N,P) = (2,2,2) antenna configuration shown in Figure 3, where both the horizontal and the vertical antenna spacing values are 0.5λ.

Figure 3: Proposed BS antenna option

# Conclusion

The proposals made in this contribution are summarized below:

Proposals:

- At all the carrier frequency, adopt 3 TRP based deployment shown in Figure 3-(a).

- Adopt the antenna patterns as in the following table:

|  | 4 GHz | 30 GHz | 70 GHz |
| --- | --- | --- | --- |
| Antenna patterns | ceiling | ceiling | ceiling |

  - Pattern 1: Ceiling mounted antenna. Each TRP has 1 sector. Antenna has θ3dB=φ3dB=130°, SLAV=Am=25 and GE,max=4 dBi.

  - Pattern 2: Ceiling mounted antenna. Each TRP has 1 sector. Omni antenna with G = 5 dBi.

- Adopt the (M,N,P) = (2,2,2) antenna configuration shown in Figure 3, where both the horizontal and the vertical antenna spacing values are 0.5λ.

# References

[1] R1-165850, WF on antenna placement in Indoor scenarios	Intel, NTT DoCoMo, Samsung, Ericsson

# Appendix A

## Results of 3 TRPs

Figure A-0: 3 TRPs and 4GHz

Figure A-1: 3 TRPs and 30 GHz

Figure A-2: 3 TRPs and 70GHz

## Results of 6TRPs

Figure B-0: 4GHz

Figure B-1: 30GHz

Figure B-2: 70GHz

## Results of 12 TRPs

Figure C-0: 4GHz

Figure C-1: 30GHz

Figure C-2: 70GHz