3GPP TSG RAN WG1 Meeting #86					R1- 166870

Gothenburg, Sweden 22nd - 26th August 2016

Agenda Item:	8.1.2.1

Source: 	LG Electronics

Title: 	Waveform evaluation results in case 4

Document for:	Discussion

- Introduction

According to [1], a study of waveforms that is a part of the approved SID on Next New Radio Access Technology is conducted to decide the basic waveform for NR. Therefore, in this contribution, we present evaluation results of waveform candidates for case 4, targeting multi-user with mixed numerology (synchronous reception) scenario.

Details of waveform specific simulation parameters

To evaluate CP-OFDM, W-OFDM, F-OFDM, and FCP-OFDM, we consider the following parameters and assumptions.

- Transceiver description for FCP-OFDM

In order to clarify FCP-OFDM proposed in [2], we first describe details of FCP-OFDM.

Figure 1 Block diagram of FCP-OFDM

FCP-OFDM is one of sub-band wise filtering based waveforms with flexible CP/ZP. The block diagram of transceiver for the generation of FCP-OFDM signal is depicted in figure 1. FCP-OFDM signal at the transmitter side is generated by following precedures: 1) pre-equalization of the filter, 2) IFFT operation, 3) insertion of CP/ZP, and 4) subband filtering.

Pre-equlization of the filter, the inverse filter frequency response, is used to prevent an increased number of error events due to weaker pass-band edge subcarriers as in [3]. FCP-OFDM utilizes the variable combination of CP and ZP with filtering per each sub-band.

At receiver side, the matched filter and windowing techniques can be adopted for the enhancement of the performance to both suppress the interfering signals and increase the power of the desired signal.

- FCP-OFDM

The specific parameters associated with FCP-OFDM are summarized in Table 1.

Table 1 FCP-OFDM parameters

| FCP-OFDM settings | Value |
| --- | --- |
| Tx filter type | Dolph-Chebyshev filter |
| Tx filter info. | - Pre-equalization of filter response is considered. - Target subband : Filter length: 61, SLA: 40 dB, CP length: 60, ZP length: 12 - Interfering subband : Filter length: 61, SLA: 40 dB, CP length: 30, ZP length: 6 |
| Rx filter type | Matched filter |
| Rx window type | Raised cosine window |
| Rx window length | 10 |
| Rx windowing operation |  |

- F-OFDM [4]

The specific parameters associated with F-OFDM are summarized in Table 2.

Table 2 F-OFDM parameters

| f-OFDM settings | Value |
| --- | --- |
| Tx filter type | Windowed Sinc (Root-Raised Cosine window) |
| Tx filter order | 512 for all subbands |
| Tx filter's tone-offset | Target subband (SC spacing 15 kHz): 2.5 SC spacing Interfering subband (SC spacing 30 kHz): 1.25 SC spacing |
| Rx filter type | Windowed Sinc (Root-Raised Cosine window) |
| Rx filter order | 512 for all subbands |
| Rx filter's tone-offset | Target subband (SC spacing 15 kHz): 2.5 SC spacing Interfering subband (SC spacing 30 kHz): 1.25 SC spacing |

- W-OFDM [5]

The specific parameters associated with W-OFDM are summarized in Table 3.

Table 3 W-OFDM parameters

| W-OFDM settings | Value |
| --- | --- |
| Tx window type | Raised cosine window |
| Tx window edge length | 52 for target subband; 26 for interfering subband |
| Tx overlapped window length | 52 for target subband; 26 for interfering subband |
| Rx window type | Raised cosine window |
| Rx window edge length | 10 |
| Tx/Rx windowing operation |  |

Evaluation results

  - Simulation parameter settings

For evaluation comparison, Table 4 shows parameters selected in evaluation parameters agreed in RAN1 #85 (refer to Appendix 6.1).

Table 4 Evaluation parameters

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Duplex | FDD, TDD considered by calculation |
| System Bandwidth and FFT size | 10 MHz, 1024 for 15 kHz subcarrier spacing |
| Subframe duration | 1 ms |
| Subcarrier spacing | Target UE: 15 kHz, Interferer pair: 30kHz |
| Guard time interval | 6.7% overhead |
| Data transmission bandwidth | Target UE: 720 kHz, Interferer pair: 720kHz (per UE) |
| Bandwidth of guard tones between neighboring UEs | 60 kHz |
| Number of uplink users | 3 (1 target user and 2 interferer uses) |
| Power offset of the interferer user | 10 dB |
| Antenna configuration | 1T1R |
| MCS | 16QAM coding rate=1/2, 64QAM coding rate=1/2, 64QAM coding rate=3/4 |
| Control Overhead | Zero |
| Time offset of interfering user | None |
| Channel estimation | Ideal |
| Channel Model | TDL-C with 300 ns delay spread, 3km/h |

RF non-linearity model should be considered in the evaluation as agreed in [6]. Therefore, the PA model and simulation setup are summarized in Table 5.

Table 5 PA model and simulation setup

| Additional assumptions | Value |
| --- | --- |
| The number of Turbo decoding iteration | 8 |
| Turbo decoding algorithm | max-log-MAP algorithm |
| Whether assume the CRC bits to be part of the information bits in the code rate calculations or not | No |
| The number of subframes for simulations | 10000 |
| The output power for the UL polynomial model | 22 dBm |
| The common phase compensation for the UL polynomial PA model | 76.3 degree |
| The baseline waveform chosen for UL | CP-OFDM |
| Whether Extrinsic information scaling (scale = 0.75) or not (scale = 1) in the max-log-MAP algorithm | 1 |
| IBO for UL Polynomial model, 4PRB, SC separation 15 kHz | 8 dB |

Also, we assume 4 guard tone arrangement as shown in Figure 2.

Figure 2 Description for guard band setting in case 4

  - Spectral efficiency

In this section, evaluation results without and with PA model are shown.

The spectral efficiency (SE) is calculated by

Spectral efficiency = TBS×(1-BLER)T×BW

with the following definitions:

- TBS: the number of bits in a transport block

- BLER: block error rate

- T: the time duration of one subframe

- BW: the actual bandwidth of target subband

In TDD system, to compute the time duration, T, the filter and windowing tails need to be considered. However, when a subframe contains 14 OFDM symbols (=1ms), these overheads can be negligible in terms of SE performance. So, it is assumed that the performance of TDD is identical to that of FDD. However, if a subframe is composed of a few OFDM symbols, the impact of filter and window tails needs to be considered.

    - Without PA

Figure 3 Spectral efficiency of different waveform candidates for case 4 without PA

Figure 4 TDL-C, 16QAM with coding rate=1/2 for case 4

Figure 5 TDL-C, 64QAM with coding rate=1/2 for case 4

Figure 6  TDL-C, 64QAM with coding rate=3/4 for case 4

As shown in Figures 3-6, we can see that F-OFDM has the best performance in terms of SE and BLER between considered waveforms due to good frequency localization in the ideal case.

Observation 1: In ideal case, F-OFDM can provide performance gain compared to the other waveforms.

    - With PA

Figure 7 Spectral efficiency of different waveform candidates for case 4 with PA

Figure 8 TDL-C, 16QAM with coding rate=1/2 for case 4

Figure 9 TDL-C, 64QAM with coding rate=1/2 for case 4

Figure 10 TDL-C, 64QAM with coding rate=3/4 for case 4

Figures 7-10 presents the performance comparison in terms of SE and BLER with the practical assumption considering realistic PA model. In this simulation setup, performance of 64QAM with code rate ¾ does not have meaningful result due to severe performance degradation. For the cases of 16QAM ½ and 64QAM ½, it can be observed that subband filtered OFDM waveforms such as FCP-OFDM and F-OFDM have better performance than windowed based OFDM waveform. This is due to better robustness against the neighboring subband interferences, which enables a mixed numerology coexistence with less guard band.

Based on the results, we address the following observations:

Observation 2: In practical case considering realistic PA model, subband-wise filtered OFDM waveforms (including FCP-OFDM) has better performance than windowed OFDM in mixed numerology case.

  - Discussion on complexity and processing delay

    - Complexity comparison

In order to calculate Tx and Rx complexity, we consider IFFT block, FFT block, matched filter block, and filtering/windowing block. For the matched filter and filtering operations, both the time domain convolution and frequency domain implementation are considered. Additionally, we apply fast Fourier transform complexity. We follow the complexity calculation method proposed in [7]. The complexity is summarized in Table 6 and 7.

Table 6 Complexity comparison in Tx

|  | Real Multiplications |  | Real Additions |  |
| --- | --- | --- | --- | --- |
| F-OFDM | TD conv. | (Mlog2M-3M+4)+2(M+LCP)LF-ODFM | TD conv. | (Mlog2M-3M+4)+2M'F-OFDM(LF-OFDM-1) |
|  | FD conv. (fast conv.) | Mlog2M-3M+4+52M'F-OFDMlog2M'F-OFDM-3M'F-OFDM+4+4M'F-OFDM | FD conv. | Mlog2M-3M+4+523M'F-OFDMlog2M'F-OFDM-3M'F-OFDM+4-M'F-OFDM+2+2M'F-OFDM+LF-ODFM |
| FCP-OFDM | TD conv. | Mlog2M-3M+4 +2M+LCPLFCP-OFDM | TD conv. | Mlog2M-3M+4+2M'FCP-OFDM(LFCP-OFDM-1) |
|  | FD conv. | Mlog2M-3M+4+52M'FCP-OFDMlog2M'FCP-OFDM-3M'FCP-OFDM+4+4M'FCP-OFDM | FD conv. | Mlog2M-3M+4+523M'FCP-OFDMlog2M'FCP-OFDM-3M'F-OFDM+4-M'FCP-OFDM+2+2M'FCP-OFDM+LFCP-OFDM |

(M: IFFT size, LF-ODFM: filter length of F-OFDM, LFCP-OFDM: filter length of FCP-OFDM,  LW: length of Rx windowing, LCP: CP length, M'F-OFDM≅M+LCP+LF-OFDM-1, M'FCP-OFDM≅M+LCP+LFCP-OFDM-1)

Table 7 Complexity comparison in Rx

|  | Real Multiplications |  | Real Additions |  |
| --- | --- | --- | --- | --- |
| F-OFDM | TD conv. | (Mlog2M-3M+4)+2(M+LCP)LF-ODFM | TD conv. | (Mlog2M-3M+4)+2M'F-OFDM(LF-OFDM-1) |
|  | FD conv. | Mlog2M-3M+4+52M'F-OFDMlog2M'F-OFDM-3M'F-OFDM+4+4M'F-OFDM | FD conv. | Mlog2M-3M+4+523M'F-OFDMlog2M'F-OFDM-3M'F-OFDM+4-M'F-OFDM+2+2M'F-OFDM+LF-ODFM |
| FCP-OFDM | TD conv. | Mlog2M-3M+4 +2M+LCPLFCP-OFDM+4LW | TD conv. | Mlog2M-3M+4+2M'FCP-OFDM(LFCP-OFDM-1) +2LW |
|  | FD conv. | Mlog2M-3M+4+52M'FCP-OFDMlog2M'FCP-OFDM-3M'FCP-OFDM+4+4M'FCP-OFDM+4LW | FD conv. | Mlog2M-3M+4+523M'FCP-OFDMlog2M'FCP-OFDM-3M'F-OFDM+4-M'FCP-OFDM+2+2M'FCP-OFDM+LFCP-OFDM+2LW |

By applying the values considered in the evaluation, we can obtain the complexity of F-OFDM and FCP-OFDM as shown in Table 8. Therefore, we can observe that F-OFDM requires more complexity compared to FCP-OFDM

Table 8 Tx and RX complexity

|  | Tx complexity |  | Rx complexity |  |
| --- | --- | --- | --- | --- |
| F-OFDM | TD conv. | 2779002 | TD conv. | 2779002 |
|  | FD conv. | 169960 | FD conv. | 169960 |
| FCP-OFDM | TD conv. | 283872 | TD conv. | 290076 |
|  | FD conv. | 119216 | FD conv. | 125420 |

    - Processing delay

As addressed in [8], time localization property is also one of important factors of waveform as well as spectral containment in frequency domain. A well time-localized waveform is essential for NR to support low latency communication with very short TTIs. Especially, the time localization of waveform is tightly coupled with UL/DL switching in TDD system. For example, in the extreme case when DL/UL switching occurs symbol by symbol, filter length should be carefully designed in order to absorb guard period. Figure 11 presents a conceptual example of DL/UL switching period. When matched filter is applied into Rx, receiver needs to capture energy within filter tail. Thus, after receiving the end of tail of filtered signal, RF could be able to transmit signal. Also, time corresponding to filter tail length is required so that the filtered transmit signal is sent. For the case without considering round trip time, minimum required switching time corresponds to filter length. This required period could impact on the guard time design with supported cell radius, especially when symbol duration is very short.

Figure 11 An example of Tx/Rx transient period

F-OFDM leads to the long group delay due to the long filter length and thus F-OFDM requires a processing delay of 1 OFDM symbol because the length of F-OFDM filter is set to be a half of OFDM symbol length. On the other hands, since the filter length of FCP-OFDM is shorter than CP length of OFDM symbol, FCP-OFDM has a short filter length in comparison with F-OFDM. Therefore, the delay due to FCP-OFDM is relatively small.

Observation 3: FCP-OFDM has low complexity and processing delay compared to F-OFDM.

Observation 4: With spectral efficiency gain, suitable complexity and processing delay also need to be considered at the same time.

Proposal 1: FCP-OFDM can be considered as one of NR waveform candidates.

Summary

In this contribution, we provided and discussed the simulation results to evaluate both the subband-wise filtering and windowing based waveforms focusing on case 4. Based on the evaluation results, the following observations are given:

Observation 1: In ideal case, F-OFDM can provide performance gain compared to the other waveforms.

Observation 2: In practical case considering realistic PA model, subband-wise filtered OFDM waveforms (including FCP-OFDM) has better performance than windowed OFDM in mixed numerology case.

Observation 3: FCP-OFDM has low complexity and processing delay compared to F-OFDM.

Observation 4: With spectral efficiency gain, suitable complexity and processing delay also need to be considered at the same time.

Proposal 1: FCP-OFDM can be considered as one of NR waveform candidates.

- Reference

- [1] RP-160671, New SID Proposal: Study on New Radio Access Technology, NTT DoCoMo

- [2] R1-162516, “Flexible CP-OFDM with variable ZP”, LG Electronics

- [3] R1-165014, “Subband-wise filtered OFDM for New Radio below 6 GHz” Nokia, Alcatel-Lucent Shanghai Bell

- [4] R1-164033, “f-OFDM scheme and filter design”, Huawei, HiSilicon

- [5] R1-162199, “Waveform candidates”, Qualcomm Incorporated

- [6] R1-163889, “Way forward on assumptions for waveform evaluation”

- [7] R1-163990, “Preliminary evaluation results for OFDMA-based waveforms”, Samsung

- [8] R1-165434, “Initial evaluation result for waveform”, LG Electronics

- Appendix

  - Summary of evaluation parameters for case 4 agreed in RAN1 #85

  - PSD w/ and w/o PA model for case 1b

For comparison in terms of PSD, PSDs of waveforms considered for evaluation are presented in the followings.

Figure 12 Spectrum of different waveform candidates for 4 PRB allocation size without PA

Figure 13  Spectrum of different waveform candidates for 4 PRB allocation size with PA

    - CP-OFDM, 4PRBs

    - FCP-OFDM, 4PRBs

    - W-OFDM, 4PRBs

    - F-OFDM, 4PRBs