3GPP TSG RAN WG1 Meeting #86                          R1-166875

Gothenburg, Sweden 22nd - 26th August 2016

Agenda Item:	8.1.2.2

Source: 	LG Electronics

Title: 	Considerations on the receiver types and NoMA schemes

Document for:	Discussion

Introduction

In NR design, the non-orthogonal multiple access (NoMA) schemes are considered for some KPIs [1][2]. The NoMA schemes have been proposed for the increase of the connectivity or the system throughput. These schemes are based on the non-orthogonal spreading code or the difference of spatial resource and power. In case of specific channel environments with system optimizations (e.g. power allocation or user scheduling); the NoMA schemes can provide the improved connectivity or the increased system throughput compared to the conventional OMA schemes. However, NoMA schemes have some defeats, such as scheduling complexity, encoding/decoding complexity, loss of BLER, and limited environments. In addition, the NoMA schemes induce multiuser interference (MUI) inherently. To mitigate the MUI, advanced receivers can be considered for NR design. As the first step toward NR design, the advanced receivers for NR are discussed in this document.

Followings are the observations on the multiple access of NR in RAN1 #84bis;

Observations:

- Examples non-orthogonal schemes include (but not limited to):

  - For UL, Multi-user shared access (MUSA) (e.g., R1-162226)

  - Resource spread multiple access (RSMA) (e.g., R1-163510)

  - Sparse code multiple access (SCMA) (e.g., R1-162153)

  - Pattern defined multiple access (PDMA) (e.g., R1-163383)

  - Non-orthogonal coded multiple access (NCMA) (e.g., R1-162517)

  - Low code rate spreading (e.g., R1-162385)

  - Frequency domain spreading (e.g., R1-162385)

  - Non-orthogonal multiple access (NOMA) (e.g., R1-163111)

Receiver Types

In the NoMA schemes, the superposition of the symbols for multiple users leads to the MUI. To mitigate the MUI, various types of receiver have been proposed, depending on the technical characteristics of each NoMA scheme. We can summarize the receiver types as follows:

  - Optimal Receiver

Maximum Likelihood (ML) (or Maximum joint A Posteriori (MAP)) detection is known that optimal performance can be provided at the receiver side, assuming only symbol level detection. If the priori symbol distribution is uniform, performance of ML is equivalent to that of MAP. Thus, in common communication systems, the ML detection can be considered as optimal detection in terms of symbol level detection. The ML detection maximizes the probability of the received signal ‘y’ on condition of the correct symbol ‘s’ is sent. The ML detector can be represented as follows: sML=argmaxP(y|s).

The enumerative search for the minimum squared Euclidean distance for received signal from all possible symbol vector combinations in ML and MAP detection make them too complicated and impractical for hardware implementation. Moreover, considering the channel coding, the codeword level ML detection can provide more enhanced performance compared to the symbol level ML detection. However, it is no longer acceptable complexity at the receiver.

  - Linear Receiver

This type of receivers is considered as a baseline receiver. In general, maximum ratio combining (MRC), zero-forcing (ZF) and minimum mean square error (MMSE) receivers can be regarded to the linear receiver in multiple antenna systems.

MRC, ZF and MMSE are methods of diversity combining. The MRC, ZF and MMSE receivers have low complexity and assume that the channel information is known at receiver side. However, although the MRC receiver can increase the SNR by maximizing signal combining, it cannot handle the interferences. In addition, although the ZF receiver can maximize the SIR by nulling the interferences, it causes noise enhancement so that it will decrease the SNR. On the other hand, the MMSE receiver reduces the noise enhancement compared to ZF and increase the SINR by handling both noise and interference level, but it can still have poor performance in a fading channel. The MRC, ZF and MMSE receivers are a principal choice considering its performance and complexity.

After passing the above linear receivers for multiple antenna system, in spreading based NoMA scheme, the receiver needs to de-spread the received signal before passing them to decoder. In other words, from multi-user detection (MUD) perspective, MUD is done by the Hermitian transpose of spreading sequence matrix. This process can view as a match filter (MF) [3]. So, the spreading, repetition or scrambling based NoMA schemes need to consider the MF process basically.

  - Iterative Receiver

This type of receivers is known that sub-optimal performance can be provided at the receiver side. The ML detector has prohibitive computational requirements for most current applications, and consequently a variety of linear and nonlinear multiuser detectors have been proposed to ease this computational burden while maintaining satisfactory performance. An obvious approach for reducing MUI is to subtract an estimate of the MUI form the received signal. Typically, this estimate will not be perfect, and there will be some residual MUI. This motivates an iterative cancellation approach for recursively improving symbol estimates and removing residual interference [4]. However, if the combined system has large dimensions (e.g., large array size, large user population, and combinations of these conditions), the implementation of these suboptimal techniques still proves to be very complex [5].

- Interference Cancellation (IC)

Interference cancellation (IC) is a well-known physical layer technique. Briefly, IC is the ability of a receiver to receive two or more signals concurrently. The IC principle is, firstly, to detect the information of the interfering users by single user detection (SUD) techniques. Here, in the spreading based NoMA schemes, SUD might be a linear receiver including channel equalizations, such as MF with MMSE. The interfering part is then reconstructed and cancelled from the received signal. Detection of the desired user is finally performed. IC types are divided to parallel interference cancellation (PIC) and ordered-successive interference cancellation (Ordered-SIC). According to the IC types, IC can be performed in parallel (PIC) or sequential (Ordered-SIC). In PIC, the part of all interfering users is cancelled simultaneously. In Ordered-SIC, the part of the strongest remaining interferer is cancelled iteratively. Ordered-SIC is generally called as SIC.

In the initial detection stage of PIC, the data symbols of all active users are detected in parallel by the SUD technique. Then, the obtained interference is subtracted from the received signal and data detection is performed again with reduced MUI.

The SIC receiver detects and cancels the part of the first selected interfering user from the received signal, then detection and cancellation of part of the second selected user follows, and so on. A specific ordering must be enforced. In the case of SIC, interferers are ordered based on their decreasing received power levels.

PIC could be preferred when the amount of interference from each user is similar. On the other hand, SIC could be used in the case of interferers with different signal strength. In general, the SIC receiver, while saving hardware complexity, takes more processing time than the PIC receiver [6]. In addition, the IC receivers have the problem of error propagation, so that their performance is highly dependent on the first estimate being fed to the IC detector.

This type of receiver has been considered in RSMA, NCMA, IDMA, Low code rate spreading and Power level NoMA (or MUST) schemes.

- Massage Passing Algorithm (MPA)

MPA is known as Sum-Product (Belief/Probability Propagation) or Max-Product (Min-Sum) Algorithm. The MPA is also known as iterative decoding algorithm as message pass back and forth between the variable node and check node iteratively until result is achieved or the process is halted.

MPA is used to compute bit LLR (converted from symbol LLR) assuming no a prior information, then the bit LLR is fed into the decoder to finish decoding each user’s packet. Here, the bit LLR can be obtained by the iterative computation based on Belief Propagation. Being assumed to the sparsity of sequence (or resource), it can be reduced to the number of pass for obtaining the bit LLR. In this case, overall complexity can be reduced. In addition, the performance of MPA might approach the optimal performance in specific environments. Although many works to reduce the complexity have been proposed, the implementation of MPA is still complex.

This type of receiver has been considered in SCMA and PDMA schemes.

Considerations on the NoMA scheme related to receiver types

In general, the details of receiver algorithm may be a receiver design choice in terms of implementation. In addition, the complexity of the above algorithms can be varied, according to the way of hardware implementation. Moreover, the receiver algorithms can be combined with each other, such as MF-SIC, MMSE-PIC, MPA-SIC, and so on. Thus, the NoMA schemes proposed in NR should be able to operate properly at various types of receiver, at least all of the common receivers mentioned in Section 2. It is not proper to design the NoMA scheme optimized to a specific receiver type. Consequently, NoMA schemes adapted to various types of receiver should be designed.

Proposal: The NoMA schemes proposed in NR should be able to operate properly at various types of receiver, at least all of the common receivers. NoMA schemes adapted to various types of receiver should be designed.

Summary

In this document, we discussed on the receiver types for the NoMA schemes. Our suggestions can be summarized as below.

Proposal: The NoMA schemes proposed in NR should be able to operate properly at various types of receiver, at least all of the common receivers. NoMA schemes adapted to various types of receiver should be designed.

Reference

R1-162517, LG Electronics, “Considerations on DL-UL multiple access for NR”

R1-164557, LG Electronics, “Initial LLS Evaluation Result for NoMA”

R1-164689, Qualcomm Incorporated, “RSMA and SCMA comparison”

Michael L. Honig, “Advances in Multiuser Detection,” Wiley

Ezio Biglieri, “MIMO Wireless Communications,” Cambridge Press

Nevio Benvenuto, “Parallel and Successive Interference Cancellation for MC-CDMA and their Near-Far Resistance,” IEEE VTC 2003.