3GPP TSG RAN WG1 Meeting #86	  				  	R1-166916

Gothenburg, Sweden, 22th - 26th August 2016

______________________________________________________________________Agenda item: 8.1.7

Source: LG Electronics

Title: Remaining issues on the evaluation assumptions for eV2X

Document for: Discussion and Decision

- Introduction

In 3GPP RAN1#85 meeting, the agreements are made for the evaluation assumptions for NR eV2X study, as shown in Appendix. Also, in the email discussion [85-15], the following performance metric is agreed:

Agreement:

CAR (Communication Availability and Resilience) = (1-X)*(1-Y)

- CAR metric is used for 1) sidelink only, 2) downlink only, or 3) uplink only simulation.

  - For the other case, the equation can be modified.

- 1-X represents the availability of the communication connection, where X is the time portion during which the communication is interrupted by handover failure or link failure.

  - X is simulated like mobility management similar to HetNet mobility in TR36.839.

  - Depending on application, X can be prefixed without evaluation of X, e.g., X=0 for sidelink.

- 1-Y represents the resilience of the communication scheme:

  - Alt. 1: (1-Y) is simulated as PRR (packet reception ratio) within a specific range as defined in TR36.885.

  - Alt. 2: (1-Y) is simulated as a function of packet reception ratio per link.

    - Detailed method for Alt. 2 is further studied in RAN1#86.

  - Depending on the application, choose Alt. 1 and/or Alt.2.

- X and Y are simulated each. And those are used to calculate (1-X)*(1-Y).

- The packet which is correctly received within D ms is regarded as successfully received packet. (NOTE: re-transmission is allowed within the latency bound.)

  - The requirement for the reliability and the latency depends on the targeting use cases.

  - The reliability and latency requirement is defined based on single communication pair

In this contribution, the remaining issues on evaluation assumptions for NR eV2X are discussed.

- Remaining issues on the evaluation assumptions for NR eV2X

- X value for communication availability

In the agreed CAR performance metric in the email discussion [85-15], X is defined by the time portion during which the communication is interrupted by handover failure or link failure. As the detailed definition, only the interruptions that cause problem in doing reliable communication can be counted in order to exclude those that last for a very short time which has negligible impact in achieving the latency requirement with the required reliability. Further details can be dependent of the requirement of each use case. As the main cause of a non-zero X value is the interruptions during the mobility management which has been treated in RAN2, detailed analysis/evaluation can be done in RAN2 with potential relation to the solutions to minimize such interruptions. On the other hand, X can be predicted without evaluation in some cases as captured in the email discussion outcome. At least for the sidelink operation based on the preconfigured resource pool with GNSS-based synchronization, which is one operation scenario to be supported by LTE V2X, UE mobility does not cause interruption in communication. X=0 can be assumed in such a case.

- Alternatives to calculate Y value for communication resilience

There are two alternatives to evaluate PRR in the agreed metric. One is to average all the links within a specific range from transmitters, and the other is to keep measurement for a single link in the evaluation. In terms of use case, it is natural to use Alt.1 for broadcast to an open group of destinations (i.e., the transmitter is unaware of the set of receivers) and Alt. 2 for unicast/groupcast to a closed group of destinations (i.e., the transmitter is aware of the set of receivers).

The alternative 1 is clear, which has already used in LTE V2X simulation. For the alternative 2, we can simply describe the method as that each link is measured with N time simulations, after dropping many of links to show the effect of interference. However, in order to obtain 10^-5 error rate, we need at least 10^7 times for the valuable result. It would be a big burden to obtain various evaluations.

- In-band emission

Studies on sidelink so far have revealed that in-band emission is one of the main cause of determining the system level performance in the sidelink operations. In LTE, SC-FDMA with 15 kHz subcarrier spacing was used for sidelink transmissions and, the corresponding in-band emission is described in TR36.843 by modifying the UE minimum requirement. In-band emission model can be dependent of the signal characteristics such as the waveform and subcarrier spacing, so it seems difficult to establish a proper in-band emission model before such things become clearer. As a general consideration point, NR sidelink design for eV2X use case should render sufficiently low in-band emission in the non-allocated frequency resources so that FDM of multiple UE transmissions can be facilitated. So, RAN1 can study what level of in-band emission is needed for the NR sidelink operations achieving the eV2X requirement. In such study, the in-band emission model of LTE sidelink can be the starting point and the need, feasibility, and solution for further reduced in-band emission can be discussed. We note that link level study can be conducted for NR sidelink for eV2X independently of the in-band emission model.

Proposal 1: RAN1 needs to study the impact of in-band emission and proper modeling for it for NR sidelink operations.

- Traffic model and UE distribution

The traffic model and UE distribution agreed in the previous meeting follows TR38.913 where some values have brackets. After RAN#72 plenary meeting, the values are a little modified. We can follow the traffic model and UE distribution determined in TR38.913.

Proposal 2: Traffic model and UE distribution follows the values determined in TR38.913.

- LLS

In the previous meeting, the only SLS evaluations assumptions are agreed. We need the LLS evaluation assumptions to evaluate link performance. Simply, we can follow the assumptions used in LTE V2X simulation assumptions.

Proposal 3: The LLS evaluation assumptions used in LTE V2X is reused for those of NR eV2X.

- Conclusion

In this contribution, we discussed the remaining issues on evaluation assumptions for NR eV2X. The detailed proposal is given as follows:

Proposal 1: RAN1 needs to study the impact of in-band emission and proper modeling for it for NR sidelink operations.

Proposal 2: Traffic model and UE distribution follows the values determined in TR38.913.

Proposal 3: The LLS evaluation assumptions used in LTE V2X is reused for those of NR eV2X.

______________________________________________________________________

- Appendix. Evaluation assumptions for eV2X

| Parameters | Urban grid for eV2X | Highway for eV2X |
| --- | --- | --- |
| Layout | Option 1: Macro only (with the road configuration in Figure 6.1.10-1 in TR38.913) Option 2: Macro + RSUs (with the road configuration in Figure 6.1.10-1 in TR38.913) Note: An RSU can be a BS type RSU or UE type RSU Out of coverage can be evaluated assuming eNB or RSU to be disabled. Sidelink evaluation with partly out of coverage Ues and partly in coverage Ues are FFS. | Option 1: Macro only (straigntline eNB placement with Road configuration in TR36.885) Option 2: Macro + RSUs (straightline eNB with Road configuration in TR36.885) Note: An RSU can be a BS type RSU or UE type RSU Out of coverage can be evaluated assuming eNB or RSU to be disabled. Sidelink evaluation with partly out of coverage Ues and partly in coverage Ues are FFS. |
| Inter-BS distance | Inter Macro: 500m Inter RSU: RSU is dropped at each intersection | Inter Macro: 1732m, 500m (optional) Inter RSU: Uniform allocation with 100m spacing in the middle of the highway |
| Carrier frequency | Macro to/from vehicle/pedestrian UE : 4 GHz Between vehicle/pedestrian UE: 6 GHz BS-type-RSU to/from vehicle/pedestrian UE : 4 GHz UE-type-RSU to/from vehicle/pedestrian UE: 6 GHz Note: Agreed value does not mean non-ITS band is precluded for real deployment for sidelink | Macro to/from vehicle/pedestrian UE : 2 GHz or 4GHz Between vehicle/pedestrian UE: 6 GHz BS-type-RSU to/from vehicle/pedestrian UE : 4 GHz UE-type-RSU to/from vehicle/pedestrian UE: 6 GHz Note: Agreed value does not mean non-ITS band is precluded for real deployment for sidelink |
| Aggregated system bandwidth | Up to 200 MHz (DL+UL) Up to 100 MHz (SL) | Up to 200 MHz (DL+UL) Up to 100 MHz (SL) |
| Simulation bandwidth | 20 or 40 MHz (DL+UL) 10 or 20 MHz (SL) | 20 or 40 MHz (DL+UL) 10 or 20 MHz (SL) |
| Channel model | Macro to/from vehicle/pedestrian UE : 3D UMa Between vehicle/pedestrian UE: V2X Channel model in TR36.885 RSU to/from vehicle/pedestrian UE : V2X Channel model in TR36.885 FFS whether V2V channel model enhancements are needed | Macro to/from vehicle/pedestrian UE: 3D UMa for 500m ISD 3D RMa for 1732m ISD (2D RMa may be used until 3D RMa is complete) Between vehicle/pedestrian UE: V2X Channel model in TR36.885 RSU to/from vehicle/pedestrian UE : V2X Channel model in TR36.885 FFS whether V2V channel model enhancements are needed |
| Tx power | Macro BS: 49dBm PA scaled with simulation BW when system BW is higher than simulation BW. Otherwise, 49dBm [TBD for TDD] BS-type-RSU: 24dBm PA scaled with simulation BW when system BW is higher than simulation BW. Otherwise, 24dBm Vehicle/pedestrian UE or UE type RSU: 23dBm Note: 33dBM for RSU or UE is not precluded | Macro BS: 49dBm PA scaled with simulation BW when system BW is higher than simulation BW. Otherwise, 49dBm [TBD for TDD] BS-type-RSU: 24dBm PA scaled with simulation BW when system BW is higher than simulation BW. Otherwise, 24dBm Vehicle/pedestrian UE or UE type RSU: 23dBm Note: 33dBM for RSU or UE is not precluded |
| BS antenna configuration | Macro BS: Up to 256 TX/RX antenna elements BS-type-RSU: Up to 8 TX/RX antenna elements | Macro BS: Up to 256 TX/RX antenna elements BS-type-RSU: Up to 8 TX/RX antenna elements |
| BS antenna pattern | Macro BS: Follow the modeling of TR 36.873 BS-type RSU: Follow the modeling of TR 36.873 Note: Further study if needed, e.g., vertical beamforming, vehicle-to-vehicle channel. | Macro BS: Follow the modeling of TR 36.873 BS-type RSU: Follow the modeling of TR 36.873 Note: Further study if needed, e.g., vertical beamforming effect, vehicle-to-vehicle channel. |
| BS antenna height | Macro BS: 25m BS-type-RSU: 5m | Macro BS: 35m for ISD 1732m 25m for ISD 500m BS-type-RSU: 5m |
| BS antenna element gain + connector loss | Macro BS: 8 dBi BS-type-RSU: 8dBi | Macro BS: 8 dBi BS-type-RSU: 8dBi |
| BS receiver noise figure | Macro BS:5dB BS-type-RSU: 5dB | Macro:5dB BS-type-RSU: 5dB |
| UE antenna elements | Vehicle/pedestrian UE: Up to 8 TX/RX antenna elements UE-type RSU: Up to 8 TX/RX antenna elements | Vehicle/pedestrian UE: Up to 8 TX/RX antenna elements. UE-type RSU: Up to 8 TX/RX antenna elements |
| UE antenna pattern | Vehicle/pedestrian UE: Half spherically uniform distribution with upper direction UE-type-RSU: Half spherically uniform distribution with bottom direction Note: directional antenna pattern is not precluded Note: uniform antenna models should be used for 2-D channel models | Vehicle/pedestrian UE: Half spherically uniform distribution with upper direction UE-type-RSU: Half spherically uniform distribution with bottom direction Note: directional antenna pattern is not precluded Note: uniform antenna models should be used for 2-D channel models |
| UE antenna height | Vehicle/pedestrian UE: 1.5m UE-type-RSU: 5 m | Vehicle/pedestrian UE: 1.5m UE-type-RSU: 5 m |
| UE antenna gain | Vehicle UE: 3dBi Pedestrian UE: 0dBi UE-type RSU: 3dBi | Vehicle UE: 3dBi Pedestrian UE: 0dBi UE-type RSU: 3dBi |
| UE receiver noise figure | Vehicle UE: 9dB UE-type RSU: 9dB | Vehicle UE: 9dB UE-type RSU: 9dB |
| Traffic model | 50 messages per 1 second with 60km/h, 10 messages per 1 second with 15km/h in TR38.913 Note: This value is tentative. After SA1 input, it can be modified. | 50 messages per 1 second with absolute average speed of either - 100-250 km/h (relative speed: 200 solute ave in TR38.913 , or - 30km/h in TR38.913 Note: This value is tentative. After SA1 input, it can be modified. |
| Traffic load (Resource utilization) | FFS | FFS |
| UE distribution | Urban grid model (car lanes and pedestrian/bicycle sidewalks are placed around a road block. 2 lanes in each direction, 4 lanes in total, 1 sidewalk, one block size: 433m x 250m) in TR38.913 Average inter-vehicle distance (between two vehicles’ center) in the same lane is 1sec * average vehicle speed (average speed 15 – 120km/h) in TR38.913 Vehicle UE location update in TR36.885 should be used for the evaluation of PRR in sidelink or communication interruption in uplink/downlink. Vehicle UE location update may not be assumed for the evaluation of PRR in uplink/downlink Note: Inter-vehicle distance is tentative. After SA1 input, it can be modified. Pedestrian UE distribution: Inter-pedestrian distance 20m, which is tentative. After SA1 input, it can be modified. | Average inter-vehicle distance (between two vehicles’ center) in the same lane is 0.5 sec or 1sec * average vehicle speed (average speed: 100-300 km/h) in TR38.913 Vehicle UE location update in TR36.885 should be used for the evaluation of PRR in sidelink or communication interruption in uplink/downlink. Vehicle UE location update may not be assumed for the evaluation of PRR in uplink/downlink Note: Inter-vehicle distance is tentative. * After SA1 input, only one value will be selected. |
| UE receiver | MMSE-IRC as the baseline receiver Note: Advanced receiver is not precluded. | MMSE-IRC as the baseline receiver Note: Advanced receiver is not precluded. |
| Feedback assumption | FFS | FFS |
| Channel estimation | Realistic | Realistic |
| Performance metric | Communication availability and resilience | Communication availability and resilience |