3GPP TSG-RAN WG1#86	R1-166926

Gothenburg, Sweden, 22-26 August 2016

Source: 	Ericsson

Title:	Further Discussion on TBCC Complexity and Enhancements

Agenda Item:	8.1.4.1

Document for: 	Discussion and Decision

# Introduction

In this document, we discuss TBCC enhancement candidates for NR. Three enhancements to LTE TBCC are investigated. The performance of the candidates are shown in comparison to the LTE TBCC.

# TBCC Enhancement Candidates

- Enhancement 1

The LTE TBCC can be enhanced by extending the mother code rate from 1/3 to lower code rates. One candidate is to extend the LTE constraint length 7 TBCC with the list of polynomials shown in Table 1.

- Enhancement 2

The LTE TBCC can be enhancement by adopting convlutional code of larger constraint length, while keeping the mother code rate of rate 1/3. One candidate is the constraint length 9 TBCC with the first 3 polynomials [561, 753, 712] (octal) in Table 2.

Note that in UMTS, constraint length-9 tailed convolutional codes with both rate ½ and rate 1/3 are used. Our analysis has shown that the proposed rate 1/3 constraint length 9 code given by [561, 753, 715] is better than the UMTS rate 1/3 constraint length 9 code given by [557, 663, 711]. See Annex.

- Enhancement 3

The LTE TBCC can be enhancement by adopting convlutional code of larger constraint length, as well as lowering the mother code rate beyond rate 1/3. This is essentially the combination of Enhancement 1 and Enhancement 2.  One candidate is the constraint length 9 TBCC with the list of polynomials shown in Table 2.

Table 1 Nested polynomials for TBCC with constraint length 𝛎+1=7

| n | Polynomial | df | Adf | Cdf |
| --- | --- | --- | --- | --- |
| 1 | 133 | - | - | - |
| 2 | 171 | 10 | 11 | 36 |
| 3 | 165 | 15 | 3 | 7 |
| 4 | 117 | 20 | 2 | 3 |
| 5 | 135 | 25 | 1 | 1 |
| 6 | 157 | 30 | 1 | 2 |
| 7 | 135 | 36 | 4 | 8 |
| 8 | 123 | 40 | 1 | 1 |
| 9 | 173 | 46 | 3 | 6 |
| 10 | 135 | 51 | 2 | 4 |
| 11 | 171 | 56 | 2 | 3 |
| 12 | 135 | 61 | 1 | 1 |

Table 2 Nested polynomials for TBCC with constraint length 𝛎+1=9

| n | Polynomial | df | Adf | Cdf |
| --- | --- | --- | --- | --- |
| 1 | 561 | - | - | - |
| 2 | 753 | 12 | 11 | 33 |
| 3 | 715 | 18 | 4 | 11 |
| 4 | 517 | 24 | 2 | 3 |
| 5 | 675 | 30 | 1 | 2 |
| 6 | 513 | 36 | 1 | 1 |
| 7 | 657 | 43 | 2 | 4 |
| 8 | 745 | 49 | 1 | 1 |
| 9 | 753 | 56 | 4 | 8 |
| 10 | 473 | 62 | 3 | 6 |
| 11 | 745 | 68 | 2 | 3 |
| 12 | 753 | 74 | 1 | 2 |

# Performance of the TBCC Enhancement Candidates

The performance of the Enhancement 1, 2, and 3 is simulated and compared to that of LTE TBCC. The comparison is shown for information block lengths K= {40, 60, 80, 120} bits in Figure 1 – Figure 4.

In Figure 5, Es/N0 (dB) improvement of Enhancements 1, 2, 3, as compared to LTE TBCC for code rate ¼ to 1/9. The Es/N0 (dB) is measured at FER = 10-3.

The study shows that for transmission rate ¼ to 1/9:

- Enhancement 1 provides 0.1 – 0.2 dB gain compared to LTE TBCC.

- Enhancement 2 provides 0.5 – 0.7 dB gain compared to LTE TBCC.

- Enhancement 3 provides 0.7– 0.9 dB gain compared to LTE TBCC.

Figure 1. FER performance of Enhancements 1, 2, 3, as compared to LTE TBCC. Info block size K=40 bits.

Figure 2. FER performance of Enhancements 1, 2, 3, as compared to LTE TBCC. Info block size K=60 bits.

Figure 3. FER performance of Enhancements 1, 2, 3, as compared to LTE TBCC. Info block size K=80 bits.

Figure 4. FER performance of Enhancements 1, 2, 3, as compared to LTE TBCC. Info block size K=120 bits.

(a)				            (b)

(c)				            (d)

(e)				            (f)

Figure 5. Es/N0 (dB) improvement of Enhancements 1, 2, 3, as compared to LTE TBCC for code rate ¼ to 1/9. The Es/N0 (dB) is measured at FER = 10-3.

# Viterbi Decoding Complexity of TBCC

For decoder complexity of TBCC, regular Viterbi decoder is the most widely used method for decoding convolutional code. Mature and efficient decoder implementation exists for regular Viterbi decoder. For one information bit in one iteration, the Viterbi decoder complexity is:

- Branch metric calculation: 2n additions, where n is the number of output bits for a input bit, i.e., code rate R = 1/n;

- 2v  2 additions for state metric branches; for a code with 2v states, and 2 branches per state;

- State metric update (i.e., selection step): one comparison/selection per state, for a total of 2v comparison/selection (equivalent of addition);

Overall, the decoding complexity is (2n + 32v) per information bit.

For Enhancement 1, 2, and 3, the Viterbi decoding complexity is compared. The LTE TBCC has 200 additions (or its equivalents) in comparison.

Between Enhancement 2 and 3, Enhancement 3 can achieve roughly 0.2 dB better FER performance, but the decoding complexity grows as the code rate decreases. Since going to a longer constraint length provides most of the performance gain, both Enhancement 2 and Enhancement 3 are acceptable for NR.

Table 3 Viterbi decoding complexity (unit=additions) of TBCC for one information bit

| Transmitted code rate | Enhancement 1 | Enhancement 2 | Enhancement 3 |
| --- | --- | --- | --- |
| 1/3 | 200 | 776 | 776 |
| 1/4 | 208 | 776 | 784 |
| 1/5 | 224 | 776 | 800 |
| 1/6 | 256 | 776 | 832 |
| 1/7 | 320 | 776 | 896 |
| 1/8 | 448 | 776 | 1024 |
| 1/9 | 704 | 776 | 1280 |

For control channel and data channel with short info block length, the LTE tail-biting convolutional code is enhanced by adopting constraint length-9 polynomials.

# Conclusion

Based on extensive analysis presented in this document, we make the following proposals.

- For control channel and data channel with short info block length, the LTE tail-biting convolutional code is enhanced by adopting constraint length-9 polynomials.

# References

R1-164356, “Performance Evaluation of TBCC and Polar Codes,” Ericsson.

R1-164357, “Analysis of Candidate Code Types for Short Block Length,” Ericsson.

Annex

The nested rate 1/3 constraint length 9 code given by [561, 753, 715] is better than the UMTS rate 1/3 constraint length 9 code given by [557, 663, 711]. The distance spectrum comparison is provided below.

|  | [561, 753, 715] |  | [557, 663, 711] |  |
| --- | --- | --- | --- | --- |
| d | Ad | Cd | Ad | Cd |
| 18 | 4 | 11 | 5 | 11 |
| 19 | 0 | 0 | 0 | 0 |
| 20 | 13 | 49 | 7 | 32 |
| 21 | 0 | 0 | 0 | 0 |
| 22 | 28 | 136 | 36 | 195 |
| 23 | 0 | 0 | 0 | 0 |
| 24 | 81 | 496 | 85 | 564 |
| 25 | 0 | 0 | 0 | 0 |
| 26 | 235 | 1652 | 204 | 1473 |
| 27 | 0 | 0 | 0 | 0 |
| 28 | 646 | 5122 | 636 | 5129 |
| 29 | 0 | 0 | 0 | 0 |
| 30 | 1889 | 16388 | 1927 | 17434 |
| 31 | 0 | 0 | 0 | 0 |
| 32 | 5608 | 53870 | 5416 | 54092 |
| 33 | 0 | 0 | 0 | 0 |
| 34 | 16007 | 167364 | 15769 | 171117 |
| 35 | 0 | 0 | 0 | 0 |
| 36 | 46407 | 529226 | 45763 | 539486 |
| 37 | 0 | 0 | 0 | 0 |
| 38 | 133484 | 1642089 | 131319 | 1667179 |
| 39 | 0 | 0 | 0 | 0 |
| 40 | 386574 | 5104883 | 380947 | 5187615 |
| 41 | 0 | 0 | 0 | 0 |
| 42 | 1113762 | 15714070 | 1100932 | 16003037 |
| 43 | 0 | 0 | 0 | 0 |
| 44 | 3220414 | 48331307 | 3173395 | 49013235 |
| 45 | 0 | 0 | 0 | 0 |
| 46 | 9297602 | 147890952 | 9186269 | 150271658 |
| 47 | 0 | 0 | 0 | 0 |