3GPP TSG RAN WG1 Meeting #86	R1-166935

Gothenburg, Sweden, 22-26 August 2016

Source:	Ericsson

Title:	Channel Coding Techniques for Control Information in NR

Agenda Item:	8.1.4.1

Document for:	Discussion and Decision

# Introduction

In RAN#85, it was agreed that NR design should strive to enable the possibility for corresponding UL data transmission shortly after reception of UL assignment as well as the possibility for corresponding retransmission shortly after the end of acknowledgement reporting.

Although the extent to which these goals are achievable depends, for example, on the UE capability/category, payload size, etc., it inevitably imposes a stringent requirement on the latency in decoding control information, which in turn imposes requirements on decoder hardware implementation.  Therefore, the choice of a channel coding scheme for control information must enable these requirements to be fulfilled.

Furthermore, NR control channels is expected to require a fine granularity (down to one bit) in block length as in LTE.  The channel coding scheme chosen for control information must be flexible enough to support such granularity.

In this paper, we examine a proposed coding scheme for control channel, namely Polar code, and compare it with TBCC, the existing coding scheme used in LTE, in terms of implementation, latency, and flexibility.

# Implementation

## TBCC

TBCC has already been used in LTE for error correction of DCI and UCI. There is no implementation effort if LTE TBCC is reused. If enhancement is applied to LTE TBCC, only modest effort is needed to implement the enhancement, e.g., a lower code rate than the rate 1/3 in LTE.

## Polar code

Although there is a number of decoder implementation proposals for Polar codes that promise high throughput and low latency, most of them implement successive cancellation (SC) decoder which performs poorly in practice.  Nearly all published list SC decoding implementations for polar codes have a list size (L) that is no larger than 16. To achieve good error-correcting performance, list SC decoding with a relatively large list size (L≥32) is needed. The implementation of list SC decoding poses significant challenges.  In particular, list management operations, such as memory copying and sorting, required to maintain an updated list of candidate paths in list SC decoding can substantially increase the implementation complexity as compared to SC decoding.  It remains uncertain whether list SC decoder with list size of at least 32 can be implemented efficiently.

# Latency

With regard to DCI decoding, Assume the UE is given ~50 µs to receive the downlink control channel.  Taking into account other processing time like channel estimation and demodulation, maybe there is ½*50 = 25 µs for left for decoding. Then assume ~40 blind decodes as in LTE, there is 25/40 ~= 0.6 µs to decode one downlink control channel candidate.

## TBCC

Viterbi decoding of TBCC can be implemented with high throughput. For example, [2] describes a convolutional decoder with throughput of 500 Mbps for 64-state NSC. Even when taking into account the processing due to the tail-biting feature of the convolutional code in LTE, the throughput can still be relatively high, in the range of 300 Mbps. Assuming the UCI and DCI with size range of K=20 ~ 80 bits, the decoding latency of one DCI candidate is

20/300 ~ 80/300 = 0.07 ~ 0.27 s

Even when assuming sequential processing of blind decoding candidates, the TBCC Viterbi decoder satisfies the low decoding latency requirement of NR.

In general, under the low delay constraints, for very short information block lengths, convolutional code has been shown to outperform block codes, including binary and non-binary LDPC codes, and turbo codes (see [3]). From a break-even point onward, convolutional codes cannot compete with block codes anymore and nonbinary LDPC codes show the best performance. The break-even point for reaching FER = 10-3 is approximately the following for the convolutional code and GF(256) LDPC codes simulated in [3]:

- K~=70 bits for R=1/4;

- K~=80 bits for R=1/3;

- K~=110 bits for R=1/2;

- K~=150 bits for R=2/3;

- K~=250 bits for R=3/4;

Since control channel (DCI, UCI) info block size is low (e.g., in the range of 20 bits to 80 bits), TBCC is a good code choice for the control channel of NR.

## Polar code

As the complexity and delay of list management operations rapidly increase with list size, the overall latency of list SC decoding grows large and limits the applicability of polar codes in high-throughput and latency sensitive applications [5][6].  List management operations, such as memory copying and sorting, required to maintain an updated list of candidate paths in list SC decoding can substantially increase the processing delay, as compared to SC decoding.  Each memory access, whether read or write, consumes one clock cycle.  Depend on the configuration of data lines, data fletching may need to be serialized, which can drastically increase the total number of clock cycles.

The fastest list SC decoder with a list size of 16 has a coded throughput of only about 200Mbps [5]. This translates into a substantially larger latency than that achievable with the existing TBCC code in LTE.  However, as mentioned above, to achieve a good error performance, typically a list of 32 is needed.  As the complexity of list management and associated logic delay increase super-linearly with list size, one can expect that the latency of list size of 32 can be substantially larger.

Although various methods have been proposed, cf. [5][6], to reduce latency of Polar codes, they typical achieve the latency reduction by modifying the list decoding algorithm to the effect of reducing the effective list size on average, at the expense of compromising performance.

Performance of polar code can be improved by using an outer CRC code.  The use of such additional outer (CRC) code, if needed, demands additional interface between hardware components and further increases latency as the CRC checking cannot be parallelized with other decoding operations.

# Flexibility

Polar code performance depends heavily on the choice of the information set (or equivalently, the set of frozen bit locations) [4]. The use of a different information set results in a different effective channel code. Ideally, the information set should be chosen to match the underlying effective binary-input memoryless channel in order to avoid performance degradation.

The most commonly used polar codes are binary polar codes.  The block lengths of binary polar codes must be powers of two.  Hence, the difference between two adjacent block lengths of polar codes grows exponentially.  To achieve fine granularity of block lengths to support NR control channels, a large number of bits may need to be punctured, and a large number of different puncturing patterns must be made available since performance degradation can result if the information set does not match well with the puncturing pattern.

Not only do these information sets need to be stored or computed in real time, but the hardware also need to have the flexibility to support the change of information set in real time.  The need for such flexibility precludes many methods proposed in literature for boosting the throughput and reducing the latency of polar codes that exploit the specific frozen bit locations of a particular information set.

In addition, as mentioned above, an extremely large number of combinations of K and N needs to be supported for NR control channels.  This further magnifies the number of different information sets needed to be either stored or computed in real time.  This imposes burden on hardware to provide adequate storage and/or accuracy of computation.

# Conclusion

In this contribution, we examine Polar code as a candidate coding scheme for control channel and compare it with the existing coding scheme used in LTE, namely TBCC, in terms of implementation, latency, and flexibility.   Based on the discussion, together with the observation that TBCC and Polar code achieve similar performance [7][8], we have the following proposal:

- Control channel information of NR adopts TBCC as the channel coding method.

# References

RP-160671, “New SID Proposal: Study on New Radio Access Technology,” NTT DOCOMO, 3GPP TSG RAN Meeting #71, Göteborg, Sweden, 7.-10. March, 2016.

F. Kienle, et al,, “On Complexity, Energy- and Implementation-Efficiency of Channel Decoders,” IEEE TRANSACTIONS ON COMMUNICATIONS, VOL. 59, NO. 12, DECEMBER 2011, pp. 3301-3310.

C. Rachinger, J. B. Huber, R. R. Müller, “Comparison of convolutional and block codes for low structural delay”, IEEE Trans. Commun., vol. 63, no. 12, pp. 4629-4638, Dec. 2015

E. Arikan, “Channl polarization: A method for constructing capacity achieving codes for symmetric binary-input memoryless channels,” IEEE Trans. Info. Thy., vol. 55, no. 7, pp. 3051-3073, May 2009.

Y.Z. Fan, C.Y. Xia, J. Chen, C.Y. Tsui, J. Jin, H. Shen, and B. Li, “A low-latency list successive cancellation decoding implementation for polar codes,”  IEEE JSAC, vol. 34, No 2, Feb 2016.

G. Sakis, A. Vardy, C. Thibeault, W.J. Gross, “Fast list decoders for polar codes,” arXiv, Nov 2015.

R1-166926, “Further Discussion on Performance and Complexity of Enhanced TBCC”

R1-166927, “Evaluation of Enhanced TBCC and Polar Codes”