3GPP TSG RAN WG1 Meeting #86                                                                                 R1-167034

Göteborg, Sweden, 22th – 26th August, 2016.

Source: 	Ericsson

Title:	On PA impact in mixed numerology with narrow band allocation

Agenda Item:	8.1.2.1

Document for:	Discussion

# Introduction

To support simultaneous services with vastly different requirements, e.g. ultra-low latency communications (short symbols and thus wide subcarrier spacing) and MBMS services (long symbols to enable long cyclic prefix and thus narrow subcarrier spacing), an OFDM based system with different numerologies multiplexed in frequency-domain on the same carrier was discussed and proposed as working assumption from the previous 3GPP TSG RAN WG1 meetings for NR.

In [1], some features of mixed numerology were described and some simulation results were presented.

In this contribution, we evaluate mixed numerologies with a narrow band victim besides a wide band aggressor.

# Discussion

## Evaluation assumptions

The PA model used in the downlink evaluation is the Rapp PA model with 46 dBm total output power and 57.6 dBm saturation power as described in [2].

As we observed in [3] that with ideal channel estimation and perfect PA phase compensation, there is almost no difference in performances with and without PA. Thus we evaluate the PA impact with estimated channel in which the phase error caused by the PA is also included. The channel estimation is based on the LTE DL DMRS allocation and please see [7] for a dedicated discussion on DMRS allocation for NR.

Table 1 summarizes simulation assumptions.

Table 1	Link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Numerologies | 15 kHz (victim) and 30 kHz (aggressor) |
| Aggressor allocation | Wideband to fill up the system BW minus the guard band minus the victim allocation - 30kHz subcarrier spacing |
| Victim allocation | narrowband 6 PRB – 72 15kHz subcarriers |
| PSD allocation between numerologies | Equal |
| Spectral confinement method | TX and RX windowing [1] with linear window slope of length 24 (15 kHz) and 12 (30 kHz) samples. |
| Guard band | 10 narrowband subcarriers [4] |
| FFT size | 15 kHz: 4096 30 kHz; 2048 |
| Cyclic prefix length | 15 kHz: 1320 + 6288 samples 30 kHz: 1160 + 6144 samples |
| Resource block size | 12 subcarriers |
| TTI length | 15 kHz: 1 ms 30 kHz: 1 ms |
| Channel estimation | Frequency and time linear interpolation |
| Link adaption | Based on ideal ACK/NACK |
| Modulations scheme | QPSK, 16QAM, 64QAM |
| Channel codec | LTE Turbo code LogMax scaling factor = 0.75 Number of iterations = 8 |
| Reference signal overhead | LTE-DL DMRS |
| Control channel overhead | not included |
| Channel model | TDL-C with 10n and 300ns RMS delay spread [5] |
| PA model | Rapp PA model with 46 dBm total output power and 57.6 dBm saturation power [2] |

## Performance results

Figure 1 and Figure 2 below showed the throughput performances with link adaption without PA and with downlink PA [2], for TDL-C RMS delay spread of 10 ns and 300 ns respectively.

Since we observed in [3] and [6] for the case 2 and case 1a that with ideal channel and ideal PA phase error compensation, the two throughput curves with PA and without PA are on each other. With estimated channel including phase error caused by the PA impairments, we can see some performance degradation due to the PA in higher SNR region. The performance loss is similar for single numerology and mixed numerologies.

Figure 1	6 PRB victim allocation; throughput with link adaptation and max 64QAM, TDL-C 10ns

Figure 2	6 PRB victim allocation; throughput with link adaptation and max 64QAM, TDL-C 300ns

# Observations and discussion

The Rapp PA model with 46 dBm total output power and 57.6 dBm saturation power as described in [2] with parameter setup according to [3] showed little performance impact to the throughput and BLER performances of the cases evaluated compared to an ideal linear PA (i.e. without PA model), with ideal channel and ideal phase compensations. But with estimated channel including phase error caused by the PA impairments, we can see some performance degradation due to the PA in higher SNR region. The performance loss is similar for single numerology and mixed numerologies.

# References

R1-165833, “Mixed Numerology in an OFDM System”

RAN1 email discussion [85-18] PA models for NR

R1-167030, “Mixed Numerology with PA impairment”

R1-167033, “On resource allocation and guard subcarriers for mixed numerologies”

R1-165989, “Way forward on calibration assumptions for NR waveform”

RAN1 email discussion [85-16] Calibration for NR waveform and Excel document NRWaveformCalibration_v32

R1-167037, “Demodulation reference signal design principles”