TSG-RAN WG1 #86	R1-167046

Gothenburg, Sweden, August 22 – 26, 2016

Source:	Ericsson

Title:	Summary of NR subframe design

Agenda Item:	8.1.3.2

Document for:	Discussion and Decision

# Introduction

This contribution summarizes Ericsson’s view on NR subframe design. Additional details and motivation can be found in the companion papers listed in the reference section.

# Discussion

The following is proposed

- The use of a common subframe length across numerologies of 0.5 ms (for subcarrier spacings of 15 kHz and higher), suitable as the transmission unit for MBB services [4][8].

  - Latency-critical transmissions use a shorter transmission duration, allowed to start at ‘any’ OFDM symbol and not only at subframe boundaries [4].

- OFDM symbol boundaries are aligned across different numerologies [9] to simplify time-multiplexing of different numerologies

- Support for dynamic TDD where the parts of a subframe carrying data can be dynamically assigned to uplink or downlink as part of the dynamic scheduling decision [4].

- “Front-loaded design” with downlink control and demodulation reference signals occurring at the beginning of the transmission [5]

- “On the fly” processing supported with a pipelining-friendly mapping (e.g. avoiding time-domain interleaving) [5]

- The possibility for [5]

  - immediate hybrid-ARQ feedback,  ~1 OFDM symbol after the end of data reception.

  - immediate uplink scheduling grants, uplink data transmission starting  ~1 OFDM symbol after reception of the scheduling grant

- Possibility for “multi-subframe” hybrid-ARQ feedback in addition to “immediate” feedback [6]

- Uplink and downlink subframes are described separately to allow for simple handling of timing relations and applicability the same subframe design to all duplex schemes [1].

- Generic frame structure applicable to TDD as well as FDD [2], and uplink/downlink as well as sidelink [3].

# References

- R1-167048, “On subframe description”, Ericsson

- R1-167042, “On FDD frame structure”, Ericsson

- R1-167043, “On sidelink frame structure”, Ericsson

- R1-167044, “On dynamic TDD”, Ericsson

- R1-167047, “On smallest schedulable unit”, Ericsson

- R1-167045, “On low-latency frame structure”, Ericsson

- R1-167039, “On different HARQ feedback reporting modes”, Ericsson

- R1-167049, “On subframe scaling”, Ericsson

- R1-167050, “On symbol scaling”, Ericsson