TSG-RAN WG1 #86	R1-167051

Gothenburg, Sweden, August 22 – 26, 2016

Source:	Ericsson

Title:	On time interval X

Agenda Item:	8.1.3.2

Document for:	Discussion and Decision

# Introduction

At RAN1#85 the following was agreed:

- At least the following should be supported for NR in a frequency portion

  - A time interval X which can contain one or more of the following

    - DL transmission part

    - Guard

    - UL transmission part

  - FFS which combinations are supported and whether they are indicated dynamically and/or semi-statically

  - Furthermore, the following is supported

    - The DL transmission part of time interval X to contain downlink control information and/or downlink data transmissions and/or reference signals

    - The UL transmission part of time interval X to contain uplink control information and/or uplink data transmissions and/or reference signals

  - FFS length(s) of time interval X

  - FFS: other characteristics of time interval X

  - Note: The usage of DL and UL does not preclude other deployment scenarios e.g., sidelink, backhaul, relay

In the following some further properties of time interval X are proposed in order to progress the frame structure discussion.

# Discussion

From the minutes, it is unclear where ‘time interval X’ is defined – at the eNodeB (i.e. with no timing advance) or at the UE (where a timing advance is applied). It also appears to implicitly assume half-duplex transmission in unpaired spectrum. To avoid these confusions, it is suggested to adopt separate structures for the uplink and downlink transmissions as outlined in [1]. If, despite this, RAN1 insists on continuing using ‘time interval X’, the following is proposed:

- Time interval X contains at most one DL part and one UL part

- Subsequent time interval Xs are non-overlapping

- The downlink control information in a time interval X may relate to the downlink data transmission in the same time interval X

- The downlink control information in a time interval X may relate to the uplink data transmission in the same time interval X

- The uplink control information in a time interval X may relate to the downlink data transmission in the same time interval X

# Conclusion

It is proposed to adopt the description approach in [1] or, as a second preference, adopt the bullet list in the previous section.

# References

- R1-167048, “On subframe description”, Ericsson