3GPP TSG-RAN WG1 #86 	R1-167119

22th – 26th August 2016

Gothenburg, Sweden

Source: 	Qualcomm

Title:	Forward compatibility considerations on NR Integrated Access and Backhaul

Agenda Item:	8.1.1

Document for:	Discussion and Decision

# Introduction

In RAN meetings #84 and #85, multiple proposals were made for wireless backhauling, referred to here as “self-backhauling” (SBH) [1][2]. These proposals stressed the importance of self-backhauling for NR deployments as well as the need for its timely availability together with NR access. The proposals for self-backhauling was focused on high frequencies such as mmWaves. Multiple proposals emphasized the need for multi-hop relaying to guarantee sufficient range extension, redundant connectivity to ensure sufficient reliability and integrated access/backhauling features to ensure resource-efficient operation. Also, relay mobility was mentioned as a desirable feature.

RAN meeting #85 achieved agreements related to the support of access and backhauling by NR [2].

In this paper, we emphasize on the timely availability of multi-hop capabilities, redundant connectivity and resource-efficient operation to ensure a viable deployment path for NR access. For these capabilities, we propose high-level requirements to ensure a future-proof NR design.

We discuss on how to evolve the Rel-10 RN concept toward extended networking topologies that meet the requirements by leveraging asymmetric access links as the atomic building block. In this manner, the desired self-backhauling capabilities can be supported in timely manner. We also discuss how spectral efficiency should be through integration of access and backhauling, and via dynamic load balancing features.

# Requirements

In agreement with proposals of [1] and [2], we would like to stress the need for timely support of self-backhauling capabilities with the first release of NR access. In this context, we interpret self-backhauling as to allow sufficient range enhancement and reliability, which we formulate via the following two requirements:

Requirement on range extension: NR design should allow for multiple-hop self-backhauling without dictating hard limits on the hop count.

Requirement on reliability: NR design should allow for topological redundant self-backhauling without confining the degree of inter-node connectivity.

It has to be recognized that especially for high-frequencies such as mmWaves, no experience on range or link reliability presently exists from actual deployments. By not setting specific hard bounds to hop-count and inter-node connectivity, these requirements ensure that NR design is future proof. These requirements do not preclude limits on hop count or inter-node connectivity to be enforced via configuration, e.g. in a deployment-specific manner.

We further like to propose the following requirement on resource efficiency:

Requirement on high resource efficiency: NR design should allow for dynamic load balancing between backhaul links as well as between backhaul- and access links.

It would be desirable to capture more refined bounds on the resource efficiency or the time scale of resource balancing in the foreseeable future.

Many RAN proposals on self-backhauling highlighted the need for relay mobility [1][2].  We believe that NR should allow for the accommodation of changes to the self-backhaul network topology, which includes addition and release of individual links as well as integration and departure of self-backhaul nodes. These capabilities are relevant for stationary relay deployments as they allow for variations in channel conditions and enable network-node maintenance and network growth. Local topology changes should be possible while other parts of the network carry life traffic.

Requirement on topology changes: NR design should allow for accommodation of local changes in network topology while other parts of the self-backhaul network carry traffic.

It would be desirable to set more refined requirements on the time scales of the various response mechanisms for stationary relay nodes in foreseeable future.

For mobile relay nodes, the response mechanism to accommodate topological changes are generally more stringent. In this context, the RAN WG1 may carefully evaluate if relay node mobility is an imperative requirement for the initial release of NR.

# Steady-state Operation

We initially focus on the steady-state operation scenario, where all nodes and links (related to network node, relay node and UE) are in place and mutually exchange traffic. The establishment of the steady-state, i.e. network setup, link discovery and connection establishment, are discussed further below.

For steady-state operation, we would like to motivate an approach that evolves the Rel-10 RN concept toward multi-hop, redundant self-backhauling in compliance with the above requirements. This approach leverages the access link as the fundamental atomic building block for a self-backhaul network with IAB features, thereby minimizing the implications of backhauling support in the NR design. The approach includes the asymmetric nature of the access link, which defines a one-to-many relation between Access Node (AN) and UEs via a DL and an UL. An analogue approach has been applied in the development of the Rel-10 Relay Node (RN) concept [3].

Figure 1: Rel-10 RN concept

Figure 1 shows a topology that can be supported by the Rel-10 Relay Node concept (in this discussion we focus on the Type-1 RN [3]). Note that the Rel-10 RN concept only supports single-hop backhauling and it does not support topological redundancy, i.e. a RN cannot have simultaneous links with multiple Donor eNBs (DeNBs). Further, resource multiplexing is applied between access and backhaul link of the RN to avoid interlink interference and to have both types of links comply with the half-duplexing constraint (e.g. RN cannot transmit and receive at the same time). This resource multiplexing follows a semi-static schedule (shown color-coded on links in Figure 1). The semi-static nature of the schedule limits its ability to dynamically respond to load variations between access and backhaul links. Consequently, the first three of the above requirements are therefore not met by the Rel-10 RN concept.

The Rel-10 RN concept adopts for the backhaul link the main framework defined for the access link such as numerology, frame structure, asymmetric link definitions (DL vs. UL), one-to-many relation between DeNB and RNs, and centralized resource management assigned to the DeNB. In this light, the RN can be regarded as a collocation of a UE function for the backhaul link and an eNB function for the access links.

Figure. 1 shows an example of the frame structure for the Rel-10 RN concept using in-band backhauling. The time-division multiplexing scheme via semi-static schedule is shown color-coded for access links (pink) and backhaul links (green). A few changes had to be made to the design for backhaul-channel over that defined for access channels to guarantee backward compatibility with Rel-8 UEs.

The adoption of access functionality for backhauling in Rel-10 resulted in reduced standardization effort and it enabled IAB functionality at the same time. These goals are the same as presently pursued in NR for the accommodation of self-backhauling. For this reason, it makes sense to adopt the Rel-10 RN concept for NR.

Figure 2: Example for adoption of Rel-10 RN concept to NR

Figure 2 shows an example of such a NR-based RN concept, where the 5G-RN consists of a UE-Function (UEF) and an AN-Function (ANF), while the 5G Donor hosts an ANF. Since NR is not constraint by backward compatibility issues, as it was the case for the Rel-10 RN, it is possible to use the same channel structure for access and backhaul. This significantly reduced the impact of self-backhauling support on the NR design. It further allows the donor to perform dynamic load balancing between access links and backhauling links. Semi-static resource multiplexing still has to be applied between donor level and relay-node level (TDM example shown via pink and green color-coding in Figure 2). Note that the 5G-RN concept of Figure 2 still does not support multiple hops or redundant connectivity.

Figure 3: Example for evolution of NR-relaying concept to spanning tree topology

The NR-relaying concept of Figure 2 can be easily evolved to a multi-hop spanning-tree topology by allowing each 5G-RN to support lower tiers of 5G-RNs (Figure 3). For that purpose, the ANF of a 5G-RN connects to the UEFs of its lower-tier 5G-RNs. To comply with a half-duplexing constraint, resource multiplexing has to be applied to the various tiers. In case inter-link interference is small, resources can be reused on every second tree level. The corresponding semi-static resource schedule (shown as pink and green in Figure 3) becomes very simple and spectrally efficient. Inter-link interference across two levels may already be small due to spatial separation. For mmWave-based backhauling, narrow-beam formation and coordination of beams across 5G-RNs may further improve in inter-link interference suppression.

While the spanning tree topology obtained in this evolutionary step enables multi-hop backhauling, it still cannot support redundant connectivity. Further, resource scheduling between layers is rather rigid.

Figure 4: Example for evolution of NR-relaying concept to mesh topology

Figure 4 takes the evolution of NR-relaying one step further by permitting allocation of an arbitrary number of ANFs and UEFs in each 5G-RN. In this manner, topological redundancy can be supported by decomposing the connectivity graph among 5G Donors and 5G-RNs into stars, and allocating one ANF to each star center and one UEF to each of the star leaves. Access links can be integrated into this graph either by having UEs connect to an ANF that is already hosted on a 5G-RN or by allocating an additional ANF on the corresponding 5G-RN. A semi-static resource multiplexing scheme has to be enforced among stars with mutually touching links to comply with the half-duplexing constraint. Stars that are disjoint can reuse the same resource. One example is shown on hand of color-coded links in Figure 4 (green, pink and yellow correspond to orthogonal resources).

Figure 5: Example for NR-relaying concept for mesh topology with dynamic load balancing

All relaying concepts discussed above rely on a semi-static resource multiplexing schedule to comply with the half-duplexing constraint. Even if this schedule optimizes resource assignment based on some average demand its semi-static nature may prove too slow to respond to rapid load fluctuations. NR should therefore consider means to ensure dynamic load balancing within such a semi-static schedule. Figure 5 shows an example, where an idle resource on one link (yellow link to top right donor) is temporarily leased to a neighbour link.  Such a local load balancing mechanism can occur fast since it only requires local signalling.

The examples of Figures 1 to 5 illustrate how the Rel-10 RN concept can be evolved to meet the requirements for self-backhauling, in particular multi-hop range extension, reliable transport via redundant connectivity and high spectral efficiency via integration of access and backhaul and via dynamic load balancing mechanisms.

# Topological changes to Self-backhaul

NR needs to consider mechanism to account for the integration and departure of 5G-RNs as well as the integration and release of inter-5G-RN links. One issue that needs to be addressed is inter-node synchronization and random access among 5G-RNs, which should ideally leverage resources that have already been allocated for access. It is perceivable, for instance, that 5G-RNs occasionally skip access-related SYNC channel transmissions to listen to their peers. In a similar manner, RACH resources can be allocated in between 5G-RNs.

For the integration of 5G-RNs into the self-backhauling network similar procedures as established for the Rel-10 RN concept can be developed. These procedures mostly relay on upper layer signalling. Further, the semi-static resource allocation among links may be centrally managed via upper-layer signalling.

# Conclusion

The following proposals are made:

Proposal 1: NR design should allow for multiple-hop self-backhauling without dictating hard limits on the hop count.

Proposal 2: NR design should allow for topological redundant self-backhauling without confining the degree of inter-node connectivity.

Proposal 3: NR design should allow for dynamic load balancing between backhaul links as well as between backhaul- and access links.

Proposal 4: NR design should allow for accommodation of local changes in network topology while the remaining self-backhaul network carries traffic.

# References

Final Report of 3GPP TSG RAN WG1 #84bis, Busan, Korea, 11th – 15th April 2016

Final Report of 3GPP TSG RAN WG1 #85, Nanjing, China 23rd - 27th May 2016

3GPP TS 36.912 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Feasibility study for Further Advancements for E-UTRA (LTE-Advanced).