3GPP TSG RAN WG1 Meeting #86	R1-167197

Gothenburg, Sweden, August 22-26, 2016

Agenda Item:	8.1.5

Source:	Huawei, HiSilicon

Title:	CSI acquisition mechanism for NR DL MIMO

Document for:	Discussion and decision

Introduction

At the previous meeting (RAN1#85), it was agreed in [1] that

- In NR multi-antenna schemes, studies on CSI acquisition framework  include

  - CSI reporting schemes

    - Implicit CSI feedback

      - Parameters indicating channel quality based on a set of transmission and receiving hypotheses associated with one particular UE, e.g. CQI, PMI, RI, CRI

    - Explicit CSI feedback: for both quantized and unquantized/analog CSI feedback

      - Parameters representing channel coefficients or some reduced-space representation thereof

    - Reciprocity-based feedback

      - For example, take into account interference and/or receiver hypothesis can be included

    - Note: including aperiodic, periodic and semi-persistent, and single/wide band and sub-band feedback

    - Mixed feedback is not precluded

  - Interference measurement

  - FFS: CSI measurement and/or reporting and/or triggering can be ‘self-contained’ in at least time domain

In NR, it is beneficial to reduce the huge amount of transmission modes in LTE/LTE-A, and support dynamic switching among multiple MIMO transmission schemes in the same transmission mode [2][3]. In this case, a unified CSI acquisition framework should be strived as much as possible in NR, supporting various CSI measurement and feedback types for different use cases, such as multi-level CSI measurement and reporting, implicit CSI feedback, explicit CSI feedback, and reciprocity based CSI acquisition.

In this contribution, we investigate the CSI acquisition mechanism for DL MIMO in NR, including CSI reporting schemes (i.e. implicit CSI feedback, explicit CSI feedback and reciprocity based CSI acquisition), and CSI measurement/reporting procedures.

CSI Reporting Schemes

All of implicit, explicit CSI feedback and reciprocity based CSI acquisition need to be investigated in NR.

Implicit CSI Feedback

To well reflect the realistic propagation channel with multiple clusters, multiple bases CSI representation (MBCR) approach is worthy to be studied for accurate CSI quantization, based on the combination of multiple basis beams. Given a predefined base codebook B=[b1 b2⋯bN], where bn is the n-th basis beam, the precoding vector νm,k for each polarization on sub-band m of user k can be expressed as a weighted sum of multiple basis beams selected from B.

νm,k≈i∈Iselαi,m,kbi	(1)

where Isel consists of the indices of Nsel (Nsel≤N) most significant basis beams from B, and αi,m,k is the weight corresponding to beam bi on sub-band m of user k. In MBCR, the indices of the selected basis beams and the corresponding weights need to be reported per subband, which requires large quantization bits to achieve accurate CSI at TRP.

To further reduce the feedback overhead, a common subset of Nsel significant basis beams is assumed for all sub-bands. Therefore, Nsel indices for the wideband and Nsel weights for each sub-band are required to obtain subband CSI. A two-stage codebook structure can be considered based on the principle of MBCR as follows.

W=W1W2=bi1 bi2⋯biNsel00bi1 bi2⋯biNselα1,m,k⋮α2Nsel,m,k	(2)

where W1 is composed of Nsel basis beams selected from B, i.e.

W1=bi1 bi2⋯biNsel00bi1 bi2⋯biNsel	(3)

and W2 consists of 2Nsel weights for the combination of the beams in W1.

W2=α1,m,k⋮α2Nsel,m,k	(4)

- W1 design

The “best W1” can be achieved by selecting Nsel best beams from the total N beams in B. The corresponding feedback overhead is log2(CNNsel) bits. To further reduce the feedback overhead, W1 codebook design needs to be considered, each codeword corresponding to a fixed beam subset. The appropriate selection of a beam subset for each W1 codeword and the size of W1 codebook need to be further studied in NR.

- W2 design

According to equation (4), a W2 codeword consists of 2Nsel components, so the total overhead of W2 using per-element quantization is 2NsubNselNb bits, where Nb is the number of bits available to quantize a complex weight including the real and imaginary parts. Advanced quantization methods of W2 need to be studied in NR, considering the tradeoff between feedback overhead and channel quantization accuracy.

Let  denote the vector of W2 for quantization with unit norm, which can be viewed as a point on a unit sphere of dimension K=2Nsel.

(5)

A cube-split quantization is proposed for the represention of , constituted by the following three steps.

Step 1: Transform the complex vector  into a real vector  in the real unit-sphere, by concatenating the real and imaginary part of a rotation of with respect to the phase of its first coordinate.

Define a rotated equivalent vector as  where . Then  is constructed by real values as follows.

(6)

Step 2: Find the maximum element of ,  where . Normalize  by , so the elements unknown is further reduced to  real values.

(7)

Step 3: Perform a mapping on  from the unit sphere of dimension  to the unit cube of dimension  by

(8)

Then  are uniformly quantized between [0, 1].

In summary, the bit sequences of  and quantization on  () together constitute the index of a codeword in W2 codebook, i.e. W2 PMI. TRP can retrieve the W2 codeword according to equations (5), (6) and (8), based on  and  () indicated by the W2 PMI.

Figure 1 shows the benefit of the above cube-split quantization compared with per-element quantization, in terms of relative beamforming gain. Simulation assumptions and parameters are listed in Appendix A. It is observed that given the same amount of bits to quantize W2, the cube-split quantization achieves larger BF gain than the per-element quantization. In addition, the proposed scheme with total 5K bits achieves similar BF gain as the per-element quantization with 6K bits, where K is the number of elements in W2. Hence, total K=2Nsel bits can be reduced by usage of the cube-split quantization scheme.

Figure 1. Comparison (relative BF gain) between the cube-split and per-element W2 quantization (32 ports)

Proposal 1: A two-stage W1W2 codebook with appropriate selection of beams in W1 and cube-split quantization of W2 should be studied for implicit CSI feedback in NR.

Explicit CSI Feedback

Explicit CSI feedback includes channel feedback, channel eigenvector feedback, and covariance matrix feedback.

Explicit CSI such as channel and eigenvectors can be quantized and reported in the similar way to the MBCR approach, based on the principle of beam combination, or the channel / channel eigenvector is directly feedback in analog way.

One Example is given as follows:

- Channel

For user k, the channel vector on the sub-band m is approximated by

hm,k≈i∈Iselβi,m,kbi                                                               (9)

- Eigenvector

For user k, the l-th dominant eigenvector of channel covariance matrix on the sub-band m is approximated by

ul,m,k≈i∈Iselγl,i,m,kbl,i                                                             (10)

Both the indices of wideband beam subset and the corresponding sub-band weights β or γ should be reported. The beam combination weights can be quantized and fed back in a digital way, or be directly reported in an analog way.

Proposal 2: Explicit CSI feedback for beam combination or direct channel/channel eigenvector should be studied in NR.

Channel covariance matrix is beneficial to spatial dimension reduction for massive MIMO especially at sub-6GHz frequency band [2][3]. Direct feedback of a large-dimensional channel covariance matrix definitely requires a large number of bits for accurate quantization, so methods need to be studied to reduce the feedback overhead. Considering the high spatial correlation in a compact planar array usually deployed in massive MIMO systems, the channel covariance matrix can be approximated by the Kronecker product of several sub-channel covariance matrices with lower dimensions. Thus, instead of quantizing and feedback of a large-dimensional channel covariance matrix, it is more efficient to report the sub-channel covariance matrices to save feedback overhead and retrieve the entire channel covariance matrix at TRP based on Kronecker product. Codebook design for these sub-channel covariance matrices needs to be further investigated. Two alternatives are listed below to elaborate the proposed scheme.

- Alternative 1

For a planar array (N1, N2, 2), the  channel covariance matrix  can be approximated by the Kronecker product of two sub-channel covariance matrices, respectively corresponding to the horizontal cross-polarized linear array (CLA) and the vertical uniformly-spaced linear array (ULA).

(11)

where  is the  sub-channel covariance matrix of the horizontal array (1, N2, 2), and   is the  sub-channel covariance matrix of the vertical array (N1, 1, 1).

Figure 3. Alternative 1 of channel covariance matrix feedback

- Alternative 2

In another alternative, the  channel covariance matrix  can be approximated by the Kronecker product of three sub-channel covariance matrices, respectively corresponding to the horizontal ULA, the vertical ULA and a 2Tx CLA.

(12)

where  is the 2x2 sub-channel covariance matrix of array (1, 1, 2), capturing the correlation between two polarizations,  is the  sub-channel covariance matrix of the horizontal array (1, N2, 1), and   is the  sub-channel covariance matrix of the vertical array (N1, 1, 1).

Figure 4. Alternative 2 of channel covariance matrix feedback

Proposal 3: Kronecker-product based channel covariance matrix feedback should be studied in NR.

Reciprocity Based CSI Acquisition

Reciprocity based feedback scheme needs to be investigated for TDD systems in NR, due to avoidance of CSI feedback delay and accurate CSI without quantization. Considering higher propagation loss in NR than LTE/LTE-A [4] and limited UE transmission power, to extend UL coverage and achieve sufficient channel estimation accuracy, multi-beam based and power boosting are two possible solutions for enhancement on SRS transmission. For multi-beam based scheme, function of beam tracking/refinement needs to be considered on SRS transmission as well as channel acquisition.

Proposal 4: Both multi-beam based and power boosting schemes should be studied to enhance reciprocity based CSI acquisition in NR.

Interference Measurement

Probing Based Interference Measurement

Interference measurement is crucial to link adaptation quality. It is a very important part of CSI feedback framework. Unfortunately, interference measurement has not been specified until LTE Rel-11. Interference measurement based on reference signal (CSI-RS/DMRS/CRS) or PDSCH as well as timing relation between interference measurement and reporting are all left for UE implementation. In Rel-11, IMR (Zero power CSI-RS) was specified for interference measurement. In Rel-13, measurement restriction for both channel and interference was also introduced.  To some extent, those enhancements help TRP choose more suitable MCS levels to UE.

Figure 5: Interference measurement probing with IMR

In spite of the specified interference measurement resource and timing relation between measurement and reporting, link adaption still suffers mismatch between measurement time and reporting time due to the dynamic variation of inter-cell interference. One way to mitigate this mismatch is to probe the interference of subframe n+L with small amount of resource at subframe n by mimicking scheduling of subframe n+L. Figure 5 shows this interference measurement probing mechanism with IMR. It can be seen M times more interference measurement overhead than TM10 single cell MIMO with only 1 IMR.

As discussed, the obvious disadvantage of IMR based interference measurement specified in Rel-11 is huge overhead. However, if NZP CSI-RS can be allowed for probing interference of upcoming L subframes, overhead can be greatly reduced as shown in Figure 6.

Figure 6:  Interference measurement probing with NZP CSI-RS

Proposal 5: Non-zero-power (NZP) CSI-RS should be studied for interference measurement in NR to enhance link adaptation.

Interference Measurement Based on Demodulation RS

With more users co-scheduled and spatially multiplexed in NR, MU interference generates larger impact on the reliability of downlink transmissions. Demodulation RS can be employed to measure interference resulting from co-scheduled users in MU-MIMO. The advantages of the aforementioned interference measurement scheme include:

- Data demodulation will be more reliable based on the interference channel measurement and interference suppression.

- Link adaptation can also be enhanced with MU-CQI measurement and reporting based on the interference measurement mentioned above.

One challenge of the aforementioned interference measurement scheme is that a given UE does not know which other UEs are to be co-scheduled during the time interval of interference measurement. Hence, multiple hypotheses of the interference are needed. To perform interference measurement, UE should know some configuration information of the related demodulation RS ports, for example,

- Antenna port index of demodulation RS for co-scheduled UE,

- Demodulation RS initialization parameters (e.g., scramble ID/Cell ID/time/frequency mapping position information).

Generally, in the demodulation RS based interference measurement scheme, UE should know all of the port indices and the initialization parameters of the demodulation RSs assigned to co-scheduled users. The aforementioned information can be acquired in an explicit or implicit way, but the latter is more preferred considering the constraint on signaling overhead.

Proposal 6: Demodulation RS needs to be studied for interference measurement in NR to enhance data demodulation and link adaptation.

CSI Measurement and Reporting Procedure

Multi-level CSI Measurement and Reporting

A two-level CSI measurement and reporting as shown in Figure 7 is required for massive MIMO with dynamic spatial dimension reduction (DSDR) especially at sub-6GHz frequency band [2][3], each level corresponding to one of the two precoding stages.

- First-level CSI measurement and reporting

TRP configures measurement on wideband/long-term channel statistics information for the DSDR precoding, then UE estimates and reports the corresponding explicit CSI, such as channel covariance matrix.

As discussed in section 3.2, the channel covariance matrix can be retrieved by Kronecker product of 2 (alternative 1) or 3 (alternative 2) sub-channel covariance matrices. So the first-level CSI measurement and reporting can be divided into 2 or 3 steps on the measurement and reporting of these sub-channel covariance matrices.

- Second-level CSI measurement and reporting

TRP configures measurement on subband/short-term instantaneous channel with a reduced dimension based on DSDR, then UE estimates and reports the corresponding CSI, either in implicit CSI feedback or explicit CSI feedback.

Following the proposed two-level CSI measurement and reporting, hybrid CSI-RS configuration should be further studied for sub-6GHz, by utilizing non-precoded CSI-RS for each step in the first-level CSI measurement and beamformed CSI-RS for the second-level CSI measurement.

Proposal 7: Multi-level CSI measurement and reporting should be supported in NR, e.g. joint use of explicit and implicit CSI feedback.

Figure 7. Procedure of multi-level CSI measurement and reporting

Aperiodic/Periodic/Semi-persistent CSI Measurement and Reporting

To meet various accuracy requirements for the aforementioned two-level CSI measurement and reporting, different periodicity for the two levels should be allowed. Longer periodicity is sufficient to measure the first-level channel statistics information, so periodic or semi-persistent CSI measurement and reporting is preferable. But for the second-level instantaneous channel, accurate CSI should be guaranteed to achieve high system capacity, especially in scenarios with fast-varying channel, so applying aperiodic CSI measurement and reporting is a beneficial way for efficient resource utilization and high CSI resolution. In summary, aperiodic, periodic and semi-persistent CSI measurement and reporting should all be supported in NR.

Proposal 8: All of aperiodic, periodic and semi-persistent CSI measurement and reporting should be supported in NR.

Self-contained CSI Measurement/Reporting/Triggering

The performance of massive MIMO is significantly dependent on the accuracy of CSI, which is affected by quantization, feedback error, measurement and feedback delay and channel variation. Self-contained CSI triggering, measurement and reporting is beneficial to eliminate CSI feedback delay and therefore improves CSI estimation accuracy. In details, as shown in Figure 8, an UL field is attached in the end of a DL time interval for channel acquisition, such as explicit/implicit CSI feedback, or SRS transmission together with interference feedback.

Figure 8. Self-contained CSI triggering, measurement and reporting

Proposal 9: Self-contained CSI triggering, measurement and reporting should be supported in NR.

Conclusions

This contribution discusses the CSI acquisition mechanism to be applied for DL MIMO in NR and concludes with the following observations and proposals:

Proposal 1: A two-stage W1W2 codebook with appropriate selection of beams in W1 and cube-split quantization of W2 should be studied for implicit CSI feedback in NR.

Proposal 2: Explicit CSI feedback with beam combination or direct channel/channel eigenvector should be studied in NR.

Proposal 3: Kronecker-product based channel covariance matrix feedback should be studied in NR.

Proposal 4: Both multi-beam based and power boosting schemes should be studied to enhance reciprocity based CSI acquisition in NR.

Proposal 5: Non-zero-power (NZP) CSI-RS should be studied for interference measurement in NR to enhance link adaptation.

Proposal 6: Demodulation RS needs to be studied for interference measurement in NR to enhance data demodulation and link adaptation.

Proposal 7: Multi-level CSI measurement and reporting should be supported in NR, e.g. joint use of explicit and implicit CSI feedback.

Proposal 8: All of aperiodic, periodic and semi-persistent CSI measurement and reporting should be supported in NR.

Proposal 9: Self-contained CSI triggering, measurement and reporting should be supported in NR.

- References

“RAN1 chairman’s notes”, Nanjing, China, May 23-27, 2016.

Huawei, HiSilicon, “CSI acquisition framework for massive MIMO in new radio”, R1-164374, Nanjing, China, May 23-27, 2016.

Huawei, HiSilicon, “Discussion on transmission schemes for NR DL MIMO”, R1-16xxxx, Gothenburg, Sweden, August 22-26, 2016.

Huawei, HiSilicon, “Coverage issues of massive MIMO in NR”, R1-164372, Nanjing, China, May 23-27, 2016.

- Appendix A. Simulation Assumptions and Parameters

Table 1. Simulation assumptions and parameters

| Performance metric | Relative beamforming gain |
| --- | --- |
| Channel Model | 3D UMi |
| Carrier frequency | 2.5 GHz |
| TRP antenna configuration | Single-polarized uniform planar array with 0.5λ horizontal antenna spacing and 0.8λ vertical antenna spacing - 32 Tx: (4, 8, 1) - 16 Tx: (4, 4, 1) |
| UE antenna configuration | Cross-polarized linear array - 2 Rx: (1, 1, 2) |
| System bandwidth | 20MHz |
| Number of subbands | 20 |
| Channel estimation | Ideal |
| DL CSI | Rank-1 precoding vector |
| Base codebook B | For TRP antenna configuration (N1,N2,2): B is a N1N2 x N1N2 DFT matrix, the number of column vectors of which is N= N1N2. |

Relative beamforming gain defined below is used as performance metric to evaluate the beamforming gain between two quantization w1 and w2.

ΔBF Gainw1,w2=BF Gainw1,ν-BF Gain(w2,ν)

where

BF Gain(w,ν)=20×log⁡(|wHν|),

ν is the downlink CSI, and w is  the quantized CSI.

In simulations, w2 is the LTE Rel-13 codebook with config 1, N1, N2=2, 8 for 32-port, N1, N2=2, 4 for 16-port, and O1, O2=(8, 8).